3GPP TSG-RAN WG4 Meeting #116	R4-2511237

Bengaluru, India, August 25th – August 29th, 2025

Source:	Nokia

Title:	Discussion on remaining issues of Rel-18 Mobility Enhancements core

Agenda item:	5.19.1

Document for:	Discussion

Introduction

This paper discusses about the remaining open issues of Rel-18 mobility enhancements. R4-25xxxxx is the CR implementing agreement from RAN4#115 related to the agreement on minimum time gap between PDCCH order command and RO. CR related to section 2.2 is R4-2511238.

Discussion

RACH occasion colliding with MG

In the last RAN4 meeting, RACH collision with MG was discussed. The issue is divided into two parts, one is about RACH collision with MG, and another part is about whether the UE should transmit or drop preamble transmission attempt when a collision occurs.

Regarding the first topic, the following agreement was reached for collision definition:

Agreement: - For PDCCH order RACH in LTM, RACH occasion colliding with the MG occasion can be defined as: - MG overlaps with RACH occasion or RF retuning time to target cell or RF retuning time back to serving cell.

After offline discussion about the issue 2, the following was noted as an outcome.

Outcome from offline discussion: UE shall transmit PDCCH order PRACH at latest in the first available RO after the RAN4 defined “PDCCH ordered Random Access delay” in 6.2.2C.2. The definition of available RO is RO where MG does not overlap with RACH occasion or RF retuning time to target cell or RF retuning time back to serving cell.

From the network point of view, what is important is that the PDCCH ordered RACH will result in at least one actual transmission of the preamble. As discussed in our contribution before, by design, LTM does not have automatic retransmission of preamble. If the UE drops the RACH transmission attempt, currently it is not clear whether the UE should attempt to transmit on a next RACH occasion or not. From the network point of view, the network has transmitted a PDCCH order, and the network is expecting UE to perform actual transmission rather than dropping the preamble transmission.

When considering that it is the current serving cell that is transmitting the PDCCH order, and the preamble is transmitted to a candidate cell (e.g. one of the neighbouring cells), the source has no visibility whether the preamble transmission is failing because the UE did not transmit preamble, or whether it is failing at the target side.

From the network point of view PDCCH ordered RACH shall result actual transmission.

Source does not know whether the failure is at the UE or network side.

We can understand the UE point of view that it may not be desirable to try to retransmit multiple times, hence we are ok if UE only performs the actual preamble transmission at the next available slot or in one of the ROs within that slot.

After PDCCH ordered RACH command to a target cell, if RACH occasion is colliding with the MG and UE has not transmitted a preamble, the UE shall transmit preamble at latest at the next available valid RO. I.e., first RO where MG does not overlap with RO or RF retuning time to target cell or RF retuning time back to serving cell.

Minimum time gap between PDCCH order command and RO

In the last meeting, the following was agreed:

Agreement: The minimum gap from the PDCCH-order to the RO, in RAN4, is re-defined as ‘Max(X ms, the current RAN4 minimum timeline requirement).’ - X = 3.5 for intra-BWP early PRACH - X = 5.5 for inter-BWP early PRACH

Our understanding is that this has impact how the current specification is defined. We think that the cleanest way to implement this agreement in CR is to define a new time component for PDCCH ordered RACH delay, and refer to the RAN1 specification for applicable components. This should be aligned with the legacy way but it also makes RAN4 specification clearer for those companies who implement the procedure according to the specification. We used similar structure as the cell switch command section 6.3 is using. The new parts are defining clearly TLTM-PDCCH-order and introducing Tfirst-RO according to the agreement above.  The CR is R4-2511238.

#### 6.2.2C.2 PDCCH ordered Random Access delay PDCCH ordered RACH delay TLTM-PDCCH-order is the delay from PDCCH ordered RACH command until the time when the UE shall complete the RACH transmission to the LTMcandidate cell. The LTM PDCCH ordered RACH delay is defined as: TLTM-PDCCH-order = Max (Tfirst-RO, NT,2 + ∆BWPSwitching + ∆Delay + Tswitch + TSSB + ∆RF/BB preparation) Where: - Tfirst-RO is the minimum time gap between the PDCCH ordered RACH command to the first candidate cell RACH occasion UE shall be able to transmit (RO), where: - If PRACH resource is within the active UL BWP of serving cell(s), the Tfirst-RO is 3.5ms - If PRACH resource is outside of the active UL BWP of serving cell(s), the Tfirst-RO is 5.5ms - NT,2, ∆BWPSwitching, ∆Delay and Tswitch are defined in TS 38.213 [3] clause 8.1 <rest of the section omitted>

As the change needs alignment across companies, we propose to discuss this directly in CR during the RAN4 meeting.

Discuss directly in CR how to implement components for minimum gap from the PDCCH-order

Conclusion

The following Observations and Proposals were made:

Observation 1: From the network point of view PDCCH ordered RACH shall result actual transmission.

Observation 2: Source does not know whether the failure is at the UE or network side.

Proposal 1: After PDCCH ordered RACH command to a target cell, if RACH occasion is colliding with the MG and UE has not transmitted a preamble, the UE shall transmit preamble at latest at the next available valid RO. I.e., first RO where MG does not overlap with RO or RF retuning time to target cell or RF retuning time back to serving cell.

Proposal 2: Discuss directly in CR how to implement components for minimum gap from the PDCCH-order