3GPP TSG-RAN WG4 Meeting #116						R4-2511240

Bengaluru, India, August 25th – August 29th, 2025

Source:	Nokia

Title:	Discussion on LP-WUR Simulation assumptions and results

Agenda item:	7.24.5.1

Document for:	Discussion

Introduction

This contribution discusses about simulation assumptions and results for LP-SS. In the last meeting, the following agreements were made relating to simulation assumptions.

Issue 2-1-5: On ideal RSRP/RSRQ in simulation Agreement: Use method 1 in future simulation alignment Issue 2-1-11: On I and Q branches in simulation Agreement: For RAN4 simulation alignment, RAN4 simulation should consider power from both I and Q branches

Discussion

Number of samples

Issue 2-2-2: Summary on proposal/observations based on simulation results Working assumption 1: The number of LP-SS samples for achieving ±3.5 dB accuracy for LP-SS RSRP at -3 dB SINR is [from 1 to 3]; When LP-SS samples number is larger than 3 at -3 dB SINR, the extra gain on accuracy for LP-SS RSRP is smaller; Working assumption 2: The number of LP-SS samples for achieving ±3.5 dB accuracy for LP-SS RSRQ for AWGN and TDL-C channel at -3 dB SINR is [2 or 3];

From the results in R4-2511241 we observed that the number samples do not impact significantly to the observed accuracy and timing error.  From our simulation results, we observed that RSRP accuracy has some dependance on sequence between serving and interfering cells.  To fulfil the 3.5dB accuracy requirement, we think 2 samples is enough.

Number of samples does not significantly impact the accuracy of the LP-SS measurements across different scenarios

The number of LP-SS samples for achieving ±3.5 dB accuracy for LP-SS RSRQ for AWGN and TDL-C channel at -3 dB SINR is 2

OOK interfering cell

Issue 2-1-7: On OOK interference

Currently RAN4 is simulating with an interference pattern that is selected from the available patterns from RAN1. Selecting overly good pattern will bias the RAN4 simulation results, and eventually the UE requirements as well as selecting opposite pattern will cause problems. In real deployment scenario it is highly likely that there will be some time offset between neighbouring OOK patterns, hence the deployment in the scenario is not realistic.

We observed some performance degradation for the case when fully opposite sequence is used for serving and neighbour cell, and we are ok to exclude the case or no need to have any particular consideration for this case. If this case needs to be treated, then it’s ok for us to apply time offset for the interfering cell.

For the case when fully opposite sequence is used for serving and neighbour cell, no need to have any particular consideration for this case. If consideration is needed, RAN4 can have a time-offset for the interfering signal.

Conclusion

The following Observations and Proposals were made:

Observation 1: Number of samples does not significantly impact the accuracy of the LP-SS measurements across different scenarios

Proposal 1: The number of LP-SS samples for achieving ±3.5 dB accuracy for LP-SS RSRQ for AWGN and TDL-C channel at -3 dB SINR is 2

Proposal 2: For the case when fully opposite sequence is used for serving and neighbour cell, no need to have any particular consideration for this case. If consideration is needed, RAN4 can have a time-offset for the interfering signal.