3GPP TSG-RAN WG4 Meeting #116                                                               R4-2511251

Bengaluru, India, 25th – 29th August, 2025

Source:	Samsung

Title:	Analysis on UE-UE CLI problem and potential solutions

Agenda item:			7.21.2.2

Document for:	Approval

# 1. Introduction

Based on Rel-18 study item on evolution of NR duplex operation, the support of subband non-overlapping full duplex has been studied, which is targeted to provide enhanced UL coverage, reduced latency, improved system capacity, and improved configuration flexibility for NR TDD operation [2].

In RAN#102, the work item on evolution of NR duplex operation (SBFD) has been approved, with WID further revised in the follow-up RAN plenary [1]. According to the objectives in WID, RAN1 is tasked to specify the mechanisms to support SBFD, including semi-static indication of time/frequency location, random access in SBFD symbols, and other transmission, reception and measurement behavior and procedures for SBFD aware UE. Furthermore, the enhancement for CLI handing, including gNB-to-gNB and UE-to-UE CLI handling, will also be specified in RAN1. Accordingly, from RAN4 perspective, it is tasked to specify BS RF, necessary RRM and demodulation requirements.

In RAN4#115, the group discussed the UE-to-UE CLI problem, focusing on intra-operator adjacent-channel inter-cell scenarios, particularly the Urban Hotspot -> Urban Hotspot Scenario 2, and proposes solutions such as reducing p-max for SBFD-aware UE TX power, using DUD configurations, and scheduling adjustments. Companies are encouraged to analyze the problem, identify potential solutions, and evaluate their benefits, limitations, and impacts on RAN4, RAN2, and RAN3 requirements [3].

In this contribution, we would like to provide our analysis and viewpoints on the UE-UE CLI problem and potential solutions.

# 2. Co-existence analysis for UE-to-UE CLI Problem

### 2.1 Background

Based on Rel-18 study, the following conclusion is provided for Case 1 (aggressor SBFD DU victim NR TDD DL) in the conclusion part of Chapter 11 in TR 38.858 [2]:

#### 11.3.1 Case 1: aggressor SBFD DU victim NR TDD DL Case 1 considers legacy TDD in DL slot as a victim while SBFD is operating in the adjacent channel for both FR1 and FR2-1. The conclusions are listed per scenario in Table 11.3.1-1. Table 11.3.1-1: Case 1 co-existence conclusions Deployment Scenario (Aggressor -> Victim); Frequency range; Co-existence conclusion. Urban Macro -> Urban Macro Scenario 1 and Scenario 6; FR1 and FR2-1; No DL throughput degradation on the victim legacy TDD DL network for both average throughput and cell edge throughput is observed for different BS Tx powers (46 dBm to 53 dBm for FR1 and 30 dBm for FR2-1), grid-shifts (5% to 100%), and different SBFD BS antenna configurations.. Urban Hotspot -> Urban Hotspot Scenario 2; FR1; DL throughput degradation is observed only at cell edge due to inter-UE CLI for different grid-shifts (5% to 100%) and BS Tx powers (46 dBm to 53 dBm).. Urban Macro -> Urban Micro Scenario 4; FR1; No DL throughput degradation for both average throughput and cell edge throughput is observed.. Indoor -> Indoor Scenario 3 and Scenario 9; FR1 and FR2-1. Urban Micro/Dense -> Urban Micro/Dense Scenario 5 and Scenario 8; FR1 and FR2-1.

In RAN1#121 (May, 2025), the following conclusion is provided on this topic:

Conclusion Separate configuration of Pcmax/p-max is a RAN4 issue and will not be discussed in RAN1. It is up to RAN4 make decision on this issue. As part of RAN1 study on this issue, one company submitted simulation results in R1-2504391.

Triggered by the RAN1 conclusion, RAN4 has discussed the UE-to-UE CLI problem, focusing on intra-operator adjacent-channel inter-cell scenarios, particularly the Urban Hotspot -> Urban Hotspot Scenario 2 [3]:

- UE-to-UE CLI problem statement: - The scenario to be considered: - Intra-operator adjacent-channel inter-cell (i.e., both channels belong to a single operator) - Use Urban Hotspot -> Urban Hotspot Scenario 2 as reference - FFS non-located assumption is always valid or not - detailed description provided in TR38.858 - The case to be considered: - SBFD-aware UE (aggressor) UL transmission to NR TDD UE (victim) DL reception

### 2.2 Co-existence analysis of UE-to-UE CLI problem

Based on the WF [3], companies are encouraged to provided analysis on the clarified UE-to-UE CLI problem (as below), in order to identify whether the above-mentioned UE-to-UE CLI problem exists or not in the practical deployment:

- WF-1: Companies are encouraged to provided analysis on the above UE-to-UE CLI problem: - The purpose is to identify whether the above-mentioned UE-to-UE CLI problem exists or not in the practical deployment - One evaluation method is to re-perform RAN4 Rel-18 co-existence study on the above scenario/case - Parameters agreed in Rel-18 will be reused unless difference identified - FFS impact from different grid shift values - FFS impact from ACLR model

#### 2.2.1 ACLR/ACS step-wise model

For the assumption of ACLR model, given it’s FFS in [3], we take into account the previous discussions documented in the TR 36.942 [6] as reference. From [6], the UE ACLR was assumed as follows.

#### The UE ACLR models in [6] For uplink, it is assumed that the ACIR is dominated by the UE ACLR. As shown in Figure 12.1 and Table 12.5, the bandwidth for each ACIR value was assumed to be the same as the transmission bandwidth of LTE-A UE. Outside ACIR1/2 regions, ACIR3 was used for all the regions. Those models are modified model, in which the ACIR3 is smaller than ACIR2 based on the actual spectrum shape. # Figure 12.1 Uplink ACIR models Table 12.5 ACIR models for LTE-A coexistence ACIR value; (Ref. LTE model). ACIR1; 30 + X; 30 + X. ACIR2; 43 + X; 43 + X. ACIR3; 50 + X; 43 + X.

In the above section in [6], the UE ACLR was assumed to be represented by a 3-step model, in which the first step assumed ACLR1 as 30+X, and second step assumed ACLR2 as 43+X, and third step assumed as even larger ACLR3 which is larger than 50+X, so that the resulting ACIR3 was then constraint by the ACS as 50+X.

Observation 1: In TR 36.942, the UE ACLR was assumed to be represented by 3-step model, i.e., ACLR1 as 30+X, ACLR2 as 43+X, and ACLR3 as higher than 50+X.

As similar case for ACS, it is assumed that the ACS step-wise model is provided as follows:

#### The UE ACS models in [6] ACS1 [dB]; 33.0. ACS2 [dB]; 34.3. ACS3 [dB]; 46.3.

As provided in the above section in [6], the UE ACS was assumed to be represented by a 3-step model with ACS1 as 33dBc, ACS2 as 34.3dBc, and ACS3 as 46.3 dBc. Each step of ACS model was assumed as half of victim UE BW.

Observation 2: In TR 36.942, the UE ACS was assumed with 3 steps as ACS1 as 33dBc, ACS2 as 34.3dBc and ACS3 as 46.3dBc, and each step assumed as half of UE channel bandwidth.

We believe the step-wise model documented in TR 36.942 can be a valid reference, and so that we propose to use these models as baseline assumption for the study.

Proposal 1: RAN4 UE-to-UE CLI simulation adopt the step-wise ACLR and ACS models referenced in TR 36.942 as the baseline.

In the target scenario where SBFD uplink UE interfering legacy TDD DL UE in adjacent channel, the resulting ACIR would be affected by the SBFD subband configurations as follows. Below figure also depicts how we apply the step-wise ACIR into the simulation.

Figure 2.2-1. Step-wise ACLR/ACS assumption

For the normal SBFD DUD configuration case, by applying above step-wise model, we assume the ACLR3 as assumed ACLR, which is larger than 50dBc, and we assume averaged ACS from ACS1 and ACS2 by using the equation below.

110Averaged_ACS/10=120*1010ACS1/10+1010ACS2/10

The averaged ACS, by above equation, is calculated as 33.6dBc. And it’s quite straightforward that this case is ACS constraint, and so that we assume the ACIR for this scenario as 33.6 dBc.

| ACLR3 (dBc) | > 50 |
| --- | --- |
| Averaged_ACS (dBc) | 33.6 |
| ACIR (dBc) | 33.6 |

Observation 3: By applying the proposed step-wise ACLR/ACS model, the resulting ACIR for this SBFD DUD UE-UE CLI case is 33.6 dBc.

#### 2.2.2 Grid-shifts and the clusters

The clusters was slightly discussed in the Rel-18. In [2], the Urban Hotspot scenario along with this cluster dropping method is only considered under the assumption of 100% grid-shift between two networks. Based on pre-RAN4 CC, it is identified that the cluster dropping method is critical and needs to be clarified. In the below figure, we demonstrate our method of how the cluster dropping is performed in the simulation.

Figure 2.2-2. Clusters in different grid shifts

From above figure, our observation is that in 100% (or 0%) grid shift case, the clusters can be randomly dropped within the grey area (in the left figure). And the UEs are randomly dropped in the sector shared by two networks. However, if we change the grid shift to a value between 0 and 100%, to avoid the clusters belongs to two aggressor sectors or two victim sectors, the cluster locations along with the associated UE locations shall be limited to certain area. Especially, the sector borders are actually crossed in the good spots of the victim sector, which means by following this way of dropping clusters it actually ruled out the cases where indoor UEs are in the cell centers, which requires less Tx power and causes less UE-UE CLI.

In conclusion, our observation is that this cluster methodology for different grid shifts seems problematic and needs more discussion and refinement.

Observation 4: By following the other company proposed cluster dropping method, the cluster will not cross the sector borders, thus limiting the indoor UE to be distributed not in all practical deployment locations.

Observation 5: The cluster dropping methodology could only be valid for 100% grid shift case, and to use it in other grid shift values can be problematic.

Proposal 2: For the Urban Hotspot scenario, only 100% grid shift should be considered.

#### 2.2.3 Preliminary simulation results

Here we firstly provide the analysis for the UE-to-UE CLI problem, by considering the above step-wise ACLR/ACS model and 80% indoor UE ratio. The results below shows in the FR1 Hotspot scenario, the adjacent channel interference from SBFD UL UE would results in 11.63% throughput degradation at cell-edge and 2.64% throughput degradation at cell-average to the legacy TDD DL UE.

Table 2.2-1. Results for the UE-to-UE CLI with baseline assumption

| Grid shift | SINR degradation @5% in dB | SINR degradation @50% in dB | TP degradation @5% in % | TP degradation @Mean in % |
| --- | --- | --- | --- | --- |
| 100% | 1.18 | 0.73 | 11.63 | 2.64 |

Observation 6: Directly compared to 5% throughput degradation criteria, with baseline assumption, the SBFD UL UE would create performance degradation to the TDD DL UE in Urban Hotspot scenario at cell-edge, while the impact on cell-average is well acceptable.

However, we’d like to point out the 5% criteria was usually put onto the averaged and 5%-tile throughput degradation for the normal scenarios, like Urban Macro, Urban Micro, Indoor. In this Urban Hotspot cases, the simulation settings focused on the UE-UE CLI where both UEs are indoor UEs (20% baseline indoor ratio, and 80% optional indoor ratio in probability under normal UMa), stay in the same cluster (Low probability, not considered under normal UMa) and in the same floor (12.5% in probability under UMa). If we took both indoor UEs are dropped in the same floor, it means this scenario is looking at an event which would happens with 0.5% (baseline) and 8% (optional) probability under the normal UMa co-ex study in RAN4.

Observation 7: The Urban Hotspot scenario generally looks at the cases which happens with 0.5% (baseline) and 8% (optional) probability under the normal UMa co-ex study in RAN4.

Considering the following, our view is that the SBFD UE UL to TDD DL UE CLI does exists under very rare and corner case, and we do not think it’s necessary to work further on solving it.

- Conclusion in [2] was that no performance degradation in TDD DL was observed in UMa;

- The scenario setting focused on an event with 0.5% (baseline) and 8% (optional) probability from UMa scenario in [2]; and,

- 2.64% average throughput degradation meets the 5% criteria while 11.63% degradation at cell-edge is acceptable with such low probability.

Moreover, if we keep 80% indoor ratio unchanged, but additionally to assume the UE would randomly dropped in different floors, the interference would be further reduced.

In [5], the UEs under UMa scenario are assumed to be dropped with a height in range of 1.5m to 22.5m, i.e. floor #1 to #8. And given the complexity of this Urban hotspot scenario, we use the following condition to add one additional penetration loss to the UE-UE CLI link.

- If UE-UE CLI was NLOS, and the probability was based on the UE-UE distance and model documented in [5];

- If both UEs are not in the same floor, where both UEs are randomly dropped in floor #1~8 as [5];

And for the penetration loss, which actually represents the loss between different floors, we assumed two different losses from [5], one is wooden and one is concrete.

Material; Penetration loss [dB]. Standard multi-pane glass; Lglass=2+0.2⋅f. IRR glass; LIRRglass=23+0.3⋅f. Concrete; Lconcrete=5+4⋅f. Wood; Lwood=4.85+0.12⋅f. Note: f is in GHz.

Table 2.2-2. Results for the UE-to-UE CLI with optional assumptions

| Grid shift | Penetration loss material | SINR degradation @5% in dB | SINR degradation @50% in dB | TP degradation @5% in % | TP degradation @Mean in % |
| --- | --- | --- | --- | --- | --- |
| 100% | Wooden | 0.76 | 0.51 | 7.64 | 1.56 |
|  | Concrete | 0.21 | 0.15 | 2.14 | 0.47 |

In above results, we only added one time of penetration loss regardless of numbers of floor differences between two UEs. Though it’s a preliminary and we agree that the assumption can be further discussed and refined, the results still represents a much wider and more practical case, and the throughput degradation seems acceptable to us at both cell-edge and cell-average.

Observation 8: By adding one penetration loss, depending on the UE-UE distance, NLOS probability and random drop of UE height, the UE-UE CLI impact under this Urban Hotspot scenario already falls into the acceptable range.

Proposal 3: The UE-to-UE CLI issue is generally not identified under this Urban Hotspot scenario by considering practical assumptions. The SBFD UL UE to TDD DL UE CLI may exist in some corner cases, which strongly depends on the simulation assumptions.

# 3. Co-existence analysis for UE-UE CLI handling solutions

In RAN4#114, the proposal is proposed to introduce new mechanism for limiting maximum UE transmit power in SBFD symbols/slots [R4-2500474]. In RAN4#114bis, it is highlighted that the mechanism is for the single operator scenario in which a single operator owning both networks, SBFD and legacy TDD. In the corresponding proposal, it is stated that UE-to-UE CLI could be mitigated, by using DUD (40%, 20%, 40%), and by reducing PCMAX of the SBFD UE power by X dB [R4-2503235]. In RAN4#115, the following proposals are provided in the Tdoc [R4-2505459]:

- Proposal 1: Reduce pMax of the SBFD UE power by XdB.

- Proposal 2: Send LS from RAN4 to RAN1 asking to approve our agreement

- Proposal 3: A single operator owning both networks, SBFD and legacy TDD, could mitigate UE-to-UE CLI, by using DUD (40%, 20%, 40%).

In RAN4#115, aside of solution 1, at least the other three solutions are identified for further analysis, while additional new solutions (including the combination of the above-mentioned solutions) are also not precluded.

- WF-2: Companies are encouraged to identify potential solution(s) to the above UE-to-UE CLI problem: - Solution-1: Reduced p-max for SBFD-aware UE TX power by X dB. - the reduced p-max is configurable per-UE - FFS X dB is a single value or a configurable value or a configurable range - Solution-2: SBFD configuration is set by using DUD (40%, 20%, 40%), or DU (by putting UL subband away from victim channel) - Solution-3: Avoid scheduling the aggressor UE, up to BS implementation. - Solution-4: Configure the aggressor UE fallback to TDD operation, up to BS implementation. - Other new solutions (including the combination of the above-mentioned solutions) are encouraged to be provided, if any.

For Solution-1 and Solution-3, the co-existence analysis can be performed, in which the baseline simulation assumption provided in Section 2 is followed.

### 3.1 Co-existence analysis of Solution-1 (reduced p-max for SBFD-aware UE)

Particularly for the above proposal 1 (named as Solution-1 in the approved WF [3]), RAN4 agree to further study the effect of this solution to alleviate the UE-to-UE CLI problem.

- WF-2: Companies are encouraged to identify potential solution(s) to the above UE-to-UE CLI problem: - Solution-1: Reduced p-max for SBFD-aware UE TX power by X dB. - the reduced p-max is configurable per-UE - FFS X dB is a single value or a configurable value or a configurable range

Here we provide the analysis for the solution 1, by simply applying the reduced p-max to all SBFD-aware UEs.

Table 2.3-1. Results for the UE-to-UE CLI with base assumptions and pmax-3dB

| Grid shift | P-max – X, | SINR degradation @5% in dB | SINR degradation @50% in dB | TP degradation @5% in % | TP degradation @Mean in % |
| --- | --- | --- | --- | --- | --- |
| 100% | X=0 | 1.18 | 0.73 | 11.63 | 2.64 |
|  | X=3 | 0.29 | 0.45 | 2.92 | 1.18 |

From the table above, with the 3dB Pmax reduction applied to all SBFD UL UE, both throughput degradation at cell-edge and cell-average are reduced. However, it should also be noted that the above analysis is based on applying 3dB Pmax reduction to all SBFD UL UE, because we still can see any mechanism which can support the victim UE/gNB to identify the aggressor UE and notify the aggressor gNB for power reduction.

Observation 9: Reducing the Pmax of SBFD UL transmission would achieve the adjacent channel interference reduction and accordingly TP degradation reduction, with the basic assumption of Urban Hotspot and applying p-max reduction to all SBFD UEs.

Since we are looking into the scenario where both networks are under same operator, we believe it’s also worthwhile to look into the performance degradation of SBFD carrier, performance improvement of legacy TDD carrier, and both carriers as a whole. The figure below shows the throughput impacts of this operator by applying this X = 3dB to the SBFD UL UE.

Figure 2.3-1. Throughput impacts on SBFD carrier (TP1), legacy TDD carrier (TP2) and both carrier as a whole (TP3)

From above figure, it can be observed that

- SBFD uplink throughput degrades;

- TDD downlink throughput improves;

- Overall throughput in both carriers degrades.

Observation 10: By applying X=3 dB pmax reduction to all SBFD UEs, the overall throughput of this operator in two carriers, including SBFD and TDD carriers, would decrease.

### 3.2 Co-existence analysis of New Solution (by scheduling victim UEs further apart in frequency domain)

We have identified the new solution, i.e., avoid scheduling the aggressor UE to mitigate UE-to-UE CLI for this case. It should be noted that the solution is up to BS implementation, which make the victim TDD DL UE to be scheduled further apart in frequency domain, and also to reduce the channel BW of the victim TDD DL UE, it is possible that the ACLR and ACS would both fall into the ACLR3/ACS3 region. Under this assumption of more frequency separation, the resulting ACIR would goes up to 46.3 dBc which is still an ACS dominant case according to [6].

Table 2.4-1. Results for the UE-to-UE CLI with baseline assumption and network scheduling

| Grid shift | SINR degradation @5% in dB | SINR degradation @50% in dB | TP degradation @5% in % | TP degradation @Mean in % |
| --- | --- | --- | --- | --- |
| 100% | 0.19 | 0.16 | 1.74 | 0.48 |

From the Table above, by network scheduling to have further frequency separation, both throughput degradation at cell-edge and cell-average are significantly reduced.

Observation 11: Network scheduling based method by simply scheduling victim and aggressor UE with enough frequency separation (regarded one implementation for Solution 3) cans till reduce the adjacent channel UE-to-UE CLI significantly.

# 4. Comparison of UE-UE CLI handling solutions

Based on the approved WF [3], Companies are encouraged to provide analysis on solutions, including the aspects of benefits, limitation, expected RAN2/3/4 requirement impacts:

- WF-3: Companies are encouraged to provide analysis on solutions, by at least considering the following aspects: - Benefit(s): - At least for Solution-1, one evaluation method is to perform RAN4 co-existence study on the above scenario/case, by using XdB lower maximum UE power for aggressor UE(s) - FFS how to determine one SBFD-aware UE is aggressor UE - The value of X can be chosen from the range of [1, 5] and other values may not be precluded. - How to determine the value of X or the range can be FFS - Baseline to be compared to is Rel-18 coexistence study re-performed in WF-1 - Detailed parameters can be further discussed by conference call and/or email discussion before RAN4#116. - Other methods are not precluded - Limitation(s), e.g., decreased coverage for certain SBFD-aware UE, etc. - Expected RAN4 requirement impact, including - the impact to TS 38.104 - the impact to TS 38.101 series, e.g., Pcmax, etc. - Expected RAN2 requirement impact: - whether new or changed RAN2 signaling(s) is required: - Expected RAN3 requirement impact: - whether new or changed RAN3 signaling(s) is required:

We provided our analysis in the follow table.

Proposal 4: RAN4 shall capture the below summary comparison table as agreement for different solutions to UE-to-UE CLI problem:

|  | Expected benefit(s) | Expected limitation(s) | Expected RAN4 requirement impact | Expected RAN2 requirement impact | Expected RAN3 requirement impact |
| --- | --- | --- | --- | --- | --- |
| Solution-1 (Reduced p-max for SBFD-aware UE TX power by X dB) | CLI mitigated | Overall throughput reduced in which SBFD UL performance is much impacted | Configured transmitted power in Clause 6 of TS38.101-1/2/3 needs to be updated for SBFD | p-Max IE or power backoff dedicated for SBFD symbols: - FFS per cell or per UE | Impact expected, because Rel-19 UE-to-UE CLI measurement is only for intra-cell, and measurement reports can’t be shared over Xn interface |
| Solution-2 (SBFD configuration is set by using DUD (40%, 20%, 40%), or DU (by putting UL subband away from victim channel) | CLI mitigated (by much improved ACS/ACLR performance for victim/aggressor UEs respectively) | Deployment restriction by not allowing certain SBFD configurations. | No impact (Rel-19 RAN4 specification would already include the requirements for these configurations) | No impact | No impact |
| Solution-3 (Avoid scheduling the aggressor UE) | CLI mitigated | Scheduling restriction for not scheduling aggressor UE in the concerned SBFD symbols | No impact | No impact | Impact expected, because Rel-19 UE-to-UE CLI measurement is only for intra-cell, and measurement reports can’t be shared over Xn interface |
| Solution-4 (Configure the aggressor UE fallback to TDD operation) | CLI mitigated | Scheduling restriction to aggressor UE by fallback to TDD | No impact | No impact | Impact expected, because Rel-19 UE-to-UE CLI measurement is only for intra-cell, and measurement reports can’t be shared over Xn interface |
| New solution (by scheduling victim UEs further apart in frequency domain) | CLI mitigated | Scheduling restriction to victim UE for DL reception | No impact | No impact | No impact |

Based on the above analysis, the following proposal can be made to conclude this UE-to-UE CLI issue:

Proposal 5: RAN4 conclude the UE-to-UE CLI problem (i.e., intra-operator adjacent-channel inter-cell UE-to-UE CLI, in Urban Hotspot Scenario):

- UE-to-UE CLI issue is generally not identified by considering practical assumptions.

- The SBFD UL UE to TDD DL UE CLI may exist in some corner cases, which strongly depends on the simulation assumptions.

- Solutions identified for mitigate this UL-to-UL CLI problem, while pros/cons and standard impacts are analyzed.

# 5. Conclusion

In this contribution, we have provided the analysis and viewpoints on the UE-UE CLI problem and potential solutions, accordingly with the following observations and proposals obtained:

Co-existence analysis for UE-to-UE CLI Problem

Observation 1: In TR 36.942, the UE ACLR was assumed to be represented by 3-step model, i.e., ACLR1 as 30+X, ACLR2 as 43+X, and ACLR3 as higher than 50+X.

Observation 2: In TR 36.942, the UE ACS was assumed with 3 steps as ACS1 as 33dBc, ACS2 as 34.3dBc and ACS3 as 46.3dBc, and each step assumed as half of UE channel bandwidth.

Proposal 1: RAN4 UE-to-UE CLI simulation adopt the step-wise ACLR and ACS models referenced in TR 36.942 as the baseline.

Observation 3: By applying the proposed step-wise ACLR/ACS model, the resulting ACIR for this SBFD DUD UE-UE CLI case is 33.6 dBc.

Observation 4: By following the other company proposed cluster dropping method, the cluster will not cross the sector borders, thus limiting the indoor UE to be distributed not in all practical deployment locations.

Observation 5: The cluster dropping methodology could only be valid for 100% grid shift case, and to use it in other grid shift values can be problematic.

Proposal 2: For the Urban Hotspot scenario, only 100% grid shift should be considered.

Observation 6: Directly compared to 5% throughput degradation criteria, with baseline assumption, the SBFD UL UE would create performance degradation to the TDD DL UE in Urban Hotspot scenario at cell-edge, while the impact on cell-average is well acceptable.

Observation 7: The Urban Hotspot scenario generally looks at the cases which happens with 0.5% (baseline) and 8% (optional) probability under the normal UMa co-ex study in RAN4.

Observation 8: By adding one penetration loss, depending on the UE-UE distance, NLOS probability and random drop of UE height, the UE-UE CLI impact under this Urban Hotspot scenario already falls into the acceptable range.

Proposal 3: The UE-to-UE CLI issue is generally not identified under this Urban Hotspot scenario by considering practical assumptions. The SBFD UL UE to TDD DL UE CLI may exist in some corner cases, which strongly depends on the simulation assumptions.

Co-existence analysis for UE-UE CLI handling solutions

Observation 9: Reducing the Pmax of SBFD UL transmission would achieve the adjacent channel interference reduction and accordingly TP degradation reduction, with the basic assumption of Urban Hotspot and applying p-max reduction to all SBFD UEs.

Observation 10: By applying X=3 dB pmax reduction to all SBFD UEs, the overall throughput of this operator in two carriers, including SBFD and TDD carriers, would decrease.

Observation 11: Network scheduling based method by simply scheduling victim and aggressor UE with enough frequency separation (regarded one implementation for Solution 3) cans till reduce the adjacent channel UE-to-UE CLI significantly.

Comparison of UE-UE CLI handling solutions

Proposal 4: RAN4 shall capture the below summary comparison table as agreement for different solutions to UE-to-UE CLI problem:

|  | Expected benefit(s) | Expected limitation(s) | Expected RAN4 requirement impact | Expected RAN2 requirement impact | Expected RAN3 requirement impact |
| --- | --- | --- | --- | --- | --- |
| Solution-1 (Reduced p-max for SBFD-aware UE TX power by X dB) | CLI mitigated | Overall throughput reduced in which SBFD UL performance is much impacted | Configured transmitted power in Clause 6 of TS38.101-1/2/3 needs to be updated for SBFD | p-Max IE or power backoff dedicated for SBFD symbols: - FFS per cell or per UE | Impact expected, because Rel-19 UE-to-UE CLI measurement is only for intra-cell, and measurement reports can’t be shared over Xn interface |
| Solution-2 (SBFD configuration is set by using DUD (40%, 20%, 40%), or DU (by putting UL subband away from victim channel) | CLI mitigated (by much improved ACS/ACLR performance for victim/aggressor UEs respectively) | Deployment restriction by not allowing certain SBFD configurations. | No impact (Rel-19 RAN4 specification would already include the requirements for these configurations) | No impact | No impact |
| Solution-3 (Avoid scheduling the aggressor UE) | CLI mitigated | Scheduling restriction for not scheduling aggressor UE in the concerned SBFD symbols | No impact | No impact | Impact expected, because Rel-19 UE-to-UE CLI measurement is only for intra-cell, and measurement reports can’t be shared over Xn interface |
| Solution-4 (Configure the aggressor UE fallback to TDD operation) | CLI mitigated | Scheduling restriction to aggressor UE by fallback to TDD | No impact | No impact | Impact expected, because Rel-19 UE-to-UE CLI measurement is only for intra-cell, and measurement reports can’t be shared over Xn interface |
| New solution (by scheduling victim UEs further apart in frequency domain) | CLI mitigated | Scheduling restriction to victim UE for DL reception | No impact | No impact | No impact |

Proposal 5: RAN4 conclude the UE-to-UE CLI problem (i.e., intra-operator adjacent-channel inter-cell UE-to-UE CLI, in Urban Hotspot Scenario):

- UE-to-UE CLI issue is generally not identified by considering practical assumptions.

- The SBFD UL UE to TDD DL UE CLI may exist in some corner cases, which strongly depends on the simulation assumptions.

- Solutions identified for mitigate this UL-to-UL CLI problem, while pros/cons and standard impacts are analyzed.

# 6. Reference

[1] RP-240789 (revision from RP-234035), “WID revision: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, RAN#103.

[2] 3GPP TR 38.858 v18.0.0, “Study on Evolution of NR Duplex Operation (Release 18)”.

[3] R4-2508708, “Way Forward on UE-UE CLI problem statement and potential solutions”, Samsung, RAN4#115.

[4] 3GPP TR 38.901 v17.0.0, “Study on channel model for frequencies from 0.5 to 100 GHz (Release 17)”.

[5] 3GPP TR 38.803 v14.4.0, “Study on new radio access technology: Radio Frequency (RF) and co-existence aspects (Release 14)”.

[6] 3GPP TR 36.942 v18.0.0, “Evolved Universal Terrestrial Radio Access (E-UTRA) Radio Frequency (RF) system scenarios (Release 18)”.