3GPP TSG-RAN WG4 Meeting #116	R4-2511269

Bengaluru, IN, 25th – 29th, August. 2025

Source: 	Nokia

Title: 					n256 A-MPR simulations and proposal

Agenda Item: 			7.6.3.2.1

Document for:	Approval

# 1	Introduction

Based on the WF from RAN4 #115 meeting [1], in this paper, we’d like to provide A-MPR simulations for n256 PC1 and PC2, NS_24. We also provide an A-MPR proposal based on our results and A-MPR regions based on the simulations.

# 2	Simulation assumptions and scenario

## 2.1	Simulation parameters

- Operating band n256 (UL 1980 MHz – 2010 MHz)

- 5MHz, 10 MHz, 15 MHz, and 20 MHz CBWs

- Power class PC1 and PC2

- WOLA processing

- I/Q image -28 dBc (default according to 38.101-1, Table 6.4.2.3-1) for other modulations than 256QAM

  - For 256QAM the I/Q image is -33.7 dBc as in MPR simulations [2].

- Carrier leakage -28 dBc (default according to 38.101-1, Table 6.4.2.3-1)

- CIM3 -60 dBc

- CIM5 -70 dBc

- Channel noise: 38.5 dBc

- PA noise: -75 dBm/MHz

- Only additional spurious emission mask from NS_24 is simulated. Other gating factors (ACLR, SEM, etc.) are excluded because they are handled by MPR.

- Phase noise and channel noise are only added for QAM256, following LTE 256-QAM MPR simulations [2].

## 2.2	NS_24 mask

Table 6.5.3.3.13-1: Additional requirements for "NS_24" (38.101-1)

| Frequency range (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 5 MHz, 10 MHz, 15 MHz, 20MHz |  |  |
| 2010 ≤ f ≤ 2025 | -50 | 1MHz | This requirement applies at a frequency offset equal or larger than 5 MHz from the upper edge of the channel bandwidth, whenever these frequencies overlap with the specified frequency band. |

# 3	Simulation results

We provide complete sets of the A-MPR triangles from the simulations at chosen relevant center frequencies in the Appendix. In this section we present some highlights of the results.

In Figs. 1 and 2 we present examples of the A-MPR triangles for 20 MHz CBWs for PC2 and PC1, respectively.

Figure 1: Examples of the simulated A-MPR to satisfy the NS_24 additional requirements for 20 MHz channel at Fc = 1995 MHz for PC2.

Figure 2: Examples of the simulated A-MPR to satisfy the NS_24 additional requirements for 20 MHz channel at Fc = 1995 MHz for PC1.

Based on the A-MPR triangles for n256 PC1 NS_24 20 MHz CBW at 1995Mhz center frequency, the A-MPR is needed for most parts of the triangles, but for smaller bandwidths the A-MPR regions are smaller. The largest A-MPR values at the bottom left corner come from CIM3 which comes from I/Q modulator nonlinearity. At the top of the triangle, we see the regrowth of the allocation that comes from the PA nonlinearity. At the bottom right corner of the triangle, we see CIM5 or IMD, depending on the case.

# 4	A-MPR proposal

We have simulated the A-MPR for n256 following NS_24 limitation. We fill the simulation results based on the A-MPR table of PC3. We marked the regions that are different from PC3 in yellow. Empty cells in tables 2 and 4 indicate that MPR is already enough according to our results.

We found that in some cases region B needs to be slightly extended to cover all the allocations that need A-MPR, which also affects region C. This extension is likely needed due to the tail of the CIM3 peak. An alternative way to cover the A-MPR allocations is to decrease the upper LCRB limit of region C or create a new region. We also found that in case of PC2 the region A at 5 MHz needs to be extended a bit due to regrowth of the allocation. and regions B and C need to be modified for 15MH The reason we think why 5MHz regions for PC1 can be the same as PC3’s is because the linearity of the PC1 PAs is better due to the difference of form factors from PC2 and PC2 PAs.

Proposal 1: RAN4 shall consider the tables 1 to table 4 below for defining the A-MPR for n256 NS_24.

Table 1: A-MPR regions for -50 dBm/MHz for PC1

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | Region A |  |  | Region B |  |  | Region C |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR | RBend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR | RBend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR |
| 5MHz | 1987.5 < Fc ≤ 1992.5 |  | >3.24 | A7 |  |  |  |  |  |  |
| 5MHz | 1992.5 < Fc ≤ 1997.5 |  | >3.24 | A4 |  |  |  |  |  |  |
| 5MHz | 1997.5 < Fc ≤ 2002.5 |  | >1.98 | A1 | >3.6 | >1.08 ≤1.98 | A2 | ≤3.6 | ≤1.98 | A3 |
|  |  |  |  |  |  | ≤1.08 | A6 |  |  |  |
| 10MHz | 1975 < Fc ≤ 1985 | >5.4 |  | A4 |  |  |  |  |  |  |
| 10MHz | 1985 < Fc ≤ 1995 |  | >4.32 | A1 | ≥7.2 | >1.08 ≤4.32 | A2 | <7.2 | ≤4.32 | A3 |
|  |  |  |  |  |  | ≤1.08 | A6 |  |  |  |
| 10MHz | 1995 < Fc ≤ 2000 | ≥5.76 |  | A5 | <3.06 |  | A5 | ≥3.06 <5.76 | >1.44 | A6 |
| 15MHz | 1972.5 < Fc ≤ 1987.5 |  | >6.84 | A1 | ≥10.8 | >1.08 ≤6.84 | A2 | <10.8 | ≤6.84 | A3 |
|  |  |  |  |  |  | ≤1.08 | A6 |  |  |  |
| 15MHz | 1987.5 < Fc ≤ 1997.5 | ≥8.64 |  | A5 | <4.14 |  | A5 | ≥4.14 <8.64 | >1.44 | A6 |
| 20MHz | 1970 < Fc ≤ 1990 | ≥12.96 |  | A5 | <4.86 |  | A5 | ≥4.86 <12.96 | >2.16 | A6 |
| 20MHz | 1990 < Fc ≤ 1995 | ≥11.52 |  | A5 | <5.76 |  | A5 | ≥5.76 <11.52 | >1.44 | A6 |
| NOTE 1: The A-MPR values are listed in Table 6.2.3.15-2. NOTE 2: For any undefined region, MPR applies |  |  |  |  |  |  |  |  |  |  |

Table 2: A-MPR for -50 dBm/MHz for PC1

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 | A6 | A7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer | Outer/Inner | Outer/Inner | Outer |
| DFT-s-OFDM PI/2 BPSK | 8 | 6.5 |  | 1.5 | 21.5 | 16.5 |  |
| DFT-s-OFDM QPSK | 9 | 8 |  | 3 | 21.5 | 16 |  |
| DFT-s-OFDM 16 QAM | 9.5 | 8 |  | 3.5 | 21.5 | 16 |  |
| DFT-s-OFDM 64 QAM | 9.5 | 8.5 |  | 3.5 | 21.5 | 16 |  |
| DFT-s-OFDM 256 QAM | 9.5 | 8.5 |  |  | 21.5 | 16 |  |
| CP-OFDM QPSK | 10 | 10 |  | 4 | 21.5 | 16.5 |  |
| CP-OFDM 16 QAM | 10 | 10 |  | 4 | 21.5 | 16.5 |  |
| CP-OFDM 64 QAM | 10 | 10 |  | 4 | 21.5 | 16.5 |  |
| CP-OFDM 256 QAM | 10 | 10 |  |  | 21.5 | 16.5 |  |
| NOTE 1: The backoff applied is max(MPR, A-MPR) where MPR is defined in Table 6.2.2-1 NOTE 2: Outer and inner allocations are defined in clause 6.2.2 |  |  |  |  |  |  |  |

Table 3: A-MPR regions for -50 dBm/MHz for PC2

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | Region A |  |  | Region B |  |  | Region C |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR | RBend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR | RBend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR |
| 5MHz | 1987.5 < Fc ≤ 1992.5 |  | >3.24 | A7 |  |  |  |  |  |  |
| 5MHz | 1992.5 < Fc ≤ 1997.5 |  | >3.06 | A4 |  |  |  |  |  |  |
| 5MHz | 1997.5 < Fc ≤ 2002.5 |  | >1.98 | A1 | >3.6 | >1.08 ≤1.98 | A2 | ≤3.6 | ≤1.98 | A3 |
|  |  |  |  |  |  | ≤1.08 | A6 |  |  |  |
| 10MHz | 1975 < Fc ≤ 1985 | >5.4 |  | A4 |  |  |  |  |  |  |
| 10MHz | 1985 < Fc ≤ 1995 |  | >4.32 | A1 | ≥7.2 | >1.08 ≤4.32 | A2 | <7.2 | ≤4.32 | A3 |
|  |  |  |  |  |  | ≤1.08 | A6 |  |  |  |
| 10MHz | 1995 < Fc ≤ 2000 | ≥5.76 |  | A5 | <3.06 |  | A5 | ≥3.06 <5.76 | >1.44 | A6 |
| 15MHz | 1972.5 < Fc ≤ 1987.5 |  | >6.84 | A1 | ≥10.8 | >1.08 ≤6.84 | A2 | <10.8 | ≤6.84 | A3 |
|  |  |  |  |  |  | ≤1.08 | A6 |  |  |  |
| 15MHz | 1987.5 < Fc ≤ 1997.5 | ≥8.64 |  | A5 | <4.14 |  | A5 | ≥4.14 <8.64 | >1.44 | A6 |
| 20MHz | 1970 < Fc ≤ 1990 | ≥12.96 |  | A5 | <4.68 |  | A5 | ≥4.68 <12.96 | >2.16 | A6 |
| 20MHz | 1990 < Fc ≤ 1995 | ≥11.52 |  | A5 | <5.58 |  | A5 | ≥5.58 <11.52 | >1.44 | A6 |
| NOTE 1: The A-MPR values are listed in Table 6.2.3.15-2. NOTE 2: For any undefined region, MPR applies |  |  |  |  |  |  |  |  |  |  |

Table 4: A-MPR for -50 dBm/MHz for PC2

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 | A6 | A7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer | Outer/Inner | Outer/Inner | Outer |
| DFT-s-OFDM PI/2 BPSK | 9 | 5 | 0.5 | 2.5 | 16.5 | 11.5 |  |
| DFT-s-OFDM QPSK | 10 | 5 | 2 | 4.5 | 16.5 | 11.5 |  |
| DFT-s-OFDM 16 QAM | 11 | 5 | 2.5 | 4.5 | 16.5 | 11.5 |  |
| DFT-s-OFDM 64 QAM | 11 | 5 |  | 4.5 | 17 | 11.5 |  |
| DFT-s-OFDM 256 QAM | 11 | 5 |  |  | 17 | 11.5 |  |
| CP-OFDM QPSK | 11.5 | 5.5 | 3.5 | 5 | 17.5 | 12.5 |  |
| CP-OFDM 16 QAM | 11.5 | 5.5 | 3.5 | 5.5 | 17.5 | 12.5 |  |
| CP-OFDM 64 QAM | 11.5 | 5.5 |  | 5.5 | 17.5 | 12.5 |  |
| CP-OFDM 256 QAM | 11.5 |  |  |  | 17.5 | 12.5 |  |
| NOTE 1: The backoff applied is max(MPR, A-MPR) where MPR is defined in Table 6.2.2-1 NOTE 2: Outer and inner allocations are defined in clause 6.2.2 |  |  |  |  |  |  |  |

# 5	Conclusions

We have simulated A-MPR for n256 for PC1 and PC2. We have made the following proposal based on our results and A-MPR regions:

Proposal 1: RAN4 shall consider the tables 1 to table 4 below for defining the A-MPR for n256 NS_24.

Table 1: A-MPR regions for -50 dBm/MHz for PC1

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | Region A |  |  | Region B |  |  | Region C |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR | RBend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR | RBend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR |
| 5MHz | 1987.5 < Fc ≤ 1992.5 |  | >3.24 | A7 |  |  |  |  |  |  |
| 5MHz | 1992.5 < Fc ≤ 1997.5 |  | >3.24 | A4 |  |  |  |  |  |  |
| 5MHz | 1997.5 < Fc ≤ 2002.5 |  | >1.98 | A1 | >3.6 | >1.08 ≤1.98 | A2 | ≤3.6 | ≤1.98 | A3 |
|  |  |  |  |  |  | ≤1.08 | A6 |  |  |  |
| 10MHz | 1975 < Fc ≤ 1985 | >5.4 |  | A4 |  |  |  |  |  |  |
| 10MHz | 1985 < Fc ≤ 1995 |  | >4.32 | A1 | ≥7.2 | >1.08 ≤4.32 | A2 | <7.2 | ≤4.32 | A3 |
|  |  |  |  |  |  | ≤1.08 | A6 |  |  |  |
| 10MHz | 1995 < Fc ≤ 2000 | ≥5.76 |  | A5 | <3.06 |  | A5 | ≥3.06 <5.76 | >1.44 | A6 |
| 15MHz | 1972.5 < Fc ≤ 1987.5 |  | >6.84 | A1 | ≥10.8 | >1.08 ≤6.84 | A2 | <10.8 | ≤6.84 | A3 |
|  |  |  |  |  |  | ≤1.08 | A6 |  |  |  |
| 15MHz | 1987.5 < Fc ≤ 1997.5 | ≥8.64 |  | A5 | <4.14 |  | A5 | ≥4.14 <8.64 | >1.44 | A6 |
| 20MHz | 1970 < Fc ≤ 1990 | ≥12.96 |  | A5 | <4.86 |  | A5 | ≥4.86 <12.96 | >2.16 | A6 |
| 20MHz | 1990 < Fc ≤ 1995 | ≥11.52 |  | A5 | <5.76 |  | A5 | ≥5.76 <11.52 | >1.44 | A6 |
| NOTE 1: The A-MPR values are listed in Table 6.2.3.15-2. NOTE 2: For any undefined region, MPR applies |  |  |  |  |  |  |  |  |  |  |

Table 2: A-MPR for -50 dBm/MHz for PC1

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 | A6 | A7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer | Outer/Inner | Outer/Inner | Outer |
| DFT-s-OFDM PI/2 BPSK | 8 | 6.5 |  | 1.5 | 21.5 | 16.5 |  |
| DFT-s-OFDM QPSK | 9 | 8 |  | 3 | 21.5 | 16 |  |
| DFT-s-OFDM 16 QAM | 9.5 | 8 |  | 3.5 | 21.5 | 16 |  |
| DFT-s-OFDM 64 QAM | 9.5 | 8.5 |  | 3.5 | 21.5 | 16 |  |
| DFT-s-OFDM 256 QAM | 9.5 | 8.5 |  |  | 21.5 | 16 |  |
| CP-OFDM QPSK | 10 | 10 |  | 4 | 21.5 | 16.5 |  |
| CP-OFDM 16 QAM | 10 | 10 |  | 4 | 21.5 | 16.5 |  |
| CP-OFDM 64 QAM | 10 | 10 |  | 4 | 21.5 | 16.5 |  |
| CP-OFDM 256 QAM | 10 | 10 |  |  | 21.5 | 16.5 |  |
| NOTE 1: The backoff applied is max(MPR, A-MPR) where MPR is defined in Table 6.2.2-1 NOTE 2: Outer and inner allocations are defined in clause 6.2.2 |  |  |  |  |  |  |  |

Table 3: A-MPR regions for -50 dBm/MHz for PC2

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | Region A |  |  | Region B |  |  | Region C |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR | RBend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR | RBend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR |
| 5MHz | 1987.5 < Fc ≤ 1992.5 |  | >3.24 | A7 |  |  |  |  |  |  |
| 5MHz | 1992.5 < Fc ≤ 1997.5 |  | >3.06 | A4 |  |  |  |  |  |  |
| 5MHz | 1997.5 < Fc ≤ 2002.5 |  | >1.98 | A1 | >3.6 | >1.08 ≤1.98 | A2 | ≤3.6 | ≤1.98 | A3 |
|  |  |  |  |  |  | ≤1.08 | A6 |  |  |  |
| 10MHz | 1975 < Fc ≤ 1985 | >5.4 |  | A4 |  |  |  |  |  |  |
| 10MHz | 1985 < Fc ≤ 1995 |  | >4.32 | A1 | ≥7.2 | >1.08 ≤4.32 | A2 | <7.2 | ≤4.32 | A3 |
|  |  |  |  |  |  | ≤1.08 | A6 |  |  |  |
| 10MHz | 1995 < Fc ≤ 2000 | ≥5.76 |  | A5 | <3.06 |  | A5 | ≥3.06 <5.76 | >1.44 | A6 |
| 15MHz | 1972.5 < Fc ≤ 1987.5 |  | >6.84 | A1 | ≥10.8 | >1.08 ≤6.84 | A2 | <10.8 | ≤6.84 | A3 |
|  |  |  |  |  |  | ≤1.08 | A6 |  |  |  |
| 15MHz | 1987.5 < Fc ≤ 1997.5 | ≥8.64 |  | A5 | <4.14 |  | A5 | ≥4.14 <8.64 | >1.44 | A6 |
| 20MHz | 1970 < Fc ≤ 1990 | ≥12.96 |  | A5 | <4.68 |  | A5 | ≥4.68 <12.96 | >2.16 | A6 |
| 20MHz | 1990 < Fc ≤ 1995 | ≥11.52 |  | A5 | <5.58 |  | A5 | ≥5.58 <11.52 | >1.44 | A6 |
| NOTE 1: The A-MPR values are listed in Table 6.2.3.15-2. NOTE 2: For any undefined region, MPR applies |  |  |  |  |  |  |  |  |  |  |

Table 4: A-MPR for -50 dBm/MHz for PC2

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 | A6 | A7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer | Outer/Inner | Outer/Inner | Outer |
| DFT-s-OFDM PI/2 BPSK | 9 | 5 | 0.5 | 2.5 | 16.5 | 11.5 |  |
| DFT-s-OFDM QPSK | 10 | 5 | 2 | 4.5 | 16.5 | 11.5 |  |
| DFT-s-OFDM 16 QAM | 11 | 5 | 2.5 | 4.5 | 16.5 | 11.5 |  |
| DFT-s-OFDM 64 QAM | 11 | 5 |  | 4.5 | 17 | 11.5 |  |
| DFT-s-OFDM 256 QAM | 11 | 5 |  |  | 17 | 11.5 |  |
| CP-OFDM QPSK | 11.5 | 5.5 | 3.5 | 5 | 17.5 | 12.5 |  |
| CP-OFDM 16 QAM | 11.5 | 5.5 | 3.5 | 5.5 | 17.5 | 12.5 |  |
| CP-OFDM 64 QAM | 11.5 | 5.5 |  | 5.5 | 17.5 | 12.5 |  |
| CP-OFDM 256 QAM | 11.5 |  |  |  | 17.5 | 12.5 |  |
| NOTE 1: The backoff applied is max(MPR, A-MPR) where MPR is defined in Table 6.2.2-1 NOTE 2: Outer and inner allocations are defined in clause 6.2.2 |  |  |  |  |  |  |  |

# 6	References

[1] R4-2508008, WF on [115][128] NR_IoT_NTN_HPUE_part1

[2] R4-1609823, Uplink 256-QAM MPR simulations

# 7	Appendix: A-MPR triangles

## 7.1	PC1

## 7.2	PC2