3GPP TSG-RAN WG4 Meeting #116	R4-2511276

Bengaluru, India, August 25th – August 29th, 2025

Source:	Nokia

Title:	Adopting the new channel model with satellite movements into RRM test cases

Agenda item:	7.6.5.2

Document for:	Discussion

Introduction

In RAN #101, RAN plenary has approved a simplification of the test framework for NGSO, amidst concerns of the time to complete the work item in RAN5 by the end of the Release 17. As a consequence, the specification work in RAN4 has concluded that the test cases for NGSO scenarios would be tested against a fixed (non-varying) propagation delay and fixed doppler delay.

As the speed of satellite in NGSO scenarios is a very relevant aspect of the NTN scenario that has not been covered by similar testing before and considering that NTN is a growing and relevant market, RAN has approved the WID RP-241985 [1] (NR_IoT_NTN_req_test_enh) to develop more meaningful and relevant test cases for this feature considering the motion trajectory of the satellite.

The following objectives were established for this WI:

NTN testing for NGSO - Specify the TE-emulated channel model with varying Doppler and delay shifts for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands - Match the satellite motion trajectory based on the ephemeris for NGSO (Non-Geostationary Orbit) scenarios - Inform RAN5 to assist specifying RF frequency error tests, if needed - Checking if the existing frequency error requirement, can be met under the TE-emulated channel model without intention of modifying the existing core requirement - Specify UE RRM performance test of uplink timing for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model - Checking if the existing RRM uplink timing requirements, can be met under the TE-emulated channel model without intention of modifying the existing core requirement - Specify UE demodulation performance requirement(s) of PDSCH for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model - Minimize the number of demodulation performance requirements - Specify the applicability of the tests under the TE-emulated channel model

The topic was initially discussed in RAN4 #112, and the discussion followed in RAN4 #113.

In the present meeting, RAN4 has to discuss the adaptation of the RRM specifications in TS 38.133 to encompass the new channel model to the test cases.

In this contribution we will discuss the following topics:

- The scope of the changes to acquire the new channel model

- How to define/configure the new channel model in the specifications.

- Whether new tolerance margins are to be added to the test cases.

Discussion

Applicability of the new channel model

The first point to be discussed in this document is related to the scope of changes of the test cases to adopt the new test cases. In the previous meeting, some companies presented their views relative to this matter, and the following options were presented. The views are captured in the moderator summary under issue 3-2-1:

Main proposals: - Proposal 1 (Xiaomi， Samsung, Ericsson, Huawei): - Yes for UL transmit timing test cases - Proposal 2 (Ericsson): - Yes for the test cases below. - For NR NTN, at least below test cases may apply the new model. In RRC_IDLE state mobility, - Cell reselection to FR1 inter-frequency NR case - Time-based measurement initiation to FR1 inter-frequency cell reselection Timing requirements In RRC_CONNECTED state mobility, - Inter-frequency SAN Handover from FR1 to FR1 - Inter-frequency SAN time-based conditional Handover from FR1 to FR1 - Inter-frequency SAN location-based conditional Handover from FR1 to FR1 In Measurement procedure, - Inter-frequency Measurements, e.g., - SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used with single gap for satellite access - Event triggered reporting tests without gap under DRX. - For NB-IoT NTN and MTC NTN, at least below test cases may apply the new model. In RRC_IDLE state for satellite access, - Cell re-selection for satellite access Timing requirements In RRC connection mobility control for satellite access, - Random Access for Satellite Access In UE measurement procedures in RRC_CONNECTED state for UE category NB1 for satellite access, - Inter-frequency neighbour cell measurement.

Our view in this topic aligns with Proposal 2. The reason behind is that Proposal 2 encompass the set of all test cases in which the UE behavior is affected by the doppler effects or varying propagation delay caused by the satellite movement. For the timing advance test cases, it is obvious the impact of the varying propagation delay, for the mobility test cases in scenarios with multiple satellites, the presence of  “K_satellite /K_multi_SMTC” was added to accommodate for the UE needs to measure different satellites with very different doppler effects.

Therefore:

Utilize the new test cases for the UL transmit timing test cases for both NR NTN and NB-IoT NTN.

For NR NTN apply the new channel model for the following test cases in RRC_IDLE:

  - Cell reselection to FR1 inter-frequency NR cell

  - Time-based measurement initiation to FR1 inter-frequency cell reselection

For NR NTN apply the new channel model for the following mobility  test cases in RRC_CONNECTED:

  - Inter-frequency SAN Handover from FR1 to FR1

  - Inter-frequency SAN time-based conditional handover from FR1 to FR1

  - Inter-frequency SAN location-based conditional Handover from FR1 to FR1.

For NB-IoT NTN and MTC NTN, apply the new channel model for the following test case:

  - Cell reselection for satellite access.

Configuring the new channel model on the test cases

In [2], there were different proposals about the definition of the test cases and how the specification should capture it:

#### Issue 3-2-2: How to define the test cases with NGSO channel model Background: Main proposals: - Proposal 1 (Xiaomi): - FFS on the test configuration - Proposal 1a (Ericsson, Huawei): - the test configuration for NGSO can be added in Clause B.5. And the model itself can refer to BDaT spec (TS38.101,102) - Proposal 1b (Samsung) - First, NGSO channel model should be added in these timing TCs. The corresponding configuration of NGSO channel should be added. - - During the tests, UE position is needed to be added and set by AT command at the beginning of T1. In successive time periods of different T, UE position should keep unchanged. The TE adds the NGSO channel model by itself.

In a more detailed analysis, the proposal 1a seems to address the problem at hand with enough details. In clause B.5, the channel model is defined, with references to the specifications that create the channel model. Then, in the respective test cases, the channel model is to be referred to clause B.5.

Based on this, we propose the following:

The channel model for NGSO can be added in Clause B.5, using references to the definition of the channel model in the BDaT specification (e.g. TS38.101,102)

In the affected test cases, add the reference of the new channel model in the test configuration, pointing towards clause B.5

New tolerance margin for the new channel model

This topic has been discussed lengthy during the preparation of the channel model. We don’t think we have evidence enough to establish that the new margin is warranted and neither how large such margin should be.  In this case, as the WI approaches the completion phase, we see no solid justification to provide the added margin for the time being. The discussion can ensue in RAN5, where the interested companies may provide their views about this issue.

Whether additional test tolerance is needed based on the new channel model is left for RAN5 discussion.

Conclusion

In this paper, we provided our views on the performance test cases utilizing the new channel model that includes satellite movement. The following proposals were made:

Proposal 1: Utilize the new test cases for the UL transmit timing test cases for both NR NTN and NB-IoT NTN.

Proposal 2: For NR NTN apply the new channel model for the following test cases in RRC_IDLE:

a.	Cell reselection to FR1 inter-frequency NR cell

b.	Time-based measurement initiation to FR1 inter-frequency cell reselection

Proposal 3: For NR NTN apply the new channel model for the following mobility  test cases in RRC_CONNECTED:

a.	Inter-frequency SAN Handover from FR1 to FR1

b.	Inter-frequency SAN time-based conditional handover from FR1 to FR1

c.	Inter-frequency SAN location-based conditional Handover from FR1 to FR1.

Proposal 4: For NB-IoT NTN and MTC NTN, apply the new channel model for the following test case:

a.	Cell reselection for satellite access.

Proposal 5: The channel model for NGSO can be added in Clause B.5, using references to the definition of the channel model in the BDaT specification (e.g. TS38.101,102)

Proposal 6: In the affected test cases, add the reference of the new channel model in the test configuration, pointing towards clause B.5

Proposal 7: Whether additional test tolerance is needed based on the new channel model is left for RAN5 discussion.

- References

- RP-241985, “Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN”, Samsung, RAN #105, Melbourne, Australia, September 9-12, 2024+

- R4-2505558, “Topic summary for [115][209] NR_IoT_NTN_req_test_enh”, Moderator (Xiaomi), Malta, MT, 19th – 23rd May, 2025