3GPP TSG-RAN WG4 Meeting #116	R4-2511280

Bengaluru, India, August 25th – August 29th, 2025

Source:	Nokia

Title:	On the measurement framework for the new IOT NTN TDD Punctured frame

Agenda item:	7.29.6

Document for:	Discussion

Introduction

In the plenary meeting RAN #105, a new WI for Rel-19 was approved to introduce a TDD frame operation for NTN [1]. The work item is designed to provide extended support of 3GPP NB-IoT features leveraging existing satellite systems deployed in Non-Geostationary Satellite Orbit (NGSO). It also aims at enabling low-complexity payloads for satellite operations that are non-reliant on a diplexer.

To achieve the aforementioned goals, the WID was designed to introduce an NTN-compatible TDD frame, that while based on minimum necessary changes on the NB-IoT NTN FDD specification provides a new approach to TDD periodicity.  RAN4 has analysed the potential impacts of such frame and provided their view on [2].

In the plenary meeting RAN #106, the WID was updated to provide the objectives for RAN4 based on identified challenges [3]. In this document we discuss the potential RRM impacts.

Discussion

Measurement framework

The measurement framework for the newly proposed IOT NTN TDD punctured frame pattern was extensively discussed in the previous RAN4 meeting. There are multiple different views and objects of discussion related to this Work Item, which did not allow further agreements to be made. RAN4 is currently discussing the parametrization of the measurement procedures, both, in consideration of the signals to be measured and in the number of samples needed of each signal to be available for the requirement to be applicable.

For the first task (signals to be measured), RAN4 has reached agreements about the signals to be measured for most procedures, but there are still two ongoing items left for further study: whether the current WI scope will consider the measurements in NSS for cell measurements in connected mode (clause 8.14A in TS 38.133), and for RRC-re-establishment measurements (6.5A).

In both cases, we see this as an UE enhancement that allows for an alternative measurement procedure, which may provide some UE gains. However, the default measurements (NRS) are already covered by the agreements, and there is still considerable workload to be performed in this WI and this requires efficient usage of the time units available for discussion. We see that, in the light of the current open issues, the NSSS measurements are less relevant for the introduction of the feature and should be dowprioritized.

RAN4 downprioritize NSSS measurements for cell measurement in connected mode and RRC Re-establishment.

For the second task (number of samples needed). RAN4 has recently agreed to have the side condition set to:

- SNR in AWGN (normal coverage): -1 dB

The current agreement has defined the number of samples available for cell measurements in mobility scenario, but has not yet reached agreement for RLM.

Agreement: online - Q2: How many samples of the reference signal(s) were needed in a normal coverage scenario with SNR ≥ -1 dB? - Issue 1 & 2 & 3: Cell reselection (4.6A.2), Cell measurement in connected mode (8.14A) and RRC re-establishment - Detection: 12 NPSS subframes + 2 NSSS subframes + 1 NSSS subframe - T_SI: 8 NPBCH subframes is included - Measurement: 8 NRS subframes - Issue 5: RLM (7.23A): further discussion

For the RLM, it has been raised by some companies that under Qout conditions, the number of samples required has to be adjusted, because 8 samples would not be enough. But also, in legacy procedures, even in non-DRX procedures, the total time allowed for Qout is 400 m, as there is some time left for filtering and spacing between samples, which might help to avoid very frequent transitions between “Qout” and “Qin” state transitions at the UE side. In this sense we propose to consider temporarily [20] and [10] NRS subframes for Qout and Qin, until these values are confirmed by simulation.

For non-DRX state, the RLM can be performed with [20] and [10] NRS subframes for Qout and Qin respectively.

For DRX state, usually 20 cycles are considered in legacy procedures. And we believe this can be maintained for the current framework.

For DRX state, the number of NRS samples needed for RLM does not need to be changed.

Issue 1: Cell Reselection

For the cell reselection the current open issue was left for discussion in the way forward from RAN4 #114bis:

Agreement (online): For the cell reselection for TDD NB-IoT in NTN, - The requirements are applicable only if UE is provided with the active periods of the neighbour cells configured for the measurements. - For earth fixed cell and earth moving cell: - The existing values for Tdetect, Tmeasure, and Tevaluate in Table 4.6A.2.2-1 and Table 4.6A.2.2-2 can be applicable if it is confirmed that the current requirement is achievable without requiring more than one NPSS/NSSS sample for half the DRX cycle length [due to the TDD pattern] - Further discuss the values if -1dB will be used. Ksatellite = 1 for the cells measured in DL opportunities of a neighbour satellite that are in non-overlapping with DL opportunities to be measured in the serving satellite Further discuss the following: - For earth moving cell: - Option 2-A: The existing values for Tdetect, Tmeasure, and Tevaluate in Table 4.6A.2.2-1 and Table 4.6A.2.2-2 can be applicable if it is confirmed that the current requirement is achievable without requiring more than one NPSS/NSSS sample for half the DRX cycle length - Option 2-B: Re-evaluate the maximum latency in cell detection, measurement, and evaluation required for this scenario, as well as the minimum number of NPSS, NSSS, and NRS samples needed for cell detection and measurement at an SNR of [-1] dB, and enhance Tdetect, Tmeasure, and Tevaluate in Table 4.6A.2.2-1 and Table 4.6A.2.2-2.

Based on our analysis provided in Table 1 and on the discussion above, where we indicate that, in a given number of DRX Cycles (see Table 4.6A.2.2-1) the number of PSS/SSS opportunities is maintained and although in smaller number the number of NRS symbols is enough to acquire multiple versions of the signal to enable L1 and L3 measurements, we propose that:

Re-use the values for Tdetect, Tmeasure and Tevaluate from Release 17/18.

However, it was raised during the meeting that some companies are concerned that although the number of samples assumed to be measured within a DRX cycle is preserved compared to the legacy reference, the total number of opportunities available to obtain such samples is reduced. This means that the UEs shall wake up for a higher percentage of the PSS/SSSoccasions. Besides, if the UE has to measure across multiple DL opportunities, the total wake time will be much larger. Therefore, when the UE has to measure cells across multiple DL opportunities (see Figure 1), the UE will be awake for most of the time just to collect measurements of the cells to be measured, which goes against the principle of NB-IoT to limit power consumption. Because of this, as a compromise, we can scale the measurements when the UE has to measure 3 or more different DL opportunities, in order to give the UE more chances to save power and space its measurements in time.

Figure 1

Scale Tmeasure and Tdetect by a factor of [2], when the UE measures cells across [3] or more different DL opportunities.

Issue 2: Cell measurements in connected mode

For the cell measurements in connected mode, the current way forward states that:

Agreement: online - Q2: How many samples of the reference signal(s) were needed in a normal coverage scenario with SNR ≥ -1 dB? - Issue 1 & 2 & 3: Cell reselection (4.6A.2), Cell measurement in connected mode (8.14A) and RRC re-establishment - Detection: 12 NPSS subframes + 2 NSSS subframes + 1 NSSS subframe - T_SI: 8 NPBCH subframes is included - Measurement: 8 NRS subframes - Issue 5: RLM (7.23A): further discussion

For intra-frequency measurements, in legacy specification, Tdetect and Tmeasure are given by:

|  | No DRX Cycle | 0.256<DRX-cycle≤10.24 |
| --- | --- | --- |
| Tdetect | 1400 ms | 6 DRX Cycles |
| Tmeasure | 800 ms (NRS-based) or 1600 ms (NSS-based) | 5 DRX Cycles |

In the agreements above, the number of NPSS needed in normal coverage (-1 dB)  is 12 NPSS subframes. In the new TDD frame, the periodicity of both symbols has been enlarged by a factor of 9 (PSS went from 10 to 90 ms and SSS from 20 to 180 ms), therefore within 1400 ms (used before for No DRX Cycle), there will be only 7 SSS opportunities and 15 PSS opportunities. This would cause the time for measurements to be very tight and demand the UE to wake up every DL subframe. We suggest to increase the value to 2000 ms in order to accommodate more measurements.

When no DRX is configured, for the new TDD punctured pattern, the value of and Tdetect shall be 2000 ms.

For Tmeasure, 800 ms seems to be enough to accommodate for the 8 NRS subframes.

When no DRX is configured, for the new TDD puctured pattern, the value of Tmeasure does not need to be changed.

In a reference used several Releases ago, it was established that the number of PSS and SSS considered “available” for measurements per DRX Cycle – in order to establish the number of DRX Cycles needed to complete a measurement – was:

  - DRX Cycle 1.28s: 5

  - DRX Cycle: 2.56s: 10

  - DRX Cycle 5.12s: 10

  - DRX Cycle: 10.24s: 20

In all the cases above, the most stringent one is the DRX Cycle with duration 1.28s. It is possible to see that at least 5 NPSS and 5 NSSS are available per DRX cycle. Based on this, and in the number of samples needed to complete the measurements we propose:

When DRX is configured, for the new TDD punctured pattern, the value of Tdetect for intra-frequency measuremetns shall be:

- 20 DRX Cycles for 0.256 s<DRX-cycle<2.56 s

- 10 DRX Cycles for 2.56≤DRX-cycle<5.12

- 5 DRX Cycles for 5.12≤DRX-cycle

Besides, based on the same reasoning aforementioned for idle mode mobility, when the UE has to measure across multiple DL opportunities the measurements can be scaled:

Scale Tmeasure and Tdetect  by a factor of [2], when the UE measures cells across 3 or more different patterns of DL opportunities.

Inter-frequency measurements

Inter-frequency measurement requirements, based on sample counts (factors N and M in the time to detect formula), already scale naturally with measurement time. Therefore, without increasing the number of samples, we propose that

Do not further scale Tmeasure and Tedetect for inter-frequency measurements due to the punctured pattern

For the length of the measurement occasions, the pre-conditions for the requirements applicability also establish that:

- Length of MOdetect_inter_NB1-NC is at least 200 ms. - Length of MOmeasure_inter_NB1-NC is at least 50 ms.

Given the improved frame structure with more defined measurement opportunities due to serving cell inactivity, we propose reducing the minimum measurement window size for time to detect below 200ms (i.e. reducing the MO window size). This allows for measurements aligned with UE-perceived downlink opportunities, minimizing overlap with serving cell downlink activity. One potential approach is to have the minimum length of measurement equal to the interval between the end of one DL occasion X (X=1,2,3 or 4) to the start of the next DL occasion X (90 ms – 8 subframes = 82 ms).

For inter-frequency measurement, the minimum length of measurement occasions is [82] ms.

Issue 3: RRC Re-establishment

The discussion for RRC Re-establishment is related to the number of PSS and SSS samples available for the UE within a period of time. As discussed in section 2.2 of the present document, the total time of 1400 ms shall be enlarged to 2000 ms to accommodate the number of samples to be collected

The search time per frequency layer for the RRC Re-establishment shall be 2000 ms per frequency layer.

Issue 4: RRC Connection Redirection to non-anchor carrier

There is one additional issue discussed in the previous WF that needs addressing. It is related to the DL-UL switch times in the RRC connection redirection.

## Issue 4: RRC Connection redirection to Non-Anchor Carrier (6.9A) Agreement (online): For the latency requirement (TRRC_procedure_delay + Tperiod_DL_bitmap + TUL_grant + TDL-UL switch + TNPUSCH) of RRC Connection redirection to Non-Anchor Carrier for TDD NB-IoT in NTN, - No update to TRRC_procedure_delay, TUL_grant, and TNPUSCH - Update the following parameters - Tperiod_DL_bitmap: to be updated based on RAN1 design - TDL-UL switch: to be updated based on RAN1 design

The discussion below was presented for the first time in our previous contribution and is repeated hereby:

The requirements for RRC Connection Redirection to Non-Anchor Carrier establish the maximum delay in time for the UE to complete the transition (RRC Redirection) to the Non-Anchor Carrier upon receiving the reconfiguration indication through the IE “CarrierConfigDedicated-NB”.  This delay depends on several components, as described below:

6.9A.2 Requirements The UE shall be capable of performing the RRC connection redirection to the non-anchor carrier within Tconnection_redirect_non-anchor. The time delay (Tconnection_redirect_non-anchor) is the time between the end of the last subframe in the repetition period of NPDSCH containing the IE, “CarrierConfigDedicated-NB” received on the anchor carrier and the end of the last subframe in the repetition period of NPUSCH transmitted on the target non-anchor carrier. The time delay (Tconnection_redirect_non-anchor) shall be less than: Tconnection_redirect_non-anchor = TRRC_procedure_delay + Tperiod_DL_bitmap + TUL_grant + TDL-UL switch + TNPUSCH - TRRC_procedure_delay: It is the RRC procedure for processing the received message “CarrierConfigDedicated-NB”. It shall be less than 110 ms. - Tperiod_DL_bitmap: It is the periodicity of the downlink subframe configuration for downlink transmission on the non-anchor carrier. It is configured via IE DL-Bitmap-NB [2] and can be 10 ms or 40 ms. TDL-UL switch: It is the time between the end of the last subframe in the repetition period of NPDCCH received on the non-anchor carrier and the start of the first subframe in the repetition period of the corresponding NPUSCH transmitted on the non-anchor carrier. TDL-UL switch is 8 ms. - TNPUSCH: It is the time required to transmit NPUSCH on the non-anchor carrier. The value of TNPUSCH depends on the number of repetitions of NPUSCH used in the non-anchor carrier. When the NPUSCH ACK transmission for the received RRC message takes longer than 110ms, the overall RRC connection redirection delay may be extended.

As we highlight above, two of the parameters need to be revisited during this WI, as they are impacted by the periodic pattern introduced.

The Tperiod_DL_bitmap defines the periodicity of the DL subframe configuration on the non-anchor carrier. It could be previously configured for 10 or 40 ms. However, in the current WI, due to periodic patterns with cycle periods of 9 radio frames, the “off time”  of a cell might be close to 90 ms. Therefore, the value of the parameter requires updating.

Update Tperiod_DL_bitmap to a value compatible with a period of 90 ms.

Also the TDL-UL switch  which measures the time between the end of the last subframe in the repetition period of the NPDCCH received on the non-anchor carrier and the start of the first subframe in the repetition period of the NPUSCH transmitted on the non-anchor carrier requires updating. Currently the delay value is set to 8 ms. But in practice this value will be much longer than 8 ms.

In fact, regarding the DL-UL switch, RAN1 has reached a significant agreement that might be used as a baseline for establishing the new requirement:

Agreement The guard period between the end of the downlink subframes and the beginning of the uplink subframes is fixed in specifications to be 50ms at the ULSRP. - NOTE: This roughly corresponds to the largest separation between DL and UL Iridium slot pairs. - Note: Adjustments to align with different UL/DL pair can be achieved by adjusting the common TA.

Update TDL-UL switch to a value equal to the guard period between DL-UL  subframes to be introduced in specifications by RAN1.

Conclusion

In this paper we discussed the TDD punctured pattern for IoT NTN UEs. The following Observations and Proposals were made:

Proposal 1: RAN4 downprioritize NSSS measurements for cell measurement in connected mode and RRC Re-establishment.

Proposal 2: For non-DRX state, the RLM can be performed with [20] and [10] NRS subframes for Qout and Qin respectively.

Proposal 3: For DRX state, the number of NRS samples needed for RLM does not need to be changed.

Proposal 4: Re-use the values for Tdetect, Tmeasure and Tevaluate from Release 17/18.

Proposal 5: Scale Tmeasure and Tdetect by a factor of [2], when the UE measures cells across [3] or more different DL opportunities.

Proposal 6: When no DRX is configured, for the new TDD punctured pattern, the value of and Tdetect shall be 2000 ms.

Proposal 7: When no DRX is configured, for the new TDD puctured pattern, the value of Tmeasure does not need to be changed.

Proposal 8: When DRX is configured, for the new TDD punctured pattern, the value of Tdetect for intra-frequency measuremetns shall be:

Proposal 9: Scale Tmeasure and Tdetect  by a factor of [2], when the UE measures cells across 3 or more different patterns of DL opportunities.

Proposal 10: Do not further scale Tmeasure and Tedetect for inter-frequency measurements due to the punctured pattern

Proposal 11: For inter-frequency measurement, the minimum length of measurement occasions is [82] ms.

Proposal 12: The search time per frequency layer for the RRC Re-establishment shall be 2000 ms per frequency layer.

Proposal 13: Update Tperiod_DL_bitmap to a value compatible with a period of 90 ms.

Proposal 14: Update TDL-UL switch to a value equal to the guard period between DL-UL  subframes to be introduced in specifications by RAN1.

- References

- RP-242415, “New WID on introduction of IoT-NTN TDD mode”, September 2024

- R4-2508290,  WF on RRM requirements for IoT_NTN_TDD,   Moderator (Qualcomm Incorporated),  R4-114bis, Malta, MT, 19th – 23rd May, 2025

- RP-243293, “Revised WID on introduction of IoT-NTN TDD mode”, Iridium Satellite LLC, RAN#106, December 2025

- R4-2504915, On the measurement framework for the new IOT NTN TDD Punctured frame, Qualcomm

- R1-2409038, “Feature lead summary #1 on IoT-NTN TDD mode” , Qualcomm, RAN1#118bis

- R4-79AH-0138, “Cell detection simulation results”, Nokia, Hong Kong, China, 28rd – 30th June 2016