3GPP TSG-RAN WG4 Meeting #116	R4-2511331

Bengaluru, India, Aug. 2025

Source	:	Qualcomm Incorporated

Title	:	On FR1 LPWUS UE RF jammer requirements

Agenda item	:	7.24.3.2

Document for	:	Approval

# 1.	Introduction

In this contribution we derive from first principles, the ACS requirement for a UE monitoring LP-WUS, based on real-world considerations. The primary goal is not let LP-WUS monitoring degrade PDCCH/paging reliability. Also included is a proposal for capturing the ‘same PSD’ requirement for the ASCS requirement.

# 2. 	Discussion

## 2.1	ACS requirements

### 2.1.1	Baseline requirement from RAN4 agreements

The following expression for ACSLPWUS can be derived based on the understanding that the same interferer applies to the UE, whether LR or MR is operational, and the ‘wanted signal’ power is set to 14 dB + REFSENS [4]

ACSLPWUS= ACSlegacy+(NFlegacy-NFLPWUS)+10logNRBRBLPWUS-DG	……….	EQ 2.3.1-1

The derivation however depends on some specific agreements on test side-conditions including some tentatively labelled ‘starting point’. A more methodical process would be for RAN4 to first focus on the ACS requirement itself and only then address the side conditions as part of ‘how to test’. For reference, the legacy ACS requirement for FR1 is captured fully in tables 7.5-1 and -2, note that there is no mention of test side conditions (like +14 dB) here.

Proposal 1: RAN4 to focus on the LP-WUS ACS requirement first, and address side conditions like +14 dB etc only after the ACS requirement is finalized.

The fundamental basis of jammer requirements in general is that the LR and MR cohabit the same UE, and the UE must withstand the same jammers whether it is receiving PDSCH or monitoring the WUS in some power-saving mode.

### 2.1.2	Real-world consideration 1

A real-world ACS performance expectation is ‘the coverage of PDSCH and the coverage of LP-WUS both degrade by similar amounts in the presence of an interferer’. This expectation can be translated to ‘for a given interferer, the desensitization suffered by the UE for LP-WUS detection is no greater than the desensitization suffered by the main radio’. This real-world scenario is analyzed below.

For the main radio with diversity gain DG, assume an interferer that causes ‘x’ dB desensitization of PDSCH. The absolute interferer level (dBm) at each UE input can be calculated as (see Annex1, EQ 5.1-1):

Intrfrlegacy=-174+NFlegacy-DG+10logCBW+10log10x10-1+ACSlegacy………. EQ 2.1.2-1

In the real world, LP-WUS is likely to have adjacent non LP-WUS RBs. Desensitization due to these adjacent non-LP-WUS RBs is assumed to be ‘d’ (dB). The ‘real-world principle’ to apply here is that that the adjacent channel interferer that causes ‘x’ dB desens in the MR also causes no more than a further ‘x’ dB desensitization at the UE for LP-WUS detection.  i.e the net desens for LP-WUS reception would be ‘d+x’. The required ACS that should apply to the UE for LP-WUS reception can then be calculated as (see Annex1, EQ. 5.2-1):

ACSLPWUS= Intrfrlegacy+174- NFLPWUS-10logBWLPWUS-10log10x10-1 -d    ….. EQ 2.1.2-2

Inserting the absolute level of the interferer from equation 2.3.2-1 into equation above,

ACSLPWUS= ACSlegacy+NFlegacy-NFLPWUS+10logNRBRBLPWUS-DG-d………. EQ 2.1.2-3

Note that this expression for ACSLPWUS based on ‘real-world’ expectation differs from the one derived from RAN4 agreements by the ASCS desensitization value ‘d’(equation 2.3.1-1).

Note also that there is no dependence on ‘14 dB’ or other magic number in the derivation based on real-world deployment arguments. Instead, it merely assumes that the LP-WUS coverage does not degrade disproportionately compared to PDSCH coverage, in the presence of an interferer. Put another way, the expression for ACS above represents a ‘basic requirement’, while the choice of test side-conditions (14.0 dB etc) must be established only as a subsequent step to verify the basic requirement.

LP-WUS

UE

MR

LR

Jammer

PDSCH

2.3.3	Real-world consideration 2

A second real-world consideration for LP-WUS ACS is the UE’s radio environment when connected to a serving cell (for PDSCH and LP-WUS) and in the presence of a neighbor cell interferer. See figure below. Here the key constraint is equal PSD for all DL from the serving cell, at least for the case of no power boost. The no power boost condition is significant here because that is also the chosen condition for the ASCS requirement.

PDSCH

PDSCH (CBW)

UE jammer (‘Ig’ of bandwidth IBW)

UE jammer

LP-WUS

NR DL RBs not shown

P dBm/Hz

The SNR degradation in the MR when the adjacent channel interferer is introduced, relative to when the UE is interference-free is given by (Annex 1, EQ. 5.1-4)

∆SNRACS,g=10log1+10IG-ACSlegacy+DG10CBW.10Nlegacy10

…. EQ 2.1.3-1

LP-WUS operation is now considered for the same pair of UE and serving cell gNB. For LP-WUS RBs, 0 dB power boost is assumed. i.e., when LP-WUS is transmitted, it has the same ‘P’ dBm/Hz PSD as PDSCH.  Desensitization due to the adjacent non-LP-WUS RBs is assumed to be ‘d’ (dB). Assuming filtering of the interferer by the amount ACSLPWUS, the SNR degradation in LP-WUS reception due to the adjacent channel jammer (Annex1, EQ. 5.3-3):

∆SNRACS,LPWUS=10log1+10Ig-ACSLPWUS10BWLPWUS.10NLPWUS10+10d10-1.10P-SNRLPWUS10 	…. EQ 2.1.3-2

The principle used for this real-world consideration is that under the condition of same PSD for PDSCH and LP-WUS, the SNR degradation for PDSCH reception is comparable to the SNR degradation of LP-WUS reception in the presence of an adjacent channel interferer. i.e.:

∆SNRACS,LPWUS=∆SNRACS,g

From equations 2.3.3-1 and -2:

10log1+10IG-ACSlegacy+DG10CBW.10Nlegacy10=10log1+10Ig-ACSLPWUS10BWLPWUS.10NLPWUS10+10d10-1.10P-SNRLPWUS10

i.e.   10IG-ACSlegacy+DG10CBW.10Nlegacy10=10Ig-ACSLPWUS10BWLPWUS.10NLPWUS10+10d10-1.10P-SNRLPWUS10

i.e.    10ACSLPWUS-ACSlegacy+DG10=CBW.10Nlegacy10BWLPWUS.10NLPWUS10+10d10-1.10P-SNRLPWUS10

ACSLPWUS-ACSlegacy+DG=Nlegacy+10logCBWBWLPWUS-10log10NLPWUS10+10d10-1.10P-SNRLPWUS10

ACSLPWUS-ACSlegacy+DG=Nlegacy-NLPWUS+10logCBWBWLPWUS-10log1+10d10-1.10P-SNRLPWUS1010NLPWUS10

Note that the threshold SNR for LP-WUS reception is given by SNRLPWUS= RLPWUS-NLPWUS where ‘R’ is the PSD of the REFSENS and N is the PSD of the antenna referred noise (-174 + NF), so:

ACSLPWUS-ACSlegacy+DG=Nlegacy-NLPWUS+10logCBWBWLPWUS-10log1+10d10-1.10P-RLPWUS10

Or…     ACSLPWUS= ACSlegacy+NFlegacy-NFLPWUS+10logNRBRBLPWUS-DG-10log1+10d10-1.10P-RLPWUS10

…. EQ 2.1.3-3

While this expression is dependent on the DL PSD ‘P’, simplifications can be made. Specifically, the expression above must apply throughout the cell, especially at cell edge. For cell edge cases, ‘P’ is close to ‘R’ (PDSCH sensitivity PSD), and therefore comparable to ‘RLPWUS’. Consequently, the last term 10log1+10d10-1.10P-RLPWUS10 contributes negligibly, and the ACS expressions align for the LR types:

ACSLPWUS≈ ACSlegacy+NFlegacy-NFLPWUS+10logNRBRBLPWUS-DG  …. EQ 2.1.3-4

### 2.1.4	UE implementation considerations

The previous sections have evaluated UE ACS requirements demanded by the system, i.e. a ‘top-down’ approach. We now evaluate if there is a feasibility problem at the UE.

Recall that the UE can autonomously and transparently flip between using the LR and MR, as part of its expected behavior (see Annex 3). The overall goal is to preserve reliability of PDCCH/paging while supporting the LP-WUS feature and under jammer conditions. This UE flexibility is key to implementing LP-WUS support with LRs with ultra-low operational power consumption goals.

For example, consider a UE with an LR implementation featuring either relaxed filtering, or relaxed phase noise performance. This UE would opportunistically use the LR when interference conditions are low enough but fall back to MR operation when interferers can affect its LR implementation’s reliability.

A UE’s implementation choice of the LR and the handover logic between the LR or legacy operation with the MR are two sides of the same coin. For example, a UE design with an ultra-low consumption LR is more likely to fall back to legacy operation with the MR due to interferers, while another UE with sufficient filtering and LO quality will not need to fall back to MR operation as often. The handover logic is unique to each UE implementation and is integral to the support of this feature.

Observation 1: LR implementations with relaxed phase noise or relaxed filtering are not precluded by a system-perspective ACS requirement because:

- The LP-WUS requirements are applicable at a UE level

- UEs are allowed to transparently and autonomously toggle between MR and LR operation.

### 2.1.5	Summary of ACS considerations

If RAN4 make the allowance that LR NF trend across bands mirrors that of the MR, the difference between LR and MR NF remains constant across bands. The expression derived for low bands then would then apply to all bands, and the requirement becomes band agnostic. The following is assumed for future argumentation: LR NF trend across bands is assumed to the same as MR NF trend across bands.

The table below summarizes the implied ACS requirements for 3 considerations or motivating principles treated in this contribution, while incorporating proposal 2:

| Motivating principle | Implied LP-WUS ACS Requirement | ACS Requirement for type 1 LR | ACS Requirement for type 2 LR |
| --- | --- | --- | --- |
| - RAN4 agreements on LP-WUS [4] | ACSLPWUS= Z+(NFlegacy-NFLPWUS) | ACSLPWUS= Z-9.5 | ACSLPWUS= Z-5.0 |
| - The coverage of PDSCH and the coverage of LP-WUS both degrade by similar amounts in the presence of an interferer | ACSLPWUS= Z+NFlegacy-NFLPWUS-d | ACSLPWUS= Z-10.0 | ACSLPWUS= Z-5.0 |
| - Under the condition of same PSD for PDSCH and LP-WUS, the SNR degradations for PDSCH reception and for LP-WUS reception are comparable | ACSLPWUS=Z -NFlegacy-NFLPWUS | ACSLPWUS= Z-9.5 | ACSLPWUS= Z-5.0 |
| NOTE: Z= ACSlegacy+10logNRBRBLPWUS-DG |  |  |  |

Regardless of system-level motivating principle, the ACS requirement for LP-WUS works out to a bandwidth-corrected version of the legacy ACS requirement (parameter ‘Z’ in table above) with some correction for number of Rx chains. There is close alignment between the requirements, so it may be reasonable to choose the more relaxed candidate for the requirement.

Proposal 2: Based on real world principle: ‘the coverage of PDSCH and the coverage of LP-WUS both degrade by similar amounts in the presence of an interferer’, the LP-WUS ACS requirement for both LR types is (‘d’ is allowed ASCS desens):

ACSLPWUS=  NFlegacy-NFLPWUS+10logNRBRBLPWUS+ACSlegacy-DG-d

The expression for ACSLPWUS in proposal 3 is translated into dCR form [5].  Note that, for case 2, where the interferer level is specified as an absolute level, the LP-WUS signal level is derived as:

SLPWUS=Abs.Intrfr-ACSLPWUS+SNRLPWUS+IM

As argued for proposal 1, the basic requirement is the starting point, and the side conditions must be derived to verify the requirement above. It is not meaningful to discuss an increase in wanted signal level while retaining the interferer level, because it loses consistency with the basic ACSLPWUS requirement.

Observation 2: Due to LP-WUS coverage and reliability implications, a comprehensive study is first needed before ACS requirements can be relaxed from system-level considerations.

Since the expression for ACSLPWUS is a function of ACSlegacy, it follows that the requirement split for ACSlegacy (be it based on frequency band or DL power regime) must carry over unchanged to ACSLR requirement.

Proposal 3: The LP-WUS ACS jammer requirements retain the same structure (requirement split based on frequency band, DL power regime or other) as the legacy requirements as a baseline. Streamlining can be discussed from this baseline.

## 2.2	ASCS requirement

RAN4 have agreed [2] that:

For ASCS test configuration, the NR RBs should be fully allocated with the same PSD of LP-WUS within the channel except for 11 LP-WUS RBs and potential guard RBs (if any).

It was further agreed [3] that the max. input power requirement would include a high-power version of the ASCS requirement:

Issue 3-2-3: Maximum input level requirements for FR1 LP-WUR - In spec, for requirements using -25dBm as single value. - The max. input power level is specified for all channel bandwidths. FFS the side condition - in the presence of legacy RBs with the same PSD as the LPWUS RBs. - And no NR RBs in the CBW

A convenient definition here is that of LP-WUS power boosting, which is agreed for use with BSRF requirements:

LP-WUS power boosting is the difference between the average power of LP-WUS REs (which occupy certain REs within a NR transmission bandwidth configuration) and the average power over all REs (from both LP-WUS and the NR carrier containing the LP-WUS REs).

Observation 3: The agreed ASCS test condition of equal PSD between LP-WUS and legacy NR RBs can be captured simply as ‘0 dB LP-WUS power boost’ in the requirements.

The CR proposal [6] incorporates the agreements above along with observation 3.

## 2.3	IBB, OBB, Intermodulation and Spurious

RAN4 have agreed [1]:

IBB, OBB requirements Same interference level as MR. further discuss and decide next meeting on relaxed wanted signal

Any ‘relaxation’ of wanted signal, assumed to mean higher wanted signal in the requirement side condition compared to legacy requirements, must first be justified from a system perspective. In section 2.3, evaluation of some real-world considerations demonstrated that retaining the same side condition format (same interferer as MR, same relation between wanted signal and its REFSENS level) is the correct method.

For consistency, this principle should be extended to any other types of jammers, including IBB, OBB, etc.

Proposal 4: Based on real-world principles, if the LP-WUS IBB, OBB, spurious and Rx intermodulation jammer requirements retain the use of the legacy jammer, they also specify the wanted signal level like the legacy case, i.e. same dB increase as the legacy requirements, but referenced to the LP-WUS REFSENS value.

This also implies that any MR requirement split by band or by power level must be carried over to the LP-WUS requirement set as a baseline. Streamlining can be further discussed based on technical arguments.

Proposal 5: As a baseline, the LP-WUS IBB, OBB, spurious and Rx intermodulation jammer requirements retain the same structure (requirement split based on frequency band, DL power regime or other) as the corresponding legacy requirements. Streamlining can be discussed from this baseline.

Proposals 4 and 5 are captured in CR format [7,8].

# 3.	Conclusions

Proposal 1: RAN4 to focus on the LP-WUS ACS requirement first, and address side conditions like +14 dB etc only after the ACS requirement is finalized.

Observation 1: LR implementations with relaxed phase noise or relaxed filtering are not precluded by a system-perspective ACS requirement because:

- The LP-WUS requirements are applicable at a UE level

- UEs are allowed to transparently and autonomously toggle between MR and LR operation.

Proposal 2: Based on real world principle: ‘the coverage of PDSCH and the coverage of LP-WUS both degrade by similar amounts in the presence of an interferer’, the LP-WUS ACS requirement for both LR types is (‘d’ is allowed ASCS desens):

ACSLPWUS=  NFlegacy-NFLPWUS+10logNRBRBLPWUS+ACSlegacy-DG-d

Observation 2: Due to LP-WUS coverage and reliability implications, a comprehensive study is first needed before ACS requirements can be relaxed from system-level considerations.

Proposal 3: The LP-WUS ACS jammer requirements retain the same structure (requirement split based on frequency band, DL power regime or other) as the legacy requirements as a baseline. Streamlining can be discussed from this baseline.

Observation 3: The agreed ASCS test condition of equal PSD between LP-WUS and legacy NR RBs can be captured simply as ‘0 dB LP-WUS power boost’ in the requirements.

Proposal 4: Based on real-world principles, if the LP-WUS IBB, OBB, spurious and Rx intermodulation jammer requirements retain the use of the legacy jammer, they also specify the wanted signal level like the legacy case, i.e. same dB increase as the legacy requirements, but referenced to the LP-WUS REFSENS value.

Proposal 5: As a baseline, the LP-WUS IBB, OBB, spurious and Rx intermodulation jammer requirements retain the same structure (requirement split based on frequency band, DL power regime or other) as the corresponding legacy requirements. Streamlining can be discussed from this baseline.

# 4.	References

[1]	R4-2505104, ‘WF for [114bis][131] NR_LPWUS_UERF’, vivo RAN4#114-Bis

[2] 	R4-2503028, ‘WF on LP-WUS UE RF requirements’, vivo, RAN4#114

[3]	R4-2507936, ‘WF for [115][136] NR_LPWUS_UERF’, vivo RAN4#115

[4]	R4-2505368, ‘On UE Rx requirements for the LPWUR’, Qualcomm, RAN4#115

[5] 	R4-2511338, ‘dCR to 38.101-1, LPWUS ACS’, Qualcomm et al., RAN4#116

[6] 	R4-2511339, ‘dCR to 38.101-1, LPWUS max. power limit’, Qualcomm et al., RAN4#116

[7] 	R4-2511342, ‘dCR to 38.101-1, LPWUS IBB, OBB’, Qualcomm et al., RAN4#116

[8] 	R4-2511340, ‘dCR to 38.101-1, spurious, intermodulation for LPWUS’, Qualcomm et al., RAN4#116

# 5.	Annex1, derivation of ACS interferer level

## 5.1	Legacy 2Rx baseline ACS treatment

Assume:

- Rlegacy is the refsens PSD for the MR requirement at each connector

- Nlegacy is the PSD of the antenna input referred noise (= -174 + NFlegacy) also at each connector

- SNRMR is the SNR that causes the receiver to run at threshold condition for MR requirements

- CBW is the channel NRB BW in Hz

- ACSMR is the MR ACS requirement for that CBW

- DG is 10log(Nchains) , where Nchain is the number of Rx chains

- IntrfrMR is the absolute interferer level per connector in dBm

For the MR 2Rx reception case without interferers, and assuming MRC, the baseband signal level (coherent combining) and baseband noise level (power combining) can be written as (to some additive constant):

SBBlegacy=Rlegacy+20logNchains+10log⁡(CBW)

NBBlegacy=Nlegacy+10logNchains+10log⁡(CBW)

SNRlegacy=Rlegacy-Nlegacy+10logNchains

The value of SNRMR for UE RF requirements is +1.5 dB, and it is derived by application of an implementation margin (MR agreement) of 2.5 dB to the theoretical expectation of -1.0 dB for threshold SNR for MCS4.

Consider an adjacent channel interference condition that causes ‘x’ dB of desensitization. In other words, the wanted signal must be increased by ‘x’ dB to recover the same signal quality:

SACSlegacy=Rlegacy+x+10log⁡(CBW)

At the baseband, post MRC, to some fixed additive constant:

SBBACSlegacy=Rlegacy+x+10log⁡(CBW)+20logNchains

NBBACSlegacy=Nlegacy+10log⁡(CBW)+10logNchains (same as case without interferers)

IBBACSlegacy=Ix+20logNchains (same combining gain as wanted signal)

Ix=Intrfrlegacy-ACSlegacy

Now, the SINR can be calculated as:

SINRBBACSlegacy=SBBACSlegacy-10log{linearNBBACSlegacy+linearIBBACSlegacy}

For a marginally successful ACS test outcome:

SINRBBACSlegacy= SNRlegacy

So,

Rlegacy+x+10logCBW+20logNchains-10logCBW.10Nlegacy10+Nchains.10Ix10-10logNchains= Rlegacy-Nlegacy+10logNchains

i.e. Nlegacy+x+10logCBW= 10log⁡( CBW.10Nlegacy10+Nchains.10Ix10)

i.e. CBW.10x10-1.10Nlegacy10= Nchains. 10Ix10

i.e. Ix= Nlegacy+10logCBW+10log10x10-1- 10logNchains

Using the previously defined relationship:

Ix=Intrfrlegacy-ACSlegacy

Intrfrlegacy= ACSlegacy+ Nlegacy+10logCBW+10log10x10-1- 10logNchains

Intrfrlegacy= ACSlegacy+ Nlegacy+10logCBW+10log10x10-1- DG

Intrfrlegacy=-174+NFlegacy-DG+10logCBW+10log10x10-1+ACSlegacy…….. EQ. 5.1-1

A more generalized treatment assumes that the input signal power is some ‘P’ dBm/Hz. The SNR condition without interferers is

SNRg= P-Nlegacy+DG				…….. EQ. 5.1-2

When an adjacent channel interferer ‘Ig’ is introduced, the SINR condition can be constructed by evaluating signal, noise and interference at BB, to some additive constant:

SACS,g=P+20logNchains+10log⁡(CBW)

NACS,g=Nlegacy+10logNchains+10log⁡(CBW)

IntACS,g=IG-ACSlegacy+20logNchains

SINRACS,g=SACS,g-10log⁡(10NACS,g10+10IntACS,g10)

SINRACS,g=P+20logNchains+10log⁡(CBW)-10log⁡(10Nlegacy+10logNchains+10log⁡(CBW)10+10IG-ACSlegacy+20logNchains10)

SINRACS,g=P+DG+10log⁡(CBW)-10log⁡(10Nlegacy+10log⁡(CBW)10+10IG-ACSlegacy+10logNchains10) …….. EQ. 5.1-3

The SNR degradation suffered by the UE with the introduction of the adjacent channel interferer is:

∆SNRACS,g= SNRg-SINRACS,g

Using EQs. 5.1-2 and -3:

∆SNRACS,g= P-Nlegacy+DG-P+DG+10log⁡(CBW)-10log⁡(10Nlegacy+10log⁡(CBW)10+10IG-ACSlegacy+DG10)

∆SNRACS,g=10logCBW.10Nlegacy10+10IG-ACSlegacy+DG10-10logCBW-Nlegacy

i.e.   ∆SNRACS,g=10log1+10IG-ACSlegacy+DG10CBW.10Nlegacy10  				.. EQ. 5.1-4

## 5.2	LR 1Rx treatment

Assume:

- RLPWUS is the refsens PSD for the LP-WUS requirement at only connector

- NLPWUS is the PSD of the antenna input referred noise (= -174 + NFLPWUS) also at connector

- SNRLPWUS is the SNR that causes the receiver to run at threshold condition for LP-WUS requirements

- CBW is the channel NRB BW in Hz

- ACSLPWUS is the LP-WUS ACS requirement for that CBW

- IntrfrLPWUS is the absolute interferer level at connector in dBm

- ‘d’ is the dB of ASCS desens

REFSENS for LP-WUS is determined for the reception case without out-of-channel interferers and without any interference contribution due to adjacent non-LP-WUS RBs. In the RESENS condition, the baseband signal level and baseband noise level can be written as (to some additive constant):

SBBLPWUS=RLPWUS+10log⁡(BWLPWUS)

NBBLPWUS=NLPWUS+10logBWLPWUS

SNRLPWUS=SBBLPWUS- NBBLPWUS= RLPWUS-NLPWUS

In a realistic scenario, the first source of interference in addition to thermal noise: leakage is from leakage from non-LP-WUS RBs. If the ASCS desense value is ‘d’, it means the DL signal level must be elevated by ‘d’ so SNR is effectively unchanged from the threshold condition. This means that the effective noise is elevated by the same ‘d’ for this condition.

When the same adjacent channel interferer is now added, we let LP-WUS reception be further desensitized by the value ‘x’, same as the desensitization suffered by the UE during PDSCH reception. This means that the signal level must be further elevated by ‘x’ to achieve the target SINR.

SBBACSLPWUS=RLPWUS+x+10logBWLPWUS+d

NBBACSLPWUS=NLPWUS+10logBWLPWUS+d

IBBACSLPWUS=Ix=Intrfrlegacy-ACSLPWUS

Noise+intereference=10log⁡(10NBBACSLPWUS10+10Ix10)⁡

For a marginally successful ACS outcome:

SINRBBACSLPWUS= SNRLPWUS

So,

RLPWUS+x+10logBWLPWUS+d-10logBWLPWUS.10NLPWUS10.10d10+10Ix10= RLPWUS-NLPWUS

This simplifies as:

NLPWUS+x+10logBWLPWUS+d= 10log⁡( BWLPWUS.10NLPWUS10.10d10+10Ix10)

BWLPWUS.10NLPWUS10.10x10.10d10= BWLPWUS.10NLPWUS10.10d10+10Ix10

BWLPWUS.10x10-1.10NLPWUS10.10d10= 10Ix10

Ix=NLPWUS+10logBWLPWUS+d+10log10x10-1

Using the previously defined relationship for Ix: 
Ix=IntrfrLPWUS-ACSLPWUS

i.e. ACSLPWUS= Intrfrlegacy+174- NFLPWUS-10logBWLPWUS-10log10x10-1-d   …. EQ. 5.2-1

## 5.3	LR 1Rx treatment with same PSD for PDSCH and LP-WUS

Assume:

- RLPWUS is the refsens PSD for the LP-WUS requirement at only connector

- NLPWUS is the PSD of the antenna input referred noise (= -174 + NFLPWUS) also at connector

- SNRLPWUS is the SNR that causes the receiver to run at threshold condition for LP-WUS requirements

- CBW is the channel NRB BW in Hz

- ACSLPWUS is the LP-WUS ACS requirement for that CBW

- IntrfrLPWUS is the absolute interferer level at connector in dBm

- ‘d’ is the dB of ASCS desens

REFSENS for LP-WUS is determined for the reception case without out-of-channel interferers and without any interference contribution due to adjacent non-LP-WUS RBs. In the RESENS condition, the baseband signal level and baseband noise level can be written as (to some additive constant):

SBBLPWUS=RLPWUS+10log⁡(BWLPWUS)

NBBLPWUS=NLPWUS+10logBWLPWUS

SNRLPWUS=SBBLPWUS- NBBLPWUS= RLPWUS-NLPWUS  		 …….. EQ. 5.3-1

In a realistic scenario, the first source of interference in addition to thermal noise is from leakage from non-LP-WUS RBs. If the ASCS desense value is ‘d’, then an elevated signal is required to overcome the ASCS leakage and still deliver the threshold SNR above to the receiver

SBBLPWUS,ASCS=RLPWUS+10logBWLPWUS+d

NBBLPWUS,ASCS=NLPWUS+10logBWLPWUS (same as before)

The interference power from non-LP-WUS RBs is proportional to the PSD common to all DL. This dependence is modelled by an additive constant ‘k’ where ‘k’ is related to the ASCS of the UE for the configured channel BW.

IBBLPWUS,ASCS=SBBLPWUS,ASCS-k

So:

Noise+intereference=10log⁡(10NBBACSLPWUS10+10IBBLPWUS,ASCS10)⁡

For a marginally successful ACS outcome:

SINRBBLPWUS,ASCS= SNRLPWUS

So,

RLPWUS+10logBWLPWUS+d-10logBWLPWUS.10NLPWUS10+BWLPWUS.10(RLPWUS+d-k)10= RLPWUS-NLPWUS

This simplifies as:

NLPWUS+d=10log⁡( 10NLPWUS10+10(RLPWUS+d-k)10)

10NLPWUS10.10d10= 10NLPWUS10+10(RLPWUS+d-k)10

10d10-1.10NLPWUS10= 10(RLPWUS+d-k)10

NLPWUS+10log10d10-1=RLPWUS+d-k

Noting that SNRLPWUS=RLPWUS-NLPWUS (EQ 5.3-1)

k=d+SNRLPWUS-10log⁡(1-10-d10)  		  …….. EQ. 5.3-2

This expression relates the ASCS for a configured channel BW to the dB of de-sense due to adjacent non-LP-WUS RBs assuming the same PSD for all DL.

With a generalized input PSD of P dBm/MHz regardless of PDSCH or LP-WUS, with adjacent non-LP-WUS RBs but no adjacent channel power, the SINR condition of the UE for LP-WUS reception can be constructed by evaluating signal, noise and interference at BB, to some additive constant:

SBBASCS,LPWUS=P+10log⁡(BWLPWUS)

NBBASCS,LPWUS=NLPWUS+10log⁡(BWLPWUS)

IntBBASCS,LPWUS=SASCS,LPWUS-k

SINRASCS,LPWUS=SBBASCS,LPWUS-10log⁡(10NBBASCS,LPWUS10+10IntBBASCS,LPWUS10)

If the adjacent channel interferer ‘Ig’ is now introduced, there is an additional interference component at baseband, which (to some additive constant) is:

IBBACS,LPWUS=Ig-ACSLPWUS

The revised SINR condition under both, adjacent channel jammer as well as non-LP-WUS RBs is:

SINRASCS,LPWUS=SBBASCS,LPWUS-10log⁡(10NBBASCS,LPWUS10+10IntBBASCS,LPWUS10+10IBBACS,LPWUS10)

The LP-WUS SINR degradation due to introduction of the adjacent channel interferer is:

∆SNRACS,LPWUS= SINRASCS,LPWUS- SINRACS,LPWUS

= 10log⁡10NBBASCS,LPWUS10+10IntBBASCS,LPWUS10+10IBBACS,LPWUS10- 10log⁡10NBBASCS,LPWUS10+10IntBBASCS,LPWUS10

=10log1+10Ig-ACSLPWUS1010NASCS,LPWUS10+10IntASCS,LPWUS10

i.e.    ∆SNRACS,LPWUS=10log1+10IACS,LPWUS1010NLPWUS+10log⁡(BWLPWUS)10+10P+10logBWLPWUS-k10

Recall from EQ. 5.3-2 that k=d+SNRLPWUS-10log⁡(1-10-d10), so:

10-k10= 10d10-1.10-SNRLPWUS10

Therefore, SNR degradation for LP-WUS detection in the presence of an adjacent channel interferer is:

∆SNRACS,LPWUS=10log1+10Ig-ACSLPWUS10BWLPWUS.10NLPWUS10+10d10-1.10P-SNRLPWUS10   …….. EQ. 5.3-3

# 6.	Annex2, LP-WUS ACS requirement trends

Graphic representation of LP-WUS ACS requirements from proposal 2 is below. Note that the representation does not imply any specific SCS applicability.

As is well understood, the fundamentally different trends come from different interferer assumptions between the low band group (<= 2.7 GHz) and the high band group (> 2.7 GHz).

# 7.	Annex3, RAN1 rule for monitoring LP-WUS

In general, RAN1 has left it up to the UE on when it should rely on the LR for triggering PDCCH/paging monitoring, and when it needs to fall back on the MR.

For connected mode:

There is no consensus in RAN1 whether to specify UE autonomous fallback to PDCCH monitoring when UE monitors LP-WUS in RRC CONNECTED mod

It means no specific condition is specified in RAN1 on how / whether a UE falls back from WUS monitoring e.g. in case the UE is out of WUS coverage. For connected mode, there is no entry/exit condition, no out-of-coverage report, etc. This practice is aligned with the legacy DCP feature (Rel-16 WUS).

Similarly for IDLE mode:

From RAN1 perspective, for the entry/exit conditions for LP-WUS monitoring in IDLE/inactive mode, - The UE may start LP-WUS monitoring if - the serving cell measurement performed by the MR is above entry threshold(s), if configured by the gNB - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied - If UE starts LP-WUS monitoring, it may stop the legacy PO monitoring before UE receives LP-WUS indicating wake-up - The UE monitors the legacy PO (and may monitor PEI) and may stop LP-WUS monitoring if - the serving cell measurement performed by the LR is below exit threshold(s), if configured by the gNB - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied - FFS the serving cell measurement metrics - The entry/exit thresholds can be configured separately for different types of LR - It is left to RAN2 discussion whether the threshold(s) are always configured by the gNB. - Note: This may be revisited based on the RAN2/RAN4 discussion.

Here too, the interesting detail is no mandate on the UE to use the LR  (i.e no ‘shall’).

Observation: RAN1 does not mandate the UE to use LPWUR in connected or idle/inactive mode, it is up to the UE whether it uses the LR or MR