3GPP TSG-RAN WG4 Meeting #116	R4-2511332

India, Aug. 2025

Source	:	Qualcomm Incorporated

Title	:	On FR2 LPWUS UE RF requirements

Agenda item	:	7.24.3.2

Document for	:	Approval

# 1.	Introduction

In this contribution we present proposals and the underlying arguments for the FR2 LPWUS requirements.

# 2. 	Discussion

## 2.1	LR types

RAN1 have already established capabilities by LR type (see FG62 in RAN1). While these are available for RAN4 to use, it is up to RAN4 to decide whether to enable both types for FR2 bands also. We have previously argued that ‘FR2 unlike FR1 does not lend itself to making a standalone LR. Practical implementations will generally require significant front end sharing with the MR.’

For practical relevance and with WI timeline considerations, it may be sufficient to define the OFDM-based LR for FR2, and the OOK-based LR can be reconsidered in the future if there are advocates.

Proposal 1: For FR2 LR, requirements are specified only for the type2 (OFDM-based) variant. The type1 (OOK-based) variant can be reconsidered if there are advocates.

## 2.2	Applicable UE power classes for LP-WUS

The LP-WUS feature is a receive feature and is orthogonal to the UE’s transmit power capabilities. The LP-WUS feature therefore should be enabled for all UE power classes by default. It is expected that spherical coverage will track that of the MR, as explained in the next section.

Proposal 2: All FR2 power classes are enabled for LP-WUS.

## 2.3	FR2 requirements

### 2.3.1	FR2 REFSENS specification

As is now well understood, 120 kHz is the relevant SCS for FR2. For 120 kHz SCS, RAN1 have already agreed that M = {1} and L = {1,2}. SNR curves for 1% FAR are presented for N = 5 (number of information bits) to the right.

The SNR curves differ based on expected frequency and time error, which in turn is related to frequency with which these parameters are tracked by the UE. In connected mode, the error window is expected to be smaller and consequently the threshold SNR is lower.

The key take away is that the SNR threshold is in the -9 dB range, much lower than in FR1, primarily due to reduction in M from 4 to 1. This value, however, is well beyond the coverage needs of MSG3.

Now, FR2 networks differ fundamentally from legacy FR1 networks, because of beam forming. Moreover, cell coverages are smaller, which means there is less opportunity to exercise the full 32 UE/UE subgroup facility allowed by RAN1 design. While OFDM-based receivers tolerate short LP-WUS, the signal duration is still tied to the number of information bits. Longer signal durations are not favorable for the network. Considering the lack of need to support as many as 32 UE subgroups, most deployment are likely to configure only 2 or 3 information bits.  It is therefore suggested that for FR2, RAN4 adopt 3 information bits as the side condition for LP-WUS requirements. The SNR comparison between 3 and 5 bits is presented in the figure to the right, and it confirms that there is no appreciable change in threshold performance.

Proposal 3: The FRC channel for LP-WUS requirements for FR2 is:

| Parameter | Unit | Value |
| --- | --- | --- |
| MR Channel bandwidth | MHz | All CBW |
| LP-WUS bandwidth | RB | 11 |
| Subcarrier spacing | kHz | 120kHz |
| RM coding | bits | 3 bits(*) |
| Chip rate |  | M=1 (1 chip in an OFDM symbol) |
| overlaid OFDM sequence |  | Length 132: generated by 131-length ZC sequence with extension |
| Number of overlaid OFDM sequence per chip to carry information |  | L = 2 |
| WUS duration for OOK |  | N/A (*) |
| WUS duration for OFDM |  | 6 OFDM symbols |
| Manchester coding for OOK |  | 1/2 |
| Number of information bits |  | 3 |

(*) Values can be revisited if the OOK-based receiver is necessary to support.

While M = 1 affords better sensitivity for FR2 compared to FR1 (M = 4), for the sake of uniformity, it is proposed to adopt the FR1 threshold value of -4.5 dB for FR2 also.

Proposal 4: For FR2 OFDM-based LR, the threshold SNR for 1% MDR and 1% FAR is chosen as -4.5dB

Single element beam

Multi-element refined beam examples

Uniform array with repeated element

Superposition of all refined beams

To derive a sensitivity requirement, it is useful to first revisit basic array theory. For a uniform array of repeated elements, it is well understood that the shape described by the envelope of all possible refined beams is congruent to the shape of the beam of each element. See figure to the right.

Specifically, for a uniform array, the refined beam envelope in any direction has 10log(Nelem) dB more gain compared to that of the prototype element. This is also justification for the previous agreement to maintain the same gain drop and spherical coverage fraction for LP-WUS reception as for legacy MR operation.

Now, sensitivity in any direction (including REFSENS in legacy beam peak direction) for LP-WUS can be calculated formulaically from the legacy MR sensitivity requirements, agreements in RAN4 and basic principles:

- NF+IM is 12 dB [6], (1dB degraded from low FR2 band conn. mode PDSCH reception)

- Diversity gain is -3 dB [6] (3dB degraded from conn. mode PDSCH reception)

- Antenna gain assumption is based on single element rather a 4-element array (6 dB degraded from conn. mode PDSCH reception)

- The SNR proposal for FR2 LP-WUS is -4.5 dB (3.5 dB improvement from MCS4)

- BW adjustment (degraded by 10log(NRB/11))

The net degradation of LP-WUS sensitivity from connected mode PDSCH requirement is therefore 6.5 + 10log(NRB/11) dB, i.e. (1+3 + 6 -3.5 +10log(NRB/11)) dB. This can be extended to high bands if it is assumed that the NF of the LR across bands tracks that of the MR across bands.

Proposal 5: For FR2 OFDM-based LR, LP-WUS sensitivity is calculated from legacy PDSCH sensitivity by adding 6.5 + 10 log (NRB/11) dB.

### 2.3.2	Max. power limit requirement specification

In general, agreements made for FR1 can be carried over to FR2.

Proposal 6: The max. power-related agreements made for FR1 type2 (OFDM-based) are applicable for FR2 type 2 (OFDM-based) LR also

### 2.3.3	Jammer requirement specification

In general, agreements made for FR1 can be carried over to FR2. One legacy FR2 requirement assumptions that is useful to co-opt is that the jammer is ‘polarization matched to the wanted signal’. For simplicity this assumption can be carried over.

Proposal 7: All jammer related agreements made for FR1 type2 (OFDM-based) are applicable for FR2 type 2 LR also. Jammers are applied in same polarization as the wanted signal, like in legacy case.

In a separate contribution [8], we derive the ACS requirement from first principles without dependence on any side conditions (like ‘14 dB’). The analysis confirmed that the 14 dB parameter is chosen merely to facilitate verification of the requirement after the requirement was set, and it is meaningless to not use the same value for both, the desired signal and the interferer. This principle is used for the FR2 ACS requirement derivation, as is retention of the ‘14 dB’ for verification side-conditions. Annex1 contains a more detailed look at the jammer situation in FR2.  For legacy operation with the main radio, the following expression (EQ. 5.1-1) relates the max. strength of the interferer to the ACS requirement and the side condition of providing the wanted signal 14 dB higher than REFSENS, where Gant=10logNelem+Gelemis the array gain in each pol. of the module

Intrfrlegacy= ACSlegacy+ Nlegacy+10logCBW+10log101410-1-Gant

Similarly, for LP-WUS reception with a single element and pol. selection diversity (EQ. 5.2-1)

IntrfrLPWUS= ACSLPWUS+ NLPWUS+10logBWLPWUS+10log101410-1-CG-Gelem

Note here that CG = -3 for LP-WUS (combining gain for pol. selection diversity) Setting both interferers to the same value,

ACSLPWUS= NFlegacy-NFLPWUS+CG-10logNelem+10logCBWBWLPWUS+ACSlegacy

For FR2 LP-WUS, NF+IM is agreed (tentatively) as 12.0 dB [6] for type 2 LR, where IM itself is 2.0 dB from legacy FR2 practice. It follows therefore that NFLPWUS is 10.0 dB. Finally, legacy assumption for the module was a 4 element array. The equation above can be specialized for type 2 LR as:

ACSLPWUS= 9.0-10.0-3.0-6.0+10logCBWBWLPWUS+ACSlegacy

ACSLPWUS,type2= ACSlegacy+10logNRBRBLPWUS-10.0

Note that this equation only applies to those bands where the MR NF is 9.0 dB. If the LP-WUS NF is assumed to track the MR NF (due to front end reuse), the equation above can be generalized to all bands.

Proposal 8: For FR2 type 2 LR, the ACS requirement is derived from legacy the MR ACS requirement as:

ACSLPWUS,type2= ACSlegacy+10logNRBRBLPWUS-10.0

Note that, for case 2, where the interferer level is specified as an absolute level, the LP-WUS signal level is derived as:

SLPWUS=Abs.Intrfr-ACSLPWUS+SNRLPWUS+IM

# 3.	Conclusions

Proposal 1: For FR2 LR, requirements are specified only for the type2 (OFDM-based) variant. The type1 (OOK-based) variant can be reconsidered if there are advocates.

Proposal 2: All FR2 power classes are enabled for LP-WUS.

Proposal 3: The FRC channel for LP-WUS requirements for FR2 is:

| Parameter | Unit | Value |
| --- | --- | --- |
| MR Channel bandwidth | MHz | All CBW |
| LP-WUS bandwidth | RB | 11 |
| Subcarrier spacing | kHz | 120kHz |
| RM coding | bits | 3 bits(*) |
| Chip rate |  | M=1 (1 chip in an OFDM symbol) |
| overlaid OFDM sequence |  | Length 132: generated by 131-length ZC sequence with extension |
| Number of overlaid OFDM sequence per chip to carry information |  | L = 2 |
| WUS duration for OOK |  | N/A (*) |
| WUS duration for OFDM |  | 6 OFDM symbols |
| Manchester coding for OOK |  | 1/2 |
| Number of information bits |  | 3 |

(*) Values can be revisited if the OOK-based receiver is necessary to support.

Proposal 4: For FR2 OFDM-based LR, the threshold SNR for 1% MDR and 1% FAR is chosen as -4.5dB

Proposal 5: For FR2 OFDM-based LR, LP-WUS sensitivity is calculated from legacy PDSCH sensitivity by adding 6.5 + 10 log (NRB/11) dB.

Proposal 6: The max. power-related agreements made for FR1 type2 (OFDM-based) are applicable for FR2 type 2 (OFDM-based) LR also

Proposal 7: All jammer related agreements made for FR1 type2 (OFDM-based) are applicable for FR2 type 2 LR also. Jammers are applied in same polarization as the wanted signal, like in legacy case.

Proposal 8: For FR2 type 2 LR, the ACS requirement is derived from legacy the MR ACS requirement as:

ACSLPWUS,type2= ACSlegacy+10logNRBRBLPWUS-10.0

# 4.	References

[1]	R1-2501397, ‘Initial RAN1 UE features list for Rel-19 NR after RAN1 #120’

[2] 	R4-2420394, ‘WF on LP-WUS UE RF requirements’, vivo, RAN4#113

[3]	R4-2505104, ‘WF for [114bis][131] NR_LPWUS_UERF’, vivo RAN4#114-Bis

[4] 	R4-2503028, ‘WF on LP-WUS UE RF requirements’, vivo, RAN4#114

[5] 	R4-2503170, ‘On UE Rx requirements for the LPWUR’, Qualcomm, RAN4#114-Bis

[6]	R4-2507936, ‘WF for [115][136] NR_LPWUS_UERF’, vivo RAN4#115

[7]	R4-2505368, ‘On UE Rx requirements for the LPWUR’, Qualcomm, RAN4#115

[8]	R4-2511331, ‘On FR1 LPWUS UE RF jammer requirements’, Qualcomm, RAN4#116

[9] 	R4-2511343, ‘dCR to 38.101-2, LPWUS feature CR’, Qualcomm et al., RAN4#116

# 5.	Annex1, derivation of ACS interferer level for FR2

## 5.1	Legacy 2Rx baseline ACS treatment

Assume:

- Rlegacy is the refsens PSD for the legacy requirement at the RIB

- Nlegacy is the PSD of the antenna input referred noise (= -174 + NFlegacy) per element

- Nelem is the number of dual pol. elements, 4 for legacy module size

- SNRlegacy is the SNR that causes the receiver to run at threshold condition for legacy requirements

- CBW is the channel NRB BW in Hz

- ACSlegacy is the legacy ACS requirement for that CBW

- Gelem is element gain in module array.

- Intrfrlegacy is the absolute interferer level at the RIB

For the legacy 2Rx reception case without interferers, and assuming MRC (0 dB combining gain), the baseband signal level (coherent combining) and baseband noise level (power combining) can be written as (to some additive constant):

SBBlegacy=Rlegacy+20logNelem+Gelem+10log⁡(CBW)

NBBlegacy=Nlegacy+10logNelem+10log⁡(CBW)

SNRlegacy=Rlegacy-Nlegacy+10logNelem+Gelem

Define Gant=10logNelem+Gelem

SNRlegacy=Rlegacy-Nlegacy+Gant

The value of SNRlegacy for UE RF requirements is +1.5 dB, and it is derived by application of an implementation margin (legacy agreement) of 2.5 dB to the theoretical expectation of -1.0 dB for threshold SNR for MCS4.

The legacy 2Rx radio under ACS conditions have the following conditions at the RIB:

TestSignalACSlegacy=Rlegacy+14.0+10log⁡(CBW)

At the baseband, post MRC, to some fixed additive constant:

TestSignalBBACSlegacy=Rlegacy+14.0+10log⁡(CBW)+20logNelem+Gelem

NBBACSlegacy=Nlegacy+10log⁡(CBW)+10logNelem (same as case without interferers)

Recall that for FR2, interferers are applied under the assumption of ‘polarization match’, i.e. the interferer is applied with the same polarization as the wanted signal, and therefore gets the same gain after MRC

IBBACSlegacy=Ix+20logNelem+Gelem

Where Ix=Intrfrlegacy-ACSlegacy

SINRBBACSlegacy=TestSignalBBACSlegacy-10log{linearNBBACSlegacy+linearIBBACSlegacy}

For a marginally successful ACS test outcome:

SINRBBACSlegacy= SNRlegacy

So,

Rlegacy+14.0+10logCBW+20logNelem+Gelem-10logNelem.CBW.10Nlegacy10+Nelem2.10Gelem10.10Ix10= Rlegacy-Nlegacy+Gant

This simplifies to:

Nlegacy+14.0+10logCBW= 10log⁡( CBW.10Nlegacy10+Nelem.10Gelem10.10Ix10)

This simplifies to:

CBW.101410-1.10Nlegacy10= Nelem. 10Gelem10.10Ix10

This simplifies to:

Ix= Nlegacy+10logCBW+10log101410-1-Gant

Using the previously defined relationship:

Ix=Intrfrlegacy-ACSlegacy

Intrfrlegacy= ACSlegacy+ Nlegacy+10logCBW+10log101410-1-Gant       …EQ. 5.1-1

The derivation above is made from first principles, and it is useful to cross-check the outcome above with the requirement in the standard. Now, the REFSENS expression can be derived from the previously derived expression for PSD of REFSENS

SNRlegacy=Rlegacy-Nlegacy+Gant

Rlegacy-SNRlegacy+10logCBW=Nlegacy-Gant+10log⁡(CBW)

REFSENSlegacy-SNRlegacy=Nlegacy-Gant+10log⁡(CBW)

The equation above subtracted from the highlighted equation:

Intrfrlegacy- REFSENSlegacy= ACSlegacy+ 10log101410-1-SNRlegacy

Assuming SNRlegacy is 1.5 dB (= -1.0 dB + 2.5 dB IM), and noting that 10log(101.4-1) approximates 14, the above simplifies to:

Intrfrlegacy- REFSENSlegacy≈ACSlegacy+ 12.5

This is readily verified by the reader as consistent with the legacy ACS requirement in 38.101-2. This cross check confirms that the treatment above is correct in its assumptions.

## 5.2	LR 1Rx treatment

Assume:

- RLPWUS is the refsens PSD for the LPWUS requirement at the RIB

- NLPWUS is the PSD of the antenna input referred noise (= -174 + NFLPWUS) also at the RIB

- SNRLPWUS is the SNR that causes the receiver to run at threshold condition for LPWUS requirements

- CBW is the channel NRB BW in Hz

- ACSLPWUS is the LPWUS ACS requirement for that CBW

- IntrfrLPWUS is the absolute interferer level at connector in dBm at the RIB

- CG is combining gain, agreed as -3 dB due to pol. selection diversity assumption

- Gelem is element gain in module array.

For the LPWUS reception case without interferers, the worst-case baseband signal level and baseband noise level can be written as (to some additive constant):

SBBLPWUS=RLPWUS+10logBWLPWUS+CG+Gelem

Note that the equation is written for single element operation, also an agreement for LPWUS.

NBBLPWUS=NLPWUS+10logBWLPWUS

SNRLPWUS=SBBLPWUS- NBBLPWUS= RLPWUS-NLPWUS+CG+Gelem

The LR under ACS conditions extended from agreements in FR1 have the following conditions at baseband (to some fixed additive constant):

TestSignalBBACSLPWUS=RLPWUS+14.0+10log⁡(BWLPWUS)+CG+Gelem

NBBACSLPWUS=NLPWUS+10logBWLPWUS (same as case without interferers)

For the case with interferers, extending the legacy agreement

IBBACSLPWUS=Ix= IntrfrLPWUS-ACSLPWUS+CG+Gelem

SINRBBACSLPWUStype1=TestSignalBBACSLPWUStype1-10log{linearNBBACSLPWUS+linearIBBACSLPWUS}

For a marginally successful ACS test outcome:

SINRBBACSLPWUS= SNRLPWUS

So,

RLPWUS+14.0+10logBWLPWUS+CG+Gelem-10logBWLPWUS.10NLPWUS10+10Ix10= RLPWUS-NLPWUS +CG+Gelem

This simplifies as:

NLPWUS+14.0+10logBWLPWUS= 10log⁡( BWLPWUS.10NLPWUS10+10Ix10)

BWLPWUS.101410-1.10NLPWUS10= 10Ix10

Ix= NLPWUS+10logBWLPWUS+10log101410-1

Using the previously defined relationship: Ix=IntrfrLPWUS-ACSLPWUS+CG+Gelem

IntrfrLPWUS= ACSLPWUS+ NLPWUS+10logBWLPWUS+10log101410-1-CG-Gelem   ….. EQ.5.2-1