3GPP TSG-RAN WG4 Meeting #116	R4-2511336

Bengaluru, India, Aug. 2025

Source: 	Qualcomm

Title: 	TP for TR 38.774: FR2 IBB requirements for LP-WUS

Agenda Item:	7.28.3.3

Document for:	Approval

# Background

This contribution provides the TP for TR 38.774 on blocking requirements for LP-WUS.

# References

[1] R4-2503830, TR 38.774 V0.3.0

# Text Proposal

<Start of Text Poposal>

### 7.1.7	IBB requirements

#### 7.1.7.x	FR2-specific considerations

In general, agreements made for FR1 LPWUS jammers can be carried over to FR2. Some legacy FR2 requirement assumptions that are useful to co-opt is that the jammer is ‘polarization matched to the wanted signal’ and that jammer performance is measured in the Rx beam peak direction.

Following the convention that same jammers should apply, whether UE is receiving PDSCH or detecting LP-WUS, the LP-WUS IBB jammer requirement is derived by retaining the jammer level from legacy practice, but the wanted signal (LP-WUS transmission bandwidth configuration) is derived by replacing legacy REFSENS value with REFSENSLPWUS.

<End of Text Poposal>