3GPP TSG-RAN WG4 Meeting #116	R4-2511337

Bengaluru, India, Aug. 2025

Source: 	Qualcomm

Title: 	TP for TR 38.774: on max. power limit and ASCS

Agenda Item:	7.24.3.2

Document for:	Approval

# Background

This contribution provides the TP for TR 38.774 on max. power limit and ASCS for LP-WUS.

# References

[1] R4-2503830, TR 38.774 V0.3.0

# Text Proposal

<Start of Text Poposal>

### 7.1.5	ASCS requirements

Adjacent subcarrier selectivity (ASCS) is a measure of a receiver's ability to receive an LP-WUS signal at its configured channel frequency in the presence of adjacent in-band NR signal(s). The in-band LP-WUS and NR signal should be same PSD and be separated by a given frequency offset (guard RB) between LP-WUS and NR, and the NR signal occupies the remaining RB resources within the maximum transmission bandwidth configuration, excluding the LP-WUS RBs

If the network uses the RBs other than those used for LP-WUS to serve other UEs, a certain amount of desensitization is expected. This desensitization is evaluated at two DL power levels, REFSENSLPWUS and at the max. power limit. The latter requirement is captured as ‘case 2’ in the max. power limit test. The requirement is framed as an ability to meet the RF metric in the presence of the adjacent RB interferers and the LPWUS signal level elevated from the REFSENS value by the desensitization value.

<End of Text Poposal>