3GPP TSG-RAN4 Meeting #116	R4-2511343

# Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR | 0822 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | dCR to 38.101-2, LPWUS feature CR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_LPWUS |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Feature CR |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify requirements based on extending existing FR1 agreements. Only OFDM-based LR (type2) is targeted. Official feature name in WID is adopted for requirement headings. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | WID objective remains incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.xM |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  |  |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-2 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

/* start change */

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

:

:

Inter-band carrier aggregation: Carrier aggregation of component carriers in different operating bands.

NOTE:	Carriers aggregated in each band can be contiguous or non-contiguous.

Intra-band contiguous carrier aggregation: Contiguous carriers aggregated in the same operating band.

Intra-band non-contiguous carrier aggregation: Non-contiguous carriers aggregated in the same operating band.

LP-WUS power boost: difference between the average power of LP-WUS REs (which occupy certain REs within a NR transmission bandwidth configuration) and the average power over all REs (from both LP-WUS and the NR carrier containing the LP-WUS REs)

Link angle: a DL-signal AoA from the view point of the UE, as described in Annex J. If the beam lock function is used to lock the UE beam(s), the link angle can become any arbitrary AoA once the beam lock has been activated.

Measurement angle: the angle of measurement of the desired metric from the view point of the UE, as described in Annex J

:

:

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

:

:

Max()	The largest of given numbers

Min()	The smallest of given numbers

MPRf,c	Maximum output power reduction for carrier f of serving cell c

MPRnarrow	Maximum output power reduction due to narrow PRB allocation

MPRWT	Maximum power reduction due to modulation orders, transmit bandwidth configurations, waveform types

nPRB	Physical resource block number

NRACLR	NR ACLR

NRB	Transmission bandwidth configuration, expressed in units of resource blocks

NRB,LP-WUS	Number of RBs for LP-WUS

NRB,low	Transmission bandwidth configurations according to Table 5.3.2-1 for the lowest assigned component carrier in clause 5.3A.1

NRB,high	Transmission bandwidth configurations according to Table 5.3.2-1 for the highest assigned component carrier in clause 5.3A.1

NREF	NR Absolute Radio Frequency Channel Number (NR-ARFCN)

:

:

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ACLR	Adjacent Channel Leakage Ratio

ACS	Adjacent Channel Selectivity

ASCS	Adjacent Subcarrier selectivity

A-MPR	Additional Maximum Power Reduction

AoA	Angle of Arrival

:

:

ITU-R	Radiocommunication Sector of the International Telecommunication Union

LP-WUR	Low Power-Wake Up Receiver

LP-WUS	Low Power-Wake Up Signal

LP-SS	Low Power-Synchronization Signal

LR	LP-WUR

MBW MBW	Measurement bandwidth

MDR	Missed-detection rate

MPR	Allowed maximum power reduction

MR	Main Radio

NR	New Radio

NR-ARFCN	NR Absolute Radio Frequency Channel Number

OCNG	OFDMA Channel Noise Generator

OOB	Out-of-band

OOK	On-Off keying

OTA	Over The Air

P-MPR	Power Management Maximum Power Reduction

PRB	Physical Resource Block

QAM	Quadrature Amplitude Modulation

RF	Radio Frequency

REFSENS	Reference Sensitivity

RedCap	Reduced Capability

RIB	Radiated Interface Boundary

RMS	Root Mean Square (value)

RSRP	Reference Signal Receiving Power

Rx	Receiver

SCS	Subcarrier spacing

SEM	Spectrum Emission Mask

SRS	Sounding Reference Symbol

SS	Synchronization Symbol

TPC	Transimission Power Control

TRP	Total Radiated Power

Tx	Transmitter

UE	User Equipment

UL MIMO	Uplink Multiple Antenna transmission

ULFPTx	Uplink Full Power Transmission

/* end change */

/* start change */

## 4.3	Specification suffix information

Unless stated otherwise the following suffixes are used for indicating at 2nd level clause, shown in Table 4.3-1.

Table 4.3-1: Definition of suffixes

| Clause suffix | Variant |
| --- | --- |
| None | Single Carrier |
| A | Carrier Aggregation (CA) |
| B | Dual-Connectivity (DC) |
| C | Supplement Uplink (SUL) |
| D | UL MIMO |
| K | Simultaneous reception or transmission in multiple directions |
| M | Low Power Wake-Up Signal (LP-WUS) |
| NOTE 1: Suffix D in this specification represents either polarized UL MIMO or spatial UL MIMO. RF requirements are same. If UE supports both kinds of UL MIMO, then RF requirements only need to be verified under either polarized or spatial UL MIMO. NOTE 2: Suffix K applies to simultaneous reception or transmission with different TCI states and different QCL-TypeD reference signals across the TCI states |  |

/* end change */

/* start change */

## 5.2K	Operating bands for simultaneous reception or transmission in multiple directions

Simultaneous reception or transmission in multiple directions is enabled for the bands defined in Table 5.2K-1.

Table 5.2K-1: NR bands for simultaneous reception or transmission in multiple directions

NR bands for simultaneous reception or transmission in multiple directions (Table 5.2-1)

n257

n258

n259

n260

n261

## 5.2M	Operating bands for LP-WUS

LP-WUS is enabled in the operating bands defined in Table 5.2M-1.

Table 5.2M-1: NR operating bands in FR2

| Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz | TDD |
| n258 | 24250 MHz | – | 27500 MHz | 24250 MHz | – | 27500 MHz | TDD |
| n259 | 39500 MHz | – | 43500 MHz | 39500 MHz | – | 43500 MHz | TDD |
| n260 | 37000 MHz | – | 40000 MHz | 37000 MHz | – | 40000 MHz | TDD |
| n261 | 27500 MHz | – | 28350 MHz | 27500 MHz | – | 28350 MHz | TDD |
| n262 | 47200 MHz | – | 48200 MHz | 47200 MHz | – | 48200 MHz | TDD |

/* end change */

/* start change */

## 5.3M	UE Channel bandwidth

The LP-WUS carrier bandwidth corresponding to the UE channel bandwidth for LP-WUS/WUR is defined as the sum of resource blocks (RBs) occupied by the LP-WUS signal and the guard RBs separating it from the NR signal. The LP-WUS carrier is embedded within the NR channel and is flexibly positionable, provided alignment with the NR PRB grid is maintained.

A guard RB is referred to as an ASCS guard RB when located between an NR RB and an LP-WUS RB, and as an ACS guard RB when positioned between the NR guardband and an LP-WUS RB.

### 5.3M.2 	Maximum transmission bandwidth configuration

The maximum transmission bandwidth configuration NRB,LP-WUS  within each NR UE channel bandwidth and subcarrier spacing is specified in Table 5.3M.2-1.

Table 5.3M.2-1: Maximum transmission bandwidth configuration NRB,LP-WUS

|  | Channel BW |  |  |  |
| --- | --- | --- | --- | --- |
| SCS (kHz) | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
|  | NRB,LP-WUS | NRB,LP-WUS | NRB,LP-WUS | NRB,LP-WUS |
| 60 | 11 | 11 | 11 | 11 |
| 120 | 11 | 11 | 11 | 11 |

/* end change */

/* start change */

## 7.1M	General for LP-WUS

The criterion for verifying all receiver core RF requirements shall be MDR of the LP-WUS, which shall not exceed 1% with the LP-WUS parameters given in Annex [A.3M]. Beam peak direction refers to RX beam peak direction as determined for clause 7.3.

## 7.2M	Diversity characteristics for LP-WUS

Unless otherwise specified, the requirements apply to all polarizations of DL. The minimum requirements on sensitivity apply to two measurements, corresponding to DL signals in orthogonal polarizations.

/* end change */

/* start change */

## 7.3M	Reference sensitivity for LPWUS

### 7.3M.1	General

For this requirement, a non-LP-WUS DL RB is defined as any RB in the DL transmission bandwidth configuration of the UE-specific channel that is not occupuied by the LP-WUS carrier as defined in sub-clause 5.3M. For this requirement, the power in any non-LP-WUS DL RB is 0 mW.

### 7.3M.2	Reference sensitivity power level

The LP-WUS reference sensitivity power level is defined as the lowest LP-WUS power level at the centre of the quiet zone in the RX beam peak direction at which the MDR criterion in clause [7.1M] is met with the side conditions in clause [7.1M]. The requirement on max. reference sensitivity for LP-WUS is given by:

PREFSENS,LPWUS= PREFSENS+6.5+10log10(NRBNRB, LPWUS) dB

Where PREFSENS is the reference sensitivity (dBm) specified in clause 7.3 for the respective power classes and channel bandwidths.

### 7.3M.3	Void

### 7.3M.4	LP-WUS [sensitivity] spherical coverage

The LP-WUS sensitivity in any direction is defined as the lowest LP-WUS power level at the centre of the quiet zone in that direction for which the MDR criterion in clause [7.1M] is met with the side conditions in clause [7.1M]. The requirement on spherical coverage of LP-WUS sensitivity consists of two parts. The first part is an LP-WUS sensitivity criterion, and the second part is a spherical coverage requirement.

The requirement on spherical coverage LP-WUS sensitivity ‘PSENS,LPWUS’ (sensitivity criterion) is given by:

PSENS,LPWUS= PSENS+6.5+10log10(NRBNRB, LPWUS) dB

Where PSENS is the EIS spherical coverage value (dBm) specified in clause 7.3.4 for the respective power classes and channel bandwidths.

The associated spherical coverage requirement (unless otherwise specified) is the covered fraction of the sphere over which the LP-WUS sensitivity is level less than or equal to the LP-WUS sensitivity criterion above and is specified in table 7.3M.4-1. (Link=Spherical coverage grid, Meas=Link angle).

Table 7.3M.4-1: Spherical coverage requirement for LP-WUS

| UE power class | Spherical coverage (min.) |
| --- | --- |
| 1 | 15 % |
| 2 | 40 % |
| 3 | 50 % |
| 4 | 80 % |
| 5 | 15 % |
| 6 | spherical coverage evaluation areas found in clause 6.2.1.6 |
| 7 | 50% |

/* end change */

/* start change */

## 7.4M	Maximum input level for LP-WUS

Maximum input level is defined as the maximum mean LP-WUS power level at the centre of the quiet zone in the RX beam peak direction, for which the MDR criterion is met with parameters specified in Table 7.4M-1 and 7.4M-2 respectively. For this requirement, a non-LP-WUS DL RB is defined as any RB in the DL transmission bandwidth configuration of the UE-specific channel that is not occupied by the LP-WUS carrier as defined in sub-clause 5.3M. The requirement applies for all channel BW included in sub-clause 5.3M.

Table 7.4M-1: Maximum input level, Case 1

| Rx Parameter | Units | Value |
| --- | --- | --- |
| Min. power in LP-WUS transmission BW config | dBm | -25.0 |
| Power of each non-LPWUS DL RB | mW | 0 |
| ASCS Guard RB2 | PRB | 0 |
| NOTE 2: See section 5.3M |  |  |

Table 7.4M-2: Maximum input level, Case 2

| RX parameter | Units | Channel BW (MHz) |
| --- | --- | --- |
| Total power including LP-WUS carrier and non-LPWUS RBs | dBm | -25.0 |
| ASCS Guard RB2 | PRB | 0 |
| LP-WUS power boost | dB | 0 |
| NOTE 2: See section 5.3M NOTE 3: non-LPWUS DL RBs consists of CP-OFDM QPSK with the same SCS as LP-WUS. |  |  |

/* end change */

/* start change */

## 7.5M	LP-WUS Adjacent channel selectivity

The LP-WUS adjacent channel selectivity (ACS) is a measure of a receiver's ability to receive an LP-WUS signal at its configured RBs inside the configured channel, in the presence of an adjacent channel signal at a given frequency offset from the centre frequency of the configured channel. ACS is the ratio of the receive filter attenuation on the LP-WUS frequency to the receive filter attenuation on the adjacent channel(s).

The requirement applies at the RIB when the AoA of the incident wave of the wanted signal and the interfering signal are both along the Rx beam peak direction. The requirement applies with the wanted and interfering signals in the same polarization. The requirement applies to all polarizations. For this requirement, a non-LP-WUS DL RB is defined as any RB in the DL transmission bandwidth configuration of the UE-specific channel that is not occupied by the LP-WUS carrier as defined in sub-clause 5.3M. For this requirement, the power in any non-LP-WUS DL RB is 0 mW.

The UE shall fulfil the minimum requirement specified in Table 7.5M-1 for all values of an adjacent channel interferer up to –25 dBm. For these parameters, the MDR criterion in clause [7.1M] shall be met with the side conditions in clause [7.1M]. The requirement applies for all channel BW included in sub-clause 5.3M.

However, it is not possible to directly measure the ACS, instead the lower and upper range of test parameters are chosen in Table 7.5M-2 and Table 7.5M-3.

Table 7.5M-1: Adjacent channel selectivity for LP-WUS

| Operating band | Units | Min. ACS (dB) |
| --- | --- | --- |
|  |  | Min. ACS (dB) |
| n257, n258, n261 | dB | 13 + 10log10(NRB/ NRB,LP-WUS) |
| n259, n260, n262 | dB | 12 + 10log10(NRB/ NRB,LP-WUS) |

Table 7.5M-2: Adjacent channel selectivity test parameters, Case 1

| Rx Parameter | Units | Value |
| --- | --- | --- |
| Power in LP-WUS transmission BW config | dBm | REFSENSLP-WUS + 14.0 dB |
| Min. PInterferer for band n257, n258, n261 | dBm | REFSENS + 35.5 dB |
| Min. PInterferer for band n259, n260, n262 | dBm | REFSENS + 34.5 dB |
| BWInterferer | MHz | BWChannel |
| FInterferer (offset) | MHz | BWChannel / -BWChannel |
| NOTE 1: The interferer consists of the Reference measurement channel specified in Annex A.3.3.2 with one sided dynamic OCNG Pattern OP.1 TDD as described in Annex A.5.2.1 and set-up according to Annex C. NOTE 2: The REFSENS power level is specified in Clause 7.3.2, which are applicable to different UE power classes. REFSENSLP-WUS is specified in clause [7.3M.2] NOTE 3: The absolute value of the interferer offset FInterferer (offset) shall be further adjusted to (CEIL(¦FInterferer(offset)¦/SCS) + 0.5)*SCS MHz with SCS the sub-carrier spacing of the wanted signal in MHz. Wanted and interferer signal have same SCS. |  |  |

Table 7.5M-3: Adjacent channel selectivity test parameters, Case 2

| Rx Parameter | Units | Value |
| --- | --- | --- |
| PInterferer | dBm | -25.0 |
| Max. Power in LP-WUS transmission BW config for bands n257, n258, n261 | dBm | -40.5 - 10log10(NRB/ NRB,LP-WUS) |
| Max. Power in LP-WUS transmission BW config for bands n259, n260, n262 | dBm | -39.5 - 10log10(NRB/ NRB,LP-WUS) |
| BWInterferer | MHz | BWChannel |
| FInterferer (offset) | MHz | BWChannel / -BWChannel |
| NOTE 1: The interferer consists of the Reference measurement channel specified in Annex A.3.3.2 with one sided dynamic OCNG Pattern OP.1 TDD as described in Annex A.5.2.1 and set-up according to Annex C. NOTE 2: The REFSENS power level is specified in Clause 7.3.2, which are applicable to different UE power classes. REFSENSLP-WUS is specified in clause [7.3M.2] NOTE 3: The absolute value of the interferer offset FInterferer (offset) shall be further adjusted to (CEIL(¦FInterferer(offset)¦/SCS) + 0.5)*SCS MHz with SCS the sub-carrier spacing of the wanted signal in MHz. Wanted and interferer signal have same SCS. NOTE 4: 10log10(x) is rounded to the next higher 0.5 dB value. |  |  |

/* end change */

/* start change */

## 7.6M	Blocking characteristics

### 7.6M.1	General

The LP-WUS blocking characteristic is a measure of the UE’s ability to receive an LP-WUS at its configured RBs inside the configured channel in the presence of an unwanted interferer at frequencies other than those of the adjacent channels.

The requirement applies at the RIB when the AoA of the incident wave of the wanted signal and the interfering signal are both along the Rx beam peak direction and share the same polarization. The requirement applies to all polarizations. For this requirement, a non-LP-WUS DL RB is defined as any RB in the DL transmission bandwidth configuration of the UE-specific channel that is not occupied by the LP-WUS carrier as defined in sub-clause 5.3M. For this requirement, the power in any non-LP-WUS DL RB is 0 mW.

The requirement applies at the RIB when the AoA of the incident wave of the wanted signal is along the Rx beam peak direction.

### 7.6M.2	In-band blocking

The UE shall meet the MDR criterion per clause [7.1M] with the side conditions specified in Table 7.6M.2-1. The requirement applies for all channel BW included in sub-clause 5.3M.

Table 7.6M.2-1: In band blocking requirements

| Rx parameter | Units | Value |
| --- | --- | --- |
| Power in LP-WUS | dBm | REFSENSLP_WUS + 14 dB |
| BWInterferer | MHz | BWChannel |
| Min. PInterferer for bands n257, n258, n261 | dBm | REFSENS + 35.5 dB |
| Min. PInterferer for bands n259, n260, n262 | dBm | REFSENS + 34.5 dB |
| FInterferer (offset)3 | MHz | ≤ - BWChannel & ≥ BWChannel |
| FInterferer | MHz | FDL_low + BWChannel/2 to FDL_high - BWChannel/2 |
| NOTE 1: The interferer consists of the Reference measurement channel specified in Annex A.3.3.2 with one sided dynamic OCNG Pattern OP.1. TDD as described in Annex A.5.2.1 and set-up according to Annex C. NOTE 2: The REFSENS power level is specified in Clause 7.3.2, which are applicable according to different UE power classes. REFSENSLP-WUS is specified in clause [7.3M.2] NOTE 3: The absolute value of the interferer offset FInterferer (offset) shall be further adjusted (CEIL(¦FInterferer(offset)¦/SCS) + 0.5)*SCS MHz with SCS the sub-carrier spacing of the wanted signal in MHz. Wanted and interferer signal have same SCS. NOTE 4: FInterferer range values for unwanted modulated interfering signals are interferer center frequencies. |  |  |

## 7.7M	Void

## 7.8M	Void

## 7.9M	Spurious emissions

The requirements in clause 7.9 apply.

## 7.10M	Void

/* end change */

/* start change */

## 7.11M	Adjacent subcarrier selectivity for LP-WUS

Adjacent subcarrier selectivity (ASCS) is a measure of a UE’s ability to receive an LP-WUS carrier at its configured RBs, in the presence of fully populated non-LPWUS DL RBs. For this requirement, a non-LP-WUS DL RB is defined as any RB in the DL transmission bandwidth configuration of the UE-specific channel that is not occupuied by the LP-WUS carrier as defined in sub-clause 5.3M.

The MDR criterion in clause [7.1M] shall be met with the side conditions in [7.1M] and the parameter set specified in Tables 7.11M-1. The requirement applies for all channel BW included in sub-clause 5.3M. The requirement applies at the RIB when the AoA of the incident wave of the wanted signal is along the Rx beam peak direction.

Table 7.11M-1: ASCS requirement for LP-WUS

| RX parameter | Units | Value |
| --- | --- | --- |
| Power in LP-WUS transmission BW config | dBm | REFSENSLP-WUS1 |
| ASCS Guard RB2 | PRB | 0 |
| LP-WUS power boost2 | dB | 0 |
| NOTE 1: REFSENSLP-WUS is the value specified in clause [7.3M.2] NOTE 2: See section 5.3M NOTE 3: non-LPWUS DL RBs consists of CP-OFDM QPSK with the same SCS as LP-WUS |  |  |

/* end change */

/* start change */

# A.3M	LP-WUS reference channel

## A.3M.1	General

Unless otherwise stated, the tables in this clause are applicable for all LP-WUS reception requirements.

Table A.3M.1-1. Common reference channel parameters for 60 KHz SCS.

| Parameter | Unit | Value |
| --- | --- | --- |
| MR Channel bandwidth | MHz | All CBW |
| LP-WUS bandwidth | RB | 11 |
| Subcarrier spacing | KHz | 60 |
| Chip rate | chip per OFDM symbol | M=1 |
| Overlaid OFDM sequence |  | Length 132: generated by 131-length ZC sequence with extension |
| Number of overlaid OFDM sequence per chip to carry information |  | 2 |
| LP-WUS duration for [type 2 LR] | OFDM symbols | 6 |
| Manchester coding for OOK |  | 1/2 |
| Number of information bits | Bits | 3 |

Table A.3M.1-2. Common reference channel parameters for 120 KHz SCS.

| Parameter | Unit | Value |
| --- | --- | --- |
| MR Channel bandwidth | MHz | All CBW |
| LP-WUS bandwidth | RB | 11 |
| Subcarrier spacing | KHz | 120 |
| Chip rate | chip per OFDM symbol | M=1 |
| Overlaid OFDM sequence |  | Length 132: generated by 131-length ZC sequence with extension |
| Number of overlaid OFDM sequence per chip to carry information |  | 2 |
| LP-WUS duration for [type 2 LR] | OFDM symbols | 6 |
| Manchester coding for OOK |  | 1/2 |
| Number of information bits | Bits | 3 |

Table A.3M.1-3. Relative power of SSB and LP-WUS

| Parameter | Unit | Value |
| --- | --- | --- |
| Ratio of LP-WUS EPRE with respect to SSB EPRE | dB | 0 |

/* end change */