# 3GPP TSG-RAN WG4 Meeting # 116 	R4-2511345

Bengaluru, IN, Aug. 2025

Title:	Reply LS on Beam Correspondence Initial Access

Source:	Qualcomm, Sony, Ericsson, Lenovo

Agenda item:	9.3

Document for:	Approval

# 1.	Introduction

The LS from RAN5 [1] seeks clarifications on the Rel-18 initial access beam correspondence core requirement in the form of three questions. Our view is provided below

# 2.	Discussion

## 2.1	RAN5 is looking to develop test procedure with and without UE Beamlock test function, can either procedure be used to verify RAN4 core requirements?

In general, BL (beam lock) and other test-related functions (‘test modes’) exist to facilitate testing in some way (RAN5 domain). UE core requirements (RAN4 domain) on the other hand are meant to establish design goals for field performance for UEs. For relevance to field performance, it is preferred for UEs to not rely on behavior-modifying test modes (like BL) for compliance.

In the context of the Rel-18 IABC (initial access and RRC_inactive beam correspondence), RAN4 have discussed BL extensively as part of testability of the IABC requirement. While there was no technical consensus among participating companies on the need for BL, a compromise was reached where RAN4 agreed to a set of requirements based on existence of a BL function for verification [2]. In our understanding of both intent and wording in the LS or the core requirement, there is no mandate from RAN4 for the UE to implement such a function for compliance.

Consequently:

Answer 1: Yes, either procedure can be used. RAN4 confirm that core requirements do not mandate UE to implement an IABC-specific BL function for compliance with the IABC requirements.

## 2.2	Regarding the verification of spherical coverage requirement for initial access and RRC_INACTIVE using UE Beamlock test function in the IDLE mode, will it prevent UE to not autonomously choose Uplink beams? Will using beam lock prevent UE from autonomously choosing other beams even if UE’s behaviour is such?

The first question contains a double negative ‘will (BL) prevent UE to not autonomously choose’ which when straightened out presumably means ‘will (BL) allow UE to autonomously choose …beams’. This seems consistent with the second question, ‘Will using beam lock prevent UE from autonomously choosing other beams even if UE’s behaviour is such?’….

There can be UE implementations whose native behavior an IABC BL may ‘muzzle’. For example, if a UE gets restless about its choice of UL beam due to lack of RAR, it may start to speculatively use beams other than what is chosen as optimal based on DL measurements. For such a UE, an IABC will freeze the UL beam in some configuration determined by arbitrary timing of the BL trigger or arbitrary implementation of the freeze. This example implementation therefore will be prevented from ‘autonomously choosing other beams even if UE’s behaviour is such’.

Answer 2: Yes, BL can prevent a UE from autonomously choosing other beams if UE’s behaviour is such.

## 2.3	Regarding the verification of spherical coverage requirement for initial access and RRC_INACTIVE without using UE Beamlock test function in the IDLE mode, would this be aligned to verifying RAN4 core requirements when the requirements were defined?

As explained in 2.1, There is no RAN4 mandate to use IABC BL for core requirement compliance.

Answer3: Yes. There is no RAN4 mandate for a UE to use IABC BL for IABC core requirement compliance. From a RAN4 perspective, there is no conflict if a UE chooses to meet core requirements without a test mode. It is up to RAN5 to develop the requisite test procedures.

# 3. 	References

- R5-253653, ‘LS on Beam Correspondence Initial Access’, RAN5, RAN5#107, May. 2025

- R4-2317764, ‘LS on testability issues of beam correspondence requirement for initial access’, RAN4#108Bis, Oct. ‘23

# 3GPP TSG-RAN WG4 Meeting # 116 	R4-2511345

Bengaluru, IN, Aug. 2025

Title:	[Draft] Reply LS on Beam Correspondence Initial Access

Response to:	(R5-253653) LS on Beam Correspondence Initial Access

Source:	RAN4

To:	RAN5

Contact Person:

Name	: Sumant Iyer

E-mail Address	: sumanti _at_ qti_dot_qualcomm_dot_com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

1. Overall Description:

RAN4 would like to thank RAN5 for their questions, which are clarified thusly:

- RAN5 is looking to develop test procedure with and without UE Beamlock test function, can either procedure be used to verify RAN4 core requirements?

Answer1: Yes, either procedure can be used. RAN4 confirm that core requirements do not mandate UE to implement an IABC-specific BL function for compliance with the IABC requirements.

- Regarding the verification of spherical coverage requirement for initial access and RRC_INACTIVE using UE Beamlock test function in the IDLE mode, will it prevent UE to not autonomously choose Uplink beams? Will using beam lock prevent UE from autonomously choosing other beams even if UE’s behaviour is such?

Answer2: Yes, BL can prevent a UE from autonomously choosing other beams if UE’s behaviour is such.

- Regarding the verification of spherical coverage requirement for initial access and RRC_INACTIVE without using UE Beamlock test function in the IDLE mode, would this be aligned to verifying RAN4 core requirements when the requirements were defined?

Answer3: Yes. There is no RAN4 mandate for a UE to use IABC BL for IABC core requirement compliance. From a RAN4 perspective, there is no conflict if a UE chooses to meet core requirements without a test mode. It is up to RAN5 to develop the requisite test procedures.

2. Date of Next TSG WG RAN4 Meetings:

TSG-RAN WG4 Meeting #116-Bis	Oct. 2025         		Prague, CZ

TSG-RAN WG4 Meeting #117	Nov. 2023         		Dallas, TX, US