3GPP TSG-RAN WG4 Meeting #116	R4-2511381

# Bangalore, India, 25th – 29th August 2025

Agenda item:		4.2.2

Source: 		Qualcomm Incorporated

Title:	 	PC2 FDD reference measurement channel

Document for:		Approval

## 1	Introduction

PC2 FDD was defined in Rel-17. In the resulting specification default 50% duty cycle is applied for PC2 FDD, and when duty cycle exceeds 50% delta-powerclass parameter is set to 3 dB resulting in power reduction. Testing with limited duty cycle is not enabled as reference measurement channel with less than 50% duty cycle is not specified. Therefore, PC2 FDD specification needs a correction.

## 2	Discussion

Power reduction when duty cycle exceeds 50% and no additional signaling is present is specified twice in TS 38.101-1: in UE maximum output power (6.2.1) as well as in configured maximum output power (6.2.4)

UE maximum output power:

Configured maximum output power:

Neither of these specification sections differentiates between TDD and FDD. Additionally, the signaling parameter maxUplinkDutyCycle-PC2-FR1 is specified with no differentiation for TDD and FDD in TS 38.306

Observation 1: Default duty cycle of 50% has been specified for all PC2 UEs, and applies equally for TDD and FDD

Reference measurement channel used in conformance testing are specified in Annex A. Currently, all UL RMCs for FDD use 100% duty cycle. This means that in conformance testing scenario PC2 FDD may reduce its output power to PC3 power level, and PC2 requirements are not properly verified at PC2 power level.

Observation 2: Missing PC2 FDD RMC with up to 50% leads in unintended UE behavior in conformance testing, and PC2 requirements may not be properly verified.

Clearly, PC2 FDD requirements are slightly broken in this area, and RAN4 needs to take an action to complete the specification. The simplest approach is the add a new RMC which includes an RMC with up to 50% duty cycle. Up to 50% duty cycle ensures that UE operates at PC2 power level, as power reduction is allowed only when duty cycle exceeds 50%.

This results in slightly different behavior in testing and in the field, as in field operation UE would keep transmitting but use lower power. However, for conformance testing it is purposeful to prioritize stable power level at PC2 level.

Observation 3: Adding a new RMC for PC2 FDD enables testing to be done aligned with PC2 FDD requirements.

Proposal 1: Specify a new RMC for PC2 FDD with up to 50% duty cycle.

A companion CR is provided in [1]

## 3	Conclusion

In this contribution NTN IoT UL capacity enhancements was discussed and following proposal was made:

Observation 1: Default duty cycle of 50% has been specified for all PC2 UEs, and applies equally for TDD and FDD

Observation 2: Missing PC2 FDD RMC with up to 50% leads in unintended UE behavior in conformance testing, and PC2 requirements may not be properly verified.

Observation 3: Adding a new RMC for PC2 FDD enables testing to be done aligned with PC2 FDD requirements.

Proposal 1: Specify a new RMC for PC2 FDD with up to 50% duty cycle.

## 4	References

[1]	R4-2511382, (NR_PC2_UE_FDD-Core) CR to TS 38.101-1: Addition of missing RMC, Qualcomm Incorporated, Nokia, Huawei