3GPP TSG-RAN WG4 Meeting #116	R4-2511397

# Bangalore, India, 25th – 29th August 2025

Agenda item:		7.27.3.2

Source: 		Qualcomm Inc.

Title:	 	NR NTN UL Capacity Enhancements

Document for:		Approval

## 1	Introduction

NR NTN UL capacity enhancement work is on-going in RAN1. RAN4 has had initial discussion on whether any specified scheme results in RAN4 specification impact. Now RAN4 received an LS from RAN1 stating that RAN1 assumes that UE is required to maintain phase continuity and power consistency for duration of one OCC group. In this contribution RAN4 specification impact is further discussed.

## 2	Discussion

An LS was sent from RAN1 [1], with contents reproduced below.

The LS mentions both phase continuity and power consistency. Currently RAN4 specification includes requirements for phase continuity for DMRS bundling. However, looking into the capability definitions for DMRS bundling, also power consistency is covered:

Observation 1: DMRS-bundling phase continuity requirements cover also power consistency.

In TS 38.331, the smallest duration which can be indited for FDD in MaxDurationDMRS-Bundling-r17 is 4 slots. Similarly the smallest duration possible for ntn-DMRS-BundlingNGSO-r18 is 4 slots. Therefore, the difficulty to achieve phase continuity does not increase compared to DMRS bundling.

Observation 2: The time scale for phase continuity for OCC is similar or smaller than for DMRS-bundling.

The fundamental purpose of the requirements is also the same: to ensure that transmission in consecutive slots can be processed together. For NTN use case it is also expected that low order modulations are used similar to DMRS-signal. As such, the same accuracy requirement can be applied to phase continuity and power consistency for OCC purpose.

Proposal 1: DMRS-bundling requirements values and side conditions for requirement applicability are applied for OCC, and “No UE specific and common TA updates” are added.

It can be also observed that separate capabilities have been defined for DMRS bundling for GSO and NGSO operation. It is possible that some implementations support both GSO and NGSO, but only support OCC for one of those. Therefore, separate capabilities should be also defined for supporting OCC in GSO and NGSO deployments. When it comes for capabilities to support OCC2 and OCC4, RAN1 has already agreed on those in RAN1#119:

In the most recent feature list, differentiation between satellite orbits is still left open, so it would useful to let RAN1 know that RAN4 thinks this differentiation is useful.

Observation 3: RAN1 has already agreed on separate capability on support of OCC2 and OCC4.

Proposal 2: Separate indication is needed for supporting OCC in GSO and NGSO deployments.

Given that RAN1 has already agreed on separate OCC2 and OCC4 capabilities, the simplest approach would be to just add separate GSO and NGSO support components to those, and enable the indication per band. This means that UE is able to tell for each band separately which OCC duration it supports for which deployment scenario. This also aligns the principle of the capability indication with DMRS-bundling.

Proposal 3: Include deployment scenario indication to the OCC2 and OCC4 capabilities, indication being per band.

Finally, RAN1 is asking for a reply on whether the same requirements can be applied. RAN4 should respond to RAN1 that the same requirements and side conditions can be applied as for DMRS-bundling. RAN4 should also ask RAN1 to enable UE to indicate for each band whether supported OCC length applies for GSO, NGSO or both deployment scenarios.

Proposal 4: Respond to RAN1 that the same requirements and side conditions can be applied as for DMRS-bundling, but also “No UE specific and common TA updates” will be included. Ask RAN1 to enable UE to indicate for each band whether supported OCC length applies for GSO, NGSO or both deployment scenarios.

## 3	Conclusion

In this contribution NR NTN UL capacity enhancements were discussed and following observation and proposal was made:

Observation 1: DMRS-bundling phase continuity requirements cover also power consistency.

Observation 2: The time scale for phase continuity for OCC is similar or smaller than for DMRS-bundling.

Proposal 1: DMRS-bundling requirements values and side conditions for requirement applicability are applied for OCC, and “No UE specific and common TA updates” are added.

Observation 3: RAN1 has already agreed on separate capability on support of OCC2 and OCC4.

Proposal 2: Separate indication is needed for supporting OCC in GSO and NGSO deployments.

Proposal 3: Include deployment scenario indication to the agreed OCC2 and OCC4 capabilities, indication being per band.

Proposal 4: Respond to RAN1 that the same requirements and side conditions can be applied as for DMRS-bundling, but also “No UE specific and common TA updates” will be included. Ask RAN1 to enable UE to indicate for each band whether supported OCC length applies for GSO, NGSO or both deployment scenarios.

## 4	References

[1]	R4-2505305/R1-2503071, LS on requirements for the phase continuity and power consistency for OCC with PUSCH in NR NTN Ph3, RAN1