3GPP TSG-RAN WG4 Meeting #116	R4-2511400

# Bangalore, India, 25th – 29th August 2025

Agenda item:		7.6.3.2.1

Source: 		Qualcomm Inc.

Title:	 	NR NTN HPUE Tx requirements

Document for:		Approval

## 1	Introduction

The work item for enhanced requirements and conductive test methodology for NR NTN and IoT NTN has been approved. One of the objectives is high power UE operation for NTN. In this contribution UE RF Tx requirements and feasibility of NTN HPUE are discussed.

## 2	Discussion

2.1 SAR solution

Fundamentally, SAR can be controlled in the UE by either controlling transmission time or transmission power. Only UE has knowledge of the time and power = energy budget available to meet SAR, as network is not aware of e.g. usage of other radios like WiFi or Bluetooth. The remaining energy is also not constant, as both cellular radio and other radio usage is dynamic over time. Ideally, network would need regularly informed on the available energy headroom to be able to optimize scheduling so that user experience is the least impacted.

Observation 1: Only UE has knowledge of available energy budget to meet SAR.

Observation 2: Energy budget is dynamic and therefore reporting should also be dynamic.

This target can be met by reporting the energy headroom to the network when UE start running close to no available headroom. As SAR measurement window can be very long, unnecessary signaling can be reduced e.g. if there has been a long time with no UL transmissions.

It is expected that in NTN use case most transmissions will be done with high UE output power. Therefore, it is reasonable to use maximum transmit power as a reference for remaining energy. We propose that UE reports to the network that it can transmit X seconds out of next Y seconds with full power. This has the following benefits

- Network has full information on the remaining energy, not only on power or time dimension of it

- Network can scale the time to match the used transmit power, as network also gets PHR

- Network can choose whether to use the budget more quickly or to reduce duty cycle to enable high power transmission over longer time span.

An alternative but equivalent option is to report that UE can transmit at X% duty cycle for next Y seconds.

Proposal 1: A new UE triggered report is specified to report energy headroom to the network in form of “UE can transmit at maximum transmit power for X seconds out of next Y seconds.”

A reasonable granularity is required for values X and Y. In our view a starting point would be a range from 100 ms to 5s, with 100 ms granularity. Also to share the same understanding on what are the next Y seconds, the counting of the time can be based on the time instant when the report is sent.

Proposal 2: For X and Y, consider value range from 100ms to 5s with 100ms granularity.

Proposal 3: Starting for the indicated values is the transmission time of the report.

Finally, RAN2 help is needed to realize the reporting. Therefore, an LS should be sent to RAN2 to specify this energy headroom report.

Proposal 4: Send an LS to RAN2 to specify energy headroom reporting.

2.2 Component power handling capability

In addition to maximizing the possibility for high power UE transmissions, also UE RF component feasibility needs to be considered. Specifically, RF filters and their power handling capabilities for PC2 and 2Tx PC1.5 are designed with 50% duty cycle in mind. There may be instances where UE needs to protect it’s components. To enable re-use of same filter technologies, it would be beneficial not to mandate UE to follow higher than 50% duty cycle for PC2. Unlike the TN solution, this should be optional: UEs with filters with higher power handling capability can transmit longer times.

Proposal 5: For NR NTN PC2 and 2Tx PC1.5, and 4Tx PC1, allow but not mandate UE to reduce it’s transmit power to PC3 power level when 50% Tx duty cycle in a radio frame is exceeded.

Proposal 6: For NR NTN 1Tx PC1, allow but not mandate UE to reduce it’s transmit power to PC3 power level when 25% Tx duty cycle in a radio frame is exceeded.

2.3 MPR for PC1

For edge allocations, the MPR is mainly due to SEM, and emissions are a combination of PA non-linearity and linear leakage from baseband. With PC1, it is expected that there would be approximately 2 dB increase for MPR to to meet SEM with 2 dB higher nominal power class compared to PC1.5.

The figures above do not capture the worst case, which happens with narrowest guard between edge RB and SEM. 5..8 RB allocation width at channel edge was still rather challenging and could require more power backoff than other outer allocations. To avoid increase in MPR for all outer allocations, it is beneficial to increase the RB allocation size for edge allocations for PC1

Proposal 7: L_CRB considered for edge allocations for PC1 is updated to [8] RB.

Based on the evaluations made so far, 4 Tx PC1.5 MPR table can be taken as starting point for PC1 MPR. Below the edge RB MPR is further increased by 2 dB compared to 4 Tx PC1.5 to account for the total power increase.

Proposal 8: Adopt the table below for 4Tx PC1 MPR definition

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 9.5 | ≤ 2.0 | ≤ 0.5 |
|  | QPSK | ≤ 10.0 | ≤ 2.5 | ≤ 0.5 |
|  | 16 QAM | ≤ 10.0 | ≤ 3.5 | ≤ 1.5 |
|  | 64 QAM | ≤ 10.0 | ≤ 4.0 | ≤ 3.5 |
| CP-OFDM | QPSK | ≤ 10.0 | ≤ 4.5 | ≤ 2.0 |
|  | 16 QAM | ≤ 10.0 | ≤ 4.5 | ≤ 2.5 |
|  | 64 QAM | ≤ 10.0 | ≤ 5.0 | ≤ 5.0 |

Generally, WOLA processing is a limiting factor edge MPR allocations. As the same modem and transceiver for 1Tx and 4Tx will not be fundamentally different, also WOLA performance will be similar and same edge MPR applies.

Proposal 9: Specify the same edge RB MPR for both 1Tx and 4Tx implementations

2.4 PC2 A-MPR

NS_02N PC2 A-MPR

In previous meeting RAN4 agreed to use 10 dB filter rejection for the frequency range 1559 – 1610 MHz. When this filter rejection is used, a small A-MPR is required to meet emission requirements.

Proposal 10: Adopt the tables below for NS_02N PC2 A-MPR

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | RBstart*12*SCS MHz | LCRB*12*SCS MHz | A-MPR |
| --- | --- | --- | --- | --- |
| 20MHz | 1636.5 ≤ Fc ≤ 1650.5 | ≤ 0.54 | < 1.08 | A1 |
|  |  |  | > 14.4 | A1 |

| Modulation/Waveform | A1 |
| --- | --- |
|  | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 2.5 |
| DFT-s-OFDM QPSK | ≤ 2.5 |
| DFT-s-OFDM 16 QAM | ≤ 3 |
| DFT-s-OFDM 64 QAM | ≤ 3.5 |
| CP-OFDM QPSK | ≤ 4 |
| CP-OFDM 16 QAM | ≤ 4 |
| CP-OFDM 64 QAM | ≤ 4 |

NS_24 PC2 A-MPR

A detailed A-MPR evaluation considering also transceiver wideband noise, transceiver non-linearities, phase noise and digital processing artefacts in addition to PA emissions was done to check how much A-MPR is appropriate for fulfilling NS_24 requirements. It was observed that transceiver noise contribution is relevant and results in increased A-MPR.

Generally, transceiver emissions are not as directly related to output power as PA emissions, and usually PA emissions are dominant. However, -50 dBm/MHz at a small offset is more stringent than nearly all other 3GPP emission limits. Therefore, in this case transceiver contribution plays a role.

Already for PC3, NS_24 A-MPR is very large, up to 19 dB in some case. As such, it seems unlikely that NTN ecosystem would flourish in an area where NS_24 requirements are needed. Therefore, it is not necessary to optimize A-MPR for cases for which link budget is already unfeasible due to high PC3 A-MPR.

As such PC2 A-MPR is proposed as follows

Proposal 11: Adopt the tables below for NS_02N PC2 A-MPR

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | Region A |  |  | Region B |  |  | Region C |  |  | Region D |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR | RBend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR | RBend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR | RBend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR |
| 5MHz | 1987.5 < Fc ≤ 1992.5 |  | >3.24 | A7 |  |  |  |  |  |  |  |  |  |
| 5MHz | 1992.5 < Fc ≤ 1997.5 |  | >3.24 | A4 |  | ≤ 3.24 | A8 |  |  |  |  |  |  |
| 5MHz | 1997.5 < Fc ≤ 2002.5 |  | >1.98 | A1 | >3.6 | >1.08 ≤1.98 | A2 | ≤3.6 | ≤1.98 | A3 |  |  |  |
|  |  |  |  |  |  | ≤1.08 | A6 |  |  |  |  |  |  |
| 10MHz | Fc = 1985 | >5.4 |  | A4 |  | ≥ 4.5 | A8 |  |  |  |  |  |  |
| 10MHz | 1985 ≤ Fc ≤ 1995 |  | >4.32 | A1 | ≥7.2 | >1.08 ≤4.32 | A2 | <7.2 | ≤4.32 | A3 |  |  |  |
|  |  |  |  |  |  | ≤1.08 | A6 |  |  |  |  |  |  |
| 10MHz | 1995 < Fc ≤ 2000 | ≥5.76 |  | A5 | <3.06 |  | A5 | ≥3.06 <5.76 | >1.44 | A6 | ≥3.06 <5.76 | ≤1.44 | A8 |
| 15MHz | Fc = 1987.5 |  | >6.84 | A1 | ≥10.8 | >1.08 ≤6.84 | A2 | <10.8 | ≤6.84 | A3 |  |  |  |
|  |  |  |  |  |  | ≤1.08 | A6 |  |  |  |  |  |  |
| 15MHz | 1987.5 < Fc ≤ 1997.5 | ≥8.64 |  | A5 | <4.04 |  | A5 | ≥4.04 <8.64 | >1.44 | A6 | ≥4.04 <8.64 | ≤1.44 | A8 |
| 20MHz | Fc = 1990 | ≥12.96 |  | A5 | <4.68 |  | A5 | ≥4.68 <12.96 | >2.16 | A6 | ≥ 4.68 < 12.96 | ≤2.16 | A8 |
| 20MHz | 1990 < Fc ≤ 1995 | ≥11.52 |  | A5 | <5.58 |  | A5 | ≥5.58 <11.52 | >1.44 | A6 | ≥ 5.58 < 11.52 | ≤1.44 | A8 |
| NOTE 1: The A-MPR values are listed in Table 6.2.3.15-2. NOTE 2: For any undefined region, MPR applies |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 | A6 | A7 | A8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 14 | ≤ 8 | ≤ 5.5 | ≤ 8.5 | ≤ 21 | ≤ 13 | ≤ 4.5 | ≤ 3 |
| DFT-s-OFDM QPSK | ≤ 14 | ≤ 8 | ≤ 5.5 | ≤ 8.5 | ≤ 21 | ≤ 13 | ≤ 4.5 | ≤ 3 |
| DFT-s-OFDM 16 QAM | ≤ 14 | ≤ 8 | ≤ 5.5 | ≤ 8.5 | ≤ 21 | ≤ 13 | ≤ 4.5 | ≤ 4 |
| DFT-s-OFDM 64 QAM | ≤ 14 | ≤ 8 | ≤ 6 | ≤ 8.5 | ≤ 22 | ≤ 13 | ≤ 4.5 | ≤ 4.5 |
| CP-OFDM QPSK | ≤ 15 | ≤ 9 | ≤ 6.5 | ≤ 8.5 | ≤ 22 | ≤ 15 | ≤ 5.5 | ≤ 4.5 |
| CP-OFDM 16 QAM | ≤ 15 | ≤ 9 | ≤ 6.5 | ≤ 8.5 | ≤ 22 | ≤ 15 | ≤ 5.5 | ≤ 5 |
| CP-OFDM 64 QAM | ≤ 15 | ≤ 9 | ≤ 6.5 | ≤ 8.5 | ≤ 22 | ≤ 15 | ≤ 5.5 | ≤ 5 |
| NOTE 1: The backoff applied is max(MPR, A-MPR) where MPR is defined in Table 6.2.2-1 NOTE 2: Outer and inner allocations are defined in clause 6.2.2 |  |  |  |  |  |  |  |  |

2.5 PC1 A-MPR

NS_02N PC1 A-MPR

In previous meeting RAN4 agreed to use 10 dB filter rejection for the frequency range 1559 – 1610 MHz. When this filter rejection is used, a small A-MPR is required to meet emission requirements.

Proposal 12: Adopt the tables below for NS_02N PC1 A-MPR

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | RBstart*12*SCS MHz | LCRB*12*SCS MHz | A-MPR |
| --- | --- | --- | --- | --- |
| 20MHz | 1636.5 ≤ Fc ≤ 1650.5 | ≤ 0.54 | < 1.08 | A1 |
|  |  |  | > 14.4 | A1 |
|  |  |  | > 10.8, ≤ 14.4 | A2 |
| 15 MHz | 1634 ≤ Fc ≤ 1645 |  | > 10.8 | A2 |

| Modulation/Waveform | A1 | A2 |
| --- | --- | --- |
|  | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 5 | ≤ 3 |
| DFT-s-OFDM QPSK | ≤ 5 | ≤ 3 |
| DFT-s-OFDM 16 QAM | ≤ 5 | ≤ 3.5 |
| DFT-s-OFDM 64 QAM | ≤ 5.5 | ≤ 4 |
| CP-OFDM QPSK | ≤ 6 | ≤ 4 |
| CP-OFDM 16 QAM | ≤ 6 | ≤ 4 |
| CP-OFDM 64 QAM | ≤ 6 | ≤ 4 |

NS_24 PC1 A-MPR

When output power is increased to 31 dBm with NS_24, most A-MPR values can be increased by 5 dB compared to PC2. This is because all CIM3, CIM5 related contributions are directly related to output power, and most A-MPR values are so large that emission levels are nearly linearly correlated with output power.

Furthermore, increase in output power increases the visibility of transceiver noise contributions, as those get further amplified in the RF front-end. For this reason, new region (A9) is needed for some allocations which had no A-MPR for PC2.

Proposal 13: Adopt the tables below for NS_24N PC1 A-MPR

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | Region A |  |  | Region B |  |  | Region C |  |  | Region D |  |  | Region E |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR | RBend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR | RBend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR | RBend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR | RBend*12*SCS MHz | LCRB*12*SCS MHz | A-MPR |
| 5MHz | 1982.5 ≤ Fc ≤ 1992.5 |  | >3.24 | A7 |  | ≤ 3.24 | A9 |  |  |  |  |  |  |  |  |  |
| 5MHz | 1992.5 < Fc ≤ 1997.5 |  | >3.24 | A4 |  | ≤ 3.24 | A8 |  |  |  |  |  |  |  |  |  |
| 5MHz | 1997.5 < Fc ≤ 2002.5 |  | >1.98 | A1 | >3.6 | >1.08 ≤1.98 | A2 | ≤3.6 | ≤1.98 | A3 |  |  |  |  |  |  |
|  |  |  |  |  |  | ≤1.08 | A6 |  |  |  |  |  |  |  |  |  |
| 10MHz | Fc = 1985 | >5.4 |  | A4 |  | ≤ 5.4 ≥ 4.5 | A8 |  | < 4.5 | A9 |  |  |  |  |  |  |
| 10MHz | 1985 < Fc ≤ 1995 |  | >4.32 | A1 | ≥7.2 | >1.08 ≤4.32 | A2 | <7.2 | ≤4.32 | A3 |  |  |  |  |  |  |
|  |  |  |  |  |  | ≤1.08 | A6 |  |  |  |  |  |  |  |  |  |
| 10MHz | 1995 < Fc ≤ 2000 | ≥5.76 |  | A5 | <3.06 |  | A5 | ≥3.06 <5.76 | >1.44 | A6 | ≥3.06 <5.76 | ≤1.44 | A8 |  |  |  |
| 15MHz | Fc = 1987.5 |  | >6.84 | A1 | ≥10.8 | >1.08 ≤6.84 | A2 | <10.8 | ≤6.84 | A3 |  |  |  |  |  |  |
|  |  |  |  |  |  | ≤1.08 | A6 |  |  |  |  |  |  |  |  |  |
| 15MHz | 1987.5 < Fc ≤ 1997.5 | ≥8.64 |  | A5 | <4.04 |  | A5 | ≥4.04 <8.64 | >1.44 | A6 | ≥4.04 <8.64 | ≤1.44 | A8 |  |  |  |
| 20MHz | Fc = 1990 | ≥12.96 |  | A5 | <4.68 |  | A5 | ≥4.68 <12.96 | >2.16 | A6 | ≥ 4.68 < 12.96 | ≤2.16 | A8 |  |  |  |
| 20MHz | 1990 < Fc ≤ 1995 | ≥11.52 |  | A5 | <5.58 |  | A5 | ≥5.58 <11.52 | >1.44 | A6 | ≥ 5.58 < 11.52 | ≤1.44 | A8 |  |  |  |
| NOTE 1: The A-MPR values are listed in Table 6.2.3.15-2. NOTE 2: For any undefined region, MPR applies |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Modulation/Waveform | A1 | A2 | A3 | A4 | A5 | A6 | A7 | A8 | A9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 19 | ≤ 13 | ≤ 10.5 | ≤ 8.5 | ≤ 26 | ≤ 18 | ≤ 7 | ≤ 8 | ≤ 4.5 |
| DFT-s-OFDM QPSK | ≤ 19 | ≤ 13 | ≤ 10.5 | ≤ 8.5 | ≤ 26 | ≤ 18 | ≤ 7 | ≤ 8 | ≤ 4.5 |
| DFT-s-OFDM 16 QAM | ≤ 19 | ≤ 13 | ≤ 10.5 | ≤ 8.5 | ≤ 26 | ≤ 18 | ≤ 7 | ≤ 9 | ≤ 4.5 |
| DFT-s-OFDM 64 QAM | ≤ 19 | ≤ 13 | ≤ 11 | ≤ 8.5 | ≤ 27 | ≤ 18 | ≤ 7 | ≤ 9.5 | ≤ 5 |
| CP-OFDM QPSK | ≤ 20 | ≤ 14 | ≤ 11 | ≤ 8.5 | ≤ 27 | ≤ 20 | ≤ 8 | ≤ 9.5 | ≤ 5.5 |
| CP-OFDM 16 QAM | ≤ 20 | ≤ 14 | ≤ 11 | ≤ 8.5 | ≤ 27 | ≤ 20 | ≤ 8 | ≤ 10 | ≤ 5.5 |
| CP-OFDM 64 QAM | ≤ 20 | ≤ 14 | ≤ 11 | ≤ 8.5 | ≤ 27 | ≤ 20 | ≤ 8 | ≤ 10 | ≤ 5.5 |
| NOTE 1: The backoff applied is max(MPR, A-MPR) where MPR is defined in Table 6.2.2-1 NOTE 2: Outer and inner allocations are defined in clause 6.2.2 |  |  |  |  |  |  |  |  |  |

## 3	Conclusion

In this contribution UE RF Tx requirements for NR NTN were discussed. Following observations and proposals were made.

2.1 SAR solution

Observation 1: Only UE has knowledge of available energy budget to meet SAR.

Observation 2: Energy budget is dynamic and therefore reporting should also be dynamic.

Proposal 1: A new UE triggered report is specified to report energy headroom to the network in form of “UE can transmit at maximum transmit power for X seconds out of next Y seconds.”

Proposal 2: For X and Y, consider value range from 100ms to 5s with 100ms granularity.

Proposal 3: Starting for the indicated values is the transmission time of the report.

Proposal 4: Send an LS to RAN2 to specify energy headroom reporting.

2.2 Component power hand capability

Proposal 5: For NR NTN PC2 and 2Tx PC1.5, and 4Tx PC1, allow but not mandate UE to reduce it’s transmit power to PC3 power level when 50% Tx duty cycle in a radio frame is exceeded.

Proposal 6: For NR NTN 1Tx PC1, allow but not mandate UE to reduce it’s transmit power to PC3 power level when 25% Tx duty cycle in a radio frame is exceeded.

2.3 ACLR requirements and MPR for PC1

Proposal 7: L_CRB considered for edge allocations for PC1 is updated to [8] RB.

Proposal 8: Adopt the table below for 4Tx PC1 MPR definition

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 9.5 | ≤ 2.0 | ≤ 0.5 |
|  | QPSK | ≤ 10.0 | ≤ 2.5 | ≤ 0.5 |
|  | 16 QAM | ≤ 10.0 | ≤ 3.5 | ≤ 1.5 |
|  | 64 QAM | ≤ 10.0 | ≤ 4.0 | ≤ 3.5 |
| CP-OFDM | QPSK | ≤ 10.0 | ≤ 4.5 | ≤ 2.0 |
|  | 16 QAM | ≤ 10.0 | ≤ 4.5 | ≤ 2.5 |
|  | 64 QAM | ≤ 10.0 | ≤ 5.0 | ≤ 5.0 |

Proposal 9: Specify the same edge RB MPR for both 1Tx and 4Tx implementations

2.4 PC2 A-MPR

NS_24N and NS_02N PC2 A-MPR was proposed in clause 2.4

2.5 PC1 A-MPR

NS_24N and NS_02N PC1 A-MPR was proposed in clause 2.4

## 4	References