3GPP TSG-RAN WG4 Meeting #115	R4-2511411

Bengaluru, India, August 25th – August 29th, 2025

Source:	Nokia

Title:	Discussion on Fast SCell activation for UE supporting Rel-18 EMR performance part

Agenda item:

Document for:	Discussion

Introduction

In the last RAN4 meeting, the following issues were left open.

Issue 2-2-1: Test case list Agreement: - Scenario: - Normal SCell activation for FR1 and FR2 - Direct SCell activation at SCell addition for FR1 - PUCCH SCell activation for FR1 and FR2 - FFS: - Direct SCell activation at SCell addition for FR2 - UE capabilities and network configurations: - Configuration 1: UE supports measValidationReportEMR and measIdleValidityDuration-r18 is configured - Configuration 2: UE supports measValidationReportReselectionMeasurements and measReselectionValidityDuration-r18 is configured - FFS: - Configuration 3: UE supports idleInactiveNR-MeasReport-r16, and neither measIdleValidityDuration-r18 nor measReselectionValidityDuration-r18 is configured and measIdleDuration-r16 hasn’t expired at the moment of initiation of RRC state transition to Connected mode

Discussion

Supported scenarios

SCells are used usually for capacity purpose. Therefore, FR2 support is important and adding FR2 also for direct SCell activation should be considered.

Support also Direct SCell activation at SCell addition for FR2

Configuration 3 was included in the core requirements and therefore we should not exclude it from the test cases.

Support configuration 3 as one of the scenarios.

Test cases and configuration combinations

Generally, there are two frameworks that requires combining, one being EMR framework and the other being SCell activation framework. For the EMR framework the following test cases are supported by legacy:

A.6.6.9.1 SA Idle mode CA/DC measurement for FR1

A.6.6.9.2 Idle mode fast CA/DC eEMR measurement for FR1 without valid reporting

A.6.6.9.3 Idle mode fast CA/DC cell reselection measurement for FR1 without valid reporting

A.6.6.9.4 Idle mode fast CA/DC cell reselection measurement for FR1 with valid reporting

A.7.6.23.1 Test case for Idle mode fast CA/DC eEMR measurement for FR2 without valid reporting

A.7.6.23.2 Test case for Idle mode fast CA/DC cell reselection measurement for FR2 without valid reporting

A.7.6.23.3 Test case for Idle mode fast CA/DC cell reselection measurement for FR2 with valid reporting

There are many Scell test cases but RAN4 can down select from them to include at least one scenario for FR1 and one for FR2.

A.5.5.3.1 SCell Activation and deactivation of SCell in FR2 intra-band

A.5.5.3.2 SCell Activation and deactivation of known SCell in FR1 for 160 ms SCell measurement cycle

A.5.5.3.5 SCell Activation and deactivation of SCell in FR2

A.5.5.3.7 Direct SCell activation at SCell addition of known SCell in FR2

A.5.5.3.8 Fast SCell Activation of SCell in FR2 intra-band

A.5.5.3.9 PUCCH SCell Activation and deactivation of known SCell in FR2

A.5.5.3.10 PUCCH SCell Activation and deactivation of unknown SCell in FR2

A.6.5.3.3 SCell Activation and deactivation of unknown SCell in FR1 in non-DRX

A.6.5.3.4 Direct SCell activation at SCell addition of known SCell in FR1

A.6.5.3.6 PUCCH SCell Activation and deactivation of known SCell in FR1

A.6.5.3.7 SCell Activation and deactivation of unknown SCell in FR1 in non-DRX

This means that there are quite many combinations of test cases, RAN4 can discuss which ones to select.

Discuss which combinations of EMR configurations are tested with each SCell scenario.

About measurement reporting in test cases

Generally, the EMR principle in the test cases can be followed.

The common part for the EMR test cases is that they can start all from RRCRelease message.

- T331 is configured Idle mode fast CA/DC eEMR measurement

- T331 is not configured for Idle mode fast CA/DC cell reselection measurement

The following figure illustrates the EMR reporting, which is quite important for the SCell activation procedure.

In EMR and eEMR the reporting is either UE information request / response in connected mode, or it is RRCReconfiguration complete. Currently the rel-18 test cases do not seem to support inactive-mode reporting but a test case can be added to support this scenario.

Support UE reporting in RRCReconfigurationComplete message in INACTIVE test case(s)

Support UE reporting in UE information request / response in IDLE-mode tests cases.

Conclusion

The following Observations and Proposals were made:

Proposal 1: Support also Direct SCell activation at SCell addition for FR2

Proposal 2: Support configuration 3 as one of the scenarios.

Proposal 3: Discuss which combinations of EMR configurations are tested with each SCell scenario.

Proposal 4: Support UE reporting in RRCReconfigurationComplete message in INACTIVE test case(s)

Proposal 5: Support UE reporting in UE information request / response in IDLE-mode tests cases.