# 3GPP TSG-RAN WG4 Meeting #115		        R4-2511417

# Bengaluru, India, 25th – 29th August, 2025

Agenda Item:	6.5.3

Source: 	Huawei, HiSilicon

Title:	On PC2 HPUE requirements for NS_48 and NS_49

Document for:	Approval

# 1	Introduction

As discussed in previous meetings [1], the PC2 A-MPR requirements for NS_48 and NS_49 needs some clean-up, including merging the 1Tx and 2Tx A-MPR as well as correcting some irregular A-MPR values.

# 2	Discussion

By comparing the PC2 A-MPR with the corresponding PC3 A-MPR, it’s found that some A-MPR values are more than 3dB higher than those for PC3. The values are highlighted in red below.

Table 6.2.3.26-3: A-MPR regions for NS_48 (Power Class 2)

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz |  |
| 10 MHz | 1925≤ FC ≤ 1975 | ≥0 | ≥8.1 | A6 |
|  |  | <1.8 | ≥0 | A6 |
| 15 MHz | 1927.5≤ FC ≤ 1972.5 | ≥0 | ≥9 | A6 |
|  |  | ≥0 | ≥max (0,12*SCS* RBend - 2.88) <9 | A6 |
| 20 MHz | 1930≤ FC ≤ 1970 | ≥0 | ≥9.72 | A4 |
|  |  | ≥0 | ≥max (0,12*SCS* RBend - 3.6) <9.72 | A6 |
| 25 MHz | 1932.5≤ FC ≤ 1967.5 | ≥0 | ≥9.72 | A4 |
|  |  | ≥18.72 | <1.08 | A3 |
|  |  | ≥0 | ≥max (0, 12*SCS*RBend - 1.08) < 9.72 | A4 (new region) |
| 30 MHz | 1935 ≤ FC ≤ 1965 | ≥0 | ≥12.96 (PC3: 13.5) | A4 |
|  |  | ≥21.6 | <1.08 | A5 |
|  |  | ≥0 | ≥max (0, 12*SCS*RBend - 3.6) < 12.96 | A4 (new region) |
| 40 MHz | 1940 ≤ FC ≤ 1960 | ≥0, <2.88 | ≥0 | A1 |
|  |  | ≥2.88, <17.1 | ≥max (0, 12*SCS*RBend - 4.68) | A3 |
|  |  | ≥17.1, <27.36 | ≥12.96 | A4 |
|  |  | ≥27.36, <34.56 | ≥12.96 (PC3: 13.5) | A2 |
|  |  | ≥27.36, <34.56 | <1.08 | A3 |
|  |  | ≥34.56 | ≥0 | A1 |
| 45 MHz | 1942.5 ≤ FC ≤ 1957.5 | ≥0, <5.22 (PC3: 4.86) | >0 | A1 |
|  |  | ≥5.22, <19 | ≥max (0, 12*SCS*RBend - 5.4) | A4 |
|  |  | ≥19, <37.44 | ≥14.04 | A2 |
|  |  | ≥30.96, <37.44 | <1.08 | A5 |
|  |  | ≥37.44 | >0 | A1 |
| 50 MHz | 1945 ≤ FC ≤ 1955 | ≥0, <7.2 (PC3: 6.12) | >0 | A1 |
|  |  | ≥7.2, <20.7 | ≥max (0, 12*SCS*RBend – 5.4) | A4 |
|  |  | ≥20.7, <41.04 | ≥15.12 (PC3: 17.1) | A2 |
|  |  | ≥33.84, <41.04 | <1.08 | A5 |
|  |  | ≥41.04 | >0 | A1 |

Table 6.2.3.26-4: A-MPR for NS_48 (Power Class 2)

| Modulation/Waveform |  | A1 | A2 | A3 | A4 | A5 | A6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | ≤[12] | ≤8.0 | ≤4.0 | ≤4.5 | ≤6.0 | ≤3.0 |
|  | QPSK | ≤[12] | ≤8.0 | ≤4.0 | ≤4.5 | ≤6.0 | ≤3.0 |
|  | 16 QAM | ≤[12] | ≤8.0 | ≤4.0 | ≤4.5 | ≤6.0 | ≤3.0 |
|  | 64 QAM | ≤[12] | ≤8.0 | ≤4.0 | ≤4.5 | ≤6.0 | ≤3.0 |
|  | 256 QAM | ≤[12] | ≤8.0 | ≤4.0 | ≤4.5 | ≤6.0 | ≤3.0 |
| CP-OFDM | QPSK | ≤[12.5] | ≤9.0 | ≤5.5 | ≤6.0 | ≤6.0 | ≤4.5 |
|  | 16 QAM | ≤[12.5] | ≤9.0 | ≤5.5 | ≤6.0 | ≤6.0 | ≤4.5 |
|  | 64 QAM | ≤[12.5] | ≤9.0 | ≤5.5 | ≤6.0 | ≤6.0 | ≤4.5 |
|  | 256 QAM | ≤[12.5] | ≤9.0 | ≤5.5 | ≤6.0 | ≤6.0 | ≤4.5 |

Table 6.2.3.27-3: A-MPR regions for NS_49 (Power Class 2)

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz |  |
| 10 MHz | 1925≤ FC ≤ 1975 | ≥0 | ≥8.1 | A3 |
|  |  | <1.8 | ≥0 | A3 |
| 15 MHz | 1927.5≤ FC ≤ 1972.5 | ≥0 | ≥9 | A3 |
|  |  | ≥0 | ≥max (0,12*SCS* RBend - 2.88) <9 | A3 |
| 20 MHz | 1930≤ FC ≤ 1970 | ≥0 | ≥9.72 | A4 |
|  |  | ≥0 | ≥max (0,12*SCS* RBend - 3.6) <9.72 | A3 |
| 25 MHz | 1932.5≤ FC ≤ 1967.5 | ≥0 | ≥7.92 | A4 |
|  |  | ≥18.72 | <1.08 | A3 |
|  |  | ≤6.48 | <3.6 | A3 |
| 30 MHz | 1935 ≤ FC ≤ 1965 | ≥0, <3.6 | ≥0 | A1 |
|  |  | ≥3.6, <7.92 | ≥0 | A5 |
|  |  | ≥7.92, <14.4 | ≥max (0,12*SCS* RBend - 4.32) | A3 |
|  |  | ≥14.4, <21.6 | ≥10.44 | A4 |
|  |  | ≥21.6 | ≥10.44 | A2 |
|  |  | ≥21.6 | <1.8 | A1 |
| 40 MHz | 1940 ≤ FC ≤ 1960 | ≥0, <9 | ≥0 | A1 |
|  |  | ≥9, <11.52 | <3.06 | A5 |
|  |  | ≥9, <18 | ≥max (3.06, 12*SCS*RBend – 6.48) | A4 |
|  |  | ≥18, <34.56 | ≥11.16, <27 | A2 |
|  |  | ≥27.36, <34.56 | <1.08 | A5 |
|  |  | <34.56 | ≥27 (PC3: 28.8) | A1 |
|  |  | ≥34.56 | ≥0 | A1 |
|  |  | ≥7.92, <12.42 | < min [1.08, max(0,12*SCS* RBend-7.92)] | A5 |
|  |  | ≥30.76, <36.72 | <1.08 | A5 |
| 45 MHz | 1942.5 ≤ FC ≤ 1957.5 | <36.72 | ≥12.24, <max (0, 12*SCS*RBend – 7.92) | A2 |
|  |  | <36.72 | ≥max (0, 12*SCS*RBend – 7.92) | A1 |
|  |  | ≥36.72 | >0 | A1 |
| 50 MHz | 1945 ≤ FC ≤ 1955 | ≥10.08, <14.4 | < min [1.08, max(0,12*SCS* RBend-10.08)] | A5 |
|  |  | ≥36, <39.6 | <1.08 | A5 |
|  |  | <39.6 | ≥13.68, <max (0, 12*SCS*RBend – 10.08) | A2 |
|  |  | <39.6 | ≥max (0, 12*SCS*RBend – 10.08) | A1 |
|  |  | ≥39.6 | >0 | A1 |

Table 6.2.3.27-4: A-MPR for NS_49 (Power Class 2)

| Modulation/Waveform |  | A1 | A2 | A3 | A4 | A5 |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | ≤[12.5] | ≤7 | ≤4 | ≤5.5 | ≤6 |
|  | QPSK | ≤[12.5] | ≤7 | ≤4 | ≤5.5 | ≤6 |
|  | 16 QAM | ≤[12.5] | ≤7 | ≤4 | ≤5.5 | ≤6 |
|  | 64 QAM | ≤[12.5] | ≤7 | ≤4 | ≤5.5 | ≤6 |
|  | 256 QAM | ≤[12.5] | ≤7 | ≤4 | ≤5.5 | ≤6 |
| CP-OFDM | QPSK | ≤[13.5] | ≤8 | ≤5.5 | ≤7.5 | ≤6 |
|  | 16 QAM | ≤[13.5] | ≤8 | ≤5.5 | ≤7.5 | ≤6 |
|  | 64 QAM | ≤[13.5] | ≤8 | ≤5.5 | ≤7.5 | ≤6 |
|  | 256 QAM | ≤[13.5] | ≤8 | ≤5.5 | ≤7.5 | ≤6 |

- Consider to correct the excessively large PC2 A-MPR values in the existing NS_48 and NS_49 A-MPR tables. One option is to replace them with the PC3 A-MPR values+3dB.

As pointed out in previous meetings, it’s feasible to merge the 1Tx and 2Tx PC2 A-MPR requirements for NS_48 and NS_49 by adding 0.5dB relaxation to the 1Tx A-MPR table.

Table 6.2.3.26-4: A-MPR for NS_48 (Power Class 2)

| Modulation/Waveform |  | A1 | A2 | A3 | A4 | A5 | A6 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | ≤12.5 | ≤8.5 | ≤4.5 | ≤5.0 | ≤6.5 | ≤3.5 |
|  | QPSK | ≤12.5 | ≤8.5 | ≤4.5 | ≤5.0 | ≤6.5 | ≤3.5 |
|  | 16 QAM | ≤12.5 | ≤8.5 | ≤4.5 | ≤5.0 | ≤6.5 | ≤3.5 |
|  | 64 QAM | ≤12.5 | ≤8.5 | ≤4.5 | ≤5.0 | ≤6.5 | ≤3.5 |
|  | 256 QAM | ≤12.5 | ≤8.5 | ≤4.5 | ≤5.0 | ≤6.5 | ≤3.5 |
| CP-OFDM | QPSK | ≤13.0 | ≤9.5 | ≤6.0 | ≤6.5 | ≤6.5 | ≤5 |
|  | 16 QAM | ≤13.0 | ≤9.5 | ≤6.0 | ≤6.5 | ≤6.5 | ≤5 |
|  | 64 QAM | ≤13.0 | ≤9.5 | ≤6.0 | ≤6.5 | ≤6.5 | ≤5 |
|  | 256 QAM | ≤13.0 | ≤9.5 | ≤6.0 | ≤6.5 | ≤6.5 | ≤5 |

Table 6.2.3.27-4: A-MPR for NS_49 (Power Class 2)

| Modulation/Waveform |  | A1 | A2 | A3 | A4 | A5 |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | ≤13.0 | ≤7.5 | ≤4.5 | ≤6.0 | ≤6.5 |
|  | QPSK | ≤13.0 | ≤7.5 | ≤4.5 | ≤6.0 | ≤6.5 |
|  | 16 QAM | ≤13.0 | ≤7.5 | ≤4.5 | ≤6.0 | ≤6.5 |
|  | 64 QAM | ≤13.0 | ≤7.5 | ≤4.5 | ≤6.0 | ≤6.5 |
|  | 256 QAM | ≤13.0 | ≤7.5 | ≤4.5 | ≤6.0 | ≤6.5 |
| CP-OFDM | QPSK | ≤14.0 | ≤8.5 | ≤6.0 | ≤7.5 | ≤6.5 |
|  | 16 QAM | ≤14.0 | ≤8.5 | ≤6.0 | ≤7.5 | ≤6.5 |
|  | 64 QAM | ≤14.0 | ≤8.5 | ≤6.0 | ≤7.5 | ≤6.5 |
|  | 256 QAM | ≤14.0 | ≤8.5 | ≤6.0 | ≤7.5 | ≤6.5 |

- Consider to merge the 1Tx and 2Tx PC2 A-MPR requirements for NS_48 and NS_49 by adding 0.5dB relaxation to the 1Tx A-MPR table.

# 3	Conclusion

Regarding the PC2 requirements for band n1, the following proposals are made:

Proposal 1:Consider to correct the excessively large PC2 A-MPR values in the existing NS_48 and NS_49 A-MPR tables. One option is to replace them with the PC3 A-MPR values+3dB.

Proposal 2:Consider to merge the 1Tx and 2Tx PC2 A-MPR requirements for NS_48 and NS_49 by adding 0.5dB relaxation to the 1Tx A-MPR table.

# References

- R4-2507909 WF on [115][108] HPUE_NR_bands, CMCC, RAN4#115

- R4-2507719 On PC2 HPUE requirements, Huawei, HiSilicon, RAN4#115

- R4-2501984 NS_05 NS_48 NS_49 PC2 A-MPR clean-up, Skyworks, RAN4#114