# 3GPP TSG-RAN WG4 Meeting # 116						R4-2511426

# Bangalore, India,  August 25 - 29, 2025

Source:		Ericsson

Title:			TP to TR38.774: BS RF requirement overview for LP-WUS

Agenda item:		7.24.4

Document for:		Approval

# Introduction

In this paper, we provide our view on BS RF requirement impact for LP-WUS.

# Discussion

In WF[1], the EVM requirement discussion is shown below:

Issue 1-3: Whether to define EVM requirements for LP-WUS

- Options

  - Option 1: No need to define EVM requirements for LP-WUS. (Huawei)

    - Because simulation results show that the impact on missed detection error (MDR) is minor for EVM up to 30%, as such the EVM requirements do not seem relevant for LP-WUS based on RAN1 assumptions for LP-WUS operation (in low SNR) and design. (Nokia)

    - Because frequency-domain EVM on LP-WUS “constellations” has minor impact on the receiving MDR. (ZTE)

  - Option 2: EVM requirements for LP-WUS is needed. (CMCC)

    - Option 2-1 (CATT):

      - RAN4 to specify EVM requirement only for “ON” chips of OOK symbols.

      - RAN4 to specify different EVM requirements for the cases with or without overlaid OFDM sequence.

      - RAN4 to specify EVM requirements for ON chips of OOK symbols without overlaid OFDM sequence as 17.5%.

      - RAN4 can wait until RAN1 finalizes the overlaid OFDM sequence design before deciding how to specify the EVM requirement for ON chips of OOK symbols with overlaid OFDM sequence.

    - Option 2-2 (Ericsson):

      - Define the EVM levels of each NR carrier for OOK4 modulation scheme on LP-WUS as 8%.

      - Wait further RAN1 progress on PAPR discussion.

- WF

  - Further check in next meeting whether EVM for LP-WUS needs to be specified.

According the current WID objective, the below objective still in line with the spirit of the study phase and keep the RF impact as min as possible.

- Specify the necessary RAN4 core requirement(s) to support the feature (RAN4).

  - Study and if necessary specify or support by declaration, the corresponding BS requirements, e.g., dynamic range for LP-WUS/LP-SS.

    - Current NR BS requirements is baseline

WID objective mandates the BS new requirement subject to the declaration and use the legacy NR requirement as baseline.

Though there is no other requirement mentioned in WID than the dynamic range for LP-WUS, RAN4 has done investigation on the LP-WUS signal quality investigation the simulation by some companies. The signal is overlaid signal OOK + OFDM symbols and most companies view are the signal quality is not demanding and EVM requirement for such signal could be quite relaxed.

Table 1: The WUR performance degradation with different EVM of LP-WUS

| Companies | LP-WUS EVM | OOK WUR degradation (dB) | OFDM WUR degradation (dB) |
| --- | --- | --- | --- |
| Ericsson | 12,5% (16QAM) | 0.6 | 0.6 |
|  | 8% (64QAM) | Minor (< 0.2) | Minor (< 0.2) |
| Nokia [5] | 30% | Minor (< 0.2) | Minor (< 0.2) |
| ZTE[6] | 20% | Minor (< 0.2) | Minor (< 0.2) |

The simulation results from different companies is summarized in above table and it can be observed a relaxed EVM is good enough for LP-WUS signal. We propose to use the BPSK EVM requirement for LP-WUS as starting point.

17.5% EVM requirement for LP-WUS.

The 64QAM modulation is mandatory tested in 38.141-1 and therefore, the LP-WUS signal quality test can be dependent on the PDSCH 64QAM and there is no need to introduce additional test. The suggested wording in clause 6.5.2.2 as below:

The OOK4 signal quality test can be dependent on the 64QAM EVM test.

#### 6.5.2.2	Minimum Requirement for BS type 1-C and BS type 1-H

For BS type 1-C and 1-H, the EVM levels of each NR carrier for OOK4 modulation scheme on LP-WUS is specified in table 6.5.2.2-2.

Table 6.5.2.2-2: EVM requirements for BS type 1-C and BS type 1-H carrier

| Modulation scheme for LP-WUS | Required EVM |
| --- | --- |
| OOK-4 | [17.5] % |

Another issue is the restriction on the LP-WUS operation.

Issue 1-4: Restriction on the applicability of LP-WUS operation

- WF

  - Option 1:

    - It is proposed to restrict in-band LP-WUS operation to cases where the NR signal employs a maximum modulation order of 64QAM. This limitation should be explicitly specified to ensure robust system performance.

  - Option 2

    - No restriction of LP-WUS operation, this is BS implementation issue.

The issue originated from some company claiming the PAPR increase for combined WUS and NR signal. As the balanced LP-SS is agreed in RAN1, we donot see the PAPR issue for NR+LP-WUS transmission.

Agreement

For LP-SS with M=4, supports set1 (balanced).

No restriction on the LP-WUS operation.

TP starts here:

### 7.2.4	Transmitted signal quality

It is agreed that overlaid OFDM sequence is designed for LP-WUS and the sequence is ZC sequence. This applies in the time domain before the DFT transform for both OOK-1 and OOK-4. RAN1 also agrees on the processing for the OOK signal, and this is illustrated in Figure 7.2.4-1. After the DFT transform, the OOK-4 frequency domain pattern is illustrated in Figure 7.2.4-2.

Figure 7.2.4-1: OOK-4 signal processing and subcarrier mapping before IFFT

Figure 7.2.4-2: ZC sequence in frequency domain after DFT processing before IFFT

The simulation results on the WUR performance degradation due to the EVM impairment from different companies are listed in Table 7.2.4-1.

Table 7.2.4-1: The WUR performance degradation with different EVM of LP-WUS

| Companies |  | LP-WUS EVM |  | OOK WUR degradation (dB) |  | OFDM WUR degradation (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| Ericsson | 12,5% (16QAM) | 0.6 | 0.6 |
|  | 8% (64QAM) | Minor (< 0.2) | Minor (< 0.2) |
| Nokia |  | 30% |  | Minor (< 0.2) |  | Minor (< 0.2) |
| ZTE | 20% | Minor (< 0.2) | Minor (< 0.2) |

# Conclusions

In this contribution, we present our view on the BS RF requirement with below observations and proposals:

Observation 1 WID objective mandates the BS new requirement subject to the declaration and use the legacy NR requirement as baseline.

Proposal-1:17.5% EVM requirement for LP-WUS.

Proposal-2:The OOK4 signal quality test can be dependent on the 64QAM EVM test.

Proposal-3: No restriction on the LP-WUS operation.

# References

- R4-2508668, WF on NR LP-WUS BS RF, Huawei

- 38.141-1, Base Station (BS) conformance testing Part 1: Conducted conformance testing

- 38.104

- R4-2502092, TP to TR38.774: BS RF requirement overview for LP-WUS, Ericsson

- R4-2503168, BS RF requirements for low-power wake-up signal for NR, Nokia

- R4-2503636, Discussion on BS RF requirements for LP-WUS/WUR, ZTE