# 3GPP TSG-RAN WG4 Meeting # 116						R4-2511427

# Bangalore, India,  August 25 - 29, 2025

Source:		Ericsson

Title:			On blocking performance of WUR

Agenda item:		7.24.2

Document for:		Approval

# Introduction

In this paper, we present our overview on blocking performance of WUR.

# Discussion

It is agreed in previous RAN4 meeting that jammer power level should be the same for both WUR and Main receiver. This is because the WUR and Main receiver will experience the same radio environment. However, the wanted signal relaxation is not decided yet and the blocking performance of a WUR compared to main receiver is not clear.

The WUR blocking performance compared to main receiver is not clear.

In study item phase, the simulation campion is done in ACS case, and the guard RB is used to relax the ACS requirement is a common understanding to accommodate different filter order design. It is agreed in the TR 38.869 and quoted as below:

“In general, NR ACS requirement is derived based on co-existence study to meet max 5% throughput loss metric. While for LP-WUS, the purpose in the study of ACS is not for justification of co-existence between contained LP-WUS and adjacent NR carrier, but to evaluate the required guard RBs in-between the different signals. Given the WUS waveform would be different from NR signal, the traditional throughput loss metric can not be reused. Therefore, RAN4 agrees to focus on the evaluation of required guard RB rather than ACS requirements with co-existence simulation.”

RAN4 has agreed in study phase not to do the coexisting study and the only evaluation of guard RB to accommodate the same ACS requirement.

The same ACS with main receiver is evaluated for WUR with required guard RB.

In the work item phase, there are companies to focus on the phase noise and limited dynamic range impact from design and request therefore to relax the ACS and potential other blocking requirement. In the last meeting, the zero guard RB is assumed in WF and consequently, the WUR will be specified with different blocking performance in specification compared to the main receiver.

With 0 guard RB, the ACS and other blocking requirement relaxation will create inferior selectivity /blocking performance for WUR compared to main receiver.

When WUR is in operation and main receiver is put in a sleep mode to save power, it is the WUR to ensure the LP-WUS signal to be received in the presence of the jammer in the same radio environment of the main receiver. To design a main receiver with super good blocking performance but putting it into sleep mode while at the same time to design a simple WUR with inferior blocking and putting it into operation, this does not make sense.  Above all, the paging success rate is a KPI (Key Performance Indicator) at network and this KPI measures the success rate of network attempts to locate and notify a UE when it receives an incoming call or other communication request from network. With an inferior blocking performance for a WUR, this can lead to miss detection of the LP-WUS signal and in turn miss the paging message detection in the presence of a jammer. As such, lowering the KPI of paging success rate.

Inferior WUR lead to the lower KPI of the paging success rate in network.

Figure 1: LR ACS matching to MR ACS 33 dB (legacy NR ACS requirement) with different guard RB size for different filter order

As the WUR is configured within the system bandwidth, a guard band can be configured at network to make sure the WUR can achieve enough filter attenuation. Figure 1 shows the simple concept of this. With guard RB size of 31 (for 15kHz SCS), the filter order of 3 LPF can achieve 33 dB attenuation which is the same with ACS requirement of a main receiver. Using the guard RB, it can also make sure the phase noise impact can be less critical as the noise contribution due to reciprocal mixing is more at the frequency of the near edge of the jammer signal than other frequency far away from the jammer.  For illustration of the configuration of guard RB, the concept Figure 7.1.2.1-1  in TR 38.869 is quoted below:

(case 1: LP-WUS within large channel)

From the above discussion and network performance perspective, there shall be no compromise on the KPI of the paging success rate. Therefore, the same blocking performance of a WUR should be specified in TS 38.101-1/2. The guard RB should be allowed to achieve this. It can be up to the network to configure the LO frequency position to guarantee this though according to the specified guard RB in 38.101-1/2.  Without this guard RB information, there is no way the network can know how to guarantee the same blocking performance of a WUR and MR and KPI could be deteriorated.

RAN4 strive to specify the same ACS/IBB/OBB blocking performance with main receiver, with necessity of introducing the guard RB.

Last meeting whether to report the type 1 /2 WUR to network is not decided.  Our understanding is that if type 1 WUR corresponding to OOK WUR and type 2 WUR corresponding to OFDM WUR. This will align with RAN1/RAN2 understanding. Such should be covered by RAN1 reported capabilities. If type 1 WUR would support both OFDM and/or OOK. The reported capability of type ½ should be further discussed.

As some companies want to further relax the blocking performance on type 1 WUR compared to type 2 WUR, our opinion is that this needs a guard RB to protect the type 1 WUR so the blocking performance of a type 1 should be as good as the a type 2 WUR.  If this would be the case, the additional guard RB should be specified in the test case and there is a need to report to network on type 1 or 2 WUR.

Further discussion is needed on the needed of guard RB on type 1 WUR and reporting capability may be needed if this would be the case.

# Conclusions

In this contribution, we present our overview on blocking performance of WUR with below proposal:

Observation 1 The WUR blocking performance compared to main receiver is not clear.

Observation 2 The same ACS with main receiver is evaluated for WUR with required guard RB.

Observation 3 With 0 guard RB, the ACS and other blocking requirement relaxation will create inferior selectivity /blocking performance for WUR compared to main receiver.

Observation 4 Inferior WUR lead to the lower KPI of the paging success rate in network.

Proposal-1: RAN4 strive to specify the same ACS/IBB/OBB blocking performance with main receiver, with necessity of introducing the guard RB.

Proposal-2: Further discussion is needed on the needed of guard RB on type 1 WUR and reporting capability may be needed if this would be the case.

# Reference

[1] R4-2507936, WF for [115][136] NR_LPWUS_UERF, Vivo