# 3GPP TSG-RAN WG4 Meeting # 116						R4-2511429

# Bangalore, India, August 25 - 29, 2025

Source:		Ericsson

Title:			TP for TR 38.774_WUR RF requirement of ACS

Agenda item:		7.24.3.2

Document for:		Approval

# Introduction

In this paper, we present our view on the WUR RF requirement of REFSENS, ASC, ASCS.

# Discussion

### ACS

Issue 2-4-1: Baseline ACS requirements (same interference) and required guard RB for SET-1 (worse NF) LP-WUR

- Proposals

  - Proposal 1: check the following input from companies and converge the range for set-1 LR.  (Moderator)

|  | vivo | CMCC | ZTE | Huawei | Nokia | E/// | Sony | QC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ACS (dB) | 28 | 22-24 | 24 | 11 (Note1) | 22 | Note 2 | Higher wanted signal (Note3) | Higher wanted signal Note4 |
| Guard RB (as test parameter, not requirements) | 1RB | 1RB | 1RB | Less RB | 2RBs |  |  |  |

Note 1: For LP-WUR type aimed at moderate power consumption, the guard RBs could be derived with better phase noise profile and the interferer level could similar to that of MR.

Note 2: The same set of the inteference signal and wanted signal defined for both two set of the LP-WUS requirments. Further discuss the number of guard RB for two set of requirment.

Note 3: A higher wanted signal can be specified for OOK type LP-WUR ACS requirement to balance the power consumption and interference mitigation capability.

Note 4: LPWUS ACS is specified as (assuming higher wanted signal is higher than refsens by 14 dB). DgMR is the diversity gain used for legacy PDSCH REFSENS derivation
ACSLPWUS=  NFlegacy-NFLPWUS+10logCBWBWLPWUS+ACSlegacy-DgMR

  - Proposal 2: Whether consider a higher the wanted signal for set-1 ACS requirements, e.g., LP REFSENS+ >14dB.  (Moderator)

In the proposal of deriving the LP-WUS ACS, there is diversity gain factor is added. We share our understanding as below.

In RAN4 report#50 (R4-093227), it is agreed that no diversity gain is used in ACS legacy test.

Executive Summary

Maintenance of E-UTRA

UE Requirements

- ACS test currently in the spec does not consider the rx diversity gain.

Considering this decision and the interpretation of ACS can be three facts as below:

- The input power of legacy wanted signal should be REFSENS + diversity gain

- The relaxation factor is 11 dB instead of the 14 dB

Referring to the formular in R4-030209, where the equation of ACS just accounts for the noise increase due to the jammer presence.

“The conversion formula between the ACS and the signal levels can be written[4] as

a N = N + I/ACS					(1)

ACS = I/{(a-1)N)					(2)

Where 	a is the margin of wanted signal level above reference sensitivity in linear unit,

N is the noise floor in linear unit,

I , the adjacent channel interference signal level expressed in linear unit,

ACS, the adjacent Channel Selectivity expressed in linear unit

If the margin parameter a is big enough, the noise floor N on the right side of the equation (1) can be neglected, in this case, the formula (2) becomes

ACS  I/(a N)						(3)

In logarithmic unit, the exact relation (2) and the approximation relation (3) become respectively

ACS = I – N – A1					(4)

with A1=10log10(a-1), ACS expressed in dB, I and N are expressed in dBm.

and

ACS  I – N – A0					(5)

with A0=10log10(a), ACS expressed in dB, I and N are expressed in dBm.	”

Then with the same formular for interference signal power with 11 dB relaxation

IntrfrMR=ACS+N+A1=ACSMR+IMACS+Thermalnoise+NMR+10logNRB+10log101110-1MR

For WUR , as no diversity gain is considered, the 14 dB relaxation is considered the same with a RedCap UE with a single RX.

IntrfrLR=ACS+N+A1=ACSLR+IMACS+Thermalnoise+NLR+10logNRB,LPWUS+10log101410-1LR

Therefore, with the equal jammer of LR and MR, we have

ACSLR=  NFMR-NFLR+10logNRBNRB,LPWUS+ACSMR+10log101110-1101410-1=NFMR-NFLR+10logNRBNRB,LPWUS+ACSMR-3

It can be observed that the scaling factor is due to the diversity assusmption difference between MR and LR. MR has no diversity gain so the relaxation factor is 11 dB and LR can use 14 dB as the same with the RedCap UE. The scaling factor is the same with diversity gain (DgMR) in this case.

Use the equation above in deriving the ACS requirements for LR.

For MR, the ACS is specified differently for f< 2.7 GHz and f> 3.3 GHz. As the WUR is a fixed received bandwidth irrespective of the MR bandwidth, we think it will be simplier to specify the ACS using the fixed interferer bandwidth of 5MHz for both f< 2.7 GHz and f> 3.3 GHz. With this, the interferer can always have the same bandwidth with the LP-WUS signal and there is no further relaxation on the wanted signal to be discussed when inteferer bandwidth is wider than LP-WUS signal bandwidth.

Uisng the fixed interfere bandwidth of 5MHz for both f< 2.7 GHz and f> 3.3 GHz.

As the guard RB is important to relax the ACS requirment for WUR and there is some companies seeing the benefit of a simple design. We propose to introduce the guard RB defined in TR 38.869 in SI phase:

“For ACS guard RB, the required guard RB(s) are RB(s) between WUS signal edge and nearest edge of guardband of channel bandwidth

-	It includes ASCS guard RB(s) (if allocated) and RBs (which could be blanked RBs or RBs used for NR transmission) between WUS carrier/BW edge and nearest edge of guardband of channel bandwidth as defined in spec TS 38.101-1”

Introduce the guard RB defined in TR 38.869 in ACS test case for the type 1 WUR.

Last meeting, there are companies to propose much big relaxation on the wanted signal level but this basically means the coverage of the WUR reduced compared to the MR in the jammer presence situation. The LP-WUS and NR signal is transmitted from the same BS and has the same PSD. Such discrepancy of the relaxation factor would result in the coverage loss.  In stead of demanding a large relaxation factor, it would be more meaningfull to use the guard RB to have a enough frequency separation to avoid the reciprocal mixing from WUR LO.

Use the guard RB instead of introducing the big relaxation on the wanted siganl for ACS requriement.

ACS TP starts here:

### 7.1.6	ACS requirements

It is agreed that the LR and MR has the same jammer level for ACS test case. The ACS of LR is derived using the below formular.

The conversion formula between the ACS and the interference signal levels can be written[2] as

a N = N + I/ACS

ACS = I/{(a-1)N)

Where 	a is the margin of wanted signal level above reference sensitivity in linear unit; N is the noise floor in linear unit; I is the adjacent channel interference signal level expressed in linear unit, ACS is the adjacent Channel Selectivity expressed in linear unit.

Assuming the relaxing factor for MR is 11 dB, the MR inteferer level can be expressed with the formular below:

IntrfrMR=ACS+N+A1=ACSMR+Thermalnoise+NMR+10logNRB+10log101110-1MR

The relaxing factor for LR can be 14 dB, the LR inteferer level can be expressed with the formular below:

IntrfrLR=ACS+N+A1=ACSLR+IMACS+Thermalnoise+NLR+10logNRB,LPWUS+10log101410-1LR

With the equal jammer level of LR and MR, the ACS of LR is derived below

ACSLR=  NFMR-NFLR+10logNRBNRB,LPWUS+ACSMR+10log101110-1101410-1=NFMR-NFLR+10logNRBNRB,LPWUS+ACSMR-3

# Conclusions

In this contribution, we present our view on WUR RF requirement for REFSENS, ASCS and ACS with below proposal and observations:

Proposal-1: Use the equation above in deriving the ACS requirements for LR.

Proposal-2:Uisng the fixed interfere bandwidth of 5MHz for both f< 2.7 GHz and f> 3.3 GHz.

Proposal-3:Introduce the guard RB defined in TR 38.869 in ACS test case for the type 1 WUR.

Proposal-4: Use the guard RB instead of introducing the big relaxation on the wanted siganl for ACS requriement.

# References

- R4-2420528, WF on UE RF requirements for LP-WUS, vivo

- R4-030209, BS ACS performance, Nortel Networks