# 3GPP TSG-RAN WG4 Meeting # 116						R4-2511438

# Bangalore, India,  August 25 - 29, 2025

Source:		Ericsson

Title:			A-IoT BS RF impact

Agenda item:		7.22.3.1

Document for:		Approval

# Introduction

In this paper, we present our overview for A-IoT BS RF requirements based on WF [1]. TP is proposed in section 5.

# Discussion

## Transmitter requirements

### Issue 1-5-2: ACLR

- Agreement:

  - Consider following options for ACLR requirements

|  | Option 1 |  | Option 2 |  | Option 3 |  | Option 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R2D CBW | offset | ACLR | offset | ACLR | offset | ACLR | offset | ACLR |
| 200 kHz | 100kHz | 40/50dB | 200kHz | 40/50dB | 100kHz | 30/40dB | 100kHz | 30/40dB |
| 400 kHz | 100kHz | 40/50dB | 200kHz | 40/50dB | 100kHz | 30/40dB | [100]kHz | 30/40dB |
| 600 kHz | 100kHz | 40/50dB | 200kHz | 40/50dB | 100kHz | 30/40dB | N/A | 30/40dB |
| 800 kHz | 100kHz | 40/50dB | 200kHz | 40/50dB | 100kHz | 30/40dB | N/A | 30/40dB |

In 36.104, the Foffset is used in the NB-IoT standalone is specified with 200kHz and this also apply when NB-IoT carrier is placed at lowest or highest carrier, meaning there will be 100kHz guard band to the RF bandwidth edge. In this sense, when there is more RB configured for A-IoT, 100kHz offset to the RF bandwidth edge should be kept. If the same Foffset concept is used, the center distance between a A-IoT carrier to the center of the first adjacent carrier for ACLR requirement will be A-IoT channel bandwidth + Foffset. The Foffset definition is illustrated in Figure 7.1.6.2-1 [2] and quoted below. There is no adjacent carrier operated in the DL for A-IoT. But as the requirement is derived from coexisting study it should be understood that 40/50 dB should be used as this is only number stated in Table 6.6.2.2-2 in TR 38.769.

For NB-IoT standalone operation, NB-IoT requirements for receiver and transmitter shall apply with a frequency offset Foffset  as defined in Table 5.6-3A.

Table 5.6-3A: Foffset for NB-IoT standalone operation

| Lowest or Highest Carrier | Foffset |
| --- | --- |
| Standalone NB-IoT | 200 kHz |

- Figure 7.1.6.2-1 [2]: ACLR for NB-IoT standalone

Option 2 of 40/50 dB can be specified for A-IoT BS.

Apply the same Foffset definition as NB-IoT standalone in TS 36.104.

### Issue 1-4-2: Modulation quality

- Agreement:

  - Consider following options

    - Option 1: Take the table below as a starting point.

| Tari | Parameter | Symbol | Mimimum | Nominal | Maximum | Units |
| --- | --- | --- | --- | --- | --- | --- |
| Tari= 103M*15 (us) M = 2, 6, 12 and 24 | Modulation Depth | (A–B)/A | 80 | 90 | 100 | % |
|  | RF Envelope Ripple | Mh = Ml | 0 |  | [0.5~0.15] (A–B) | V/m orA/m |
|  | RF Envelop Rise Time | Tr,10-90 |  |  | 0.33Tari | µs |
|  | RF Envelop Fall Time | Tf,10-90 |  |  | 0.33Tari | µs |
|  | RF Pulsewidth | PW |  | 0.5Tari | [0.65/0.525] Tari | µs |

      - Note 1: The ripple is calculated based on per-chip, comparing peak value of each chip to its own average.

      - Note 2: SIP is excluded for ripple calculation.

    - Option 2: No need to define RF envelop parameters.

The RF envelop rise/fall time is discussed in Tx transient and for modulation depth, Ripple defined in EPC specification. Figure 1 to Figure 2 that these are relating to the baseband synthesized signal. For example, the DFT approach is compared with ideal brick waveform with different subcarrier number and M segments in one OFDM symbol. It can be observed that the modulation depth is relating to the algorithm used to synthesize the OOK and also the ripple performance is relating to the configuration of the number of the subcarrier and M-Chip per OFDM symbol for baseband signal.

Ripple performance in time domain relates to both baseband signal and also the RF transmitter transfer function.

The ripple performance of the OOK signal seems important for a device with simple integrator implemented as the data demodulator. The device may use the ripple limitation to set properly some threshold in the demodulator. Consider this, there are several options below if the timing mask to be specified:

  - No modification of the baseband signal, the maximum ripple measured at the baseband together with some margin can be specified.

  - Modification of the baseband signal, some method in baseband to be applied so the ripple can be reduced, this is illustrated in Figure 4.

We understand that the option 1 in last WF is to follow the EPC Gen2 UFID standard, however, there is also the similar timing mask in ISO/IEC 14443-2 but simpler. The ripple requirement in Gen2 is quite stringent which is 5% (A-B) and convert it to power with below equation, this is +0.4dB/-0.44 dB power ripple relative to 100% A level when B=0 and +0.34/-0.35 dB. Comparing to the GSM GMSK timing mask where the power ripple is +/1 dB, this ripple requirement is quite stringent and from above discussion the baseband waveform introduced the ripple and the margin left for radio is even less.  Consider this, we believe there is a need to reduce the ripple in baseband and propose below LS to RAN1 to consult if it is possible to remove/lower the ripple magnitude in baseband signal.

X (dB) = 20*log10(x[%]/100)

A LS can be sent to RAN1 asking below questions:

RAN4 will specify the OOK waveform ripple requirement. However, current OOK waveform introduce ripple in OOK waveform. RAN4 want to ask if there is any method in baseband which can reduce the ripple in OOK generated signal and if so, RAN4 would like to ask if the ripple suppression can be introduced in OOK waveform generation or it can be up to implementation.

LS to RAN1 to ask if the baseband can ripple suppression in OOK signal generation.

The confirmation from the RAN1 on the ripple suppression will leave more room on the radio imperfection and therefore make it possible to specify the timing mask. The EPC Gen2 timing mask also coupled to the level B of the envelop level of the OOK bit 0. As this inherently relates to baseband waveform, we think it is ok to set a level only relative to level A (OOK bit 1 envelop level). Therefore the timing mask can be simplified. This is illustrated in Figure 4 with the Table below listing the relevant requirement.

The overshooting level for the reference field strength suggests 10% and this corresponding to + 0.83 /-0.91 dB in Table below.

| Percentage | Difference in dB at 100% dB value |
| --- | --- |
| 110% | +0.83 |
| 90% | -0.91 |
| 10% | -20 |

Considering to introducing the below timing mask when the baseband waveform ripple can be suppressed within a limited range.

The corresponding requirement test suggests below:

This envelope of electric filed strength for OOK Bit 0 RF signal shall comply the timing mask in Figure 3 and meet the requirements in Table 1.

The envelope of electric filed strength for OOK Bit 0 shall decrease monotonically from 90% to less than 10 % of its initial value Einitial during t1 and remain less than 10 % during t2.  The envelope of electric filed strength for OOK Bit 0 shall increase monotonically from 10% to less than 90 % of its initial value Einitial during t3.

The t1 measures the fall/ramp-down time of the OOK bit 0 pulse and starts when envelop falls to 90% level of the initial value Einitial and ends when the envelop falls to 10% of Einitial. The t2 starts when the envelop drops to the 10% level of initial value Einital and ends when envelop rise to 10% level of initial value. The t3 measures the rise/ramp-up time of the OOK bit 0 pulse and starts when envelop rises to 10% level of the initial value Einitial and ends when the envelop rises to 90% of Einitial.

The initial value is 100% reference value is measured for OOK bit 1 preceding to the OOK Bit 0, where the measurement duration is [-1/6*Tchip, +1/6*Tchip] reference to the centre of the OOK bit 1 pulse.

In case of an overshoot the field shall remain within 90 % of Einitial and 110 % of Einitial.

Table 1: Timing mask paratmer (Tchip = 103M*15 (us) )

| Parameter | Min | Max |
| --- | --- | --- |
| t1 | 0 | 1/3 Tchip |
| t2 | 1/3 Tchip | [Tchip] |
| t3 | 0 | 1/3 Tchip |

Figure 1: Timing mask for OOK bit-0

Figure 2: Ripple and modulation depth with FFT size N=1024 and different size of K

Figure 3: modulation depth and ripple with FFT size N=1024 and K=24

Figure 4: The filtered OOK waveform

Figure B.1[TS 45.005]: Time mask for normal duration bursts (NB, FB, DB, SB and Extended AB) at GMSK modulation

## Receiver requirement

### Issue 2-1-1: Reference sensitivity level

- Agreement:

  - Define REFSENS for both OOK and BPSK. Depending on the simulations, they may or may not be the same.

  - Use [X dB] as the desens target, with CW single tone input through a signal generator, CW input level to BS Ant is assumed as [TBD] dBm;

For REFSENS requirement, the CW received power should be considered when testing the REFSENS. In Figure 8, the received CW power and D2R signal power is illustrated with 20 m ISD between CWT and BS. The CW received power is higher (e.g -15.5 dBm for LOS) with CW Tx of 33 dBm then the received CW power of -25.5 dBm with CW Tx power of 23 dBm for ISD of 20 meter.

With the consideration in our paper[4] last meeting, the de-sensitization factor are summarized in table below.

Table 2: Desensitization factors

| Factors for de-sens | De-sensitization value (dB) | Conditions |
| --- | --- | --- |
| Receiver linearity | 3 | 0 dBm IIP3 LNA and -15.5 dBm CW power |
| Baseband filter | 11~31 | Assuming a -15.5 dBm CW and RF cancellation of 20~ 40 dB. The baseband input CW power is -35.5 ~ -55.5 dBm. The noise floor is -127 dBm ( -174 + 5+ 10log10(15000)), this needs the attention of the 91~71 dB. Assuming the filter providing 60 dB attenuation, the desens will be 31 or 11 dB. |
| Reciprocal-mixing from receiver LO | 22 | The CW signal is strong compared to the wanted signal and LO phase noise can be reciprocal mixed with CW signal and therefore, the phase noise in the wanted signal BW will de-sens the receiver. This is similar with the CW phase noise generated in CWT node. BS LO phase noise and CWT phase noise in this situation will compete each other and |

From above analysis, the dominant de-sense factor is phase noise or if there is no RF cancellation, the baseband filter may be bottleneck. The filter and phase noise contribution will be accumulated as the phase noise is in pass band and no attenuation for it. The CW received power is therefore both critical for reciprocal mixing and filter attenuation. -25.5 dBm of CW received power level could be used assuming a 20m distance to CWT and 23 dBm CWT power level. 30 dB desensitization could be used for REFSENS.

30 dB de-sensitization for REFSENS.

### Issue 2-1-2: LLS assumptions and parameters for SNR

- Agreement:

  - Update simulation results base on 31bits preamble.

  - 1-10% SFO can be considered and declared in the evaluation.

### Issue 2-1-3: SNR value

- Agreement:

  - Encourage companies update simulation results based on 31bit preamble.

As the REFSENS is testing the BS receiver noise figure and linearity in the presence of a CW signal, we think there is no need to consider the sample clock error in the D2R signal. Preferably the clock imperfection can be specified in the demodulation requirement. The signal generator can generate a test signal with no clock error and even the signal generator can be synchronized with BS clock so there is no impact on clock setting.  As the preamble is used to estimate and compensate the clock error, we did not simulate this and as we propose, no SFO could be assumed for SNR alignment between companies and test vector of D2R signal can be generated without SFO for BS receiver test.

It can be observed that the D2R OOK modulation SNR is -4 dB when BLER is 10^-1, SFO=0.

D2R OOK modulation SNR is -4 dB when BLER is 10^-1.

No SFO can be assumed for BS receiver RF test.

Figure 5: BLER for D2R decoding

Last meeting it is agreed only UE outdoor is considered. However, in our previous analysis on co-location requirement, a NR UE operating in adjacent channel can roam into the indoor A-IoT coverage before being hand overed to another frequency. Therefore, we think ACS and IBB legacy requirement should apply.

The frequency offset between the ACS interfere and BS RF bandwidth edge should meet the condition where there is 7.5kHz offset between subcarrier grid of wanted signal and subcarrier grid of interferer signal. While A-IoT D2R signal is not OFDM signal and therefore, such condition does not apply. Therefore, the frequency offset between ACS interferer can be simplified. Reusing the legacy frequency offset or any new frequency offset are both fine.

NR UE operating in adjacent channel can roam into the A-IoT coverage before handover occurs.

Reuse the ACS and IBB requirement in legacy NR BS.

No need to consider the 7.5kHz misalignment between a wanted signal and ACS/IBB interferer.

Intermodulation response rejection is a measure of the capability of the receiver to receive a wanted signal on its assigned channel frequency at the antenna connector for BS type 1-C in the presence of two interfering signals which have a specific frequency relationship to the wanted signal. There is no other requirement for receiver linearity so this would be needed. Though the CW signal also drives the linearity requirement, it is not the dominant factor as reciprocal mixing and the filter leakage may be the main issues.

Reuse the RX intermodulation requirement.

# Conclusions

In this contribution, we present our overview on the A-IoT BS RF with below proposal and observations:

Observation 1 Ripple performance in time domain relates to both baseband signal and also the RF transmitter transfer function.

Proposal-1: Option 2 of 40/50 dB can be

Proposal-2: Apply the same Foffset definition as NB-IoT standalone in TS 36.104.

Observation 1 Ripple performance in time domain relates to both baseband signal and also the RF transmitter transfer function.

Proposal-3: LS to RAN1 to ask if the baseband can ripple suppression in OOK signal generation.

Proposal-4: Considering to introducing the below timing mask when the baseband waveform ripple can be suppressed within a limited range.

Proposal-5:30 dB de-sensitization for REFSENS.

Proposal-6: No SFO can be assumed for BS receiver RF test.

Observation 3 NR UE operating in adjacent channel can roam into the A-IoT coverage before handover occurs.

Proposal-7: Reuse the ACS and IBB requirement in legacy NR BS.

Proposal-8: No need to consider the 7.5kHz misalignment between a wanted signal and ACS/IBB interferer.

Proposal-9: Reuse the RX intermodulation requirement.

# References

- R4-2508101, WF on requirements for A-IoT BS and CW, Huawei

- EPC™ Radio-Frequency Identity Protocols Generation-2 UHF RFID

- TR 36.802

- R4-2507495 , TP to TS 38.914  Inband blocking OOB and Spurious, Ericsson