# 3GPP TSG-RAN WG4 Meeting # 116						R4-2511440

# Bengaluru, India, August 25-29, 2025

Source:		Ericsson

Title:			RedCap UE RF impact on HD-FDD

Agenda item:		7.27.3.1

Document for:		Approval

# Introduction

In this paper, we present our views on the RedCap UE RF remaining issue.

# Discussion

In WF[1], the remaining issue is below:

Agreements:

- Define new UE capabilities for UE to indicate support for the additional power boosting per band and network to enable the boosting

  - FFS for details, capabilities will be needed for feature list

- When UE indicates support for boosting and network enables the boosting, UE reference power is increased, i.e. both Pcmax_L and Pcmax_H are increased

  - MPR is not changed for DFT-s-OFDM and QPSK and pi/2 BPSK, [all RB allocations]

  - For CP-OFDM and DFT-s-OFDM 16QAM and 64QAM, boosting is allowed by UE implementation and MPR is correspondingly increased by the amount of the boost

- Boost is 0.5 dB

  - FFS on duplex modes

  - For information only: HD-FDD/FD-FDD support is indicated per band

    - This means that UE has option to e.g. be HD-FDD in band A and support boosting and be FD-FDD in band B and not support boosting

- Rel-18 and Rel-19 power boosting schemes can be simultaneously supported

  - FFS for details

- PC3 ACLR requirements apply for power boosted transmissions

- Configured maximum output power and MPR updates related to power boosting need to be captured to TS 38.101-5

This capability is network controlled as similar as the Rel-18 power boosting capability. As the new power boosting scheme originates in NTN Rel-19 but it can be applied to TN RedCap UE also. It will be up to UE to report and associate to either TN or NTN band as an optional feature.  However, current specification of 38.101-5 is referring to 38.101-1 but not vice versa, it would mean that a CR of 38.101-1 would be appropriate if such feature is generic both TN and NTN.

RAN4 decide whether to introduce a CR of 38.101-1 or CR of 38.101-5 depending on whether the power boosting would be supported only in NTN only or TN&NTN.

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| [powerBoosting-pi2BPSKQPSK-Rel-19] Indicates whether [(e) RedCap] UE supports power boosting for pi/2 BPSK and QPSK, when applicable as defined in 6.2 of TS 38.101-5 [2]. | Band | NO | NO | FR1 only |

Use the above capability definition to introduce the power boosting scheme for RedCap UE

As this is network controlled power boosting, a new IE needs to be specified. Together with the new UE capability, an LS to RAN2 should be initiated, the suggested text is below:

Below capability is agreed to be added in Rel-18:

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| [powerBoosting-pi2BPSKQPSK-Rel-19] Indicates whether [(e) RedCap] UE supports power boosting for pi/2 BPSK and QPSK, when applicable as defined in 6.2 of TS 38.101-1 [2]. | Band | NO | NO | FR1 only |

Additionally, RAN4 agrees to enable/disable the Rel-19 power boosting with RRC signalling and agree to add separate one RRC IE, [powerBoostPi2BPSKQPSK-Rel-19] power boosting.

Send LS to RAN2 for the added UE capability and IE to configure the power boosting.

Regarding on the differentiation of the operating mode, we think there is no need to limit the power boosting to a certain operation mode. For example, when the (e) RedCap UE indicate the HD-FDD mode, it will not support the FD-FDD mode. If (e) RedCap UE does not support this boosting capability in FD-FDD mode, it can indicate this capability when it indicate the HD-FDD mode capability. In this sense, there is no need to limit the power boosting to HD-FDD for a (e ) RedCap UE.

For a (e) RedCap UE indicating the HD-FDD mode, it will not support the FD-FDD mode

Not limited the power boosting capability to HD-FDD mode, UE can optionally report either operation mode using legacy signalling structure.

For simultaneous support of the Rel-18 and Rel-19 power boosting, the suggested text is below:

ΔPPowerBoost defined in clause 6.2.4 of 3GPP TS 38.101-1 [5] equals

- 1.5 dB for power class 3, when all of the following conditions are met:

-	If the UE indicates support for UE capability powerBoosting-pi2BPSK-QPSK-r18 and/or powerBoosting-pi2BPSK-QPSK-Modified-r18, and if IE powerBoostPi2BPSK-r18 and/or powerBoostQPSK-r18 is set to 1

-	and PEMAX,c, if configured, is increased by at least ΔPPowerBoost

- 	If the UE indicates support for UE capability [powerBoosting-Pi2BPSK-QPSK-NTN-r19] and if IE [powerBoost Pi2BPSKQPSKNTNr19] is set to 1 and PEMAX,c, if configured, is increased by at least ΔPPowerBoost

-	If UE indicates power class 2 in a TDD band or power class 3

-	IfΔPPowerClass is 0dB

-	If scheduled UL transmission is DFT-s-OFDM with either PI/2 BPSK modulation (with either Rel-15 DMRS or with pi/2 BPSK DMRS) or QPSK modulation

-	If the RB allocation belongs to the inner region defined in clause 6.2.2 of 3GPP TS 38.101-1 [5]  .

-	If UE indicates power class 3, the percentage of uplink symbols transmitted in a certain evaluation period is less than 70%

Discuss the above text in the companion CR.

# Conclusion

In this contribution, we present our view on the remaining issue for  Redcap UE RF:

Proposal-1: RAN4 decide whether to introduce a CR of 38.101-1 or CR of 38.101-5 depending on whether the power boosting would be supported only in NTN only or TN&NTN.

Proposal-2: Use the above capability definition to introduce the power boosting scheme for RedCap UE

Proposal-3: Send LS to RAN2 for the added UE capability and IE to configure the power boosting.

Observation 1 For a (e) RedCap UE indicating the HD-FDD mode, it will not support the FD-FDD mode

Proposal-4: Not limited the power boosting capability to HD-FDD mode, UE can optionally report either operation mode using legacy signalling structure.

Proposal-5: Discuss the above text in the companion CR.

# References

- R4-2508764, Way Forward for [115][309] NR_NTN_Ph3_General_UE_SAN_RF, Qualcomm Inc