3GPP TSG-RAN WG4 Meeting #116                                                               R4-2511452

Bengaluru, India, 25th – 29th August, 2025

Agenda item:			7.21.1

Source:	Moderator (Samsung)

Title:	Topic summary for [116][306] NR_duplex_evo_General

Document for:	Information

# Introduction

In RAN#102, the work item on evolution of NR duplex operation (SBFD) has been approved [RP-234035], with WID further revised in the follow-up RAN plenary [RP-240789]. According to the objectives in WID, RAN1 is tasked to specify the mechanisms to support SBFD, including semi-static indication of time/frequency location, random access in SBFD symbols, and other transmission, reception and measurement behavior and procedures for SBFD aware UE. Furthermore, the enhancement for CLI handing, including gNB-to-gNB and UE-to-UE CLI handling, will also be specified in RAN1. Accordingly, from RAN4 perspective, it is tasked to “Specify BS RF requirements for SBFD operation at gNB [RAN4]”.

Over RAN4#115, there was extensive discussion on SBFD general aspects and potential new requirements for SBFD operation for FR1 and FR2-1, which results in the progress captured in the approved WF [R4-2508707]. Furthermore, the WF on UE-UE CLI problem and potential solutions has been captured in WF [R4-2511251].

This document is provided for the moderator summary of thread [306] on Rel-19 work item on evolution of NR duplex operation (SBFD) for general aspect and potentially new requirements for SBFD, in which the following highlighted agenda items are supposed to be covered specifically:

- Evolution of NR duplex operation: Sub-band full duplex (SBFD) [NR_duplex_evo] - Moderator summary and conclusions [NR_duplex_evo] - General aspects (including RAN4 aspects for SBFD system parameters) [NR_duplex_evo-Core] - General system parameters for SBFD operation [NR_duplex_evo-Core] - Analysis on intra-operator adjacent-channel inter-cell UE-UE CLI and solutions [NR_duplex_evo-Core] - BS RF and conformance requirements [NR_duplex_evo-Core/Perf] - Potentially new requirements for SBFD operation for FR1 and FR2-1 including BS conformance [NR_duplex_evo-Core/Perf] - Modification of existing Tx requirements for FR1 and FR2-1 including BS conformance [NR_duplex_evo-Core/Perf] - Modification of existing Rx requirements for FR1 and FR2-1 including BS conformance [NR_duplex_evo-Core/Perf] - RRM core requirements [NR_duplex_evo-Core] - RRM requirements for UE-to-UE CLI handling [NR_duplex_evo-Core] - RRM impacts for SBFD operation [NR_duplex_evo-Core] - RRM performance requirements [NR_duplex_evo-Perf] - UE and BS Demodulation performance requirements [NR_duplex_evo-Perf]

# Topic #1: General system parameters for SBFD operation

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2510420 | Qualcomm Germany | Proposal 1: RAN4 to consider the X = 38 dBm as MR SBFD-capable power threshold value (X dBm) as a compromise between MR maximum BS rated carrier TRP output power limit given in Table 9.3.1-1, TS 38.104 (i.e., 43 dBm) and representative values adopted in Section 9.3, TR 38.858. |
| R4-2510961 | Ericsson | Observation 1 While the introduced power level threshold may appear to limit the maximum number of subband configurations for FR1 MR SBFD-capable BS, it is important to note that above this threshold, an analog filter is required for each subband configuration. As a result, there is a practical limit to the number of analog filters that can be supported by a single base station. Therefore, the selection of the power level threshold should be based on practical considerations. Proposal 1 Use 33 dBm either as TRP or total conducted power as the power level threshold for FR1 MR SBFD capable BSs related to testing. |
| R4-2509322 | CATT | Proposal 1: About declaration and flexibility for SBFD operation, the power level threshold for FR1 MR SBFD capable BSs can be 33dBm. Proposal 2: About restriction on the number of UL subband sizes supported, the power level threshold for FR1 MR SBFD capable BSs can be 33dBm. Proposal 3: About restriction on the number of DL subband sizes supported, the power level threshold for FR1 MR SBFD capable BSs can be 33dBm. |
| R4-2510132 | CMCC | Proposal 1: Considering the transmission power of MR BS in current networks, use 47 dBm either as TRP or total conducted power as the power level threshold for FR1 MR SBFD capable BSs. |
| R4-2511105 | Huawei, HiSilicon | Proposal 1: Use 38dBm conducted output power as the power level threshold for FR1 MR SBFD capable BS. Proposal 2: Not to use terminology “SBFD period” for specification drafting. Moderator Note: Proposal 2 is summarized in thread 307. |
| R4-2511137 | ZTE Corporation, Sanechips | Proposal 1: for FR1 MR SBFD BS, propose to use the 33dBm as TRP or total conducted power as the power threshold. |
| R4-2511250 | Samsung | Proposal 1: For FR1 MR BS, use X = 38 dBm as TRP or total conducted power as the power level threshold for FR1 MR SBFD capable BSs related to testing. • Note: As agreed, above this threshold, all configurations shall be tested; below this threshold, analog filters are not expected to be used, and the configurations with the narrowest and widest UL/DL subband sizes out of all the declared configurations for each channel bandwidth and each SBFD pattern (either DUD or DU/UD) shall be tested. Proposal 2: For FR1 LA, FR1 MR with TRP or conductive power less than the pre-defined threshold, and all FR2 BS classes, the principle of “for each declared channel bandwidth and each declared SBFD pattern (either DUD or DU/UD), the configurations with the narrowest and widest UL/DL subband sizes out of all declared configurations shall be tested” shall be interpreted as: • For each declared channel bandwidth and each declared SBFD pattern (either DUD or DU/UD), out of all declared configurations, o The configuration with the narrowest/widest UL subband size and accordingly the worst case SBFD DL transmission bandwidth configuration shall be tested:  For DU/UD case, the worst case SBFD DL transmission bandwidth configuration is the one with the largest NRB,SBFD,DL  For DUD case, the worst case SBFD DL transmission bandwidth configuration is the one with the largest NRB,SBFD,DL,1 Proposal 3: For FR1 MR SBFD capable BSs, adopt the same power level threshold as the one to differentiate whether the restriction of maximum 3 subband configurations is applied or not: • i.e, X = 38 dBm as TRP or total conducted power adopted as the power level threshold. Proposal 4: For FR1 MR SBFD capable BSs, adopt the same power level threshold as the one to differentiate whether the restriction on the number of DL subband sizes supported is applied or not: • i.e, X = 38 dBm as TRP or total conducted power adopted as the power level threshold. Observation 1: For the restriction on guardband size of SBFD, • From testing perspective, as provided in Proposal 2, the configuration with the narrowest/widest UL subband size and accordingly the worst case SBFD DL transmission bandwidth configuration shall be tested, therefore it is guaranteed that the guardband size should be minimized to ensure the benefits from SBFD operation for the relevant BS classes. • From declaration perspective, the principle “it is expected that guardband size should be minimized to ensure the benefits from SBFD operation” is hard to be captured in RAN4 specification. |
| R4-2511255 | Samsung | Reserved BigCR to TS38.104 to introduce BS RF requirement for SBFD-capable BS, for post-meeting email approval. |
| R4-2511257 | Samsung | draftCR to TS38.104 on general clauses for SBFD-capable BS |
| R4-2511526 | Nokia | Observation 1: The guardband plays a big role in terms of avoiding inter-subband interference. Smaller guardband does not always translate into better performance. Proposal 1: Use 33dBm either as TRP or total conducted power as the power level threshold for FR1 MR SBFD capable BSs related to testing. Proposal 2: Use 33dBm either as TRP or total conducted power as the power level threshold for FR1 MR SBFD capable BSs related to UL subband sizes supported. Proposal 3: Use 33dBm either as TRP or total conducted power as the power level threshold for FR1 MR SBFD capable BSs related to DL subband sizes supported. Proposal 4: If it is decided that a clause is introduced to the RAN4 specification related to guardband size, the text should say: “It is expected that guardband size should be minimized to reduce performance degradation due to overhead.” |

## Open issues summary

### Sub-topic 1-1: Declaration and flexibility for SBFD operation

- [Moderator] In previous RAN4 meetings, the following agreements are achieved on declaration and flexibility for SBFD operation:

Agreement in RAN4#114: - The SBFD capable BS product shall not operate any declared SBFD configuration (DL/UL subband) other than those that have been tested. - This restriction shall not be applied to FR1 LA BS - FFS FR1 MR BS Agreement in RAN4#114bis: - For FR1 LA SBFD capable BSs, which are not expected to require analog filter(s), for each declared channel bandwidth and each declared SBFD pattern (either DUD or DU/UD), the configurations with the narrowest and widest UL/DL subband sizes out of all declared configurations shall be tested. - For FR1 MR SBFD capable BSs, a power level threshold could be established: Above this threshold, all configurations shall be tested; Below this threshold, analog filters are not expected to be used, and the configurations with the narrowest and widest UL/DL subband sizes out of all the declared configurations for each channel bandwidth and each SBFD pattern (either DUD or DU/UD) shall be tested. - Use X dBm either as TRP or total conducted power as the power level threshold for FR1 MR SBFD capable BSs related to testing. - FFS the value of X, e.g., X = 33dBm as proposed by one company as candidate value, and study the value for different number of TX chains. Agreement in RAN4#115: - For all FR2 BS classes, a which are not expected to require analog filter(s), for each declared channel bandwidth and each declared SBFD pattern (either DUD or DU/UD), the configurations with the narrowest and widest UL/DL subband sizes out of all declared configurations shall be tested. - For FR1 MR SBFD capable BSs, a power level threshold could be established: Above this threshold, all configurations shall be tested; Below this threshold, analog filters are not expected to be used, and the configurations with the narrowest and widest UL/DL subband sizes out of all the declared configurations for each channel bandwidth and each SBFD pattern (either DUD or DU/UD) shall be tested. - Use X dBm selected from the range of [33, 38] dBm either as TRP or total conducted power as the power level threshold for FR1 MR SBFD capable BSs related to testing. - X value will be selected in next meeting.

##### Issue 1-1-1: Declaration and flexibility for SBFD operation

- As agreed, for FR1 MR SBFD capable BSs, a power level threshold could be established. In this meeting, proposals on FR1 MR BS for the threshold are provided:

  - Option 1 (Ericsson, CATT, ZTE, Nokia): X = 33dBm

  - Option 2 (Samsung, Qualcomm, Huawei): X = 38dBm

  - Option 3 (CMCC): X = 47dBm

- Reasoning for the above option selection:

  - Ericsson: While the introduced power level threshold may appear to limit the maximum number of subband configurations for FR1 MR SBFD-capable BS, it is important to note that above this threshold, an analog filter is required for each subband configuration. As a result, there is a practical limit to the number of analog filters that can be supported by a single base station. Therefore, the selection of the power level threshold should be based on practical considerations.

  - Qualcomm: RAN4 to consider the X = 38 dBm as MR SBFD-capable power threshold value (X dBm) as a compromise between MR maximum BS rated carrier TRP output power limit given in Table 9.3.1-1, TS 38.104 (i.e., 43 dBm) and representative values adopted in Section 9.3, TR 38.858.

- Moderator Recommendation:

  - Try to discuss X = 35.5dBm as compromised value. If no compromise can be reached, postpone the BS declaration discussion to BS conformance requirement in performance part.

##### Issue 1-1-2: How to interpret “the configurations with the narrowest and widest UL/DL subband sizes out of all declared configurations”

- [Moderator] For FR1 LA, FR1 MR with TRP or conductive power less than the pre-defined threshold, and all FR2 BS classes, the principle of “for each declared channel bandwidth and each declared SBFD pattern (either DUD or DU/UD), the configurations with the narrowest and widest UL/DL subband sizes out of all declared configurations shall be tested” is agreed, while how to interpret could be further aligned.

- Proposals on how to interpret “the configurations with the narrowest and widest UL/DL subband sizes out of all declared configurations”:

  - Proposal 1 (Samsung): For each declared channel bandwidth and each declared SBFD pattern (either DUD or DU/UD), out of all declared configurations,

    - The configuration with the narrowest/widest UL subband size and accordingly the worst case SBFD DL transmission bandwidth configuration shall be tested:

      - For DU/UD case, the worst case SBFD DL transmission bandwidth configuration is the one with the largest NRB,SBFD,DL

      - For DUD case, the worst case SBFD DL transmission bandwidth configuration is the one with the largest NRB,SBFD,DL,1

- Moderator Recommendation:

  - Confirm the above understanding and implement this interpretation in conformance requirement in performance part.

### Sub-topic 1-2: DL/UL subband configurations support for SBFD

- [Moderator] Based upon previous RAN4 discussion, it is “within RAN4 scope to study/specify the limitation or restriction on the size of subband/guardband”. In RAN4#113, the following agreement is provided:

Agreement: - For Rel-19, option 2b is adopted, with the following restrictions on SBFD configuration: - For DUD case, the UL subband should be placed in the middle of channel bandwidth, i.e., symmetric DUD - DL subband 1 and DL subband 2 shall have the same size or different sizes by only 1RB - For a specific channel BW supported for SBFD, maximum [2 or 3] specific UL subband sizes are allowed to be declared and shall be selected from the channel bandwidth sizes from Table 5.3.2-1 in TS38.104. - FFS different sizes for specific UL subband for DUD and DU/UD. - No further restriction is needed on the range of the ratio of the UL subband size relative to the channel bandwidth. - This restriction shall be applied to FR1 WA BS only, as the starting point - FFS the restriction is also applied to FR1 MR BS - For a specific channel BW and a specific UL subband size supported, only one DL subband size is allowed to be declared and tested for DU/UD case, and only one DL subband size is allowed to be declared and tested for DUD case - This restriction shall be applied to FR1 WA BS only, as the starting point - FFS the restriction is also applied to FR1 MR BS

In which option 2b is given in RAN4#112:

Agreement: - To specify DL/UL subband configuration in BS RF requirements, the following 3 options for DL/UL subband configuration are identified: - Option-2b: - UL subband configuration shall be selected from the existing values of BS transmission bandwidth configuration, as provided in Table 5.3.2-1 for FR1 and Table 5.3.2-2 for FR2-1. - DL subband size(s) and guardband size(s) depends on BS implementation - Example (FR1 DU or UD in 100MHz and 30kHz SCS, 80-20MHz case): As long as BS transmission configuration for SBFD shall be 273 (NRB,SBFD for 100MHz and 30kHz SCS) and the value of NRB,SBFD,UL is selected from Table 5.3.2-1 (according to the 30kHz SCS), from BS RF requirement perspective, no limitation on the values NRB,SBFD,DL and NRB,SBFD,GB.

##### Issue 1-2-1: Restriction on the number of UL subband sizes supported

- [Moderator] For the restriction-2 (i.e., restriction on the number of UL subband sizes supported), the following agreement is reached in RAN4#115:

<Agreement in RAN4#115> - For a specific channel BW supported for SBFD, the maximum number of specific UL subband sizes are allowed to be declared - Maximum 3 subband configurations allowed for FR1 WA BS - These 3 suband configurations shall cover all DUD and DU/UD ones supported by this BS. - One UL subband size for DU/UD cases, should be counted as 2 configurations, e.g., UD 20-80MHz and DU 80-20MHz should be counted as 2 configurations. - For FR1 MR SBFD capable BSs, a power level threshold could be established: Above this threshold, maximum 3 subband configurations allowed as FR1 WA BS; Below this threshold, the restriction on the number of UL subband sizes supported is not applicable. - Adopt the same power level threshold as the one to differentiate whether all declared configurations shall be tested or not.

- Moderator Recommendation:

  - As long as Issue 1-1-1 is resolved, this issue can be closed automatically. No need further discussion.

##### Issue 1-2-2: Restriction on the number of DL subband sizes supported

- [Moderator] For the restriction-3 (i.e., Restriction on the number of DL subband sizes supported), the following agreement is achieved in RAN4#115:

Agreement: - For a specific channel BW and a specific UL subband size supported, only one DL subband size is allowed to be declared and tested for DU/UD case, and only one DL subband size is allowed to be declared and tested for DUD case - This restriction shall be applied to FR1 WA BS - For FR1 MR BS SBFD-capable BSs, a power level threshold could be established: Above this threshold, this restriction is applicable as FR1 WA BS; Below this threshold, the restriction on the number of DL subband sizes supported is not applicable. - Adopt the same power level threshold as the one to differentiate whether all declared configurations shall be tested or not.

- Moderator Recommendation:

  - As long as Issue 1-1-1 is resolved, this issue can be closed automatically. No need further discussion.

##### Issue 1-2-3: Restriction on guardband size for SBFD

- [Moderator] Based on RAN4#114bis agreement:

Agreement: - Restriction on guardband size for SBFD: - Option 3a: No restriction (manufacturer declaration-based), and no restriction on maximum/minimum number of RBs for guardband. - The declaration of guardband size should ensure the benefits from SBFD operation. - In the RAN4 specification, the clause “It is expected that guardband size should be minimized to ensure the benefits from SBFD operation.” could be captured, and the detailed wording can be further refined.

- Observation/Proposal on the guardband

  - Observation 1 (Samsung): For the restriction on guardband size of SBFD,

    - From testing perspective, as provided in Proposal 2, the configuration with the narrowest/widest UL subband size and accordingly the worst case SBFD DL transmission bandwidth configuration shall be tested, therefore it is guaranteed that the guardband size should be minimized to ensure the benefits from SBFD operation for the relevant BS classes.

    - From declaration perspective, the principle “it is expected that guardband size should be minimized to ensure the benefits from SBFD operation” is hard to be captured in RAN4 specification.

  - Observation/Proposal from Nokia:

    - Observartion: The guardband plays a big role in terms of avoiding inter-subband interference. Smaller guardband does not always translate into better performance.

    - Proposal: If it is decided that a clause is introduced to the RAN4 specification related to guardband size, the text should say: “It is expected that guardband size should be minimized to reduce performance degradation due to overhead.”

- Moderator Recommendation:

  - FFS the text proposal “It is expected that guardband size should be minimized to reduce performance degradation due to overhead.”

##### Issue 1-2-4: Restriction on DL subband size

- [Moderator] Agreement from RAN4#115

Agreement:

- If the SSB symbol is overlapped with SBFD symbol,

  - there is no RAN4 impact at least on RAN4 core requirement for expected regarding DL subband size configuration

  - FFS the test configuration during performance part discussion.

Agreement:

- If the SSB symbol is overlapped with SBFD symbol,

  - there is no RAN4 impact at least on RAN4 core requirement for expected regarding DL subband size configuration

  - FFS the test configuration during performance part discussion.

- Observation/Proposal on the test configuration

  - For DU /UD configurations and DUD configuration, testing only needs to consider DU or UD configuration, and retesting under DUD configuration is not required (CATT).

- Moderator Recommendation:

  - Discuss it in conformance part for gNB declaration.

### Sub-topic 1-3: draftCR to TS 38.104

- [Moderator] As the worksplitting planned for dCR drafting, the following dCR is prepared which are related to general parts for SBFD operation.

  - R4-2511257, draftCR to TS38.104 on general clauses for SBFD-capable BS, Samsung

    - Moderator Recommendation:

      - View collection before and during the meeting.

  - R4-2511255, BigCR to TS38.104 to introduce BS RF requirement for SBFD-capable BS, Samsung

    - Moderator Recommendation:

      - Tdoc number is reserved to for post-meeting big CR approval.

# Topic #2: SBFD-aware UE related issues

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2510420 | Qualcomm Germany | Observation 1: The HPUE duty cycle evaluation is based on UL transmissions carried by the UE and not only on the symbol assignment by the network. Observation 2: Duty cycle evaluation considers UL transmissions by a (SBFD-aware or non SBFD-aware) UE independent if such UL transmissions were based on UL symbols, flexible symbols assigned to UL by the network, or UL subband of an SBFD symbol. Proposal 2: No additional considerations for SBFD-aware UE is expected to the HPUE duty cycle evaluation given in TS 38.101-1. |
| R4-2510263 | vivo | Observation 1: The term ‘uplink symbols transmitted’ as a whole, not ‘uplink symbols’ standalone, means ‘uplink time’, ‘uplink transmission time’ or ‘uplink transmission’ during the evaluation period for HPUE SAR. Proposal 1: It is suggested to leave the issue for HPUE duty cycle evaluation to UE RF maintenance part if company wants to further clarify the wording. |
| R4-2511250 | Samsung | Proposal 5: RAN4 recognize the followings as RAN4 common understanding (which can be captured as RAN4 agreement): • For non-SBFD-aware UE, the “uplink symbols transmitted” used for uplink duty cycle evaluation are symbols with uplink transmission; • For SBFD-aware UE, the symbols used for uplink duty cycle evaluation are symbols with uplink transmission, including the non-SBFD symbols and SBFD symbols with UL transmission. Observation 2: The existing TDD UE TX RF requirement is specified only on active UL slots. Observation 3: Further clarification on “the symbols used for uplink duty cycle evaluation” will not change anything for UE conformance requirement. Proposal 6: For SBFD-aware UE, no RAN4 requirement impact is expected for UE specification series. |
| R4-2511494 | Ericsson | Observation 1 Any UL symbols transmitted, whether TDD UL symbols or SBFD symbols—are considered as UL symbols for the purpose of UL duty cycle evaluation. Proposal 1 The SBFD symbol does not affect the UL duty cycle evaluation in the RAN4 UE specification, and no further clarification is needed. |
| R4-2511105 | Huawei, HiSilicon | Observation 1: Might be no harm to further update the spec as e.g. “uplink symbols and SBFD symbols transmitted in a certain evaluation period” with the intention of better implementing max duty cycle evaluation for an SBFD-aware UE max duty cycle evaluation. |
| R4-2511137 | ZTE Corporation, Sanechips | Proposal 2: in order to simplify the spec change, propose to add the following two sentence in the general section 6.1. For non-SBFD-aware UE, the “uplink symbols transmitted” used for uplink duty cycle evaluation are symbols with uplink transmission; For SBFD-aware UE, the symbols used for uplink duty cycle evaluation are symbols with uplink transmission, including the non-SBFD symbols and SBFD symbols with UL transmission. |
| R4-2509385 (moved from AI 7.21.3.2) | CATT |  |
| R4-2509386 (moved from AI 7.21.3.2) | CATT | draftCR to TS 38.101-1 on updating duty cycle evaluation with the introduction of SBFD |

## Open issues summary

### Sub-topic 2-1: Update for HPUE duty cycle evaluation

##### Issue 2-1-1: The necessity to update for HPUE duty cycle evaluation

- [Moderator] The necessity to update the wording for HPUE duty cycle evaluation due to introduction of SBFD has been discussed. The following way forward is achieved in RAN4#114bis and no further agreement achieved in RAN4#115:

Way forward: - RAN4 has discussed the two following points: - For non-SBFD-aware UE, the “uplink symbols transmitted” used for uplink duty cycle evaluation are symbols with uplink transmission; - For SBFD-aware UE, the symbols used for uplink duty cycle evaluation are symbols with uplink transmission, including the non-SBFD symbols and SBFD symbols with UL transmission. - FFS RAN4 specification impact by taking into account the above understanding for SBFD-aware UE.

- Proposals on the necessity to update for HPUE duty cycle evaluation:

  - Option 1 (ZTE, CATT): Needed.

  - Option 2 (Samsung, vivo, Qualcomm, Ericsson): No further clarification/change is needed.

- Proposed compromised WF:

  - Proposal 1 (Samsung): RAN4 recognize the followings as RAN4 common understanding (which can be captured as RAN4 agreement):

    - For non-SBFD-aware UE, the “uplink symbols transmitted” used for uplink duty cycle evaluation are symbols with uplink transmission;

    - For SBFD-aware UE, the symbols used for uplink duty cycle evaluation are symbols with uplink transmission, including the non-SBFD symbols and SBFD symbols with UL transmission.

  - Proposal 2 (vivo): It is suggested to leave the issue for HPUE duty cycle evaluation to UE RF maintenance part if company wants to further clarify the wording.

- Relevant observations to this issue:

  - Observation 1 (Samsung): The existing TDD UE TX RF requirement is specified only on active UL slots.

  - Observation 2 (Samsung): Further clarification on “the symbols used for uplink duty cycle evaluation” will not change anything for UE conformance requirement.

  - Observation 3 (vivo): The term ‘uplink symbols transmitted’ as a whole, not ‘uplink symbols’ standalone, means ‘uplink time’, ‘uplink transmission time’ or ‘uplink transmission’ during the evaluation period for HPUE SAR.

  - Observation 4 (vivo): The HPUE duty cycle evaluation is based on UL transmissions carried by the UE and not only on the symbol assignment by the network.

  - Observation 5 (Qualcomm): Duty cycle evaluation considers UL transmissions by a (SBFD-aware or non SBFD-aware) UE independent if such UL transmissions were based on UL symbols, flexible symbols assigned to UL by the network, or UL subband of an SBFD symbol.

  - Observation 6 (Ericsson): Any UL symbols transmitted, whether TDD UL symbols or SBFD symbols—are considered as UL symbols for the purpose of UL duty cycle evaluation.

  - Observation 7 (Huawei): Might be no harm to further update the spec as e.g. “uplink symbols and SBFD symbols transmitted in a certain evaluation period” with the intention of better implementing max duty cycle evaluation for an SBFD-aware UE max duty cycle evaluation.

- Moderator Recommendation:

  - Suggest the use proposed compromised WF to solve this issue: Proposal 1 (Samsung) and Proposal 2 (vivo)

  - Note: Currently TS 38.101-1/2/3 are not listed as impacted spec for Rel-19 SBFD WI.

##### Issue 2-1-2: If the update is needed, how to introduce in RAN4 spec

- Detailed proposals for how to update for HPUE duty cycle evaluation, if the update is confirmed to be required:

  - Option 1: Patching to clause 6.1 to simplify the spec change

    - Option 1a (ZTE): Simplify the spec change, and propose to add the following two sentence in the general section 6.1.

      - For non-SBFD-aware UE, the “uplink symbols transmitted” used for uplink duty cycle evaluation are symbols with uplink transmission;

      - For SBFD-aware UE, the symbols used for uplink duty cycle evaluation are symbols with uplink transmission, including the non-SBFD symbols and SBFD symbols with UL transmission.

    - Option 1b (CATT): RAN4 to add a clarification sentence under sub-clause 6.1 as “For an SBFD-aware UE, uplink transmissions in SBFD and non-SBFD symbols shall be counted as uplink symbols when evaluating the uplink duty cycle”.

- Moderator Recommendation:

  - Depends on the outcome Issue 2-1-1.

  - Note: Currently TS 38.101-1/2/3 are not listed as impacted spec for Rel-19 SBFD WI.

# Topic #3: Analysis on UE-UE CLI problem and potential solutions

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2511538 | ISSDU | Observation 1: The rightmost sub-band of the left-hand operator must be DL ⇒ its configuration must end in “D” (i.e., UD or DUD). Observation 2: The leftmost sub-band of the right-hand operator must be DL ⇒ its configuration must begin with “D” (i.e., DU or DUD). Observation 3: The DL sub-bands of both operators are relatively wide, so the RF filter requirements are less stringent compared to the DUD case. Observation 4: In the UD/DU configuration, both UL sub-bands are located on the outer sides. Observation 5: The right-side operator uses the DUD configuration, in which each DL sub-band is narrower compared to those in DU and UD configurations. Observation 6: The UD configuration still requires restrictions on the adjacent third-party operator, because the UL sub-band is located on the outer edge. Observation 7: The left-side operator uses the DUD configuration, in which each DL sub-band is narrower compared to those in DU and UD configurations. Observation 8: The DU configuration requires restrictions on the adjacent third-party operator, because its UL sub-band is located on the outer edge. Observation 9: Both side operators use the DUD configuration, in which each DL sub-band is narrower compared to those in DU and UD configurations. Observation 10: No special restrictions are needed on the adjacent third-party operators’ frequency allocations. Proposal 1: we propose four deployment configurations that satisfy the DL–DL boundary requirement: - UD / DU - UD / DUD - DUD / DU - DUD / DUD Proposal 2: RAN4 to restrict the frequency allocations of the third-party operator on both adjacent sides in the UD / DU scenario. Proposal 3: RAN4 to restrict the frequency allocations of the third-party operator on the left adjacent side in the UD / DUD scenario. Proposal 4: RAN4 to restrict the frequency allocations of the third-party operator on the right adjacent side in the DUD / DU scenario. Proposal 5: RAN4 to specify that no uplink restrictions from third-party adjacent-band operators are required in the DUD / DUD scenario. |
| R4-2509120 | Charter Communications, Inc | Observation 1: The UE-UE CLI was concluded in Rel-18 SI and TR 38.858 clause 11.3.1: TDD DL throughput degradation is observed at cell edge due to inter-UE CLI. The TDD DL throughput degradation is up to 36% based on the simulation results [1]. Proposal 1: We propose the following two ACLR models for aggressor UEs in the SBFD network: • Option 1 (baseline): Flat model (ACLR = 30 dBc) as specified in TS 38.101-1 and used in TR 38.858. • Option 2: Stepwise model (ACLR = 43 dBc for the DUD case) based on TR 36.942. Proposal 2: In the SBFD DUD configuration, the UL sub-band spans both the ACS1 (33 dBc) zone and the ACS2 (34.3 dBc) zone. By aggregating the power contributions from both zones, an effective ACS value of 33.6 dBc (ACS1.5) is recommended for use in the stepwise ACS model in the TDD/SBFD DUD coexistence study. Proposal 3: We propose the following two ACS models for victim UEs in the legacy TDD network: • Option 1 (baseline): Flat model (ACS = 33 dBc) as specified in TS 38.101-1 and used in TR 38.858. • Option 2: Stepwise model (ACS = 33.6 dBc) based on TR 36.942 and Proposal 2. Proposal 4: We propose the following two ACIR models for evaluating UE-UE CLI derived from the ACLR models in Proposal 1 and the ACS models in Proposal 3, with more details listed in Table 1: • Option 1 (baseline): Flat model (ACIR = 32.7 dBc for UE-UE CLI) based on in TS 38.101-1 and used in TR 38.858. • Option 2: Stepwise model (ACIR = 33.1 dBc for UE-UE CLI) based on TR 36.942. Proposal 5: A 5% throughput degradation threshold is recommended as the criteria for determining whether the UE-UE CLI issue is present. Observation 2: The simulation results confirm that the UE-UE CLI issue is present in both cases: the flat ACIR model, with a 27.7% throughput degradation at the cell edge; and the stepwise model with a 11.6% throughput degradation at the cell edge. Observation 3: The simulations indicate that GS variation has minimal effect on UE-UE CLI, with all tested GS values causing TDD DL throughput degradation well above the 5% threshold. Therefore, RAN4 could focus on 100% GS for the remainder of the study and prioritize assessing other assumptions. Proposal 6: To protect the victim TDD DL without causing significantly degradation to the SBFD UL, only a subset of SBFD UEs that potentially generating harmful UE-UE CLI should undergo Pmax reduction. In the coexistence simulation, an X-dB Pmax reduction is applied exclusively to SBFD UEs located within the same cluster as the victim TDD UEs. Observation 4: For the flat ACIR model, an X range of 0 to 5.5 dB is recommended, based on the criteria of 5% throughput degradation at the victim TDD DL cell edge. Observation 5: For the stepwise ACIR model, an X range of 0 to 3.5 dB is recommended, based on the criteria of 5% throughput degradation at the victim TDD DL cell edge. Proposal 7: Identify a unique Pmax parameter for SBFD and include this parameter in the specifications. Proposal 8: Define an X factor for Pmax reduction based on the simulation results. We can decide to add a range, for example, [0-6] dB. |
| R4-2510417 | Qualcomm Germany | Observation 1: One of the interference sources of the Urban Hotspot scenario is the inter-UE CLI from SBFD-aware UEs transmitting in the UL subband towards the adjacent legacy TDD UEs receiving DL transmissions. Observation 2: To investigate the inter-UE CLI case in an extreme scenario, we consider each victim shares the cluster, in which UEs are dropped, with an aggressor cell. Observation 3: Based on Annex E assumptions, UEs falling in the same cluster will not experience any penetration losses, which ignores the fact that there might be additional losses due to wall penetration or UEs scheduled being in different rooms/ floors. Observation 4: Flat ACIR model with minimum RF requirements for UE ACLR/ACS can lead to over-estimation of the inter-UE CLI. Observation 5: Based on assumptions given in Annex E TR 38.858, along with flat baseline ACLR/ACS with minimum RF requirements, no penetration loss between UEs in the same cluster and 0 % grid shift, we have observed throughput loss due to inter-UE CLI when SBFD is operating in the adjacent channel. Observation 6: The throughput degradation due to inter-UE CLI is sensitive to the UE deployment and clustering assumption as well as the % of UEs that are dropped indoor/ within a cluster. Observation 7: There exists no methodology to identify the set of interfering SBFD-aware UEs to the victim TDD UEs. Observation 8: With p-Max reduction, TP-loss reduction can be achieved due to lower inter-UE CLI interference, however, such reduction comes with the cost of SBFD UL network performance degradation. Observation 9a: If this solution is agreed by RAN4, more analysis is needed to derive meaningful X dB values for the different power classes for FR1 deployments. Observation 9b: If this solution is agreed by RAN4, it has to be UE configured, in order not to impact all SBFD-aware UEs. Observation 10: Mandating DUD configuration might be limiting to SBFD deployments. Observation 11: Underlying assumptions on the SBFD configuration affect the experienced inter-UE CLI. Adopting ACIR2 or ACIR3 in the simulation provided gains in terms of inter-UE CLI reduction. Observation 12: From our RAN1 studies, simple CLI-aware scheduler resulted in up to 90% recovery of DL performance degradation from inter-UE CLI. Observation 13: Summary of the cost/ gain analysis of the different solutions is as follows: |
| R4-2511106 | Huawei, HiSilicon | Observation 1: For Solution-1, the basis of aggressor UE identification is absent, because neither Rel-16 nor Rel-19 RRM CLI measurement mechanism can support adjacent frequency measurement. Observation 2: If the network can only do a rough selection on aggressor, then the cost of network coverage performance must be expensive over a very limited gain on UE-UE CLI reduction. Observation 3: For co-ex study, the ACIR generated by flat model reflects the worst case, while stepwise model can be relatively closer to real implementation and somehow shows the possibility on UE-UE CLI handling by frequency selective scheduling. Observation 4: MSD issue is somehow similar with UE-UE CLI issue, but spec didn’t introduce any Tx power limitation but only a REFSENS exemption as a reference for deployment/scheduling. Proposal 1: In Rel-19, do not specify Solution-1 in Rel-19. How to accommodate UE-UE CLI can be up to gNB implementation. |
| R4-2511138 | ZTE Corporation, Sanechips | Observation 1: to reduce the transmission power by 3-4dB for the aggressor UE would almost halve the coverage gain achieved through the SBFD BS configuration, rendering the overall improvement in uplink performance minimal. Observation 2: for 20MHz uplink subband configuration with 100MHz carrier, the ACLR3 60dBc should considered for DUD case, ACLR5 80dBc should be considered for UD case. Proposal 1: from the receiver side of victim UE in the TDD bands, the roll off of ACS should be considered with the realistic digital filter assumption. Proposal 2: from UE2UE CLI analysis, the UE antenna efficiency should be considered in the simulation evaluation. |
| R4-2511251 | Samsung | Co-existence analysis for UE-to-UE CLI Problem Observation 1: In TR 36.942, the UE ACLR was assumed to be represented by 3-step model, i.e., ACLR1 as 30+X, ACLR2 as 43+X, and ACLR3 as higher than 50+X. Observation 2: In TR 36.942, the UE ACS was assumed with 3 steps as ACS1 as 33dBc, ACS2 as 34.3dBc and ACS3 as 46.3dBc, and each step assumed as half of UE channel bandwidth. Proposal 1: RAN4 UE-to-UE CLI simulation adopt the step-wise ACLR and ACS models referenced in TR 36.942 as the baseline. Observation 3: By applying the proposed step-wise ACLR/ACS model, the resulting ACIR for this SBFD DUD UE-UE CLI case is 33.6 dBc. Observation 4: By following the other company proposed cluster dropping method, the cluster will not cross the sector borders, thus limiting the indoor UE to be distributed not in all practical deployment locations. Observation 5: The cluster dropping methodology could only be valid for 100% grid shift case, and to use it in other grid shift values can be problematic. Proposal 2: For the Urban Hotspot scenario, only 100% grid shift should be considered. Observation 6: Directly compared to 5% throughput degradation criteria, with baseline assumption, the SBFD UL UE would create performance degradation to the TDD DL UE in Urban Hotspot scenario at cell-edge, while the impact on cell-average is well acceptable. Observation 7: The Urban Hotspot scenario generally looks at the cases which happens with 0.5% (baseline) and 8% (optional) probability under the normal UMa co-ex study in RAN4. Observation 8: By adding one penetration loss, depending on the UE-UE distance, NLOS probability and random drop of UE height, the UE-UE CLI impact under this Urban Hotspot scenario already falls into the acceptable range. Proposal 3: The UE-to-UE CLI issue is generally not identified under this Urban Hotspot scenario by considering practical assumptions. The SBFD UL UE to TDD DL UE CLI may exist in some corner cases, which strongly depends on the simulation assumptions. Co-existence analysis for UE-UE CLI handling solutions Observation 9: Reducing the Pmax of SBFD UL transmission would achieve the adjacent channel interference reduction and accordingly TP degradation reduction, with the basic assumption of Urban Hotspot and applying p-max reduction to all SBFD UEs. Observation 10: By applying X=3 dB pmax reduction to all SBFD UEs, the overall throughput of this operator in two carriers, including SBFD and TDD carriers, would decrease. Observation 11: Network scheduling based method by simply scheduling victim and aggressor UE with enough frequency separation (regarded one implementation for Solution 3) cans till reduce the adjacent channel UE-to-UE CLI significantly. Comparison of UE-UE CLI handling solutions Proposal 4: RAN4 shall capture the below summary comparison table as agreement for different solutions to UE-to-UE CLI problem: Proposal 5: RAN4 conclude the UE-to-UE CLI problem (i.e., intra-operator adjacent-channel inter-cell UE-to-UE CLI, in Urban Hotspot Scenario): - UE-to-UE CLI issue is generally not identified by considering practical assumptions. - The SBFD UL UE to TDD DL UE CLI may exist in some corner cases, which strongly depends on the simulation assumptions. - Solutions identified for mitigate this UL-to-UL CLI problem, while pros/cons and standard impacts are analyzed. |
| R4-2511314 | Ericsson | Observation 1 For Scenario 2/Case 1, with GS 100%, coexistence of an SBFD Macro network with a DL legacy TDD Macro network, imposes a 31.4% degradation to the TDD network’s DL 5%tile throughput due to UE-to-UE CLI, assuming baseline ACIR and 23 dBm of maximum UE transmission power. The mean user throughput remains largely unaffected. Observation 2 For Scenario 2/Case 1, with GS 100%. reducing the UE maximum transmission power by 8 dB (down to 15 dBm), results in a degradation of 13%. Observation 3 For Scenario 2/Case 1, increasing the SBFD ACLR to 43 dB significantly reduces the TDD DL 5%tile throughput degradation compared to an ACLR of 30 dB. However, the degradation remains significant at approximately 19%, depending on the TDD UE ACS assumption. Reducing the UE maximum transmission power by up to 8 dB further mitigates the degradation to around 9%. Observation 4 For Scenario 2/Case 1, with GS 5%, coexistence of an SBFD Macro network with a DL legacy TDD Macro network imposes a 21.4% degradation to the TDD network’s DL 5%tile throughput due to UE-to-UE CLI, assuming baseline ACIR and 23 dBm of maximum UE transmission power. The mean user throughput remains largely unaffected. Observation 5 For Scenario 2/Case 1, with GS 5%. reducing the UE maximum transmission power by 8 dB (down to 15 dBm), results in a degradation of 10%. Observation 6 For Scenario 2/Case 1, with GS 5%, TDD network’s DL 5%tile throughput experiences less degradation compared to GS 100% due to UE-to-UE CLI the impact. The reason is that GS 100% represents a worst-case scenario for DL, so that the DL of TDD results weaker and more sensitive to the UE-to-UE CLI when GS is 100%. Observation 7 For Scenario 2/Case 1, increasing the SBFD ACLR to 43 dB significantly reduces the TDD DL 5%tile throughput degradation compared to an ACLR of 30 dB. However, the degradation remains significant ranging from 13.1% to 15.6% depending on the TDD UE ACS assumption. Reducing the UE maximum transmission power by 8 dB further mitigates the degradation to approximately 7%. |
| R4-2511530 | Nokia | Observation 1: On the scenario of interest for the UE-to-UE CLI problem, Solution-1 to Solution-4 for CLI handling are relevant regardless of whether it is an intra-operator or inter-operator scenario. In other words, the assumption of intra-operator scenario doesn’t bring any significant technical advantages for handling the UE-to-UE CLI compared to the inter-operator scenario studied in Release 18. Observation 2: Current UE-to-UE CLI measurements only support co-channel CLI measurements, and adjacent channel aggressor UE identification is currently not feasible. The introduction of measurements across channels requires additional efforts across multiple working groups. Observation 3: The urban-hotspot scenario is the scenario used to evaluate the UE-to-UE CLI and the potential solutions. Observation 4: In an urban-hotspot scenario, the presence of UE-to-UE CLI in the TDD DL operator due to its adjacent SBFD operator degrades the DL performance. Observation 5: Reducing Pmax for SBFD UEs up to 6 dB is impacting the performance of SBFD while not fully solving the UE-to-UE CLI problem. Proposal 1: Assume a single value of 100% grid shift for the UE-to-UE CLI evaluations to reduce the simulations effort. Proposal 2: Use flat ACLR model of 30 dBc and a step-wise ACLR model of 50 dBc for evaluating the UE-to-UE CLI. Proposal 3: Use a flat ACS model of 33 dBc and a step-wise ACS model of 33.6 dBc for evaluating the UE-to-UE CLI. |

## Open issues summary

### Sub-topic 3-1: Co-existence assumptions and analysis for UE-to-UE CLI problem

- [Background] UE-to-UE CLI problem statement: - The scenario to be considered: - Intra-operator adjacent-channel inter-cell (i.e., both channels belong to a single operator) - Use Urban Hotspot -> Urban Hotspot Scenario 2 as reference - FFS non-located assumption is always valid or not - detailed description provided in TR38.858 - The case to be considered: - SBFD-aware UE (aggressor) UL transmission to NR TDD UE (victim) DL reception

- WF-1: Companies are encouraged to provided analysis on the above UE-to-UE CLI problem: - The purpose is to identify whether the above-mentioned UE-to-UE CLI problem exists or not in the practical deployment - One evaluation method is to re-perform RAN4 Rel-18 co-existence study on the above scenario/case - Parameters agreed in Rel-18 will be reused unless difference identified - FFS impact from different grid shift values - FFS impact from ACLR model

#### Issue 3-1-1: ACLR/ACS step-wise and antenna model

- Proposals from Charter:

  - Proposal 1 (Charter): We propose the following two ACLR models for aggressor UEs in the SBFD network:

    - Option 1 (baseline): Flat model (ACLR = 30 dBc) as specified in TS 38.101-1 and used in TR 38.858.

    - Option 2: Stepwise model (ACLR = 43 dBc for the DUD case) based on TR 36.942.

  - Proposal 2 (Charter): We propose the following two ACS models for victim UEs in the legacy TDD network:

    - Option 1 (baseline): Flat model (ACS = 33 dBc) as specified in TS 38.101-1 and used in TR 38.858.

    - Option 2: Stepwise model (ACS = 33.6 dBc) based on TR 36.942 and Proposal 2.

  - Proposal 3 (Charter): We propose the following two ACIR models for evaluating UE-UE CLI derived from the ACLR models in Proposal 1 and the ACS models in Proposal 3, with more details listed in Table 1:

    - Option 1 (baseline): Flat model (ACIR = 32.7 dBc for UE-UE CLI) based on in TS 38.101-1 and used in TR 38.858.

    - Option 2: Stepwise model (ACIR = 33.1 dBc for UE-UE CLI) based on TR 36.942.

- Proposals from Samsung:

  - Proposal 1 (Samsung): RAN4 UE-to-UE CLI simulation adopt the step-wise ACLR and ACS models referenced in TR 36.942 as the baseline.

    - In TR 36.942, the UE ACLR was assumed to be represented by 3-step model, i.e., ACLR1 as 30+X, ACLR2 as 43+X, and ACLR3 as higher than 50+X.

    - In TR 36.942, the UE ACS was assumed with 3 steps as ACS1 as 33dBc, ACS2 as 34.3dBc and ACS3 as 46.3dBc, and each step assumed as half of UE channel bandwidth.

    - By applying the proposed step-wise ACLR/ACS model, the resulting ACIR for this SBFD DUD UE-UE CLI case is 33.6 dBc.

- Proposals from Qualcomm:

  - Observation 1 (Qualcomm): Flat ACIR model with minimum RF requirements for UE ACLR/ACS can lead to over-estimation of the inter-UE CLI.

- Proposals from Huawei:

  - Observation 1 (Huawei): For co-ex study, the ACIR generated by flat model reflects the worst case, while stepwise model can be relatively closer to real implementation and somehow shows the possibility on UE-UE CLI handling by frequency selective scheduling.

- Proposals from ZTE:

  - Observation 1 (ZTE): for 20MHz uplink subband configuration with 100MHz carrier, the ACLR3 60dBc should considered for DUD case, ACLR5 80dBc should be considered for UD case.

  - Proposal 1 (ZTE): from the receiver side of victim UE in the TDD bands, the roll off of ACS should be considered with the realistic digital filter assumption.

  - Proposal 2 (ZTE): from UE2UE CLI analysis, the UE antenna efficiency should be considered in the simulation evaluation.

- Proposals from Nokia:

  - Proposal 1 (Nokia): Use flat ACLR model of 30 dBc and a step-wise ACLR model of 50 dBc for evaluating the UE-to-UE CLI

  - Proposal 2 (Nokia): Use a flat ACS model of 33 dBc and a step-wise ACS model of 33.6 dBc for evaluating the UE-to-UE CLI.

- Moderator Recommendation:

  - Based on multiple companies’ observations, conclude that at least stepwise ACLR/ACS model needs to be considered.

#### Issue 3-1-2: Grid-shifts and the clusters

- Observations/Proposals:

  - Proposal 1 (Samsung): For the Urban Hotspot scenario, only 100% grid shift should be considered.

    - By following the other company proposed cluster dropping method, the cluster will not cross the sector borders, thus limiting the indoor UE to be distributed not in all practical deployment locations.

    - The cluster dropping methodology could only be valid for 100% grid shift case, and to use it in other grid shift values can be problematic.

  - Proposal 2 (Nokia): Assume a single value of 100% grid shift for the UE-to-UE CLI evaluations to reduce the simulations effort.

  - Observation 1 (Qualcomm): To investigate the inter-UE CLI case in an extreme scenario, we consider each victim shares the cluster, in which UEs are dropped, with an aggressor cell.

  - Observation 3 (CableLabs and Charter): The simulations indicate that GS variation has minimal effect on UE-UE CLI, with all tested GS values causing TDD DL throughput degradation well above the 5% threshold. Therefore, RAN4 could focus on 100% GS for the remainder of the study and prioritize assessing other assumptions.

- Moderator Recommendation:

  - Based on multiple companies’ observations, conclude 100% grid shift can be considered as baseline.

#### Issue 3-1-3: Preliminary results of co-existence analysis for UE-to-UE CLI problem

- Observations/Proposals for baseline assumption:

  - Proposals from Charter:

    - Proposal 5: A 5% throughput degradation threshold is recommended as the criteria for determining whether the UE-UE CLI issue is present.

    - Observation 1: The simulation results confirm that the UE-UE CLI issue is present in both cases: the flat ACIR model, with a 27.7% throughput degradation at the cell edge; and the stepwise model with a 11.6% throughput degradation at the cell edge.

    - Observation 2: The simulations indicate that GS variation has minimal effect on UE-UE CLI, with all tested GS values causing TDD DL throughput degradation well above the 5% threshold. Therefore, RAN4 could focus on 100% GS for the remainder of the study and prioritize assessing other assumptions.

  - Proposal 1 (Samsung): The UE-to-UE CLI issue is generally not identified under this Urban Hotspot scenario by considering practical assumptions. The SBFD UL UE to TDD DL UE CLI may exist in some corner cases, which strongly depends on the simulation assumptions.

Table 2.2-1. Results for the UE-to-UE CLI with baseline assumption

| Grid shift | SINR degradation @5% in dB | SINR degradation @50% in dB | TP degradation @5% in % | TP degradation @Mean in % |
| --- | --- | --- | --- | --- |
| 100% | 1.18 | 0.73 | 11.63 | 2.64 |

    - Directly compared to 5% throughput degradation criteria, with baseline assumption, the SBFD UL UE would create performance degradation to the TDD DL UE in Urban Hotspot scenario at cell-edge, while the impact on cell-average is well acceptable.

    - The Urban Hotspot scenario generally looks at the cases which happens with 0.5% (baseline) and 8% (optional) probability under the normal UMa co-ex study in RAN4.

  - Observation 1 (Qualcomm):

    - Based on assumptions given in Annex E TR 38.858, along with flat baseline ACLR/ACS with minimum RF requirements, no penetration loss between UEs in the same cluster and 0 % grid shift, we have observed throughput loss due to inter-UE CLI when SBFD is operating in the adjacent channel.

    - The throughput degradation due to inter-UE CLI is sensitive to the UE deployment and clustering assumption as well as the % of UEs that are dropped indoor/ within a cluster.

  - Observation 2 (Nokia):

    - In an urban-hotspot scenario, the presence of UE-to-UE CLI in the TDD DL operator due to its adjacent SBFD operator degrades the DL performance.

  - Observations from Ericsson:

    - Observation 1	For Scenario 2/Case 1, with GS 100%, coexistence of an SBFD Macro network with a DL legacy TDD Macro network, imposes a 31.4% degradation to the TDD network’s DL 5%tile throughput due to UE-to-UE CLI, assuming baseline ACIR and 23 dBm of maximum UE transmission power. The mean user throughput remains largely unaffected.

    - Observation 2	For Scenario 2/Case 1, increasing the SBFD ACLR to 43 dB significantly reduces the TDD DL 5%tile throughput degradation compared to an ACLR of 30 dB. However, the degradation remains significant at approximately 19%, depending on the TDD UE ACS assumption.

    - Observation 3	For Scenario 2/Case 1, with GS 5%, coexistence of an SBFD Macro network with a DL legacy TDD Macro network imposes a 21.4% degradation to the TDD network’s DL 5%tile throughput due to UE-to-UE CLI, assuming baseline ACIR and 23 dBm of maximum UE transmission power. The mean user throughput remains largely unaffected.

    - Observation 4	For Scenario 2/Case 1, with GS 5%, TDD network’s DL 5%tile throughput experiences less degradation compared to GS 100% due to UE-to-UE CLI the impact. The reason is that GS 100% represents a worst-case scenario for DL, so that the DL of TDD results weaker and more sensitive to the UE-to-UE CLI when GS is 100%.

    - Observation 5	For Scenario 2/Case 1, increasing the SBFD ACLR to 43 dB significantly reduces the TDD DL 5%tile throughput degradation compared to an ACLR of 30 dB. However, the degradation remains significant ranging from 13.1% to 15.6% depending on the TDD UE ACS assumption.

- Observations/Proposals for additional penetration loss within the cluster:

  - Observation 1 (Qualcomm): Based on Annex E assumptions, UEs falling in the same cluster will not experience any penetration losses, which ignores the fact that there might be additional losses due to wall penetration or UEs scheduled being in different rooms/ floors.

  - Observation 2 (Samsung):

Table 2.2-2. Results for the UE-to-UE CLI with optional assumptions

| Grid shift | Penetration loss material | SINR degradation @5% in dB | SINR degradation @50% in dB | TP degradation @5% in % | TP degradation @Mean in % |
| --- | --- | --- | --- | --- | --- |
| 100% | Wooden | 0.76 | 0.51 | 7.64 | 1.56 |
|  | Concrete | 0.21 | 0.15 | 2.14 | 0.47 |

    - By adding one penetration loss, depending on the UE-UE distance, NLOS probability and random drop of UE height, the UE-UE CLI impact under this Urban Hotspot scenario already falls into the acceptable range.

- Moderator Recommendation:

  - Different views on the severity level of this UE-to-UE CLI problem.

### Sub-topic 3-2: Co-existence and other analysis for UE-UE CLI handling solutions

- WF-2: Companies are encouraged to identify potential solution(s) to the above UE-to-UE CLI problem: - Solution-1: Reduced p-max for SBFD-aware UE TX power by X dB. - the reduced p-max is configurable per-UE - FFS X dB is a single value or a configurable value or a configurable range

#### Issue 3-2-1: Result for UE-UE CLI handling solution-1 (reduced p-max for SBFD-aware UE)

- Observations from Charter (for applying reduced p-max to selected interfering SBFD-aware UE):

  - Proposal 1: To protect the victim TDD DL without causing significantly degradation to the SBFD UL, only a subset of SBFD UEs that potentially generating harmful UE-UE CLI should undergo Pmax reduction. In the coexistence simulation, an X-dB Pmax reduction is applied exclusively to SBFD UEs located within the same cluster as the victim TDD UEs.

  - Observation 1: For the flat ACIR model, an X range of 0 to 5.5 dB is recommended, based on the criteria of 5% throughput degradation at the victim TDD DL cell edge.

  - Observation 2: For the stepwise ACIR model, an X range of 0 to 3.5 dB is recommended, based on the criteria of 5% throughput degradation at the victim TDD DL cell edge.

- Observations from Samsung (for applying reduced p-max to all SBFD-aware UE):

  - Observation 1 (Samsung): Reducing the Pmax of SBFD UL transmission would achieve the adjacent channel interference reduction and accordingly TP degradation reduction, with the basic assumption of Urban Hotspot and applying p-max reduction to all SBFD UEs.

  - Observation 2 (Samsung): By applying X=3 dB pmax reduction to all SBFD UEs, the overall throughput of this operator in two carriers, including SBFD and TDD carriers, would decrease.

- Observations from Qualcomm:

  - Observation 1 (Qualcomm): There exists no methodology to identify the set of interfering SBFD-aware UEs to the victim TDD UEs.

  - Observation 2 (Qualcomm): With p-Max reduction, TP-loss reduction can be achieved due to lower inter-UE CLI interference, however, such reduction comes with the cost of SBFD UL network performance degradation. (co-existence study by assuming that an operator is able to identify aggressor UEs)

  - Observation 3a (Qualcomm): If this solution is agreed by RAN4, more analysis is needed to derive meaningful X dB values for the different power classes for FR1 deployments.

  - Observation 3b (Qualcomm): If this solution is agreed by RAN4, it has to be UE configured, in order not to impact all SBFD-aware UEs.

- Observations from Huawei:

  - Observation 1 (Huawei): For Solution-1, the basis of aggressor UE identification is absent, because neither Rel-16 nor Rel-19 RRM CLI measurement mechanism can support adjacent frequency measurement.

  - Observation 2 (Huawei): If the network can only do a rough selection on aggressor, then the cost of network coverage performance must be expensive over a very limited gain on UE-UE CLI reduction.

- Observations from ZTE:

  - Observation 1 (ZTE): to reduce the transmission power by 3-4dB for the aggressor UE would almost halve the coverage gain achieved through the SBFD BS configuration, rendering the overall improvement in uplink performance minimal.

- Observations from Nokia:

  - Observation 1 (Nokia): Observation 5:	Reducing Pmax for SBFD UEs up to 6 dB is impacting the performance of SBFD while not fully solving the UE-to-UE CLI problem.

- Observations from Ericsson:

  - Observation 1	For Scenario 2/Case 1, with GS 100%. reducing the UE maximum transmission power by 8 dB (down to 15 dBm), results in a degradation of 13%.

  - Observation 2	For Scenario 2/Case 1, increasing the SBFD ACLR to 43 dB significantly reduces the TDD DL 5%tile throughput degradation compared to an ACLR of 30 dB. However, the degradation remains significant at approximately 19%, depending on the TDD UE ACS assumption. Reducing the UE maximum transmission power by up to 8 dB further mitigates the degradation to around 9%.

  - Observation 3	For Scenario 2/Case 1, with GS 5%. reducing the UE maximum transmission power by 8 dB (down to 15 dBm), results in a degradation of 10%.

  - Observation 4	For Scenario 2/Case 1, increasing the SBFD ACLR to 43 dB significantly reduces the TDD DL 5%tile throughput degradation compared to an ACLR of 30 dB. However, the degradation remains significant ranging from 13.1% to 15.6% depending on the TDD UE ACS assumption. Reducing the UE maximum transmission power by 8 dB further mitigates the degradation to approximately 7%.

- Moderator Recommendation:

  - Further discussion is needed for solution 1, and Issue 3-4-1 can be discussed together.

#### Issue 3-2-2: Result for UE-UE CLI handling solution-2 (DUD SBFD configuration)

- Observations from Qualcomm (co-existence study):

  - Observation 1: Underlying assumptions on the SBFD configuration affect the experienced inter-UE CLI. Adopting ACIR2 or ACIR3 in the simulation provided gains in terms of inter-UE CLI reduction.

- In RAN4#115, one contribution analysed the SBFD configuration for frequency neighboring operators, and possible configuration by considering 3rd-party operators. Proposals from ISSDU:

  - Proposal 1: we propose four deployment configurations that satisfy the DL–DL boundary requirement:

    - UD / DU

    - UD / DUD

    - DUD / DU

    - DUD / DUD

  - Proposal 2: RAN4 to restrict the frequency allocations of the third-party operator on both adjacent sides in the UD / DU scenario.

  - Proposal 3: RAN4 to restrict the frequency allocations of the third-party operator on the left adjacent side in the UD / DUD scenario.

  - Proposal 4: RAN4 to restrict the frequency allocations of the third-party operator on the right adjacent side in the DUD / DU scenario.

  - Proposal 5: RAN4 to specify that no uplink restrictions from third-party adjacent-band operators are required in the DUD / DUD scenario.

- Moderator Recommendation:

  - It is good analysis for SBFD configuration restriction especially considering coexistence scenarios, and it gives good guidance for UE-to-UE CLI handling solution-2.

  - FFS how to capture the proposals in the UE-to-UE CLI handling comparison conclusion.

#### Issue 3-2-3: Result for UE-UE CLI handling solution-3 (Scheduling-oriented solution)

- Observations from Qualcomm:

  - Observation 1: From our RAN1 studies, simple CLI-aware scheduler resulted in up to 90% recovery of DL performance degradation from inter-UE CLI.

- Moderator Recommendation:

  - Check the conclusion for Issue 3-3-1.

#### Issue 3-2-4: Result for UE-UE CLI handling new solution (by scheduling victim UEs further apart in frequency domain)

- Observations from Samsung:

  - Observation 1 (Samsung): Network scheduling based method by simply scheduling victim and aggressor UE with enough frequency separation (regarded one implementation for Solution 3) cans till reduce the adjacent channel UE-to-UE CLI significantly.

- Moderator Recommendation:

  - Check the conclusion for Issue 3-3-1.

### Sub-topic 3-3: Comparison of UE-to-UE CLI handling solutions

#### Issue 3-3-1: Collected views for UE-to-UE CLI handling solution comparison

- [Moderator] In RAN4#115, companies are encouraged to compare different CLI handling solutions:

- WF-3: Companies are encouraged to provide analysis on solutions, by at least considering the following aspects: - Benefit(s): - At least for Solution-1, one evaluation method is to perform RAN4 co-existence study on the above scenario/case, by using XdB lower maximum UE power for aggressor UE(s) - FFS how to determine one SBFD-aware UE is aggressor UE - The value of X can be chosen from the range of [1, 5] and other values may not be precluded. - How to determine the value of X or the range can be FFS - Baseline to be compared to is Rel-18 coexistence study re-performed in WF-1 - Detailed parameters can be further discussed by conference call and/or email discussion before RAN4#116. - Other methods are not precluded - Limitation(s), e.g., decreased coverage for certain SBFD-aware UE, etc. - Expected RAN4 requirement impact, including - the impact to TS 38.104 - the impact to TS 38.101 series, e.g., Pcmax, etc. - Expected RAN2 requirement impact: - whether new or changed RAN2 signaling(s) is required: - Expected RAN3 requirement impact: - whether new or changed RAN3 signaling(s) is required:

- Views collected based on agreed table for comparison:

  - Green part means the item received the supports from all relevant companies’ Tdocs.

  - The below table will be further discussed based on additional inputs from other companies.

|  | Expected benefit(s) | Expected limitation(s) | Expected RAN4 requirement impact | Expected RAN2 requirement impact | Expected RAN3 requirement impact |
| --- | --- | --- | --- | --- | --- |
| Solution-1 (Reduced p-max for SBFD-aware UE TX power by X dB) | CLI mitigated | Samsung/Qualcomm /Nokia: Degradation in SBFD-UL network performance | Samsung/Nokia: At least configured transmitted power in Clause 6 of TS38.101-1/2/3 needs to be updated for SBFD Qualcomm: Derive max power reduction values for different UE power classes and modifications to TS 38.101-1. | Samsung/Nokia /Charter: new p-Max IE or power backoff dedicated for SBFD symbols: - FFS per cell or per UE Qualcomm: FFS on additional signaling needed to identify set of interfering SBFD-aware UEs. | Samsung: Impact expected, because Rel-19 UE-to-UE CLI measurement is only for intra-cell, and measurement reports can’t be shared over Xn interface Qualcomm: FFS on additional signaling needed to identify set of interfering SBFD-aware UEs. |
| Solution-2 (SBFD configuration is set by using DUD (40%, 20%, 40%), or DU (by putting UL subband away from victim channel) | CLI mitigated (by much improved ACS/ACLR performance for victim/aggressor UEs respectively) | Deployment restriction by not allowing certain SBFD configurations. Nokia: On its own, this solution might be insufficient to completely solve the UE-to-UE adjacent CLI problem | Samsung/Nokia: No impact (Rel-19 RAN4 specification would already include the requirements for these configurations) Qualcomm: Mandate a given SBFD configuration in RAN4 specs | No impact | No impact |
| Solution-3 (Avoid scheduling the aggressor UE) | CLI mitigated | Samsung: Scheduling restriction for not scheduling aggressor UE in the concerned SBFD symbols Qualcomm: Additional information exchange between BSs to avoid inter-UE CLI. Nokia: Reduction of UL coverage or cell-edge performance depending on the level of power reduction | No impact | Samsung/Nokia: No impact Qualcomm: FFS if additional signaling is needed between legacy and SBFD-capable BSs belonging to same/ different operator. | Samsung/Nokia: Impact expected, because Rel-19 UE-to-UE CLI measurement is only for intra-cell, and measurement reports can’t be shared over Xn interface Qualcomm: FFS if additional signaling is needed between legacy and SBFD-capable BSs belonging to same/ different operator. |
| Solution-4 (Configure the aggressor UE fallback to TDD operation) | Samsung: CLI mitigated Nokia: avoids completely UE to UE CLI | Samsung: Scheduling restriction to aggressor UE by fallback to TDD Nokia: no SBFD UL coverage benefit (i.e. same or worse performance than legacy TDD) | No impact | No impact | Samsung/Nokia: Impact expected, because Rel-19 UE-to-UE CLI measurement is only for intra-cell, and measurement reports can’t be shared over Xn interface |
| New solution (by scheduling victim UEs further apart in frequency domain) | CLI mitigated | Scheduling restriction to victim UE for DL reception | No impact | No impact | No impact |

- Moderator Recommendation:

  - Discussion Issue 3-4-1 to conclude the observations, and capture conclusions in WF.

### Sub-topic 3-4: Follow-up action for UE-to-UE CLI problem

#### Issue 3-4-1: Follow-up action for UE-to-UE CLI problem

- Proposals:

  - Proposal 1 (Huawei): In Rel-19, do not specify Solution-1 in Rel-19. How to accommodate UE-UE CLI can be up to gNB implementation.

  - Proposal 2 (Charter): Identify a unique Pmax parameter for SBFD and include this parameter in the specifications. Define an X factor for Pmax reduction based on the simulation results. We can decide to add a range, for example, [0-6] dB.

- Moderator Recommendation:

  - Discussion Issue 3-4-1 to conclude the observations, and capture conclusions in WF.

  - In Rel-19, no additional follow up for this UE-to-UE CLI issue, and WF can be referred for Rel-20 scope discussion in RAN-P if needed.

    - Considering the analyzed impacts to different working groups (analyzed by multiple companies), the solution involves multiple working groups’ involvement and RAN-P for deciding the scope.

# Topic #4: New BS RF requirements for SBFD

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2509323 | CATT | Proposal 1: The absolute limits for in-channel adjacent subband leakage of FR1 WA and MR SBFD-capable BSs can reuse the values specified for FR1 NR BS. Proposal 2: For in-channel adjacent sub-band blocking requirement, -32dBm can be used for MR BS and -34dBm can be used for LA BS. Proposal 3: If co-site inter-sector case is considered, the isolation assumption between two sectors BSs should be added as a standard note. |
| R4-2510962 | Ericsson | Observation 1 Operators cannot deploy SBFD selectively at just a few locations where UL coverage enhancement is desired, while other sites deploy regular TDD. Observation 2 Operators cannot deploy SBFD with varying subband configurations across sites. Observation 3 In-channel adjacent subband blocking requirement measures the interaction between sectors in the near field, and CLI handling schemes differ from what is studied in RAN1 and have not been studied so far. Observation 4 CLI handling schemes for the case of inter-site interference could suppress a portion of the interference, but they cannot completely eliminate it, because it is not always feasible, and RX digital beamforming cannot be considered. Observation 5 For in-channel adjacent subband blocking requirement, the interfering signals originate from the intra-operator inter-sector/inter-site DL subbands. Therefore, the bandwidth of the interfering signal should be equivalent to the DL subband, and its center frequency should align with the center of the corresponding DL subband. Proposal 1 Add a note in TS 38.104 where appropriate that “Operators shall deploy SBFD uniformly across all sites with exactly the same subband configuration.” Proposal 2 Reuse the ACLR absolute value as defined in Table 6.6.3.2-2 in TS 38.104 for FR1. Proposal 3 Define in-channel adjacent subband blocking requirement without considering CLI handling schemes. Proposal 4 For FR1 WA, a SBFD capable BS with 53 dBm TRP, specify -27 dBm as the requirement level for the SBFD in-channel adjacent subband blocking, on the premise that operators do proper site dimensioning design in real networks and achieve 80 dB spatial isolation between sectors. Proposal 5 Introduce a new symbol FC,SBFD,DL for SBFD DL subband, which is similar to FC used in ICS, to represent the reference point for interfering signal. Proposal 6 Use an NR DL signal with 30 kHz SCS and size specified by NRB,SBFD,DL as the type of interfering signal. Proposal 7 For FR2 WA, a SBFD capable BS with 40 dBm TRP, specify -47 dBm as the requirement level for the SBFD in-channel adjacent subband blocking, on the premise that operators do proper site dimensioning design in real networks and achieve 87 dB spatial isolation between sectors. Proposal 8 Endorse draft CR R4-2510965 for introduction of in-channel adjacent subband blocking requirement in TS 38.104. |
| R4-2511107 | Huawei, HiSilicon | Proposal 1: For FR1 WA and MR SBFD-capable BS in-channel adjacent subband leakage requirements, check if the legacy absolute ACLR limit (for DU/UD case) and absolute CACLR limit (for DUD case) can be reused unless further relaxation is deemed necessary. Proposal 2: For in-channel adjacent subband blocking requirement: • It should be within the SBFD DL subband. • A fixed offset between the edge of SBFD UL subband and the centre frequency of interference signal is preferred. Proposal 3: For FR1 WA BS type, adopt -31dBm as the in-channel adjacent subband blocking requirement. Proposal 4: Exemplary text proposal on in-channel adjacent subband blocking requirement with 20MHz and 40MHz UL subband bandwidth: Table: SBFD base station in-channel adjacent subband blocking requirement Proposal 5: Adopt the following assumption in the specification: • For FR1 WA BS, in-channel adjacent subband blocking requirement assumes 80 dB coupling loss between interfering BS transmitter and SBFD BS receiver. |
| R4-2511139 | ZTE Corporation, Sanechips | Proposal 1: for FR1 WA and MR SBFD BS, reuse the existing absolute ACLR/CALCR limit. Observation 1: BS co-channel coordination .e.g. beam nulling or beam coordination could alleviate the in-band blocking level. Observation 2: Without CLI With CLI coordination NOTE 1 In-band blocking FR1 WA SBFD BS -51.28dBm -63.8dBm -43 FR1 MR SBFD BS -51.656dBm N/A -38 FR1 LA SBFD BS -45.24dBm N/A -35dBm NOTE 1: to find the worst interference, then considering the beam sweeping to alleviate the in-channel blocking by beam coordination solution. Proposal 2: propose to further check the assumptions if in-channel blocking requirements is close to in-band blocking requirements; |
| R4-2511140 | ZTE Corporation, Sanechips | Draft CR to TS 38.104 on In-channel adjacent subband leakage for SBFD |
| R4-2511252 | Samsung | Proposal 1: The following text proposal is adopted for clause 3.1 definitions: • SBFD operation: SBFD-capable BS operation in non-SBFD symbols. • Non-SBFD operation: SBFD-capable BS operation in non-SBFD symbols. Proposal 2: The y-axis title of the illustrative figure for transient period between SBFD and non-SBFD needs to be updated as “Transmitter output power in frequency regions for transition” Proposal 3: The following text proposal is adopted for the clauses of transient period between SBFD and non-SBFD (take clause 12.2.8 for conducted TX requirement as example): Proposal 4: For in-channel adjacent subband leakage requirement, the absolute basic limit values for FR1 WA and MR BS in SBFD DU or UD operation shall be 3dB relaxation compared to corresponding ACLR limits, i.e., BS category / BS class In-channel adjacent subband leakage absolute basic limit for SBFD DU or UD operation Category A Wide Area BS -10 dBm/MHz (3dB relaxation) Category B Wide Area BS -12 dBm/MHz (3dB relaxation) Medium Range BS -22 dBm/MHz (3dB relaxation) Local Area BS -32 dBm/MHz Proposal 5: For in-channel adjacent subband leakage requirement, the absolute basic limit values for FR1 WA and MR BS in SBFD DUD operation shall be 8dB relaxation compared to corresponding ACLR limits, i.e., BS category / BS class In-channel adjacent subband leakage absolute basic limit for SBFD DUD operation Category A Wide Area BS -5 dBm/MHz (8dB relaxation) Category B Wide Area BS -7 dBm/MHz (8dB relaxation) Medium Range BS -17 dBm/MHz (8dB relaxation) Local Area BS -32 dBm/MHz Proposal 6: In-channel adjacent subband blocking requirement shall apply within transmission bandwidth configuration for SBFD DL subband(s), i.e., NRB,SBFD,DL for DU/UD case, and both NRB,SBFD,DL,1 and NRB,SBFD,DL,2 for DUD case. Proposal 7: For in-channel adjacent subband blocking requirement, the interfering signal centre frequency minimum and maximum offset shall be specified, to guarantee: • The similar minimum offset as in-band general blocking requirement, depending on SBFD UL subband BW; • The whole bandwidth of interfering signal is within transmission bandwidth configuration for SBFD DL subband. Proposal 8: For in-channel adjacent subband blocking requirement, • for SBFD DU/UD case, the interfering signal centre frequency offset is defined from the edge of NRB,SBFD,DL which is closer to the UL subband. • for SBFD DUD case, the interfering signal centre frequency offset is defined from the edge of NRB,SBFD,DL,1 (or NRB,SBFD,DL,2) which is closer to the UL subband, if the interfering signal is within NRB,SBFD,DL,1 (or NRB,SBFD,DL,2) correspondingly. • Proposal 9: The following table is adopted for in-channel adjacent subband blocking requirement (taken 1-H as example). |
| R4-2511256 | Samsung | draftCR to TS38.104 on transient period requirements for SBFD-capable BS |
| R4-2511527 | Nokia | Observation 1: Sectorized setup is also valid for FR1 MR BS Proposal 1: Use the existing ACLR absolute limits as the new in-channel adjacent subband leakage requirements. Proposal 2: Add 3dB also to FR1 MR BS requirement due to sectorized setup. Proposal 3: Introduce the in-channel adjacent subband blocking level of -25dBm for FR1 WA BS. |
| R4-2510965 (moved from AI 7.21.3.2) | Ericsson | Draft CR to 38.104 on adding receiver in-channel requirements for SBFD |

## Open issues summary

### Sub-topic 4-1: Transient period between SBFD and non-SBFD

- [Moderator] In RAN4#114, #114bis and #115, the following WF is achieved for “Location of transient period between SBFD and non-SBFD”:

Agreement in RAN4#114: - RAN4 reach the conclusion of the location of transient periods: - Case-A: in TA_offset in SBFD symbol(s); - Case-B: in guard symbol(s) reserved before UL non-SBFD symbols - FFS guard symbol(s) in this case is SBFD symbol or not. - Case-C: FFS in guard symbol(s) reserved in non-SBFD DL symbol(s) or blanked SBFD symbols in the beginning of SBFD UL transmission: - RAN4 recognize that it can be dependent on RAN1 discussion for guard period design. - If still no RAN4 agreement can be achieved in RAN4#114bis, RAN4 shall - either send LS to RAN1 to ask for clarification, - or conclude the location of transient period between SBFD and non-SBFD as gNB implementation issue without RAN4 requirement impact. - Case-D: in TA_offset in non-SBFD UL symbol(s). - Note: The illustrative figure for Case A, B, C and D is provided below: Agreement in RAN4#114bis: - For Case-B, guard symbol(s) should be reserved before UL symbols - The transient period can be reserved only in the DL part(s) of SBFD symbol(s) or in the whole channel bandwidth, up to gNB implementation. - For the location of transient periods Case-C, - Both Option 1 (guard symbols reserved in non-SBFD DL symbol(s)) and Option 2 (blanked SBFD symbols in the beginning of SBFD UL transmission) shall be allowed, up to gNB implementation. - For the location of transient period between SBFD and non-SBFD symbols, - No need to be specified in RAN4 core requirement; - For conformance testing for relevant requirements, - FFS test configurations. Agreement in RAN4#115: - Text proposal related to transient period requirement between SBFD and non-SBFD - Terminology “non-SBFD” need to be introduced. - Use below figure for the illustration of transient period between SBFD and non-SBFD in TS 38.104. Transmitter output power in UL subband/DL subband(s) Time Transmitter ON period (DL transmission) Transmitter OFF period Transmitter OFF period Transient period between SBFD and non-SBFD OFF power level ON power level UL transmission GP or UL transmission

##### Issue 4-1-1: Text proposal related to transient period between SBFD and non-SBFD

- Proposals on text proposal related to transient period between SBFD and non-SBFD:

  - Proposal 1 (Samsung): The following text proposal is adopted for clause 3.1 definitions:

    - SBFD operation: SBFD-capable BS operation in non-SBFD symbols.

    - Non-SBFD operation: SBFD-capable BS operation in non-SBFD symbols.

  - Proposal 2 (Samsung): The y-axis title of the illustrative figure for transient period between SBFD and non-SBFD needs to be updated as “Transmitter output power in frequency regions for transition”.

- Moderator Recommendation:

  - dCR which contains the above two text proposals can be endorsed, if no objection received on this two text proposals.

### Sub-topic 4-2: In-channel adjacent subband leakage

- [Moderator] In RAN4#113, RAN4 agree to introduce new requirement for in-channel adjacent subband leakage ratio, and in RAN4#114 and 114bis, the following agreement is achieved:

Agreement in RAN4#114: - New requirement for In-channel adjacent subband leakage ratio: - In-channel adjacent subband leakage ratio requirement is defined with the existing ACLR requirement in absolute PSD limit value as baseline (i.e., Table 6.6.3.2-2 in TS 38.104 for FR1 and Table 9.7.3.3-2 for FR2 in TS 38.104) - FFS the value for absolute PSD limit - FFS the naming of this requirement, by considering the absolute PSD limit used instead of the ratio. Note that the current naming in TS 38.104 is ACLR absolute limit. Way forward in RAN4#114bis: - FFS whether/how this assumption can be captured in the RAN4 requirement for in-channel adjacent subband leakage. - Absolute limit for in-channel adjacent subband leakage: - FFS by considering the following exemplary cases by performing analysis including measurement or simulation: - FR1: - DU/UD: 80MHz (DL) – 20MHz (UL) - DUD: 40MHz (DL) – 20MHz (UL) – 40MHz (DL) - Other cases are not precluded - FR2-1: - DU/UD: 50MHz (DL) – 50MHz (UL); 100MHz (DL) – 100MHz (UL) - DUD: 75MHz (DL) – 50MHz (UL) – 75MHz (DL) - Other cases are not precluded - Note: The study on different configuration does not mean RAN4 will define separate requirements for different configuration. RAN4 still try to use a unified requirements for absolute limit. Agreement - For absolute limit for in-channel adjacent subband leakage: - For FR2 BS, legacy ACLR absolute basic limit (Table 9.7.3.3-2 for FR2 in TS 38.104) is followed. - For FR1 BS, - For FR1 LA SBFD BS, no relaxation compared with absolute ACLR limit (DU configuration) or absolute CACLR limit (DUD) are needed. - For FR1 WA and MR SBFD-capable BS, - Option 1: [3]dB relaxation is provided. - Option 2: no relaxation from corresponding existing requirement

##### Issue 4-2-1: Adding note for uniform SBFD configuration across sites

- Observations/Proposals:

  - Observation 1: Operators cannot deploy SBFD selectively at just a few locations where UL coverage enhancement is desired, while other sites deploy regular TDD.

  - Observation 2: Operators cannot deploy SBFD with varying subband configurations across sites.

  - Proposal 1: Add a note in TS 38.104 where appropriate that “Operators shall deploy SBFD uniformly across all sites with exactly the same subband configuration.”

- Moderator Recommendation:

  - Discuss Proposal 1 to see whether it can be accepted.

##### Issue 4-2-2: In-channel adjacent subband leakage for FR1 WA/MR BS, for DU/UD case

- Absolute limit for in-channel adjacent subband leakage for DU/UD case

  - Option 1 (CATT, Ericsson, Huawei, ZTE, Nokia): reuse legacy ACLR absolute basic limit (i.e., Table 6.6.3.2-2 in TS 38.104 for FR1)

Table 6.6.3.2-2 in TS 38.104 [4]: Base station ACLR absolute basic limit

| BS category / BS class | ACLR absolute basic limit |
| --- | --- |
| Category A Wide Area BS | -13 dBm/MHz |
| Category B Wide Area BS | -15 dBm/MHz |
| Medium Range BS | -25 dBm/MHz |
| Local Area BS | -32 dBm/MHz |

  - Option 2 (Samsung): 3dB relaxation compared to corresponding ACLR limits for FR1 WA/MR BS:

| BS category / BS class | In-channel adjacent subband leakage absolute basic limit for SBFD DU or UD operation |
| --- | --- |
| Category A Wide Area BS | -10 dBm/MHz (3dB relaxation) |
| Category B Wide Area BS | -12 dBm/MHz (3dB relaxation) |
| Medium Range BS | -22 dBm/MHz (3dB relaxation) |
| Local Area BS | -32 dBm/MHz |

- Moderator Recommendation:

  - Encourage companies to align the understanding, since most of Rel-18 analysis assume 45dB relative leakage level.

##### Issue 4-2-3: In-channel adjacent subband leakage for FR1 WA/MR BS, for DUD case

- Absolute limit for in-channel adjacent subband leakage for DUD case

  - Option 1 (CATT, Ericsson, Huawei, Nokia): reuse legacy CACLR absolute basic limit (i.e., Table 6.6.3.2-2 in TS 38.104 for FR1)

  - Option 2 (Samsung): 8dB relaxation compared to corresponding CACLR limits for FR1 WA/MR BS:

| BS category / BS class | In-channel adjacent subband leakage absolute basic limit for SBFD DUD operation |
| --- | --- |
| Category A Wide Area BS | -5 dBm/MHz (8dB relaxation) |
| Category B Wide Area BS | -7 dBm/MHz (8dB relaxation) |
| Medium Range BS | -17 dBm/MHz (8dB relaxation) |
| Local Area BS | -32 dBm/MHz |

- Moderator Recommendation:

  - Encourage companies to align the understanding, since most of Rel-18 analysis assume 45dB relative leakage level.

### Sub-topic 4-3: In-channel adjacent subband blocking

- [Moderator] In RAN4#113, the following agreement is obtained, i.e., RAN4 agree to introduce new requirement for in-channel adjacent subband blocking requirement, while no need to introduce new requirement for in-channel adjacent subband selectivity. There is some further way forward achieved in RAN4#114bis:

Agreement from RAN4#113: - New requirement for in-channel adjacent subband blocking should be defined - FFS the impact of in-channel interference source(s), with and without gNB-to-gNB CLI handling schemes, based on reasonable expected gNB-to-gNB isolation. - FFS in-channel selectivity (ICS) requirement can cover in-channel adjacent subband blocking requirement. - The requirement level shall be driven by feasibility consideration. Way forward in RAN4#114bis: - New requirement for in-channel adjacent subband blocking: - The requirement can be derived by considering co-site inter-sector case and/or inter-site case - If co-site inter-sector case is considered, FFS whether/how the assumption of isolation is reflected in specification. - The range of [-35dBm to -27dBm] can be used as the starting point for further study. Agreement in RAN4#115 - Wanted signal mean power, i.e., degradation level for Refsens level - For conducted in-channel adjacent subband blocking requirement, as defined for in-band blocking requirement, 6dB degradation from Refsens - 6dB degradation on top of OTA REFSENS degradation for in-channel adjacent subband blocking requirement, and own Tx is ON. - On the level of in-channel adjacent subband blocking requirement: - For FR1 WA BS: - [-31+3 = -28] dBm - FFS isolation between sectors can captured in specification - For FR1 MR BS: - [-32] dBm. - FFS isolation between sectors can captured in specification - For FR1 LA BS: - [-34] dBm.

##### Issue 4-3-1: Interfering signal level for FR1

- Proposals on interfering signal level from UL subband for FR1 WA BS

  - Option 1 (endorsed in RAN4#115): [-31+3 = -28] dBm

  - Option 2 (Ericsson): For FR1 WA, a SBFD capable BS with 53 dBm TRP, specify -27 dBm as the requirement level for the SBFD in-channel adjacent subband blocking, on the premise that operators do proper site dimensioning design in real networks and achieve 80 dB spatial isolation between sectors.

  - Option 3 (Huawei): For FR1 WA BS type, adopt -31dBm as the in-channel adjacent subband blocking requirement.

  - Option 4 (ZTE): Propose to further check the assumptions if in-channel blocking requirements is close to in-band blocking requirements

|  | Without CLI | With CLI coordination NOTE 1 | In-band blocking |
| --- | --- | --- | --- |
| FR1 WA SBFD BS | -51.28dBm | -63.8dBm | -43 |
| FR1 MR SBFD BS | -51.656dBm | N/A | -38 |
| FR1 LA SBFD BS | -45.24dBm | N/A | -35dBm |
| NOTE 1: to find the worst interference, then considering the beam sweeping to alleviate the in-channel blocking by beam coordination solution. |  |  |  |

  - Option 5 (Nokia): Introduce the in-channel adjacent subband blocking level of -25dBm for FR1 WA BS.

- Proposals on interfering signal level from UL subband for FR1 MR BS

  - Option 1 (CATT): For in-channel adjacent sub-band blocking requirement, -32dBm can be used for FR1 MR BS.

- Proposals on interfering signal level from UL subband for FR1 LA BS

  - Option 1 (CATT): For in-channel adjacent sub-band blocking requirement, -34dBm can be used for FR1 LA BS.

- Moderator Recommendation:

  - Checking ZTE’s analysis firstly.

  - If no big issues identified for existing agreement, suggest to still keep the original agreement in RAN#115.

##### Issue 4-3-2: Interfering signal level for FR2-1

- Proposals on interfering signal level from UL subband for FR2-1 WA BS

  - Proposal 1 (Ericsson): For FR2 WA, a SBFD capable BS with 40 dBm TRP, specify -47 dBm as the requirement level for the SBFD in-channel adjacent subband blocking, on the premise that operators do proper site dimensioning design in real networks and achieve 87 dB spatial isolation between sectors.

- Moderator Recommendation:

  - Adopt Proposal 1.

##### Issue 4-3-3: Assumption for coupling loss

- Proposals on adopting the assumption in the specification

  - Proposal 1 (Huawei): Adopt the following assumption in the specification:

    - For FR1 WA BS, in-channel adjacent subband blocking requirement assumes 80 dB coupling loss between interfering BS transmitter and SBFD BS receiver.

  - Proposal 2 (CATT): Adopt the following assumption in the specification:

- If co-site inter-sector case is considered, the isolation assumption between two sectors BSs should be added as a standard note.Moderator Recommendation:

  - Check if Proposal 1 can be accepted as a compromise, which means the assumption is captured in the specification while no need to further discuss Issue 4-3-4.

##### Issue 4-3-4: Assumption of gNB-to-gNB CLI handling

- Proposals/Observations:

  - Option 1 (Ericsson, Nokia): Define in-channel adjacent subband blocking requirement without considering CLI handling schemes.

    - Observation 1 (Ericsson): In-channel adjacent subband blocking requirement measures the interaction between sectors in the near field, and CLI handling schemes differ from what is studied in RAN1 and has not been studied so far.

    - Observation 2 (Ericsson): CLI handling schemes for the case of inter-site interference could suppress a portion of the interference, but they cannot completely eliminate it, because it is not always feasible, and RX digital beamforming cannot be considered.

  - Option 2 (ZTE): Unlike in-band blocking, the interference source of in-channel adjacent subband blocking can be mitigated by CLI handling techniques.

    - Observation 3 (ZTE):

|  | Without CLI | With CLI coordination | In-band blocking |
| --- | --- | --- | --- |
| FR1 WA SBFD BS | -51.28dBm | -63.8dBm | -43 |
| FR1 MR SBFD BS | -51.656dBm | N/A | -38 |
| FR1 LA SBFD BS | -45.24dBm | N/A | -35dBm |

- Moderator Recommendation:

  - The discussion on assumptions will closely related to Issue 4-3-3.

##### Issue 4-3-5: Minimum/maximum offset

- Proposals on interfering signal minimum/maximum offset from UL subband

  - Option 1 (Ericsson): bandwidth of the interfering signal should be equivalent to the DL subband

    - Introduce a new symbol FC,SBFD,DL for SBFD DL subband, which is similar to FC used in ICS, to represent the reference point for interfering signal.

    - TP provided in dCR:

- Table 12.3.9.2-1: In-channel adjacent subband blocking requirement for SBFD-capable BS type 1-H

| SBFD UL subband bandwidth (MHz) | Wanted signal mean power (dBm) (Note 1) | Interfering signal mean power (dBm) | SBFD UD/UD or DUD case | Interfering signal centre frequency/frequencies | Type of interfering signal(s) |
| --- | --- | --- | --- | --- | --- |
| 3, 5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90 | PREFSENS + 6 dB | Wide Area BS: -28 Medium Range BS: -32 Local Area BS: -34 | DU or UD | FC,SBFD,DL | NR signal 30 kHz SCS NRB,SBFD,DL |
|  |  |  | DUD | FC,SBFD,DL,1 FC,SBFD,DL2 | NR signal 30 kHz SCS NRB,SBFD,DL,1 NRB,SBFD,DL,2 |
| NOTE 1: PREFSENS depends on the SBFD UL subband bandwidth as specified in clause 12.3.2. |  |  |  |  |  |

  - Option 2 (Samsung, Huawei): similar as in-band blocking requirement, minimum offset is needed:

    - Proposal 1 (Samsung, Huawei): For in-channel adjacent subband blocking requirement, the interfering signal centre frequency minimum and maximum offset shall be specified, to guarantee:

      - The similar minimum offset as in-band general blocking requirement, depending on SBFD UL subband BW;

      - The whole bandwidth of interfering signal is within transmission bandwidth configuration for SBFD DL subband.

    - Proposal 2 (Samsung): For in-channel adjacent subband blocking requirement,

      - for SBFD DU/UD case, the interfering signal centre frequency offset is defined from the edge of NRB,SBFD,DL which is closer to the UL subband.

      - for SBFD DUD case, the interfering signal centre frequency offset is defined from the edge of NRB,SBFD,DL,1 (or NRB,SBFD,DL,2) which is closer to the UL subband, if the interfering signal is within NRB,SBFD,DL,1 (or NRB,SBFD,DL,2) correspondingly.

    - Proposal 3 (Samsung): The following table is adopted for in-channel adjacent subband blocking requirement (taken 1-H as example).

- Table 12.3.9.x-x: Base station in-channel adjacent subband blocking requirement

| SBFD UL subband (MHz) | Wanted signal mean power (dBm) (Note 2) | Interfering signal mean power (dBm) | SBFD DU/UD or DUD case | Interfering signal centre frequency minimum offset (MHz) (Note 3) | Interfering signal centre frequency maximum offset (MHz) (Note 3) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- | --- |
| 3 | PREFSENS + x dB | Wide Area BS: -28 Medium Range BS: -32 Local Area BS: -34 | DU or UD | ±4.5 | ±(NRB,SBFD,DL *0.18 *2μ - 1.35) | 3 MHz OFDM NR signal 15 kHz SCS, 15 RBs |
|  |  |  | DUD | ±4.5 | -(NRB,SBFD,DL,1 *0.18 *2μ - 1.35) if interfering signal at NRB,SBFD,DL,1 +(NRB,SBFD,DL,2 *0.18 *2μ- 1.35) if interfering signal at NRB,SBFD,DL,2 |  |
| 5, 10, 15, 20 | PREFSENS + x dB | Wide Area BS: -28 Medium Range BS: -32 Local Area BS: -34 | DU or UD | ±7.5 | ±(NRB,SBFD,DL *0.18 *2μ – 2.25) | 5 MHz OFDM NR signal 15 kHz SCS, 25 RBs |
|  |  |  | DUD | ±7.5 | -(NRB,SBFD,DL,1 *0.18 *2μ – 2.25) if interfering signal at NRB,SBFD,DL,1 +(NRB,SBFD,DL,2 *0.18 *2μ- 2.25) if interfering signal at NRB,SBFD,DL,2 |  |
| 25, 30, 35, 40, 45, 50, 60, 70, 80, 90 | PREFSENS + x dB | Wide Area BS: -28 Medium Range BS: -32 Local Area BS: -34 | DU or UD | ±30 | ±(NRB,SBFD,DL *0.18 *2μ - 9) | 20 MHz OFDM NR signal 15 kHz SCS, 100 RBs |
|  |  |  | DUD | ±30 | -(NRB,SBFD,DL,1 *0.18 *2μ – 9) if interfering signal at NRB,SBFD,DL,1 +(NRB,SBFD,DL,2 *0.18 *2μ- 9) if interfering signal at NRB,SBFD,DL,2 |  |
| NOTE 1: PREFSENS depends on the SBFD UL subband bandwidth as specified in clause 12.3.2.1. NOTE 2: For a BS capable of single band operation only, "x" is equal to 6 dB. NOTE 3: For SBFD DU/UD case, the interfering signal centre frequency offset is defined from the edge of NRB,SBFD,DL which is closer to the UL subband. For SBFD DUD case, the interfering signal centre frequency offset is defined from the edge of NRB,SBFD,DL,1 (or NRB,SBFD,DL,2) which is closer to the UL subband, if the interfering signal is within NRB,SBFD,DL,1 (or NRB,SBFD,DL,2) correspondingly. |  |  |  |  |  |  |

    - Proposal 4 (Huawei): Exemplary text proposal on in-channel adjacent subband blocking requirement with 20MHz and 40MHz UL subband bandwidth:

- Table: SBFD base station in-channel adjacent subband blocking requirement

| SBFD UL subband (MHz) | Wanted signal mean power (dBm) (Note 2) | Interfering signal mean power (dBm) | SBFD DU/UD or DUD case | Interfering signal centre frequency offset (MHz) (Note 3) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 20, 40 | PREFSENS + x dB | Wide Area BS: -31 Medium Range BS: -32 Local Area BS: -34 | DU or UD | 30 (for UD case) or -30 (for DU case) | 20 MHz CP-OFDM NR signal 15 kHz SCS, 100 RBs |
|  |  |  | DUD | 30 (for lower DL subband) and -30 (for higher DL subband) |  |
| NOTE 1: PREFSENS depends on the SBFD UL subband bandwidth as specified in clause 12.3.2.1. NOTE 2: For a BS capable of single band operation only, "x" is equal to 6 dB. NOTE 3: The interfering signal is defined within the DL subband and the offset is from the edge of UL subband to the interfering signal centre frequency. |  |  |  |  |  |

- Moderator Recommendation:

  - Discuss on the above two options firstly, and after the option is selected, detailed requirement can be discussed based on corresponding TP.

##### Issue 4-3-6: Interfering signal type

- Proposals on interfering signal type (take FR1 as example)

  - Option 1 (Samsung): OFDM NR signal, 15kHz SCS, Signal BW depends on SBFD UL subband size, similar as in-band blocking requirement.

  - Option 2 (Ericsson): OFDM NR signal, 30kHz SCS

- Moderator Recommendation:

  - For FR1, Option 1 can be compatible with both 15/30kHz SCS, while Option 2 can’t work with 15kHz SCS for UL subband. Suggest to use Option 1.

### Sub-topic 4-4: draftCR to TS 38.104

- [Moderator] As the worksplitting planned for dCR drafting, the following dCRs are prepared which are related to new BS requirements for SBFD-capable BS.

  - R4-2511256, draftCR to TS38.104 on transient period requirements for SBFD-capable BS, Samsung

    - Moderator Recommendation:

      - View collection before and during the meeting.

  - R4-2511140, Draft CR to TS 38.104 on In-channel adjacent subband leakage for SBFD, Ericsson

    - Moderator Recommendation:

      - View collection before and during the meeting.

  - R4-2510965, Draft CR to 38.104 on adding receiver in-channel requirements for SBFD, Ericsson

    - Moderator Recommendation:

      - View collection before and during the meeting.

# Topic #5: BS RF conformance requirements for SBFD

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2509322 (for general part) | CATT | Proposal 4: For DU /UD configurations and DUD configuration, testing only needs to consider DU or UD configuration, and retesting under DUD configuration is not required. Proposal 5: The general principle that SBFD doesn’t support type 1-C needs to be captured in TS 38.141-1. Proposal 6: The operating bands supported by SBFD need to be added to TS 38.141-1 and TS 38.141-2. Proposal 7: The channel bandwidths of SBFD capable BS and SBFD subband configuration(s) per supported channel bandwidth need to be added to TS 38.141-1 and TS 38.141-2. Proposal 8: When testing the SBFD mode, it is recommended to configure the SBFD carrier at the bottom and top positions of the operating frequency band. |
| R4-2509323 (for new requirement) | CATT | Proposal 4: Following new requirements for SBFD capable BS need to be added to both TS 38.141-1 and TS 38.141-2. • Transient period between SBFD and non-SBFD • Absolute limit for in-channel adjacent subband leakage • In-channel adjacent subband blocking Proposal 5: For transient period between SBFD and non-SBFD testing, it is needed to add a new test model which can switch between SBFD and non-SBFD. Proposal 6: For in-channel adjacent subband leakage requirement testing of SBFD capable BS, the desired test model and frequency location of wanted signal are the same as those used for BS ACLR absolute basic limit testing, but with different test frequency ranges. Proposal 7: For in-channel adjacent subband leakage requirement testing of SBFD capable BS, the desired test model and frequency location of wanted signal are necessary to added to both TS 38.141-1 and TS 38.141-2. Proposal 8: For in-channel adjacent subband leakage requirement testing of SBFD capable BS, the test frequency range is necessary to added to both TS 38.141-1 and TS 38.141-2. Proposal 9: For in-channel adjacent subband blocking testing of SBFD capable BS, the transmitters must be turned on. Proposal 10: For in-channel adjacent subband blocking testing of SBFD capable BS, the desired Rx input signal and its frequency location are the same as those used for in-band blocking requirement. Proposal 11: For SBFD capable BS, the test frequency range of in-channel adjacent subband blocking is necessary to added to both TS 38.141-1 and TS 38.141-2. |
| R4-2511107 | Huawei, HiSilicon | Observation 1: The interference signal generation for specific requirement shall be aligned with the assumptions for the corresponding requirement. Observation 2: Based on SBFD core requirements definition, test complexity reduction study is deemed necessary given that e.g. it is going to cover all the declared SBFD configurations. |
| R4-2510961 | Ericsson | Observation 2 Based on the discussion so far, the following declarations shall be captured in TS 38.141-1/-2: TX power for SBFD; Bands supported for SBFD; Channel bandwidth(s) supported for SBFD, from existing channel bandwidth defined in TS 38.104; SBFD subband configuration(s) per supported channel bandwidth, including SBFD pattern (DUD or DU/UD), size of DL subband(s), size of UL subband, and size of guardband(s) between subbands. |

### Sub-topic 5-1: General assumptions of conformance testing

##### Issue 5-1-1: System parameters of conformance testing

- Proposals:

  - Proposal 1 (CATT): The operating bands supported by SBFD need to be added to TS 38.141-1 and TS 38.141-2.

  - Proposal 2 (CATT): The channel bandwidths of SBFD capable BS and SBFD subband configuration(s) per supported channel bandwidth need to be added to TS 38.141-1 and TS 38.141-2.

- Moderator Recommendation:

  - Discuss the proposals firstly.

##### Issue 5-1-2: BS type for SBFD conformance testing

- Proposals:

  - Proposal 1 (CATT): The general principle that SBFD doesn’t support type 1-C needs to be captured in TS 38.141-1.

- Moderator Recommendation:

  - Discuss the proposals firstly.

##### Issue 5-1-3: Declaration of supported SBFD configuration

- Proposal on the declaration of supported SBFD configuration

  - Observation 1 (Ericsson): Based on the discussion so far, the following declarations shall be captured in TS 38.141-1/-2: TX power for SBFD; Bands supported for SBFD; Channel bandwidth(s) supported for SBFD, from existing channel bandwidth defined in TS 38.104; SBFD subband configuration(s) per supported channel bandwidth, including SBFD pattern (DUD or DU/UD), size of DL subband(s), size of UL subband, and size of guardband(s) between subbands.

- Moderator Recommendation:

  - Discuss the proposals firstly.

##### Issue 5-1-4: Testing applicability for supported SBFD configuration

- [Moderator] Based on RAN4#115 agreement:

Agreement: - If the SSB symbol is overlapped with SBFD symbol, - there is no RAN4 impact at least on RAN4 core requirement for expected regarding DL subband size configuration - FFS the test configuration during performance part discussion.

- Proposal on the test configuration

  - Proposal 1 (CATT): For DU /UD configurations and DUD configuration, testing only needs to consider DU or UD configuration, and retesting under DUD configuration is not required. For the restriction on guardband size of SBFD

  - Observation 1 (Huawei): Based on SBFD core requirements definition, test complexity reduction study is deemed necessary given that e.g. it is going to cover all the declared SBFD configurations.

- Moderator Recommendation:

  - Discuss the proposals firstly.

##### Issue 5-1-5: RF channels to be tested for SBFD operation

- Proposals:

  - Proposal 1 (CATT): When testing the SBFD mode, it is recommended to configure the SBFD carrier at the bottom and top positions of the operating frequency band.

- Moderator Recommendation:

  - Discuss the proposals firstly.

### Sub-topic 5-2: Conformance testing for new RF requirements

##### Issue 5-2-1: Conformance testing related to transient period between SBFD and non-SBFD

- Proposals from CATT:

  - Proposal 1: Following new requirements for SBFD capable BS need to be added to both TS 38.141-1 and TS 38.141-2.

    - Transient period between SBFD and non-SBFD

    - Absolute limit for in-channel adjacent subband leakage

    - In-channel adjacent subband blocking

  - Proposal 2: For transient period between SBFD and non-SBFD testing, it is needed to add a new test model which can switch between SBFD and non-SBFD.

- Moderator Recommendation:

  - Discuss the proposals firstly.

##### Issue 5-2-2: Conformance testing related to in-channel adjacent subband leakage

- Proposals from CATT:

  - Proposal 1: For in-channel adjacent subband leakage requirement testing of SBFD capable BS, the desired test model and frequency location of wanted signal are the same as those used for BS ACLR absolute basic limit testing, but with different test frequency ranges.

  - Proposal 2: For in-channel adjacent subband leakage requirement testing of SBFD capable BS, the desired test model and frequency location of wanted signal are necessary to added to both TS 38.141-1 and TS 38.141-2.

  - Proposal 3: For in-channel adjacent subband leakage requirement testing of SBFD capable BS, the test frequency range is necessary to added to both TS 38.141-1 and TS 38.141-2.

- Moderator Recommendation:

  - Discuss the proposals firstly.

##### Issue 5-2-3: Conformance testing related to in-channel adjacent subband leakage

- Proposals from CATT:

  - Proposal 1: For in-channel adjacent subband blocking testing of SBFD capable BS, the transmitters must be turned on.

  - Proposal 2: For in-channel adjacent subband blocking testing of SBFD capable BS, the desired Rx input signal and its frequency location are the same as those used for in-band blocking requirement.

  - Proposal 3: For SBFD capable BS, the test frequency range of in-channel adjacent subband blocking is necessary to added to both TS 38.141-1 and TS 38.141-2.

- Moderator Recommendation:

  - Discuss the proposals firstly.