3GPP TSG-RAN WG4 Meeting #116										    	R4-2511458

Bengaluru, India, August 25th – 29th, 2025

Agenda item:			7.7.1

Source:	Moderator (SES)

Title:	Topic summary for [116][313] NR_NTN_Ku_Band_Coexistence

Document for:	Information

# Introduction

In this contribution we present a summary of the work undertaken on the Ku Band co-existence analysis.

The co-existence analysis consists of first a calibration phase, where the simulation platforms are calibrated to asset of agreed parameters and secondly co-existence analysis where ACIR values are extracted for a number of interfering scenarios.

The latest version of the simulation assumptions for the Ku-band co-existence study are summarized in the running document [1]. In the previous RAN4#115 Meeting a way forward has been agreed in [2].

- R4-2508774, “Ku band co-existence simulation assumptions (running document)”, Ericsson, 3GPP RAN4#115, Malta, May 2025.

- R4-2508773, “Way Forward for [115][313] NR_NTN_Ku_Band_Coexistence”, Intelsat (Moderator), 3GPP RAN4#115, Malta, May 2025.

Based on [1] and [2], further calibration simulations and co-existence simulations have been performed by companies and the following contributions have been submitted to the Agenda Item 7.7.3 “Coexistence study based on ITU regulations” for this RAN4#116 meeting. Proposals and observations from these contributions are summarized in the respective topic sections.

| TDoc | Title | Source |
| --- | --- | --- |
| R4-2509121 | Summary of Ku Band Co-existence Analysis Results | SES S.A. |
| R4-2509335 | Further discussion on coexistence evaluations for Ku-band for NR NTN | CATT |
| R4-2509336 | Co-existence calibration result for Ku-band for NR NTN | CATT |
| R4-2509337 | Coexistence evaluation result for Ku-band for NR NTN | CATT |
| R4-2509770 | Temporary calibration results and simulation results for the Ku-band NTN coexistence study | Xiaomi |
| R4-2510182 | Discussion on NTN Ku band coexistence study | MediaTek (Wuhan) Inc. |
| R4-2510215 | On co-existence simulation results for NR NTN Ku band | Ericsson |
| R4-2510240 | Coexistence simulation results between TN and NTN for Ku band | Qualcomm Incorporated |
| R4-2510356 | Simulation results of NR NTN Ku band co-existence study | ZTE Corporation, Sanechips |

# Topic #1: Calibration

## Companies’ contributions summary

The following Table 1summarizes the observations and proposals of companies’ contributions regarding calibration.

Table 1 Summary of observations and proposals related to calibration

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2509121 | SES S.A., Fraunhofer IIS, Lockhead Martin | Observation 1 Good correlation of NTN calibration results for GEO 45x45 UL. Observation 2 Good correlation of NTN calibration results for LEO 45x45 UL. Observation 3 Good correlation of NTN calibration results for GEO Parabolic UL. Observation 4 Good correlation of NTN calibration results for LEO Parabolic UL. Observation 5 Good correlation of NTN calibration results for GEO 45x45 DL. Observation 6 Good correlation of NTN calibration results for LEO 45x45 DL. Observation 7 Good correlation of NTN calibration results for GEO Parabolic DL. Observation 8 Good correlation of NTN calibration results for LEO Parabolic DL. Observation 9 Good correlation of TN calibration results for TN uplink. Observation 10 Good correlation of TN calibration results for TN downlink. Observation 11 The observed “stretched” CDF curves for coupling loss in the NTN calibration are due to the underlying bimodal PDFs of clutter loss distributions. Observation 12 The observed “stretched” CDF curves for SINR in the NTN calibration are due to the underlying bimodal PDFs of clutter loss distributions. Proposal 1 Companies are expected to update ACLR and ACS results based on calibrated NTN and TN setups. Proposal 2 Update Ku band co-existence simulation assumptions in [1] to clarify that clutter loss and shadow fading with values for Ka band from TS 38.811 are included in the NTN propagation modeling for both calibration and co-existence simulations. Proposal 3 Update Ku band co-existence simulation assumptions in [1] to use ISD = 450m for TN calibration. Proposal 4 Discuss which UEs to include in co-existence simulations dependent on SINR. |
| R4-2509335 | CATT | Proposal 1: The following satellite Parameters for Ku-band co-existence is more reasonable. Table 3 Satellite Parameters Satellite orbit; GEO; LEO-600. Satellite altitude; 35786 km; 600 km. Satellite antenna pattern; Section 6.4.1 in [2]; Section 6.4.1 in [2]. Payload characteristics for DL transmissions. Equivalent satellite antenna aperture (Note 1); Ku-band (i.e. 11 GHz for DL); 4 m; 0.91 m. Satellite EIRP density; 43 dBW/MHz; 7 dBW/MHz. Satellite Tx max Gain; 51.4 dBi; 38.5 dBi. 3dB beamwidth; 0.400 deg; 1.768 deg. Satellite beam diameter (Note 2); 250 km; 19 km. Payload characteristics for UL transmissions. Equivalent satellite antenna aperture (Note1); Ku-band (i.e. 14 GHz for UL); 3.14 m; 0.714 m. G/T; 23.8 dB K-1; 10.9 dB K-1. Satellite RX max Gain; 51.4 dBi; 38.5 dBi. NOTE 1: This value is equivalent to the antenna diameter in Sec. 6.4.1 of [2]. NOTE 2: This beam size refers to the Nadir pointing of the satellite NOTE 3: All these satellite parameters are applied per beam. NOTE 4: The EIRP density values are considered identical for all frequency re-use factor options. NOTE 5: The EIRP density values are provided assuming the satellite HPA is operated with a back-off of [5] dB.. Proposal 2: Update the parameters in Table 6a.2.2.2-2 to align those in “Circular Aperture TR 38.863 ” column of Table 6a.2.2.2-1 in co-existence simulation assumptions running document. Table 6a.2.2.2-2: Other NTN UE parameters Characteristics; VSAT (Parabolic). Frequency band; Ku-band (i.e. 11 GHz DL and 14 GHz UL). Antenna type and configuration; Directional Section 6.4.1 of [2] with 60 cm equivalent aperture diameter. Polarisation; Circular. Rx Antenna gain; 34.3 dBi. Antenna temperature; 150 K. Noise figure; 2.5 dB for both LEO and GEO. Tx transmit power; 10 Watts for GEO and for LEO. Tx antenna gain; 36.6 dBi. Antenna elevation angle; 35˚ and 90˚. NOTE 1: Moving platforms (e.g., aircrafts, vessels), building mounted devices. These values are provided for information.. |
| R4-2509336 | CATT | - |
| R4-2509337 | CATT | - |
| R4-2509770 | Xiaomi | - |
| R4-2510182 | MediaTek (Wuhan) Inc. | - |
| R4-2510215 | Ericsson | Observation 1 The clutter loss assumption plays significant role in the path loss calculations at 35 degrees elevation angle assumption. Observation 2 If the link budget fails at 35 degrees or lower elevation angles, there are several other ways to make it work without changing the propagation model. Proposal 1 RAN4 to use the Ka band clutter loss reference value at 35 degrees elevation angle in Urban scenario for co-existence simulation assumptions. |
| R4-2510240 | Qualcomm Incorporated |  |
| R4-2510356 | ZTE Corporation, Sanechips |  |

## Calibration summary

Following several meeting and co-ordination of simulation parameters, calibration results were produced by various companies.

Following several calibration iterations, close correlations were obtained between various plots. It follows that there is high confidence in the performance o the simulation platforms.

Moderator’s summary: Due to close correlations observed between various simulation platforms, it is proposed to proceed with the calculation of co-existence analysis

### Issue 1-1: NTN Calibration Assumptions: Clutter Loss

For the calibration phase, a 35 degree elevation angle was selected for the lowest elevation angle and Urban propagtion model was selected.

To simulate the above senario, it was collectivly decided to incorporate the clutter loss  in the calibration phaase and consequently in the co-existence analysis with values for Ka band from TS 38.811, as no values are available for Ku band.

Issue 1-1: NTN Calibration Assumptions: Clutter Loss

Moderator’s recomendation:

- Clutter loss to be included in the calibration and co-existence analysis with values for Ka band from TS 38.811.

- Update NTN Ku band co-existence simulation assumptions in [1] accordingly.

### Issue 1-2: TN Calibration Assumptions: ISD

TN calibration assumptions have been based on the way forward in [2], i.e.  ISD = 450m has been used by companies, which is later used in the TN/NTN co-existence evaluation study. However, the running document [1] states ISD = 750m in Table C.1-2 for TN calibration.

Issue 1-2: TN Calibration Assumptions: ISD

Moderator’s recommendation:

- Update Ku band co-existence simulation assumptions in [1] to use ISD = 450m for TN calibration and co-existence study

# Topic #2: ACIR, ACLR and ACS

## Companies’ contributions summary

The following Table 2 summarizes the observations and proposals of companies’ contributions regarding ACIR, ACLR and ACS.

Table 2 Summary of observations and proposals related to ACIR, ACLR and ACS values

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2509121 | SES S.A., Fraunhofer IIS, Lockhead Martin | - |
| R4-2509335 | CATT | - |
| R4-2509336 | CATT | - |
| R4-2509337 | CATT | Observation 1: The worst interpolated ACIR@5% throughput loss for Scenarios 1, 4, 5 and 6 are shown in the following table. ACIR（worst case btw 35/90 deg）; worst interpolated ACIR[dB] @ 5% throughput loss. scenario 1 NTN UL aggressor TN UL victim VSAT ACLR to be defined; GEO; Phase array; 31.55. Parabolic; 18.67. LEO; Phase array; 27.77. Parabolic; 14.52. scenario 4 TN DL aggressor NTN UL victim SAN ACS to be defined; GEO; Phase array; 37.77. Parabolic; 38.43. LEO; Phase array; 40. Parabolic; 40. scenario 5 TN DL aggressor NTN DL victim VSAT ACS to be defined; GEO; Phase array; 40. Parabolic; 33.58. LEO; Phase array; 40. Parabolic; 38.41. scenario 6 NTN DL aggressor TN DL victim SAN ACLR to be defined; GEO; Phase array; 0. Parabolic; 0. LEO; Phase array; 0. Parabolic; 0. Observation 2: From perspective of co-existence, simulation result shows that VSAT ACLR needs to be defined as 31.55dB. Observation 3: From perspective of co-existence, simulation result shows that SAN ACS for GEO and LEO needs to be defined as 40dB. Observation 4: From perspective of co-existence, simulation result shows that VSAT ACS needs to be defined as 40dB. Observation 5: Define 13dB SAN ACLR for GEO and LEO class for Ku-band NTN. |
| R4-2509770 | Xiaomi | Observation 1: Basic on the simulation results, the ACIR requirement for case 1 the GEO 60 cm Parabolic should be 3.29 dB. Observation 2: Basic on the simulation results, the ACIR requirement for case 5 the GEO 60 cm Parabolic should be 41.47 dB. Observation 3: 5% throughput loss of case 5 the GEO 60 cm Parabolic has no value since the 5% throughput is zero which SINR lower than the minimum requirement of SINR. Observation 4: Basic on the simulation results, the ACIR requirement for case 6 the GEO 60 cm Parabolic should be 0 dB. Observation 5: Basic on the simulation results, the ACIR requirement for case 1 the LEO-600 60 cm Parabolic should be 0 dB. Observation 6: Basic on the simulation results, the ACIR requirement for case 5 the LEO-600 60 cm Parabolic should be 39.84 dB. Observation 7: 5% throughput loss of case 5 the LEO-600 60 cm Parabolic has no value since the 5% throughput is zero which SINR lower than the minimum requirement of SINR. Observation 8: Basic on the simulation results, the ACIR requirement for case 6 the LEO-600 60 cm Parabolic should be 0 dB. Observation 9: Basic on the simulation results, the ACIR requirement for case 1 the GEO 45 x 45 Phased Array case should be 2.88 dB. Observation 10: Basic on the simulation results, the ACIR requirement for case 5 the GEO 45 x 45 Phased Array case should be 40.09 dB. Observation 11: 5% throughput loss of case 5 the GEO 45 x 45 Phased Array has no value since the 5% throughput is zero which SINR lower than the minimum requirement of SINR. Observation 12: Basic on the simulation results, the ACIR requirement for case 6 the GEO 45 x 45 Phased Array case should be 0 dB. Observation 13: Basic on the simulation results, the ACIR requirement for case 1 the LEO-600 45 x 45 Phased Array case should be 0 dB. Observation 14: Basic on the simulation results, the ACIR requirement for case 5 the LEO-600 45 x 45 Phased Array case should be 40.34 dB. Observation 15: 5% throughput loss of case 5 the LEO 45 x 45 Phased Array has no value since the 5% throughput is zero which SINR lower than the minimum requirement of SINR. Observation 16: Basic on the simulation results, the ACIR requirement for case 6 the LEO-600 45 x 45 Phased Array case should be 0 dB. Observation 17: Basic on the simulation results, the ACIR requirement for case 1 the GEO 78 x 78 Phased Array case should be 4.61 dB. Observation 18: Basic on the simulation results, the ACIR requirement for case 5 the GEO 78 x 78 Phased Array case should be 35.23 dB. Observation 19: 5% throughput loss of case 5 the GEO 78 x 78 Phased Array has no value since the 5% throughput is zero which SINR lower than the minimum requirement of SINR. Observation20: Basic on the simulation results, the ACIR requirement for case 6 the GEO 78 x 78 Phased Array case should be 0 dB. Proposal 1: RAN4 can accept Xiaomi’s calibration results. SAN GEO; SAN LEO-600; VSAT - 45 x 45 Phased Array; VSAT - 60 cm Parabolic; VSAT - 78 x 78 Phased Array. ACIR requirements to determine ACLR; 0; 0; 3.29; 2.88; 4.61. ACIR requirements to determine ACS; 40; 41.5; 35. Proposal 2: RAN4 can consider the simulation results of Xiaomi as a starting point of ACIR requirement discussion. |
| R4-2510182 | MediaTek (Wuhan) Inc. | Proposal 1: Simulation results based on the 5%-tile throughput are provided for scenarios 1, 4, 5, and 6, as listed in Table 2. Observation 1: Based on the simulation results in Table 2, there is no stringent ACIR requirement for the following scenarios: - Scenario 1: From NTN UL aggressor to TN UL victim. - Scenario 4: From TN DL aggressor to NTN UL victim. - Scenario 6: From NTN DL aggressor to TN DL victim. Observation 2: Based on the simulation results in Table 2 for the VSAT side: - ACIR requirement for scenario 1 with a required ACIR of < 22 dB (the worst ACIR is 21.53 dB for scenario 1 GEO with VSAT parabolic at 35° elevation angle). - Very high ACIR requirement for scenario 5 with required ACIR of < 37 dB (the worst ACIR is 36.26 dB for LEO-600 with VSAT phase array 45*45 at 35° elevation angle) Proposal 2: RAN4 to consider ACLR of 24dB for Ku-band VSATs. Proposal 3: Simulation results based on the average throughput loss ratio are provided for scenario 5, as listed in Table 3. Observation 3: Based on the simulation results in Table 3: - The ACIR requirement for scenario 5 with a required ACIR of < 34 dB (the worst ACIR is 33.21 dB with VSAT parabolic and LEO-600 at 35° elevation angle). Observation 4: ACIR requirements of average throughput loss ratio is also too stringent for scenario 5. Proposal 4: FFS on how to relax VSAT ACS requirement. |
| R4-2510215 | Ericsson | Observation 3 For GEO Scenario 1 with interference from NTN UL to TN UL, VSAT phased array antenna shows more stringent requirements possibly because of the effect of VSAT sidelobes towards the TN BS. Observation 4 For GEO Scenario 4 with interference from TN DL to NTN UL, the interference from TN BS towards satellite is not very high at 90 degrees elevation angle. However, at 35 degrees elevation angle, the 5%ile throughput cannot be achieved as the NTN UEs are out of coverage. The ACIR here surprisingly is lower than Scenario 8. Observation 5 For GEO Scenario 5 with interference from TN DL to NTN DL, the interference from TN BS towards VSAT UE is very high due to higher TN BS antenna height with VSAT phased array representing a more stringent case. However, at 35 degrees elevation angle, the 5%ile throughput cannot be achieved as the NTN UEs are out of coverage. Observation 6 For GEO Scenario 6 with interference from NTN DL to TN DL, there is negligible effect from the satellite interference towards the TN UE. Observation 7 For LEO600 Scenario 1 with interference from NTN UL to TN UL, VSAT phased array antenna shows more stringent requirements possibly because of the effect of VSAT sidelobes towards the TN BS. Observation 8 For LEO600 Scenario 4 with interference from TN DL to NTN UL, the interference from TN BS towards satellite is not very high at 90 degrees elevation angle. However, at 35 degrees elevation angle, the 5%ile throughput cannot be achieved as the NTN UEs are out of coverage. The ACIR here surprisingly is lower than Scenario 8. Observation 9 For LEO600 Scenario 5 with interference from TN DL to NTN DL, the interference from TN BS towards VSAT UE is very high due to higher TN BS antenna height with VSAT phased array representing a more stringent case. However, at 35 degrees elevation angle, the 5%ile throughput cannot be achieved as the NTN UEs are out of coverage. Observation 10 For LEO600 Scenario 6 with interference from NTN DL to TN DL, there is negligible effect from the satellite interference towards the TN UE. Observation 11 Scenarios 5 represents the most challenging case to derive NTN-VSAT ACS and needs further discussion within RAN4. Proposal 2 Based on the simulation results, the SAN-VSAT ACLR ACS values highlighted in Table 2.2-5 can be considered for co-existence between TN and NTN. Proposal 3 RAN4 to treat the co-existence simulation assumptions for Ku-band provided in the Annex, as a running document. |
| R4-2510240 | Qualcomm Incorporated | The summary of the required ACIR for every scenario is summarized in Table 13. Table 13: Required ACIR values for all the scenarios Scenario; Satellite; Antenna Type; ACIR Elevation 90 deg; ACIR Elevation 35 deg. Scenario 1 NTN UL aggressor TN UL victim VSAT ACLR to be defined; GEO; Phase array 45x45; 25; 25. Phase array 78x78; 15; 30. Parabolic; 15; 20. LEO; Phase array; 0; 25. Parabolic; 0; 20. Scenario 6 TN DL aggressor NTN UL victim SAN ACS to be defined; GEO; Phase array 45x45; 15; 20. Phase array 78x78; 15; 15. Parabolic; 15; 20. LEO; Phase array; 25; 25. Parabolic; 25; 30. Scenario 5 TN DL aggressor NTN DL victim VSAT ACS to be defined; GEO; Phase array; 40; 45. Parabolic; 40; 40. LEO; Phase array; 45; 45. Parabolic; 40; 40. Scenario 4 NTN DL aggressor TN DL victim SAN ACLR to be defined; GEO; Phase array; 0; 0. Parabolic; 0; 0. LEO; Phase array; 0; 0. Parabolic; 0; 0. In this paper, we provided a coexistence simulation result between TN and NTN and vice versa for both elevation angles of 35 and 90 degrees considering both NTN VSAT parabolic and phases array antennas. We have made the following observations as shown below: Observation 1: The required ACIR values for Scenario 1 for VSAT under worst case assumption with 35 degrees of elevation can be 20 dB and the TN BS ACS of 30 dB can be respected. Hence the ACLR of the NTN UE VSAT can be defined with 20 dB for the Parabolic antenna case. Observation 2: The required ACIR values for Scenario 1 for VSAT with phased array antenna under worst case assumption with 35 degrees of elevation can be 25 dB and the TN BS ACS of 30 dB can still be respected. The ACLR of the NTN UE VSAT can be defined with 25 dB for the phased array antenna case. Observation 3: The ACIR values for Scenario 1 for VSAT with phased array antenna 78x78 for GEO under worst case assumption with 35 degrees of elevation is 30dB. With 90degree of elevation, the ACIR is 15dB. In the real development, the interference with 78*78 from NTN UL to TN UL would be smaller than that with 45*45 case, due to smaller side lobes. Observation 4: The interference from NTN DL to TN DL for Scenario 6 is negligible for all the satellite altitudes and at different elevation angles. The TN UE ACS of 24 dB can be respected. Hence the ACLR of the NTN Satellite can be defined with even less value than 24 dB or based on other scenarios. Observation 5: The required ACIR value for scenario 5 is 40 dB considering different elevation angles for VSAT using parabolic antenna which is exceeding the TN BS ACLR of 37 dB. The main reason for the high ACIR requirement is the very weak NTN link relative to the adjacent channel interference from the TN. Under the current simulation assumptions, even in the absence of TN interference, the NTN performance is already poor, making the system highly sensitive to additional interference. Imposing a stringent adjacent channel coexistence requirement may not be effective in ensuring the desired overall performance. Observation 6: The ACIR value for scenario 5 is 45 dB considering different elevation angles for VSAT using phased array antenna which is exceeding the TN BS ACLR of 37 dB. Similarly, the main reason for the high ACIR requirement is the very weak NTN link relative to the adjacent channel interference from the TN. Under the current simulation assumptions, even in the absence of TN interference, the NTN performance is already poor, making the system highly sensitive to additional interference. Imposing a stringent adjacent channel coexistence requirement may not be effective in ensuring the desired overall performance. Observation 7: For scenario 5, 78x78 case shows similar results as 45x45 case with up to 45 dB ACIR. Observation 8: The ACIR values for Scenario 4 are between 15 to 30 dB for different satellite altitudes and different elevation angles. The TN BS ACLR of 30 dB can be respected. Hence the ACS of the satellite can be defined with 30 dB. Furthermore, the system parameters for the elevation angle at 35 degrees need to be reconsidered to avoid the out of coverage for UL with GEO. Observation 9: For scenario 4, 78x78 case shows better results compared to 45x45 case. Based on the above observations we have the following proposals: Proposal 1: Based on the observations 1, and 2, the phased array antenna case should consider stringent requirements compared to parabolic antenna case. Proposal 2: Based on the observations 1, and 2, the ACLR for NTN UE VSAT is 20 dB and 25 dB for parabolic and phased array antenna respectively. Proposal 3: Based on observation 2, and 3, the GEO and LEO VSAT types can be defined a unified ACLR requirements. Proposal 4: Based on observation 4, the ACLR of the satellite can be defined with value less than 24 dB. It is proposed to define ACLR of GEO/LEO as 14dBc comparing with Ka-band ACLR, Proposal 5: Based on observation 5, in case that cell edge throughput is “NaN” due to the out of coverage, RAN4 can conclude based on average throughput loss. Proposal 6: When deriving VSAT ACS, several factors should be considered including the likelihood of encountering similar geometrical conditions (i.e., a very weak NTN link and a very strong adjacent TN link) in realistic VSAT operations. The VSAT ACS can be defined as [26-30]dB, and the justification of defining a more relaxed value compared with coexistence simulations should be captured in the TR. Proposal 7: Based on observation 8 and 9, the ACS of the satellite can be defined as 30dB for LEO, and 20dB for GEO. Proposal 8: Summary of proposed ACLR and ACS for Ku-band is given as below table: SAN; VSAT. GEO; LEO; Parabolic; Phased array. ACLR (dBc); 14; 14; 20; 25. ACS (dBc); 20; 30; [26-30]; [26-30]. |
| R4-2510356 | ZTE Corporation, Sanechips | In this contribution, we shared coexistence simulation results for NTN Ku band and the observations are made as follows: Observation 1: For case 4 and scenario 4 and 5, interpolated ACIR values to meet the 5% throughput loss criteria is NaN when elevation angle is 35°due to large interference. Observation 2: The required ACIR for NTN Ku band coexistence is as follows: Scenario; 1; 4; 5; 6. Frequency; 14GHz; 14GHz; 11GHz; 11GHz. Scenario case; Aggressor NTN UL to Victim TN UL; Aggressor TN DL to Victim NTN UL; Aggressor TN DL to Victim NTN DL; Aggressor NTN DL to Victim TN DL. Required ACIR[dB]; GEO; 34.03; 34.25; 35.8; 0. LEO600; 30.15; 13.75; 46.37; 0. |

## Open issues summary

### Issue 2-1: ACIR

Companies have provided ACIR results for the scenarios 1, 4, 5 and 6 which have been prioritized in the last WF [2].  The results of the contributions for this meeting are summarized in the following table. Where provided, two values for 35 and 90 degrees of elevation are listed in the table (separated by /). The values represent the ACIR @ 5% throughput loss. Where no value can be provided, the average values were taken. In case both, 5%-tile and average are provided, the worst case is listed per elevation.  The values for phase array are based on 45x45 panel as it was prioritized for ACIR simulations. The rightmost columns represent the average of the worst-case values of all contributions. See following Table 2.2.1-1

Table 2.2.1-1: ACIR Values

| ACIR (worst case btw 35/90 deg) |  |  | CATT R4-2509337 | Xiaomi R4-2509770 | Mediatek R4-2510182 | Ericsson R4-2510215 | Qualcomm R4-2510240 | ZTE R4-2510356 | Avg. of worst case |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| scenario 1 NTN UL aggressor TN UL victim VSAT ACLR to be defined | GEO | Phase array 45x45 | 31.55 | 2.88 | 21.16 / 17.92 | 16 | 25 / 25 | 22.57 / 20.71 | 19.9 |
|  |  | Phase array 78x78 | - | 4.61 | 21.45/3.04 | - | 30 / 15 | 20.77 / 7.32 | 19.2 |
|  |  | Parabolic | 18.67 | 3.29 | 21.53 / 15.51 | 9 | 20 /15 | 34.03 / 33.04 | 17.7 |
|  | LEO-600 | Phase array 45x45 | 27.77 | 0 | 0/0 | 6 | 25 / 0 | 18.91 / 0 | 12.9 |
|  |  | Parabolic | 14.52 | 0 | 0/0 | 0 | 20 / 0 | 30.15 / 8.43 | 10.8 |
| scenario 4 TN DL aggressor NTN UL victim SAN ACS to be defined | GEO | Phase array 45x45 | 37.77 | - | 0/0 | 14 | 0/0 | 33.04 / 8.82 | 17.0 |
|  |  | Phase array 78x78 | - | - | 0/0 | - | 0/0 | 34.25 / 7.35 | 11.4 |
|  |  | Parabolic | 38.43 | - | 0/0 | 10 | 0/0 | 33.36 / 8.98 | 16.4 |
|  | LEO-600 | Phase array 45x45 | 40 | - | 10.72/14.51 | 12 | 0/0 | >26 / 13.75 | 18.5 |
|  |  | Parabolic | 40 | - | 10.72/14.51 | 13 | 0/0 | >30 / 12.94 | 19.5 |
| scenario 5 TN DL aggressor NTN DL victim VSAT ACS to be defined | GEO | Phase array 45x45 | 40 | 40.09 | 35.92/35.3 | 42 | 45 / 40 | 13.51 / 30.04 | 38.8 |
|  |  | Phase array 78x78 | - | 35.23 | 32.44/30.77 | - | 40 / 40 | 9.29 / 20.57 | 32.1 |
|  |  | Parabolic | 33.58 | 41.47 | 35.53/34.35 | 32 | 40 / 40 | 33.22 / 35.8 | 36.4 |
|  | LEO-600 | Phase array 45x45 | 40 | 40.34 | 36.26/35.3 | 38 | 45 / 45 | 16.77 / 14.65 | 36.1 |
|  |  | Parabolic | 38.41 | 39.84 | 35.88/34.36 | 29 | 40 / 40 | 37.74 / 46.37 | 38.3 |
| scenario 6 NTN DL aggressor TN DL victim SAN ACLR to be defined | GEO | Phase array 45x45 | 0 | 0 | 0/0 | 0 | 20 / 15 | 0 | 3.3 |
|  |  | Phase array 78x78 | - | 0 | 0/0 | - | 15 / 15 | 0 | 3.75 |
|  |  | Parabolic | 0 | 0 | 0/0 | 0 | 20 / 15 | 0 | 3.3 |
|  | LEO-600 | Phase array 45x45 | 0 | 0 | 0/0 | 0 | 25 / 25 | 0 | 4.2 |
|  |  | Parabolic | 0 | 0 | 0/0 | 0 | 30 / 25 | 0 | 5.0 |

Issue 2-1: ACIR

Moderator’s Proposals:

- Option 1: To discuss and agree on ACIR values based on average values provided in table above.

  - Agree on a compromise set of ACIR values as a Way Forward

- Option 2: Allow more time to companies to update their ACIR Values, if necessary

### Issue 2-2: ACLR and ACS

The values of ACIR, ACLR and ACS are related by the following Equation (1):

|  | ACIR=11ACLR+1ACS | Equation (1) |
| --- | --- | --- |

For TN, the following ACLR/ACS values have been selected based on [1] see following Table 2.2.2-1:

Table 2.2.2-1: ACLR and ACS for Ku-band TN

|  | TN |  | Values |
| --- | --- | --- | --- |
| 11 GHz (IMT studies for 10 GHz from TR 38.921) | BS | ACLR | 37 dB |
|  |  | ACS | 40 dB |
|  | UE | ACLR | 24 dB |
|  |  | ACS | 31 dB |
| 14 GHz (IMT studies for 15 GHz from TR 38.922) | BS | ACLR | 30 dB |
|  |  | ACS | 30 dB |
|  | UE | ACLR | 24 dB |
|  |  | ACS | 24 dB |

Based on ACIR results from previous subsection, respective values for ACLR and ACS can be derived for both, SAN and VSAT. The following table summarizes the proposals and observations regarding ACLR and ACS made in contributions. The values in square brackets represent the range of the values, the values in braces are the detailed values from contributions. See following Table 2.2.2-2:

Table 2.2.2-2: ACLR and ACS NR-NTN at 11/14 GHz (Ku band)

|  | SAN |  | VSAT |  |
| --- | --- | --- | --- | --- |
|  | GEO | LEO | Parabolic | Phased array 45x45 |
| ACLR (dBc) | [13-18] (13, 18, 14) | [13-18] (13, 18, 14) | [16-31.55] (31.55, 24, 16, 20) | [16-31.55] (31.55, 24, 16, 25) |
| ACS (dBc) | [18-40] (40, 18, 20) | [20-40] (40, 20, 30) | [25-40] (40, 25, [26-30]) | [26-40] (40, 31, [26-30]) |

Issue 2-2: ACLR and ACS

Moderator’s proposals

- Option 1: Further discuss and agree on values for SAN/VSAT ACLR/ACS based on values from Table 2.2.2-2.

  - To discuss whether to take worst-case or average values and which values to consider for each scenario from contributions (e.g. handling of outliers)

  - Agree on a compromise set of ACLR and ACS values as a Way Forward.

- Option 2: Allow more time for companies to review and to submit updated results