3GPP TSG RAN WG4 Meeting #116		 R4-2511492

# Bengaluru, India, August 25th – 29th, 2025

Source:	Boost Mobile Network

Title:	On possibility of extending Tx-Rx separation of n66

Agenda item:			5.28.1

Document for:	Discussion

# 1. Introduction

This paper explains the current issue to flexibly and effectively utilize n66, specifically DL 2180 – 2200 MHz, due to restriction of Tx-Rx separation value, and outlines operators requirements and proposes a way forward.

# 2. Discussion

As per 38.101-1, n66 is defined as UL 1710 – 1780 MHz, DL 2110 – 2200 MHz, with the default Tx-Rx separation of 400MHz.

Figure 1a: n66 operating band definition

Figure 1b: Tx-RX separation for n66

However, the following case shows a requirement of usage of UL/DL pair in n66 that is not possible as per current specifications.

Figure 2: UL/DL pair of n66 exceeding default Tx-Rx separation

As shown in the Figure2, the configuration shows in Config2 (yellow) is not possible since the Tx-Rx separation exceeds the default value defined in Table 5.4.4.1-1 of TS 38.101-1. Similarly as Config3 (grey) is also not possible due to the same reason.

One may argue that, since the initial design of Tx-Rx separation for n66 is already specified and UE commercial product is already available, one workaround for the problem shown in Config2 or Config3 is by configuring DL CA_n66(2A) between the DL H block and DL A1/B1 block, or between DL J block and DL A1/B1 block.

Although this work around may work in some cases, the workaround does not give a complete freedom and flexibility for the operator to utilize its spectrum assets.

Requirement 1: 	Operator should be able to flexibly pair UL/DL channel according to the defined range of UL/DL of the operating band.

Furthermore, in this case, the above workaround restricts the UE with e.g. 2CC DL CA capability to always configure intra-band non-contiguous CA (DL CA_n66(2A)) in order to utilize the DL A1/B1 block, while the UE can enjoy better performance if it is allowed to perform inter-band CA (e.g. in Boost case with DL n71 or n70).

Requirement 2: 	Operator should be able to flexibly utilize n66 DL A1/B1 block (2180-2200 MHz) in inter-band CA without the restriction to configuring it as intra-band CA (DL CA_n66(2A)).

Proposal: 	RAN4 to discuss ways/mechanisms to allow the UL/DL configuration of n66 where the Tx-Rx exceeding the default 400MHz value.

# 3. Conclusion

The problem to flexibly and effectively utilize DL n66 2180 – 2200 MHz was explained. The following are Boost Mobile requirements and proposal.

Requirement 1: 	Operator should be able to flexibly pair UL/DL channel according to the definition of the operating band.

Requirement 2: 	Operator should be able to flexibly utilize n66 DL A1/B1 block (2180-2200 MHz) in inter-band CA without the restriction to configuring it as intra-band CA (DL CA_n66(2A)).

Proposal: 	RAN4 to discuss ways/mechanisms to allow the UL/DL configuration of n66 where the Tx-Rx exceeding the default 400MHz value.

# Reference

- TS 38.101-1, “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”