3GPP TSG-RAN WG4 Meeting #116									                          R4- 2511494

Bangaluru, India, 25th – 29th Aug. 2025

Agenda item:			7.21.2.1

Source:	Ericsson

Title:	Discussion on SBFD symbol on UL duty cycle evaluation

Document for:	Discussion

# Introduction

In the last RAN4 #115 meeting, with regards of the SBFD symbols, no conclusions made on the HPUE duty cycle evaluation [1].

Sub-topic 2-2: Update for HPUE duty cycle evaluation Issue 2-2-1: The necessity to update for HPUE duty cycle evaluation - No agreement in this meeting and the way forward from RAN4#114bis can still be followed.

The WF in RAN4 #114bis meeting are captured below [2]:

Issue 1-7-1: Update for HPUE duty cycle evaluation Way forward: RAN4 has discussed the two following points: - For non-SBFD-aware UE, the “uplink symbols transmitted” used for uplink duty cycle evaluation are symbols with uplink transmission; - For SBFD-aware UE, the symbols used for uplink duty cycle evaluation are symbols with uplink transmission, including the non-SBFD symbols and SBFD symbols with UL transmission. FFS RAN4 specification impact by taking into account the above understanding for SBFD-aware UE. Issue 2-1-1: RF requirement impact for PUSCH with resource muting Way forward: RAN4 has discussed this issue and will try to draft a reply LS to RAN1 based on existing results in next meeting. - Evaluation on remaining scope can be continued, and the issues can be raised if found in future.

# Discussion

## SBFD symbols on the UL duty cycle evaluation

In the RAN4 #114bis meeting, one company proposed to update 38.101-1 clause 6.2.1 by adding the “SBFD symbol” together with the existing “UL symbol transmitted “in the UL duty cycle evaluation. The current specification of the UL duty cycle evaluation was excerpted below [3]:

If a UE supports a different power class than the default UE power class for the band and the supported power class enables the higher maximum output power than that of the default power class:

-	if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is absent and the field of UE capability maxUplinkDutyCycle-PC1dot5-MPE-FR1 is absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than 50% (The exact evaluation period is no less than one radio frame); or

-	if the field of UE capability maxUplinkDutyCycle-PC2-FR1 is not absent and the percentage of uplink symbols transmitted in a certain evaluation period is larger than maxUplinkDutyCycle-PC2-FR1 as defined in TS 38.306 (The exact evaluation period is no less than one radio frame); or

From UE’s perspective, as long as the UE transmits in the evaluation period, it is counted as ‘uplink symbols transmitted’ —whether TDD UL symbols or SBFD symbols—are considered as UL symbols for the purpose of UL duty cycle evaluation. At this stage, we do not anticipate any impact on RAN4 UE specification, nor do we see a need for clarification. Furthermore, there is no definition of SBFD-aware UE or SBFD-capable UE, adding such a term in Rel-19 makes the clause redundant.

- Any UL symbols transmitted, whether TDD UL symbols or SBFD symbols—are considered as UL symbols for the purpose of UL duty cycle evaluation.

- The SBFD symbol does not affect the UL duty cycle evaluation in the RAN4 UE specification, and no further clarification is needed.

# Conclusion

In the previous sections, we made the following observations:

Observation 1	Any UL symbols transmitted, whether TDD UL symbols or SBFD symbols—are considered as UL symbols for the purpose of UL duty cycle evaluation.

Based on the discussion in the previous sections, we propose the following:

Proposal 1	The SBFD symbol does not affect the UL duty cycle evaluation in the RAN4 UE specification, and no further clarification is needed.

# References

- R4-2508707, Way forward for [115][306] NR_duplex_evo_General, Samsung.

- R4-2504751, Way Forward for [114bis][306] NR_duplex_evo_General, Samsung.

- TS 38.101-1, v19.1.0.