3GPP RAN WG4 Meeting #116																R4-2511497

Bangalore, India, 25th – 29th August 2025

Agenda item:	7.6.3.2

Source:	Apple

Title:	On differentiation between PC1.5 handheld and non-handheld RF requirements

WI/SI:	NR_IoT_NTN_req_test_enh

Release:	Rel-19

Document for:	Decision

# 1	Introduction

After the RAN#86 meeting, the Rel-17 WI was approved that aimed at enabling the 5G/NR radio access technology for non-terrestrial satellite deployments [1]. That time the only available power class for NTN was PC3 offering the maximum nominal output power of 23dBm. During the RAN#104 meeting a new WI was agreed [2] with one of the objectives to enable higher power classes for NTN.

During the previous meetings RAN WG4 discussed whether to specify or not PC1.5 power class. Since there was uncertainty regarding PC1.5 implementation options for the handheld devices, RAN WG4 concluded that Rel-19 would focus on the non-handheld devices leaving handheld devices as one of the potential topics for Rel-20. However, since the PC1.5 power class may support both handheld and non-handheld devices, it would create a precedent that the same power class covers two device classes, which may have quite different RF requirements. Thus, in this discussion paper we present our further considerations on a need to different between certain RF requirements that could be different both handheld and non-handheld devices.

# 2	PC1.5 handheld and non-handheld RF requirements

As discussed and agreed by RAN WG4, PC1.5 for NTN is going to be introduced in Rel-19 but will be limited to the non-handheld devices. Practically speaking it will mean that all RF requirements defined in Rel-19 will be based on the assumptions, which are not necessarily applicable to the handheld devices; at least RAN WG4 never made a decision that RF assumptions for the PC1.5 non-handheld devices shall be fully aligned with the handheld devices.

Observation 1a:	RAN WG4 agreed to enable PC1.5 for the non-handheld devices in Rel-19.

Observation 1b:	RAN WG4 never made an agreement that RF assumptions for the PC1.5 non-handheld devices shall be aligned with handhelds.

Observation 1c:	Both PC1.5 1TX and 2TX non-handheld architectures are allowed whereupon RAN WG4 never made an agreement which RF architecture would be feasible for PC1.5 handhelds.

Based on that we anticipate that RF requirements devised for the PC1.5 non-handheld devices would be directly applicable to the PC1.5 handhelds, which could be defined in Rel-20. While some of the requirements and principles can be definitely re-used, there more sensitive aspects, such as RSD and A-MPR, which most likely will be different for handhelds.

Observation 2:	We anticipate that certain RF requirements, such as RSD and A-MPR, will be different for non-handheld and handheld devices and thus should be differentiated accordingly.

Based on these considerations we ask RAN WG4 to discuss and agree about a way to differentiate, already in Rel-19, between the non-handheld and handheld RF requirements.

# 3	Conclusions

In this discussion paper we have presented our view on a need to differentiate between certain RF requirements, which will be most likely different for the PC1.5 handheld and non-handheld devices. Since PC1.5 for the non-handheld devices is going to be introduce in Rel-19, there should be a framework to be able to differentiate between handheld requirements that will be added in Rel-20.

Observation 1a:	RAN WG4 agreed to enable PC1.5 for the non-handheld devices in Rel-19.

Observation 1b:	RAN WG4 never made an agreement that RF assumptions for the PC1.5 non-handheld devices shall be aligned with handhelds.

Observation 1c:	Both PC1.5 1TX and 2TX non-handheld architectures are allowed whereupon RAN WG4 never made an agreement which RF architecture would be feasible for PC1.5 handhelds.

Observation 2:	We anticipate that certain RF requirements, such as RSD and A-MPR, will be different for non-handheld and handheld devices and thus should be differentiated accordingly.

Proposal 1:	Since PC1.5 handheld and non-handheld devices can be based on different RF architectures (and use different RF components), at least the following RF requirements should be differentiated already in Rel-19: Rx RSD and Tx A-MPR.

Proposal 2:	If PC1.5 A-MPR requirements are defined for non-handheld devices in Rel-19, they should be differentiated from handheld A-MPR that will be defined in Rel-20.

Proposal 3:	If PC1.5 RSD requirements are defined for non-handheld devices in Rel-19, they should be differentiated from handheld RSD that will be defined in Rel-20.

# 4	References

- RP-193234, "Solutions for NR to support non-terrestrial networks (NTN)", Thales

- RP-240857, "New WID: Enhanced requirements and test methodology for NR and IoT NTN"