3GPP RAN WG4 Meeting #116																	R4-2511498

Bangalore, India, 25th – 29th August 2025

Agenda item:	7.6.3.2.1

Source:	Apple

Title:	On practical aspects of defining HPUE AMPR for the satellite bands

WI/SI:	NR_IoT_NTN_req_test_enh

Release:	Rel-19

Document for:	Decision

# 1	Introduction

After the RAN#86 meeting, the Rel-17 WI was approved that aimed at enabling the 5G/NR radio access technology for non-terrestrial satellite deployments [1]. One of the WI objectives was to define and introduce the corresponding bands into the 3GPP specifications. The outcome of these discussions was two NTN bands – L-band and S-band – which were added as bands n255 and n256, respectively, to the Rel-17 core specifications. However, already that time it was noted that there exist other satellite deployments. To limit the overall workload in Rel-17, it was decided by RAN WG4 to focus on L- and S-bands with the common understanding that other potential NTN bands should follow the "normal" process of submitting a new spectrum WI.

So, during the RAN#98 meeting a new Rel-18 NTN spectrum WI was agreed with the objective of adding support for a new NTN band where the UL is on the L-band and the DL is on the S-band [2]. And a so-called extended L-band was also completed in Rel-18. After the RAN#104 meeting another spectrum WI was agreed for a satellite band operating in the S-band frequency range [3]. In fact, it is anticipated that more satellite bands will be defined by 3GPP as the outcome of the corresponding regulatory decisions, especially the ones that allow usage of the satellite services in the existing terrestrial band ranges.

Every time 3GPP decides to standardise a new band, all the regulatory requirements are carefully checked. And if there is a need to protect other services running in the neighbour frequencies, then 3GPP also conducts a set of simulations to define whether additional power back-off is needed. In fact, the regulatory requirements have to be checked not only for new bands, but also for existing ones if 3GPP decides to adopt higher power classes; for instance, PC2 has been agreed a new high power class for all FR1 NTN bands that will trigger the corresponding work in RAN WG4 [4].  However, running power back-off simulations and defining, if needed, the corresponding A-MPR values is a non-trivial task that requires a lot of offline efforts and online discussions to calibrate results, agree on a common baseline from which a harmonized set of the power back-off values is derived. However, some of the simulations and/or scenarios are not really needed for the satellite bands. Thus, in this discussion paper we present a set of simplifications that can be applied to all satellite bands thus saving a lot of time and efforts.

# 2	A-MPR for the satellite bands

## 2.1	General considerations

While running power back-off simulations for any given (satellite) band, usually all the possible combinations are tested that usually involve the following key aspects:

-	Channel bandwidth(s). Simulations are run for every channel bandwidth defined for a particular band.

-	Channel bandwidth placement. If a band is larger than the considered channel bandwidth, then companies usually simulate several different channel bandwidth locations within the band because inner channel usually need smaller power back-off.

-	RB start and number of RBs. For any given channel bandwidth at a particular location within the band, RAN4 usually simulates all possible combinations of the RB allocations and where they start.

-	Waveforms. While running simulations RAN4 checks performance of all the existing waveforms, i.e. DFT-s-OFDM and CP-OFDM.

-	Higher-order modulations. While running initial simulations RAN4 usually focuses first on more important modulations, such as PI/2 BPSK and QPSK. However, also higher order modulations should be explicitly simulated instead of applying some fixed relaxation.

-	Power class. Since the amount of power back-off depends on the maximum UE Tx power and the PA assumptions, the corresponding simulations are conducted for PC3 as a baseline and, if applicable, also for higher power classes.

It is also worth noting that it is not only about how much time it takes to run all power back-off simulations, but also about time it takes all companies to bring the corresponding numbers to 3GPP, align them, revise, run if needed more simulations and to agree on the final numbers. And what takes a lot of time is to agree on the frequency and A-MPR region portioning, for which the final A-MPR values are specified.

Unlike terrestrial bands, the satellite bands are characterised by a much more challenging link budget, especially in the UL direction. And to achieve a proper link budget in UL, the network ensures scheduling in such a way that the corresponding modulation schemes are used in conjunction with a limited number of scheduled RBs that provides better PSD at the satellite node receiver. Based on these observations, our view is that 3GPP can focus efforts on optimising A-MPR values for regions with low number of RBs because larger allocations are typically not useable due to low PSD and large power back-off.

## 2.2	A-MPR simplifications for the satellite bands

To exemplify the nature of "simplifications" for A-MPR definitions we present below two figures showing anticipated A-MPR values for edge 10MHz channels for the existing NTN bands n254 and n256. And it should be understood that larger channels, such as 15 and 20MHz, would require even larger power back-off values.

Figure 2.2-1: Anticipated power back-off for the PC2 10MHz channel for n254 (left) and n256 (right).

So, referring to power back-off results in Figure 2.2-1 one can make the following observations:

-	Large allocations are not usable for NTN because the resulting PSD would be quite low. As an example, if we take PC2 26dBm/15kHz as a reference point for the maximum PSD that can be achieved for a satellite link, then 25RB allocation would degrade PSD by 14dB. Similarly, a 50RB allocation would degrade PSD by 17dB.

-	In addition to the huge PSD degradation caused by a large allocation size, some bands would impose additional power back-off because of out-of-band emission requirements which are typically present for all satellite bands.

Thus, while 3GPP still can define A-MPR for all possible channel bandwidth and allocation sizes, it would be a huge waste of time to define and optimise A-MPR regions for allocations of 25RBs and larger (in fact, it can be even argued that 25RB allocations are not practical either). Based on that we suggest adopting the following principles to make sure that we focus on right regions while defining A-MPR for the satellite bands:

-	3GPP can define A-MPR for all channel bandwidths and allocations following the existing procedures.

-	The focus for A-MPR optimisations should be on small allocations of around 1..6 RBs.

-	Allocations larger than 25RBs can be regarded as "large A-MPR area" with a single A-MPR value, which can be taken as a worst-case A-MPR value from all proponents.

Using PC2 10MHz A-MPR power back-off for the NTN band n256 as an example, RAN WG4 can put more efforts into optimising A-MPR in the "low" area constrained by small RB allocations. A-MPR "mid" region up to 25RB can be also considered for more detailed analysis, even though in some cases A-MPR values can be quite high meaning that the room for optimisations is quite marginal. As for allocations that are larger than 25RBs, a fixed value can be assigned corresponding to the highest recorded A-MPR value (e.g. 18.5dB in this case).

Figure 2.2-2: Example of A-MPR regions – low, mid, and high – for the NTN bands.

Proposal 1:	While defining A-MPR for the satellite bands, RAN WG4 can follow existing procedures and define A-MPR for all applicable channel bandwidth and allocation size.

Proposal 2:	The focus area for optimised A-MPR should be small allocations of 1..6RB.

Proposal 3:	Allocations larger than 25RB can be put into a single "large A-MPR region" and assigned the corresponding single A-MPR value (e.g. taken as the worst-case number from submitted results).

# 3	Conclusions

In this discussion paper we have presented our initial considerations on a potential way to reduce specification efforts in RAN WG4 to determine and define, when needed, A-MPR values for the satellite bands. As explained in the discussion paper, since the satellite bands have quite tight link budget, especially in the UL direction, there is no practical need in certain scenarios, such as large RB allocations, specification efforts for which can be significantly reduced as explained in the paper.

Proposal 1:	While defining A-MPR for the satellite bands, RAN WG4 can follow existing procedures and define A-MPR for all applicable channel bandwidth and allocation size.

Proposal 2:	The focus area for optimised A-MPR should be small allocations of 1..6RB.

Proposal 3:	Allocations larger than 25RB can be put into a single "large A-MPR region" and assigned the corresponding single A-MPR value (e.g. taken as the worst-case number from submitted results).

# 4	References

- RP-193234, "Solutions for NR to support non-terrestrial networks (NTN)", Thales

- RP-223431, "New WID on Introduction of the satellite L-/S-band", Globalstar, Apple

- RP-241661, "New WID on Introduction of NR-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL)", DISH Network, EchoStar, TerreStar, OmniSpace, Telus

- RP-240857, "New WID: Enhanced requirements and test methodology for NR and IoT NTN"