3GPP TSG-RAN WG4 Meeting #116	R4-2511518

Bengaluru, India, August 25th – August 29th, 2025

Source:	Nokia

Title:	Discussion on BS OTA test reductions

Agenda item:	7.9.4.2.1	Identification and reduction of BS OTA test scope (TxIM, RX OoB blocking)

Document for:	Approval

Introduction

During last RAN4#115 meeting in Malta discussion on OTA test scope reduction was continued. A way forward was agreed in [1] which provides more conclusions on some detailed issues regarding the OTA test enhancements for this objective for BS RF evolution work item. This contribution continues further Nokia’s views on the topic.

Discussion

  - BS OTA test scope reduction

Following agreements on OTA scope reductions are included in the agreed Way forward after RAN4#115 meeting [1]:

Issue 1-1-1: test scope reduction for OBUE - Agreement: - Further discuss how to reduce the test simplification for OBUE limits. - Option 1: 1xCBW offset - Option 2: 3xCBW offset - Option 3: 3xCBW offset if 3rd interfering signal is falling within the range of FDL_low to FDL_high of the operating bands and 1xCBW and 2xCBW if 3rd interfering signal is falling higher than FDL_high+ ΔfOBUE or lower than FDL_low-ΔfOBUE Issue 1-1-2: test scope reduction for spurious emission - Agreement: - Option 1: 3xCBW offset Issue 1-2-1: Out of band blocking - Agreement on step size of OOBB requirement for further discussion: - Option 1: 1/3 of the minimum supported bandwidth - Option 2: Use 1% of the momentary frequency as the step size for OTA receiver out-of-band blocking testing for frequencies up to 6000 MHz. Use a fixed step size of 60 MHz for frequencies higher than 6000 MHz - Option 3: mixed EMC and option 1 approach, FFS on the details. - Option 4: To use 2/3 of the test signal configuration channel bandwidths - Option 5: Use 1MHz steps size below 100MHz and 1% of the momentary frequency as the step size for OTA receiver out-of-band blocking testing for frequencies up to 6000 MHz. Use a fixed step size of 60 MHz for frequencies higher than 6000 MHz. - Other options are not precluded Issue 2-2: Reference method and preferred method - Agreement: - Further discuss TRP estimation simplifications and the applicability improvements on TRP sampling grid - FFS on how to incorporate the OTA chamber related issues into the discussion on the reference test method.

    - OTA Transmitter Intermodulation (TX IM)

OBUE

The OTA transmitter intermodulation requirement is a measure of the capability of the transmitter unit to inhibit the generation of signals in its non-linear elements caused by presence of the wanted signal and an interfering signal reaching the transmitter unit via the RDN and antenna array from a co-located base station. The requirement applies during the transmitter ON period and the transmitter transient period.

The test purpose is to verify the ability of the transmitter units associated with the RIB under test to restrict the generation of intermodulation products in its nonlinear elements caused by presence of the wanted signal and an interfering signal reaching the transmitter unit via the RDN and antenna array from a co-located base station to below specified levels.

The transmitter intermodulation level shall not exceed the TRP unwanted emission limits specified for OTA transmitter spurious emission in clause 6.7.5 (except co-location with other base stations), OTA out-of-band emissions in clause 6.7.4 and OTA ACLR 6.7.3. Naturally due to TRP test method and CLTA usage, execution of these cases requires quite a much of test effort.

Currently there are 6 interferer frequency offsets in test case, 3 on each side of the carrier defined as the following:

, for n=1, 2 and 3

For NC operation this is twice more so it is 12. Historically this approach was introduced in UTRA specification and later was reused in E-UTRA, MSR and NR. However currently number of the offsets should be reconsidered which would effectively impact the test efforts while still keeping the requirement coverage.

For additional requirement e.g for BC3 there are further 6 (or more) interferer position as below:

Observation 1: Currently, for TX IM there are 6 interferer offsets (or 12 for NC), 3 on each side of the carriers. In case of additional test requirement is counted, then it is even more

Both OBUE and ACLR tests are rather similar cases where the measurement is taken on adjacent frequencies, it would be logical to apply the same interferer frequency offset as for ACLR. However, OBUE requirement varies depends on frequency offset. The requirement is most stringent for 10MHz or higher offset from carrier frequency.

As example below is table from TS 38.104 with BS channel bandwidths for band n1 and n78. For some bands smallest CBW is 5MHz and for some others 10 MHz. For example, for band n78 5MHz CBW is not supported, thus for OBUE interference frequency offset 10 MHz CBW has to be used as the smallest one.

Table 5.3.5-1: BS channel bandwidths and SCS per operating band in FR1 (excerpt from TS 38.104)

| NR Band | SCS (kHz) | BS channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
|  | 15 |  | 5 | 10 | 15 | 20 | 25 | 30 |  | 40 | 45 | 50 |  |  |  |  |  |
| n1 | 30 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 | 45 | 50 |  |  |  |  |  |
|  | 60 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 | 45 | 50 |  |  |  |  |  |
| n78 | 15 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 |  |  |  |  |  |
|  | 30 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |
|  | 60 |  |  | 10 | 15 | 20 | 25 | 30 |  | 40 |  | 50 | 60 | 70 | 80 | 90 | 100 |

By placing 10 MHz interferer next to carrier (1st frequency offset), start frequency for the measurements is 10MHz from interferer edge where the requirement is most stringent.

Interfering signal positions partially or completely outside of BS operation band are excluded from the requirement (expect for n77, n78, n79 in Japan), but visualization below applies for Japan case as well.

It is worth to notice that relevant for interference position selection is not whether it is outside of operating band (as it is excluded as default), but rather whether possible IM products are falling within the range of FDL_low-ΔfOBUE to FDL_high+ ΔfOBUE

Further checking of existing test results shows that 1st and/or 3rd interferer offset gives the most less margin whilst having similar. Therefore, we propose 1st interferer offset as being analogue to ACLR.

Figure 2.1.1-1 Visualization of TX IM for OBUE when interfere is at 1st interferer offset

Figure 2.1.1-2 Visualization of TX IM for OBUE when interfere is at 3rd interferer offset

Observation 2: The most stringent requirement is at >=10MHz offset from channel edge

From options in WF [1] we support option 1.

Proposal 1: For OBUE test scope reduction we support Option 1: 1st interferer offset.

OTA out-of-band blocking

The OTA out-of-band blocking characteristics are a measure of the receiver unit ability to receive a wanted signal at the RIB at its assigned channel in the presence of an unwanted interferer.

The test stresses the ability of the receiver unit associated with the RIB under test to withstand high-level interference from unwanted signals at specified frequency bands, without undue degradation of its sensitivity.

The OTA general blocking interferer is injected via test antenna, with the blocking interferer producing specified interferer field strength level for each supported polarization. CW interfering signal is swept with a step size of 1 MHz within the frequency range corresponding to 30 MHz to FUL_low - ΔfOOB and from FUL_high + ΔfOOB up to 12750 MHz for FR1. Whilst for FR2, 1 MHz step size applies for frequency range 30 MHz to 6000 MHz, and above 6000MHz bigger step sizes is defined.

The OTA co-location blocking interferer for FR1 is injected via the CLTA. The CLTA is fed with the specified co-location blocking interferer power per supported polarization. CW interfering signal is swept with a step size of 1 MHz within the frequency range corresponding to downlink operating bands related to co-located systems

Observation 3: Interfering signal step size is 1 MHz in frequency range 30-12750MHz for FR1 and in frequency range 30-6000MHz for FR2.

Observation 4: Out of all test requirements of radiated conformance test specification TS 38.141-2, the OTA out-of-band blocking case is one of the cases which requires the largest test efforts.

This is largely due to 1 MHz step size used and, therefore, the number of measurements points is significantly large. Increase of step size from 1 MHz to X MHz effects test efforts proportionally.

Observation 5: Increase of interferer step size from 1 MHz to X MHz effects test efforts proportionally.

To reduce the amount of time used for the time-consuming OTA conformance testing, while continue to ensure that any spurious response within the receive bandwidth will still be verified with sufficient granularity, the measurement step size for interfering signal can be set larger than 1 MHz.

In 114bis WF we have 2 options defined, in here we discuss a bit about those:

Option 1: 1/3 of the minimum supported bandwidth. Instead of the minimum supported bandwidth, it is more relevant to use the test signal configuration channel bandwidths. In addition, other than 1/3 fraction will also provide the same or similar coverage e.g. 2/3 of the test signal configuration channel bandwidths.

It should be noted that in specification TS 37.145-2 defines test signal configuration ATCR7/ANTCR7 for OTA blocking. Furthermore, subclauses 4.11.2.10.1 and 4.11.2.12.1 text states:

“Unless otherwise stated, for all test configurations in this section, the narrowest supported NR channel bandwidth and lowest SCS for that bandwidth for the operating band shall be used in the test configuration.

Unless otherwise stated, the E-UTRA bandwidth shall be 5 MHz unless the BS does not support 5 MHz E-UTRA, in which case the E-UTRA bandwidth shall be the lowest supported bandwidth for the operating band.”

For 5MHz channel bandwidths, 1/3 leads back to 1 MHz, which is not meeting the test effort reduction target.

“Option 2: Use 1% of the momentary frequency as the step size for OTA receiver out-of-band blocking testing for frequencies up to 6000 MHz. Use a fixed step size of 60 MHz for frequencies higher than 6000 MHz.” For this option, we recommend not to use 1% on below 100MHz, as it leads even to smaller than 1MHz step size.

Taking into account WF [1] For spurious emission we support option 5.

Proposal 2: Use 1MHz steps size below 100MHz and 1% of the momentary frequency as the step size for OTA receiver out-of-band blocking testing for frequencies up to 6000 MHz. Use a fixed step size of 60 MHz for frequencies higher than 6000 MHz

Conclusion

This document shares Nokia’s views on BS OTA test scope reduction objective. The following observations and proposals have been made:

Observation 1: Currently, for TX IM there are 6 interferer offsets (or 12 for NC), 3 on each side of the carriers. In case of additional test requirement is counted, then it is even more

Observation 2: The most stringent requirement is at >=10MHz offset from channel edge

Proposal 1: For OBUE test scope reduction we support Option 1: 1st interferer offset.

Observation 3: Interfering signal step size is 1 MHz in frequency range 30-12750MHz for FR1 and in frequency range 30-6000MHz for FR2.

Observation 4: Out of all test requirements of radiated conformance test specification TS 38.141-2, the OTA out-of-band blocking case is one of the cases which requires the largest test efforts.

Observation 5: Increase of interferer step size from 1 MHz to X MHz effects test efforts proportionally.

Proposal 2: Use 1MHz steps size below 100MHz and 1% of the momentary frequency as the step size for OTA receiver out-of-band blocking testing for frequencies up to 6000 MHz. Use a fixed step size of 60 MHz for frequencies higher than 6000 MHz

- References

- R4-2508658, Way Forward for [115][304] NR_BS_RF_Part3_OTA_TRP, ZTE Corporation