3GPP TSG-RAN WG4 Meeting #116	R4-2511526

Bengaluru, India, 25th – 29th August, 2025

Agenda item:		7.21.2.1

Source:	Nokia

Title:	On SBFD general aspects and system parameters

Document for:		Approval

# Introduction

A Way Forward was agreed in RAN4#115 [1] regarding evolution of NR duplex operation. There were many agreements made regarding SBFD general aspects and system parameters. This contribution displays Nokia’s view on SBFD general aspects and system parameters.

# Discussion

## Declaration and flexibility for SBFD operation

### Declaration and flexibility for SBFD operation

The following was agreed in RAN4#115:

Agreement: - For all FR2 BS classes, a which are not expected to require analog filter(s), for each declared channel bandwidth and each declared SBFD pattern (either DUD or DU/UD), the configurations with the narrowest and widest UL/DL subband sizes out of all declared configurations shall be tested. - For FR1 MR SBFD capable BSs, a power level threshold could be established: Above this threshold, all configurations shall be tested; Below this threshold, analog filters are not expected to be used, and the configurations with the narrowest and widest UL/DL subband sizes out of all the declared configurations for each channel bandwidth and each SBFD pattern (either DUD or DU/UD) shall be tested. - Use X dBm selected from the range of [33, 38] dBm either as TRP or total conducted power as the power level threshold for FR1 MR SBFD capable BSs related to testing. - X value will be selected in next meeting.

Our view has always been that the restrictions should also be applied to FR1 MR BS due to the high output power FR1 MR BS are allowed have, which causes demanding isolation requirement between UL and DL, which would require using analog filters. After discussion in the last two RAN4 meetings, we are willing to compromise to the proposal of using a power level threshold for FR1 MR SBFD-capable BS. SBFD-capable FR1 MR BS could be realized without using analog filters when the maximum output power is 33dBm. Therefore, we propose to set the power level threshold of 33dBm.

Use 33dBm either as TRP or total conducted power as the power level threshold for FR1 MR SBFD capable BSs related to testing.

### UL subband configurations support for SBFD

The following was agreed in RAN4#115:

Agreement: - For a specific channel BW supported for SBFD, the maximum number of specific UL subband sizes are allowed to be declared - Maximum 3 subband configurations allowed for FR1 WA BS - These 3 suband configurations shall cover all DUD and DU/UD ones supported by this BS. - One UL subband size for DU/UD cases, should be counted as 2 configurations, e.g., UD 20-80MHz and DU 80-20MHz should be counted as 2 configurations. - For FR1 MR SBFD capable BSs, a power level threshold could be established: Above this threshold, maximum 3 subband configurations allowed as FR1 WA BS; Below this threshold, the restriction on the number of UL subband sizes supported is not applicable. - Adopt the same power level threshold as the one to differentiate whether all declared configurations shall be tested or not.

The restriction on the number of UL subband sizes supported for FR1 MR SBFD capable BSs is closely related to the discussion on 2.2.1. The same method of approach should be followed on both. Therefore, with the same justification as above, we propose to choose the power level of 33dBm as the power level threshold for FR1 MR SBFD-capable BS.

Use 33dBm either as TRP or total conducted power as the power level threshold for FR1 MR SBFD capable BSs related to UL subband sizes supported.

### DL subband configurations support for SBFD

The following was agreed in RAN4#115:

Agreement: - For a specific channel BW and a specific UL subband size supported, only one DL subband size is allowed to be declared and tested for DU/UD case, and only one DL subband size is allowed to be declared and tested for DUD case - This restriction shall be applied to FR1 WA BS - For FR1 MR BS SBFD-capable BSs, a power level threshold could be established: Above this threshold, this restriction is applicable as FR1 WA BS; Below this threshold, the restriction on the number of DL subband sizes supported is not applicable. - Adopt the same power level threshold as the one to differentiate whether all declared configurations shall be tested or not.

The restriction on the number of DL subband sizes supported for FR1 MR SBFD capable BSs is closely related to the discussion on 2.2.1. The same method of approach should be followed on both. Therefore, with the same justification as above, we propose to choose the power level of 33dBm as the power level threshold for FR1 MR SBFD-capable BS.

Use 33dBm either as TRP or total conducted power as the power level threshold for FR1 MR SBFD capable BSs related to DL subband sizes supported.

### Restriction on guardband size for SBFD

No agreement was reached in RAN4#115 and the way forward from RAN4#114bis can still be followed:

Agreement: - Restriction on guardband size for SBFD: - Option 3a: No restriction (manufacturer declaration-based), and no restriction on maximum/minimum number of RBs for guardband. - The declaration of guardband size should ensure the benefits from SBFD operation. - In the RAN4 specification, the clause “It is expected that guardband size should be minimized to ensure the benefits from SBFD operation.” could be captured, and the detailed wording can be further refined.

In RAN4#114bis, there was a discussion about restricting the maximum number of RBs for guardband. We have been supporting to limit the GB size from the beginning of the WI but as there was no agreement reached and specification needed to support different sizes of GB for different implementations, there was no consensus on limiting the GB size. As we agreed no restriction on GB size in RAN4#114bis, there was an agreement to add clause stating that “GB size should be minimized to ensure the benefits from SBFD operation”. This sentence as such is misleading as the guardband plays a big role in terms of avoiding inter-subband interference, so smaller guardband does not always translate into better performance. If such note is needed, we think a more accurate formulation would be: “It is expected that guardband size should be minimized to reduce performance degradation due to overhead”.

The guardband plays a big role in terms of avoiding inter-subband interference. Smaller guardband does not always translate into better performance.

If it is decided that a clause is introduced to the RAN4 specification related to guardband size, the text should say: “It is expected that guardband size should be minimized to reduce performance degradation due to overhead.”

# Conclusion

In this contribution, our views on the SBFD BS RF requirements are presented. The following proposals were listed:

- The guardband plays a big role in terms of avoiding inter-subband interference. Smaller guardband does not always translate into better performance.

- Use 33dBm either as TRP or total conducted power as the power level threshold for FR1 MR SBFD capable BSs related to testing.

Use 33dBm either as TRP or total conducted power as the power level threshold for FR1 MR SBFD capable BSs related to UL subband sizes supported.

Use 33dBm either as TRP or total conducted power as the power level threshold for FR1 MR SBFD capable BSs related to DL subband sizes supported.

If it is decided that a clause is introduced to the RAN4 specification related to guardband size, the text should say: “It is expected that guardband size should be minimized to reduce performance degradation due to overhead.”

# References

- R4-2508707 “Way Forward for [115][306] NR_duplex_evo_General”, RAN4#115, Malta, 19th-23rd May, 2025