3GPP TSG-RAN WG4 Meeting #116	R4-2511529

Bengaluru, India, 25th – 29th August, 2025

Agenda item:		7.21.3.3

Source:	Nokia

Title:	On existing BS RF RX requirements for SBFD operation

Document for:		Approval

# Introduction

A Way Forward was agreed in RAN4#115 [1] regarding evolution of NR duplex operation. There were some agreements made that have impact on the existing BS RF requirements for SBFD operation. Also, the open items are listed in the WF. The requirement with the biggest impact is the RX in-band blocking. This contribution displays Nokia’s view on the existing BS RF RX requirements for SBFD operation and emphasizes the importance of setting correctly the RX in-band blocking requirement to enable functionality in real-life deployment.

# Discussion

## In-band blocking

### FR1 WA BS in-band blocking requirement

The following WF was agreed during RAN4#115 regarding the in-band blocking requirement for FR1 WA BS:

- Way forward - For in-band blocking FR1 WA BS, FFS on the value by taking the below range as starting point: - [-28, -16.7] dBm, in which the min and max are coming from co-ex simulations based on 100% grid shift. - For requirements assumption as deployment guidance, FFS on the following options - Option 1: The in-band blocking requirements assume a [TBD] dB coupling loss between interfering NR BS transmitter operating in and NR SBFD BS receive - Option 2: Distance, need to solve the issue that different frequency bands/scenarios would have different values - Option 3: One note can be added to generally describe the deployment restriction. - For requirement definition, FFS on the following aspects - Interference source: interference from BS operating at adjacent channel(s) - Interference signal CBW: TBD - Interference signal mean power: TBD - Interference signal frequency: - For DU configuration: [TBD] offset from the lower Base Station RF Bandwidth edge - For UD configuration: [TBD] offset from the upper Base Station RF Bandwidth edge - For DUD configuration: [TBD] offset from the lower/upper Base Station RF Bandwidth edge - For ACS/blocking requirement and test, own Tx is ON and the corresponding REFSENS degradation will be considered

The WF lists a range of [-28, -16.7] dBm as a starting point for the in-band blocking requirement for FR1 WA BS. This range is taken from the companies' contributions on the co-existence simulations based on 100% GS. The grid shift assumption of 100% is a best-case scenario from interference point of view. In this scenario, the coupling loss between the two networks is maximized resulting in a lowest interfering power and hence, a looser in-band blocking requirement. SBFD operation with lower inter network distances is not guaranteed as the in-band blocking power would be higher. Therefore, we propose to select a value for the in-band blocking power from the high end of the range mentioned in the WF. According to our results, the in-band blocking power was simulated to be -16.7dBm. This can be rounded down for the specification limit of -17dBm.

Define the in-band blocking requirement for FR1 WA BS of -17dBm.

Regarding how to capture the deployment assumptions used to determine the in-band blocking requirement: starting with Option 1, it might be challenging to define and agree on a single value of coupling loss considering the fact that, it is not always one, but there could be instead two or more base stations acting as the main source of interference to a victim base station. Furthermore, the coupling gain between a pair of base stations changes significantly depending on the respective transmit and receive beam angles. Therefore, a simpler way forward is to instead include a detailed description of the deployment restrictions, including the assumptions on minimum distance between base stations, i.e. a mix of Option 2 and Option 3. Thus, we have the following text proposals:

=================== Start of Text Proposal ===================

    - 12.3.4	In-band selectivity and blocking for SBFD

      - 12.3.4.1	General

The FR1 WA SBFD BS in-band blocking requirements are derived based on an urban macro scenario with 500 meters inter-site distance between SBFD BSs. The adjacent TDD DL network has the same inter-site distance. The SBFD BSs and its adjacent channel network are equally spaced with a distance of 288 meters, which corresponds to a grid-shift of 100% between the 2 networks. The assumption results in the best-case scenario from an interference point of view as the coupling loss between the 2 networks is maximized. The in-band blocking power is expected to be higher with lower inter-site distances. The in-band blocking requirements are derived without accounting for SBFD BS self-interference and inter-sector interference, which would also impact the receiver performance since they are also received by the BS LNA. This requirement is derived based on coexistence simulations and the relevant assumptions can be found in Annex E TR 38.858. For those parameters captured with multiple options, baseline and 1st priority values were assumed.

=================== End of Text Proposal ===================

=================== Start of Text Proposal ===================

    - 12.6.5	OTA In-band selectivity and blocking for SBFD

      - 12.6.5.1	General

The FR1 WA SBFD BS in-band blocking requirements are derived based on an urban macro scenario with 500 meters inter-site distance between SBFD BSs. The adjacent TDD DL network has the same inter-site distance. The SBFD BSs and its adjacent channel network are equally spaced with a distance of 288 meters, which corresponds to a grid-shift of 100% between the 2 networks. The assumption results in the best-case scenario from an interference point of view as the coupling loss between the 2 networks is maximized. The in-band blocking power is expected to be higher with lower inter-site distances. The in-band blocking requirements are derived without accounting for SBFD BS self-interference and inter-sector interference, which would also impact the receiver performance since they are also received by the BS LNA. This requirement is derived based on coexistence simulations and the relevant assumptions can be found in Annex E TR 38.858. For those parameters captured with multiple options, baseline and 1st priority values were assumed.

=================== End of Text Proposal ===================

Include the text proposals for clause 12.3.4 and 12.6.5 for requirements assumption as deployment guidance.

### FR1 MR BS in-band blocking requirement

The following WF was agreed during RAN4#115 regarding the in-band blocking requirement for FR1 MR BS:

- Way forward - For simulation RAN4 to use the same grid shift assumption as FR1 Macro BS as starting point - Companies to check the blocking level in the range of [-20, -32] dBm

In-band blocking for FR1 MR BS should follow the same logic than FR1 WA BS. Following the same rationale as mentioned in 2.1.1 for FR1 WA BS, we propose to select the value for interference power from the higher end of the range mentioned in the WF. However, as our simulation results showed in-band blocking power of -24.6dBm, we are willing to compromise to a level of -25dBm for the FR1 MR BS.

Define the in-band blocking requirement for FR1 MR BS of -25dBm.

### FR2-1 WA BS in-band blocking requirement

The following WF was agreed during RAN4#114-bis regarding the in-band blocking requirement for FR2-1 WA BS:

- Way forward - Encourage companies to have offline discussion for the simulation assumptions before next meeting.

Regarding the FFS above, we agree that the current FR2-1 WA BS in-band blocking requirements can be re-used, as supported by the results shown below in Table 3. We rely on the ratio of in-band blocking power versus wanted signal to reach such conclusion. From Table 10.5.2.3-1 in TS 38.104, we note that wanted and interfering signal mean powers are defined relative to the EIS_REFSENS and EIS_REFSENS_50M. Given the relation between EIS_REFSENS and EIS_REFSENS_50M, we can define the in-band blocking requirements as the ratio between both signal components. For instance, the current FR2-1 in-band blocking requirement assumes that the OTA interfering signal power is 27 dB higher than the OTA wanted signal power.

[TS 38.104, Section 10.5.2.3]

Use the ratio of interfering signal power and wanted signal power for the FR2-1 WA BS in-band blocking requirement derivation.

#### Analysis of the in-band blocking requirements

Table 3 includes absolute in-band blocking power and wanted signal power as well as the ratio of in-band blocking power versus wanted signal power at 99th percentile for the FR2-1 WA SBFD BS at 10% grid shift.

Table 3. Ratio of in-band blocking power and wanted signal [dBm] for FR2-1 scenarios

|  | Interfering signal power at 99th percentile | Wanted signal power at 99th percentile | Ratio of interfering signal power versus wanted signal at 99th percentile | Current ratio of interfering signal power versus wanted signal in TS 38.104 |
| --- | --- | --- | --- | --- |
| Scenario 6 (Urban macro FR2) | -37.4 dBm | -66.1 dBm | 28.7 dB | FR2-1 WA BS: 27 dB |

Based on the results above the following is observed:

For FR2-1 WA SBFD BS, the ratio of in-band blocking power versus wanted signal at the 99th percentile is 30 dB, which is within the limits defined by the current FR2-1 WA BS requirements in TS 38.104.

Therefore, there is no need to define new in-band blocking requirements for FR2-1 WA SBFD BS given that the obtained ratio of in-band blocking power versus wanted signal is within the current FR2-1 WA BS requirements.

The current FR2-1 WA BS in-band blocking requirements can be re-used for FR2-1 WA SBFD BS.

### Clarification on the in-band blocking requirement derivation

In the previous RAN4#112 meeting, companies agreed to not account for the SBFD specific co-channel interferences to derive the in-band blocking requirement. In other words, BS self-interference and inter-sector interferences are not considered. However, these interferences also impact the receiver performance since they are also received by the BS LNA. This is an important assumption that should be clearly stated as part of the in-band blocking requirement definition for SBFD-capable BS in order to avoid possible misunderstandings.

RAN4 specifications should include a clear statement indicating that the in-band blocking requirements for SBFD BS are derived without accounting for BS self-interference and inter-sector interference.

## ACS Requirement

The following was agreed in RAN4#115:

- Way forward - FFS on whether the assumptions that will be applied for in-band blocking requirements derivation can be applied for ACS requirement.

Adjacent channel selectivity is an important measure of the receiver's ability to receive a wanted signal with interfering signal in the adjacent channel. In our view, the definition of ACS can be reused for SBFD. The general in-band blocking interference level defined for SBFD with interference level of 9dB offset should be used to define the interfering signal power level for the ACS requirement.

Definition of ACS is reused for SBFD and the interfering signal power level keeps 9dB offset to the in-band blocking interference level defined for SBFD.

## RX intermodulation

### Necessity of New RX intermodulation requirement with 1 or 2 interfering signal

The following was agreed in RAN4#115:

- Way forward - Down-select to one of the below two options in the next meeting - Option 1: New IMD requirement is not needed. Such a requirement is implicitly captured by the SBFD RX ACS/blocking requirement, i.e. the IMD between the ACS/blocking interference and own SBFD DL can be regarded as the worst case - Option 2: New IMD requirement is needed, the following scenario with 2 input signals can be regarded as the worst case - Interfering signal 1: CW - Interfering signal 2: TBD MHz NR - Power level: the final agreed in-band blocking level -9 dB

The WF lists two options for handling the new RX IMD requirement. In our view, the option one will not always guarantee that the IMD will land on the own UL sub-band. It all depends on the sub-band configuration and placement of DL and UL sub-bands. One cannot say that such a requirement is implicitly captured by the RX blocking requirement. Therefore, we feel that there should be a requirement for RX IMD with two input signals. It has been discussed that it would be the worst case of the previously listed three options and it would guarantee performance.

New RX IMD requirement is needed with two input signals.

# Conclusion

In this contribution, our views on the existing BS RF RX requirements for SBFD operation are presented. There were observations and proposals as listed below.

- For FR2-1 WA SBFD BS, the ratio of in-band blocking power versus wanted signal at the 99th percentile is 30 dB, which is within the limits defined by the current FR2-1 WA BS requirements in TS 38.104.

- Define the in-band blocking requirement for FR1 WA BS of -17dBm.

- Include the text proposals for clause 12.3.4 and 12.6.5 for requirements assumption as deployment guidance.

- Define the in-band blocking requirement for FR1 MR BS of -25dBm.

- Use the ratio of interfering signal power and wanted signal power for the FR2-1 WA BS in-band blocking requirement derivation.

- The current FR2-1 WA BS in-band blocking requirements can be re-used for FR2-1 WA SBFD BS.

RAN4 specifications should include a clear statement indicating that the in-band blocking requirements for SBFD BS are derived without accounting for BS self-interference and inter-sector interference.

Definition of ACS is reused for SBFD and the interfering signal power level keeps 9dB offset to the in-band blocking interference level defined for SBFD.

New RX IMD requirement is needed with two input signals.

# References

- R4-2508713 “Way Forward for [115][307] NR_duplex_evo_BSRF”, RAN4#115, Malta, 19th-23rd May, 2025