3GPP TSG-RAN4 Meeting #116	R4-2511542

# Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.145-2 | CR | 0398 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 37.145-2: 7MHz channel bandwidth introduction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FR1_7MHz_BW-Perf |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduction of 7MHz channel bandwidth for NR. Technical content of this CR was Endorsed during RAN4#115 in R4-2507925. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of 7MHz channel bandwidth for NR. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Agreement on 7 MHz channel would not be properly reflected in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.2.2, 6.4.4.5.3, 6.7.2.4.2, 6.7.3.5.1.4, 7.2.5.5, 7.3.5.4, 7.4.5.3, 7.8.5.1.1, 7.8.5.1.2, 7.9.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | 37.105, 37.113, 38.104, 38.106, 38.113 |  |  |
| affected: |  | X |  | Test specifications |  |  |  | 37.141, 37.145-1, 38.115-1, 38.141-1, 38.141-2, 38.176-2 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------------------------ Modified section ------------------------------

#### 4.1.2.2	Measurement of transmitter

Table 4.1.2.2-1: Maximum Test System uncertainty for transmitter tests

| Clause | Maximum Test System Uncertainty | Derivation of Test System Uncertainty |
| --- | --- | --- |
| 6.2 Radiated transmit power (normal conditions) | ±1.1 dB, f ≤ 3.0 GHz ±1.3 dB, 3.0 GHz < f ≤ 4.2 GHz | For the derivation of test system measurement uncertainty, uncertainty budget contributors as well as uncertainty budget assessment, refer to TR 37.941 [38]. |
| 6.2 Radiated transmit power (extreme conditions) | ±2.5 dB, f ≤ 3.0 GHz ±2.6 dB, 3.0 GHz < f ≤ 4.2 |  |
| 6.3.2 OTA maximum output power | ±1.4 dB, f ≤ 3.0 GHz ±1.5 dB, 3.0 GHz < f ≤ 4.2 GHz |  |
| 6.3.3 OTA E-UTRA DL RS power | 1.3 dB, f ≤ 3.0 GHz 1.5 dB, 3.0 GHz < f ≤ 4.2 GHz |  |
| 6.4.2 OTA UTRA inner loop power control in the downlink | 0.1 dB |  |
| 6.4.3 OTA power control dynamic range | 1.1 dB |  |
| 6.4.4 OTA total power dynamic range | 0.3 dB UTRA 0.4 dB E-UTRA & NR |  |
| 6.4.5 OTA IPDL time mask | 0.7 dB |  |
| 6.5 OTA transmit ON/OFF power | ±3.4 dB, f ≤ 3.0 GHz ±3.6 dB, 3.0 GHz < f ≤ 4.2 GHz (NOTE 1) |  |
| 6.6.2 OTA frequency error | 12 Hz |  |
| 6.6.3 OTA TAE | 25 ns |  |
| 6.6.4 OTA modulation Quality | 1 % |  |
| 6.7.2 OTA occupied bandwidth | 30 kHz: BWChannel 1.4 MHz, 3 MHz 100 kHz: BWChannel 5 MHz, 7 MHz, 10 MHz 300 kHz: BWChannel 15 MHz, 20 MHz 25 MHz, 30 MHz, 40 MHz, 50 MHz 600 kHz: BWChannel 60 MHz, 70 MHz, 80 MHz, 90 MHz, 100 MHz |  |
| 6.7.3 OTA ACLR/CACLR | ±1.0 dB, f ≤ 3.0 GHz ±1.2 dB, 3.0 GHz < f ≤ 4.2 Absolute limit ±2.2 dB, f ≤ 3.0 GHz ±2.7 dB, 3.0 GHz < f ≤ 4.2 GHz |  |
| 6.7.4 OTA spectrum emission mask | ±1.8 dB, f ≤ 3.0 GHz ±2.0 dB, 3.0 GHz < f ≤ 4.2 GHz |  |
| 6.7.5 OTA operating band unwanted emissions | ±1.8 dB, f ≤ 3.0 GHz ±2.0 dB, 3.0 GHz < f ≤ 4.2 GHz |  |
| 6.7.6.2 OTA transmitter spurious emissions, mandatory requirements | ±2.3 dB, 30 MHz < f ≤ 6 GHz ±4.2 dB, 6 GHz < f ≤ 19 GHz |  |
| 6.7.6.3 OTA transmitter spurious emissions, protection of BS receiver | ±3.1 dB, f ≤ 3.0 GHz ±3.3 dB, 3.0 GHz < f ≤ 4.2 GHz (NOTE 1) |  |
| 6.7.6.4 OTA transmitter spurious emissions, additional spurious emission requirements | ±2.6 dB, f ≤ 3.0 GHz ±3.0 dB, 3.0 GHz < f ≤ 4.2 GHz |  |
| 6.7.6.5 OTA transmitter spurious emissions, co-location | ±3.1 dB, f ≤ 3.0 GHz ±3.3 dB, 3.0 GHz < f ≤ 4.2 GHz (NOTE 1) |  |
| 6.8 OTA transmitter intermodulation (interferer requirements) (NOTE 2) | The value below applies only to the interfering signal and is unrelated to the measurement uncertainty of the tests (6.6.1, 6.6.2 and 6.6.4) which have to be carried out in the presence of the interferer. ±3.2 dB, f ≤ 3.0 GHz ±3.4 dB, 3.0 GHz < f ≤ 4.2 GHz (NOTE 1) |  |
| NOTE 1: Fulfilling the criteria for CLTA selection and placement in clause 4.15 is deemed sufficient for the test purposes. When these criteria are met, the measurement uncertainty related to the selection of the co-location test antenna and its alignment as specified in the appropriate measurement uncertainty budget in TR 37.941 [38], shall be used for evaluating the test system uncertainty. NOTE 2: This tolerance applies to the stimulus and not the measurements defined in clause 6.8. |  |  |

------------------------------ Next modified section -------------------------

##### 6.4.4.5.3	NR

The downlink (DL) total power dynamic range for each NR carrier shall be larger than or equal to the level in table 6.4.4.5.3-1.

Table 6.4.4.5.3-1: Total power dynamic range

| BS channel bandwidth (MHz) | Total power dynamic range (dB) |  |  |
| --- | --- | --- | --- |
|  | 15 kHz SCS | 30 kHz SCS | 60 kHz SCS |
| 5 | 13.5 | 10 | N/A |
| 7 | 15.0 | N/A | N/A |
| 10 | 16.7 | 13.4 | 10 |
| 15 | 18.5 | 15.3 | 12.1 |
| 20 | 19.8 | 16.6 | 13.4 |
| 25 | 20.8 | 17.7 | 14.5 |
| 30 | 21.6 | 18.5 | 15.3 |
| 35 | 22.7 | 19.6 | 16.4 |
| 40 | 22.9 | 19.8 | 16.6 |
| 45 | 23.8 | 20.7 | 17.6 |
| 50 | 23.9 | 20.8 | 17.7 |
| 60 | N/A | 21.6 | 18.5 |
| 70 | N/A | 22.3 | 19.2 |
| 80 | N/A | 22.9 | 19.8 |
| 90 | N/A | 23.4 | 20.4 |
| 100 | N/A | 23.9 | 20.9 |

------------------------------ Next modified section -------------------------

##### 6.7.2.4.2	Procedure

1)	Place the AAS BS at the positioner.

2)	Align the manufacturer declared coordinate system orientation (see table 4.10-1, D9.2) of the AAS BS with the test system.

3)	Orient the positioner (and BS) in order that the direction to be tested aligns with the test antenna.

4)	Configure the beam peak direction of the AAS BS according to the declared beam direction pair.

5)	Set the AAS BS to transmit signal.

6)	For UTRA FDD, measure the spectrum of the transmitted signal across a span of 10 MHz, based on an occupied bandwidth requirement of 5 MHz. The selected resolution bandwidth (RBW) filter of the analyser shall be 30 kHz or less. The spectrum shall be measured at 400 or more points across the measurement span.

For E-UTRA and NR measure the spectrum emission of the transmitted signal using at least the number of measurement points, and across a span, as listed in table 6.7.2.4.2-1. The selected resolution bandwidth (RBW) filter of the analyser shall be 30 kHz or less.

NOTE:	The detection mode of the spectrum analyzer will not have any effect on the result if the statistical properties of the out-of-OBW power are the same as those of the inside-OBW power. Both are expected to have the Rayleigh distribution of the amplitude of Gaussian noise. In any case where the statistics are not the same, though, the detection mode is power responding. There are at least two ways to be power responding. The spectrum analyser can be set to "sample" detection, with its video bandwidth setting at least three times its RBW setting. Or the analyser may be set to respond to the average of the power (root-mean-square of the voltage) across the measurement cell.

Table 6.7.2.4.2-1: Span and number of measurement points for OBW measurements for FR1

| Bandwidth | BS channel bandwidth BWChannel (MHz) |  |  |  |  |  | Aggregated BS channel bandwidth BWChannel_CA (MHz) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 | 7 | 10 | 15 | 20 | > 20 | > 20 |
| Span (MHz) | 10 | 14 | 20 | 30 | 40 | 2×BWChannel |  |
| Minimum number of measurement points | 400 | 400 | 400 | 400 | 400 | 2×BWChannel100kHz |  |

------------------------------ Next modified section -------------------------

6.7.3.5.1.4	NR test requirement

For the NR OTA ACLR requirement either the OTA ACLR limits in tables 6.7.3.5.1.4-1/2a or the OTA ACLR absolute limits in tables 6.7.3.5.1.4-2 shall apply, whichever is less stringent. The OTA CACLR limits in clause 6.7.3.5.1.3 shall also apply.

For a RIB operating in non-contiguous spectrum, the OTA ACLR requirement applies inside sub block gaps for the frequency ranges defined in table 6.7.3.5.1.4-2a, while the CACLR requirement applies inside sub block gaps for the frequency ranges defined in table 6.7.3.5.1.3-1.

For a multi-band RIB, the OTA ACLR test requirement applies inside Inter RF Bandwidth gaps for the frequency ranges defined in table 6.7.3.5.1.4-2a, while the OTA CACLR requirement applies inside Inter RF Bandwidth gaps for the frequency ranges defined in table 6.7.3.5.1.3-1.

For operation in paired and unpaired spectrum, the OTA ACLR measurement result shall not be less than the OTA ACLR limit specified in table 6.7.3.5.1.4-1.

Table 6.7.3.5.1.4-1: NR ACLR limit

| BS channel bandwidth of lowest/highest NR carrier transmitted BWChannel [MHz] | BS adjacent channel centre frequency offset below the lowest or above the highest carrier centre frequency transmitted | Assumed adjacent channel carrier (informative) | Filter on the adjacent channel frequency and corresponding filter bandwidth | OTA ACLR limit (0 – 3 GHz) | OTA ACLR limit (3 – 6 GHz) |
| --- | --- | --- | --- | --- | --- |
| 5, 7, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90,100 | BWChannel | NR of same BW (Note 2) | Square (BWConfig) | 44 dB | 43.8 dB |
|  | 2 x BWChannel | NR of same BW (Note 2) | Square (BWConfig) | 44 dB | 43.8 dB |
|  | BWChannel /2 + 2.5 MHz | 5 MHz E-UTRA | Square (4.5 MHz) | 44 dB (Note 3) | 43.8 dB (Note 3) |
|  | BWChannel /2 + 7.5 MHz | 5 MHz E-UTRA | Square (4.5 MHz) | 44 dB (Note 3) | 43.8 dB (Note 3) |
| NOTE 1: BWChannel and BWConfig are the BS channel bandwidth and transmission bandwidth configuration of the lowest/highest NR carrier transmitted on the assigned channel frequency. NOTE 2: With SCS that provides largest transmission bandwidth configuration (BWConfig). NOTE 3: The requirements are applicable when the band is also defined for E-UTRA or UTRA. |  |  |  |  |  |

The absolute total power measurement shall not exceed the OTA ACLR absolute limit specified in table 6.7.3.5.1.4-2.

Table 6.7.3.5.1.4-2: NR ACLR absolute limit

| BS category / BS class | OTA ACLR absolute limit |
| --- | --- |
| Category A Wide Area BS | -4 dBm/MHz |
| Category B Wide Area BS | -6 dBm/MHz |
| Medium Range BS | -16 dBm/MHz |
| Local Area BS | -23 dBm/MHz |

For operation in non-contiguous spectrum or multiple bands, the OTA ACLR measurement result shall not be less than the OTA ACLR limit specified in table 6.7.3.5.1.4-2a.

Table 6.7.3.5.1.4-2a: NR ACLR limit in non-contiguous spectrum or multiple bands

| BS channel bandwidth of NR carrier transmitted adjacent to sub-block gap or inter RF Bandwidth gap BWChannel [MHz] | Sub-block or Inter RF Bandwidth gap size (Wgap) where the limit applies [MHz] | BS adjacent channel centre frequency offset below or above the sub-block or Base Station RF Bandwidth edge (inside the gap) | Assumed adjacent channel carrier | Filter on the adjacent channel frequency and corresponding filter bandwidth | OTA ACLR limit (0-3GHz) | OTA ACLR limit (3-6GHz) |
| --- | --- | --- | --- | --- | --- | --- |
| 5, 7, 10, 15, 20 | Wgap ≥ 15 (Note 3) Wgap ≥ 45 (Note 4) | 2.5 MHz | 5 MHz NR (Note 2) | Square (BWConfig) | 44 dB | 43.8 dB |
|  | Wgap ≥ 20 (Note 3) Wgap ≥ 50 (Note 4) | 7.5 MHz | 5 MHz NR (Note 2) | Square (BWConfig) | 44 dB | 43.8 dB |
| 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | Wgap ≥ 60 (Note 4) Wgap ≥ 30 (Note 3) | 10 MHz | 20 MHz NR (Note 2) | Square (BWConfig) | 44 dB | 43.8 dB |
|  | Wgap ≥ 80 (Note 4) Wgap ≥ 50 (Note 3) | 30 MHz | 20 MHz NR (Note 2) | Square (BWConfig) | 44 dB | 43.8 dB |
| NOTE 1: BWConfig is the transmission bandwidth configuration of the assumed adjacent channel carrier. NOTE 2: With SCS that provides largest transmission bandwidth configuration (BWConfig). NOTE 3: Applicable in case the BS channel bandwidth of the carrier transmitted at the other edge of the gap is 5, 7, 10, 15, 20 MHz. NOTE 4: Applicable in case the BS channel bandwidth of the NR carrier transmitted at the other edge of the gap is 25, 30, 40, 50, 60, 70, 80, 90, 100 MHz. |  |  |  |  |  |  |

------------------------------ Next modified section -------------------------

#### 7.2.5.5	NR Test Requirements

For each measured carrier, the throughput measured in step 9 of clause 7.2.4.2 shall be ≥ 95 % of the maximum throughput of the reference measurement channel as specified in TS 38.104 [33] annex A.1 with parameters specified in table 7.2.5.5-1.

Table 7.2.5.5-1: EIS levels

| BS channel | Sub-carrier | Reference | EIS level [dBm] |  |
| --- | --- | --- | --- | --- |
| bandwidth [MHz] | spacing [kHz] | measurement channel | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz |
| 5, 7, 10, 15 | 15 | G-FR1-A1-1 in clause A.1 in [33] | Declared minimum EIS + 1.3 | Declared minimum EIS + 1.4 |
| 10, 15 | 30 | G-FR1-A1-2 in clause A.1 in [33] | Declared minimum EIS + 1.3 | Declared minimum EIS + 1.4 |
| 10, 15 | 60 | G-FR1-A1-3 in clause A.1 in [33] | Declared minimum EIS + 1.3 | Declared minimum EIS + 1.4 |
| 20, 25, 30, 35, 40, 45, 50 | 15 | G-FR1-A1-4 in clause A.1 in [33] | Declared minimum EIS + 1.3 | Declared minimum EIS + 1.4 |
| 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 in clause A.1 in [33] | Declared minimum EIS + 1.3 | Declared minimum EIS + 1.4 |
| 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | 60 | G-FR1-A1-6 in clause A.1 in [33] | Declared minimum EIS + 1.3 | Declared minimum EIS + 1.4 |

------------------------------ Next modified section -------------------------

#### 7.3.5.4	NR Test Requirements

For each measured carrier, the throughput measured in step 9 of clause 7.3.4.2 shall be ≥ 95 % of the maximum throughput of the reference measurement channel as specified in annex TS 38.104 [33] A.1 with parameters specified in tables 7.3.5.4-1 to 7.3.5.4-3.

Table 7.3.5.4-1: Wide Area BS EISREFSENS levels

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement | EISREFSENS (dBm) |  |
| --- | --- | --- | --- | --- |
|  |  | channel | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz |
| 5, 7, 10, 15 | 15 | G-FR1-A1-1 | -100.4 – ΔOTAREFSENS | -100.3 – ΔOTAREFSENS |
| 10, 15 | 30 | G-FR1-A1-2 | -100.5 – ΔOTAREFSENS | -100.4 – ΔOTAREFSENS |
| 10, 15 | 60 | G-FR1-A1-3 | -97.6 – ΔOTAREFSENS | -97.5 – ΔOTAREFSENS |
| 20, 25, 30, 35, 40, 45, 50 | 15 | G-FR1-A1-4 | -94 – ΔOTAREFSENS | -93.9 – ΔOTAREFSENS |
| 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 | -94.3 – ΔOTAREFSENS | -94.2 – ΔOTAREFSENS |
| 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | 60 | G-FR1-A1-6 | -94.4 – ΔOTAREFSENS | -94.3 – ΔOTAREFSENS |
| NOTE: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |  |

Table 7.3.5.4-2: Medium Range BS EISREFSENS levels

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement | EISREFSENS (dBm) |  |
| --- | --- | --- | --- | --- |
|  |  | channel | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz |
| 5, 7, 10, 15 | 15 | G-FR1-A1-1 | -95.4 – ΔOTAREFSENS | -95.3 – ΔOTAREFSENS |
| 10, 15 | 30 | G-FR1-A1-2 | -95.5 – ΔOTAREFSENS | -95.4 – ΔOTAREFSENS |
| 10, 15 | 60 | G-FR1-A1-3 | -92.6 – ΔOTAREFSENS | -92.5 – ΔOTAREFSENS |
| 20, 25, 30, 35, 40, 45, 50 | 15 | G-FR1-A1-4 | -89 – ΔOTAREFSENS | -88.9 – ΔOTAREFSENS |
| 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 | -89.3 – ΔOTAREFSENS | -89.2 – ΔOTAREFSENS |
| 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | 60 | G-FR1-A1-6 | -89.4 – ΔOTAREFSENS | -89.3 – ΔOTAREFSENS |
| NOTE: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |  |

Table 7.3.5.4-3: Local Area BS EISREFSENS levels

| BS channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement | EISREFSENS (dBm) |  |
| --- | --- | --- | --- | --- |
|  |  | channel | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz |
| 5, 7, 10, 15 | 15 | G-FR1-A1-1 | -92.4 – ΔOTAREFSENS | -92.3 – ΔOTAREFSENS |
| 10, 15 | 30 | G-FR1-A1-2 | -92.5 – ΔOTAREFSENS | -92.4 – ΔOTAREFSENS |
| 10, 15 | 60 | G-FR1-A1-3 | -89.6 – ΔOTAREFSENS | -89.5 – ΔOTAREFSENS |
| 20, 25, 30, 35, 40, 45, 50 | 15 | G-FR1-A1-4 | -86 – ΔOTAREFSENS | -85.9 – ΔOTAREFSENS |
| 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 | -86.3 – ΔOTAREFSENS | -86.2 – ΔOTAREFSENS |
| 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | 60 | G-FR1-A1-6 | -86.4 – ΔOTAREFSENS | -86.3 – ΔOTAREFSENS |
| NOTE: PREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |  |

------------------------------ Next modified section -------------------------

#### 7.4.5.3	NR operation

For each measured carrier, the throughput measured in step 6 of clause 7.4.4.2 shall be ≥ 95 % of the maximum throughput of the reference measurement channel as specified in TS 38.104 [33] annex A.2 with parameters specified in tables 7.4.5.3-1 to 7.4.5.3-3.

Table 7.4.5.3-1: Wide Area BS dynamic range

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) |  | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz |  |  |
| 5 | 15 | G-FR1-A2-1 | -70.4 – ΔOTAREFSENS | -70.4 – ΔOTAREFSENS | -82.5 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -71.1 – ΔOTAREFSENS | -71.1 – ΔOTAREFSENS |  |  |
| 7 | 15 | G-FR1-A2-1 | -70.4 – ΔOTAREFSENS | -70.4 – ΔOTAREFSENS | -81.0 – ΔOTAREFSENS | AWGN |
| 10 | 15 | G-FR1-A2-1 | -70.4 – ΔOTAREFSENS | -70.4 – ΔOTAREFSENS | -79.3 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -71.1 – ΔOTAREFSENS | -71.1 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-3 | -68.1 – ΔOTAREFSENS | -68.1 – ΔOTAREFSENS |  |  |
| 15 | 15 | G-FR1-A2-1 | -70.4 – ΔOTAREFSENS | -70.4 – ΔOTAREFSENS | -77.5 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -71.1 – ΔOTAREFSENS | -71.1 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-3 | -68.1 – ΔOTAREFSENS | -68.1 – ΔOTAREFSENS |  |  |
| 20 | 15 | G-FR1-A2-4 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -76.2 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS |  |  |
| 25 | 15 | G-FR1-A2-4 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -75.2 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS |  |  |
| 30 | 15 | G-FR1-A2-4 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -74.4 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS |  |  |
| 35 | 15 | G-FR1-A2-4 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -73.7– ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS |  |  |
| 40 | 15 | G-FR1-A2-4 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -73.1 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS |  |  |
| 45 | 15 | G-FR1-A2-4 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -72.6 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS |  |  |
| 50 | 15 | G-FR1-A2-4 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -72.1 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS |  |  |
| 60 | 30 | G-FR1-A2-5 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -71.3 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS |  |  |
| 70 | 30 | G-FR1-A2-5 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -70.7 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS |  |  |
| 80 | 30 | G-FR1-A2-5 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -70.1 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS |  |  |
| 90 | 30 | G-FR1-A2-5 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -69.5 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS |  |  |
| 100 | 30 | G-FR1-A2-5 | -64.2 – ΔOTAREFSENS | -64.2 – ΔOTAREFSENS | -69.1 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -64.5 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS |  |  |
| NOTE: The wanted signal mean power is the power level of a single instance of the corresponding reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |  |  |  |

Table 7.4.5.3-2: Medium Range BS dynamic range

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) |  | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz |  |  |
| 5 | 15 | G-FR1-A2-1 | -65.4 – ΔOTAREFSENS | -65.4 – ΔOTAREFSENS | -77.5 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -66.1 – ΔOTAREFSENS | -66.1 – ΔOTAREFSENS |  |  |
| 7 | 15 | G-FR1-A2-1 | -65.4 – ΔOTAREFSENS | -65.4 – ΔOTAREFSENS | -76.0 – ΔOTAREFSENS | AWGN |
| 10 | 15 | G-FR1-A2-1 | -65.4 – ΔOTAREFSENS | -65.4 – ΔOTAREFSENS | -74.3 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -66.1 – ΔOTAREFSENS | -66.1 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-3 | -63.1 – ΔOTAREFSENS | -63.1 – ΔOTAREFSENS |  |  |
| 15 | 15 | G-FR1-A2-1 | -65.4 – ΔOTAREFSENS | -65.4 – ΔOTAREFSENS | -72.5 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -66.1 – ΔOTAREFSENS | -66.1 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-3 | -63.1 – ΔOTAREFSENS | -63.1 – ΔOTAREFSENS |  |  |
| 20 | 15 | G-FR1-A2-4 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -71.2 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS |  |  |
| 25 | 15 | G-FR1-A2-4 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -70.2 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS |  |  |
| 30 | 15 | G-FR1-A2-4 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -69.4 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS |  |  |
| 35 | 15 | G-FR1-A2-4 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -68.7 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS |  |  |
| 40 | 15 | G-FR1-A2-4 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -68.1 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS |  |  |
| 45 | 15 | G-FR1-A2-4 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -67.6 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS |  |  |
| 50 | 15 | G-FR1-A2-4 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -67.1 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | --59.2 – ΔOTAREFSENS | --59.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS |  |  |
| 60 | 30 | G-FR1-A2-5 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -66.3– ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS |  |  |
| 70 | 30 | G-FR1-A2-5 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -65.7 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS |  |  |
| 80 | 30 | G-FR1-A2-5 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -65.1 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS |  |  |
| 90 | 30 | G-FR1-A2-5 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -64.5 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS |  |  |
| 100 | 30 | G-FR1-A2-5 | -59.2 – ΔOTAREFSENS | -59.2 – ΔOTAREFSENS | -64.1 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -59.5 – ΔOTAREFSENS | -59.5 – ΔOTAREFSENS |  |  |
| NOTE: The wanted signal mean power is the power level of a single instance of the corresponding reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |  |  |  |

Table 7.4.5.3-3: Local Area BS dynamic range

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) |  | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz |  |  |
| 5 | 15 | G-FR1-A2-1 | -62.4 – ΔOTAREFSENS | -62.4 – ΔOTAREFSENS | -74.5 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -63.1 – ΔOTAREFSENS | -63.1 – ΔOTAREFSENS |  |  |
| 7 | 15 | G-FR1-A2-1 | -62.4 – ΔOTAREFSENS | -62.4 – ΔOTAREFSENS | -73.0 – ΔOTAREFSENS | AWGN |
| 10 | 15 | G-FR1-A2-1 | -62.4 – ΔOTAREFSENS | -62.4 – ΔOTAREFSENS | -71.3 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -63.1 – ΔOTAREFSENS | -63.1 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-3 | -60.1 – ΔOTAREFSENS | -60.1 – ΔOTAREFSENS |  |  |
| 15 | 15 | G-FR1-A2-1 | -62.4 – ΔOTAREFSENS | -62.4 – ΔOTAREFSENS | -69.5 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -63.1 – ΔOTAREFSENS | -63.1 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-3 | -60.1 – ΔOTAREFSENS | -60.1 – ΔOTAREFSENS |  |  |
| 20 | 15 | G-FR1-A2-4 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -68.2 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS |  |  |
| 25 | 15 | G-FR1-A2-4 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -67.2 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS |  |  |
| 30 | 15 | G-FR1-A2-4 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -66.4 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS |  |  |
| 35 | 15 | G-FR1-A2-4 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -64.6 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS |  |  |
| 40 | 15 | G-FR1-A2-4 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -65.1 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS |  |  |
| 45 | 15 | G-FR1-A2-4 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -64.6 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS |  |  |
| 50 | 15 | G-FR1-A2-4 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -64.1 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-5 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-6 | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS |  |  |
| 60 | 30 | G-FR1-A2-5 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -63.3 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS |  |  |
| 70 | 30 | G-FR1-A2-5 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -62.7 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS |  |  |
| 80 | 30 | G-FR1-A2-5 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -62.1 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS |  |  |
| 90 | 30 | G-FR1-A2-5 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -61.5 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS |  |  |
| 100 | 30 | G-FR1-A2-5 | -56.2 – ΔOTAREFSENS | -56.2 – ΔOTAREFSENS | -61.1 – ΔOTAREFSENS | AWGN |
|  | 60 | G-FR1-A2-6 | -56.5 – ΔOTAREFSENS | -56.5 – ΔOTAREFSENS |  |  |
| NOTE: The wanted signal mean power is the power level of a single instance of the corresponding reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full BS channel bandwidth. |  |  |  |  |  |  |

------------------------------ Next modified section -------------------------

Table 7.8.5.1.1-2: Interfering signals for intermodulation requirement

| RAT of the carrier adjacent to the upper/lower Base Station RF Bandwidth edge | Interfering signal centre frequency offset from the Base Station RF Bandwidth edge [MHz] | Type of interfering signal |
| --- | --- | --- |
| E-UTRA 1.4 MHz | ±2,0 (BC1 and BC3) / ±2,1 (BC2) | CW |
|  | ±4,9 | 1,4 MHz E-UTRA signal |
| E-UTRA 3 MHz | ±4,4 (BC1 and BC3) / ±4,5 (BC2) | CW |
|  | ±10,5 | 3 MHz E-UTRA signal |
| UTRA FDD and | ±7,5 | CW |
| E-UTRA 5 MHz | ±17,5 | 5 MHz E-UTRA signal |
| E-UTRA 10 MHz | ±7,375 | CW |
|  | ±17,5 | 5 MHz E-UTRA signal |
| E-UTRA 15 MHz | ±7,25 | CW |
|  | ±17,5 | 5 MHz E-UTRA signal |
| E-UTRA 20 MHz | ±7,125 | CW |
|  | ±17,5 | 5 MHz E-UTRA signal |
| GSM/EDGE | ±7,575 | CW |
|  | ±17,5 | 5 MHz E-UTRA signal |
| 1,28 Mcps UTRA | ±2,3 (BC3) | CW |
| TDD | ±5,6 (BC3) | 1,28 Mcps UTRA TDD signal |
| NR 5 MHz | ±7.5 | CW |
|  | ±17.5 | 5 MHz E-UTRA signal |
| NR 7 MHz | ±7.45 | CW |
|  | ±17.5 | 5 MHz E-UTRA signal |
| NR 10 MHz | ±7.465 | CW |
|  | ±17.5 | 5 MHz E-UTRA signal |
| NR 15 MHz | ±7.43 | CW |
|  | ±17.5 | 5 MHz E-UTRA signal |
| NR 20 MHz | ±7.395 | CW |
|  | ±17.5 | 5 MHz E-UTRA signal |
| NR 25 MHz | ±7.465 | CW |
|  | ±25 | 20 MHz E-UTRA signal |
| NR 30 MHz | ±7.43 | CW |
|  | ±25 | 20 MHz E-UTRA signal |
| NR 35 MHz | ±7.44 | CW |
|  | ±25 | 20MHz E-UTRA signal |
| NR 40 MHz | ±7.45 | CW |
|  | ±25 | 20 MHz E-UTRA signal |
| NR 45 MHz | ±7.37 | CW |
|  | ±25 | 20MHz E-UTRA signal |
| NR 50 MHz | ±7.35 | CW |
|  | ±25 | 20 MHz E-UTRA signal |
| NR 60 MHz | ±7.49 | CW |
|  | ±25 | 20 MHz E-UTRA signal |
| NR 70 MHz | ±7.42 | CW |
|  | ±25 | 20 MHz E-UTRA signal |
| NR 80 MHz | ±7.44 | CW |
|  | ±25 | 20 MHz E-UTRA signal |
| NR 90 MHz | ±7.46 | CW |
|  | ±25 | 20 MHz E-UTRA signal |
| NR 100 MHz | ±7.48 | CW |
|  | ±25 | 20 MHz E-UTRA signal |

------------------------------ Next modified section -------------------------

Table 7.8.5.1.2-2: Interfering signals for narrowband intermodulation requirement

| RAT of the carrier adjacent to the upper/lower Base Station RF Bandwidth edge or edge of the sub-block | CW or 1RB interfering signal centre frequency offset from the Base Station RF Bandwidthedge or edge of sub-block inside a gap [kHz] | Type of interfering signal |
| --- | --- | --- |
| E-UTRA 1.4 MHz | ±260 (BC1 and BC3) / ±270 (BC2) | CW |
|  | ±970 (BC1 and BC3) / ±790 (BC2) | 1,4 MHz E-UTRA signal, 1 RB (NOTE 1) |
| E-UTRA 3 MHz | ±260 (BC1 and BC3) / ±270 (BC2) | CW |
|  | ±960 (BC1 and BC3) / ±780 (BC2) | 3,0 MHz E-UTRA signal, 1 RB (NOTE 1) |
| E-UTRA 5 MHz | ±360 | CW |
|  | ±1 060 | 5 MHz E-UTRA signal, 1 RB (NOTE 1) |
| E-UTRA 10 MHz | ±325 | CW |
| (NOTE 2) | ±1 240 | 5 MHz E-UTRA signal, 1 RB (NOTE 1) |
| E-UTRA 15 MHz | ±380 | CW |
| (NOTE 2) | ±1 600 | 5 MHz E-UTRA signal, 1 RB (NOTE 1) |
| E-UTRA 20 MHz | ±345 | CW |
| (NOTE 2) | ±1 780 | 5 MHz E-UTRA signal, 1 RB (NOTE 1) |
| UTRA FDD | ±345 (BC1 and BC2) | CW |
|  | ±1 780 (BC1 and BC2) | 5 MHz E-UTRA signal, 1 RB (NOTE 1) |
| GSM/EDGE | ±340 | CW |
|  | ±880 | 5 MHz E-UTRA signal, 1 RB (NOTE 1) |
| 1,28 Mcps UTRA | ±190 (BC3) | CW |
| TDD | ±970 (BC3) | 1,4 MHz E-UTRA signal, 1 RB (NOTE 1) |
| NR 5 MHz | ±360 | CW |
|  | ±1420 | E-UTRA signal, 1 RB (NOTE 1) |
| NR 7 MHz | ±400 | CW |
|  | ±1240 | E-UTRA signal, 1 RB (NOTE 1) |
| NR 10 MHz | ±370 | CW |
|  | ±1960 | E-UTRA signal, 1 RB (NOTE 1) |
| NR 15 MHz | ±380 | CW |
| (Note 2) | ±1960 | E-UTRA signal, 1 RB (NOTE 1) |
| NR 20 MHz | ±390 | CW |
| (Note 2) | ±2320 | E-UTRA signal, 1 RB (NOTE 1) |
| NR 25 MHz | ±325 | CW |
| (Note 2) | ±2350 | E-UTRA signal, 1 RB (NOTE 1) |
| NR 30 MHz | ±335 | CW |
| (Note 2) | ±2350 | E-UTRA signal, 1 RB (NOTE 1) |
| NR 35 MHz (Note 2) | ±345 | CW |
|  | ±2710 | E-UTRA signal, 1 RB (NOTE 1) |
| NR 40 MHz | ±355 | CW |
| (Note 2) | ±2710 | E-UTRA signal, 1 RB (NOTE 1) |
| NR 45 MHz (Note 2) | ±365 | CW |
|  | ±2710 | E-UTRA signal, 1 RB (NOTE 1) |
| NR 50 MHz | ±375 | CW |
| (Note 2) | ±2710 | E-UTRA signal, 1 RB (NOTE 1) |
| NR 60 MHz | ±395 | CW |
| (Note 2) | ±2710 | E-UTRA signal, 1 RB (NOTE 1) |
| NR 70 MHz | ±415 | CW |
| (Note 2) | ±2710 | E-UTRA signal, 1 RB (NOTE 1) |
| NR 80 MHz | ±435 | CW |
| (Note 2) | ±2710 | E-UTRA signal, 1 RB (NOTE 1) |
| NR 90 MHz | ±365 | CW |
| (Note 2) | ±2530 | E-UTRA signal, 1 RB (NOTE 1) |
| NR 100 MHz | ±385 | CW |
| (Note 2) | ±2530 | E-UTRA signal, 1 RB (NOTE 1) |
| NOTE 1: Interfering signal consisting of one resource block positioned at the stated offset, the channel bandwidth of the interfering signal is located adjacently to the Base Station RF Bandwidth edge. NOTE 2: This requirement shall apply only for an E-UTRA FRC A1-3 mapped to the frequency range at the channel edge adjacent to the interfering signals. |  |  |

------------------------------ Next modified section -------------------------

#### 7.9.5.2	NR test requirement

The requirement shall apply at the RIB when the AoA of the incident wave of the received signal and the interfering signal are the same direction and are within the minSENS RoAoA

The wanted and interfering signals applies to all supported polarizations, under the assumption of polarization matching.

Details of the reference measurement channels can be found in TS 38.141-2 [34] annex A.

Table 7.9.5.2-1: Wide Area BS in-channel selectivity

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) |  | Interfering signal mean power (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz |  |  |
| 5 | 15 | G-FR1-A1-7 | -98.9-ΔminSENS | -98.5-ΔminSENS | -81.4 - ΔminSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 10 PRBs |
| 7 | 15 | G-FR1-A1-7 | -98.9-ΔminSENS | -98.5-ΔminSENS | -79.6 - ΔminSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 15 RBs |
| 10, 15, 20, 25, 30, 35 | 15 | G-FR1-A1-1 | -97-ΔminSENS | -96.6-ΔminSENS | -77.4 - ΔminSEN | DFT-s-OFDM NR signal, 15 kHz SCS, 25 PRBs |
| 40, 45, 50 | 15 | G-FR1-A1-4 | -90.6-ΔminSENS | -90.2-ΔminSENS | -71.4 - ΔminSEN | DFT-s-OFDM NR signal, 15 kHz SCS, 100 PRBs |
| 5 | 30 | G-FR1-A1-8 | -99.6-ΔminSENS | -99.2-ΔminSENS | -81.4 - ΔminSEN | DFT-s-OFDM NR signal, 30 kHz SCS, 5 PRBs |
| 10, 15, 20, 25, 30, 35 | 30 | G-FR1-A1-2 | -97.1-ΔminSENS | -96.7-ΔminSENS | -78.4 - ΔminSEN | DFT-s-OFDM NR signal, 30 kHz SCS, 10 PRBs |
| 40, 45, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 | -90.9-ΔminSENS | -90.5-ΔminSENS | -71.4 - ΔminSEN | DFT-s-OFDM NR signal, 30 kHz SCS, 50 PRBs |
| 10, 15, 20, 25, 30, 35 | 60 | G-FR1-A1-9 | -96.5-ΔminSENS | -96.1-ΔminSENS | -78.4 - ΔminSEN | DFT-s-OFDM NR signal, 60 kHz SCS, 5 PRBs |
| 40, 45, 50, 60, 70, 80, 90, 100 | 60 | G-FR1-A1-6 | -91-ΔminSENS | -90.6-ΔminSENS | -71.6 - ΔminSEN | DFT-s-OFDM NR signal, 60 kHz SCS, 24 PRBs |
| NOTE: Wanted and interfering signal are placed adjacently around Fc, where the Fc is defined for BS channel bandwidth of the wanted signal according to the table 5.4.2.2-1 in TS 38.104 [37].The aggregated wanted and interferer signal shall be centred in the BS channel bandwidth of the wanted signal. |  |  |  |  |  |  |

Table 7.9.5.2-2: Medium Range BS in-channel selectivity

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) |  | Interfering signal mean power (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz |  |  |
| 5 | 15 | G-FR1-A1-7 | -93.9-ΔminSENS | -93.5-ΔminSENS | -76.4 - ΔminSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 10 PRBs |
| 7 | 15 | G-FR1-A1-7 | -93.9-ΔminSENS | -93.5-ΔminSENS | -74.6 - ΔminSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 15 RBs |
| 10, 15, 20, 25, 30, 35 | 15 | G-FR1-A1-1 | -92-ΔminSENS | -91.6-ΔminSENS | -72.4 - ΔminSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 25 PRBs |
| 40, 45, 50 | 15 | G-FR1-A1-4 | -85.6-ΔminSENS | -85.2-ΔminSENS | -66.4 - ΔminSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 100 PRBs |
| 5 | 30 | G-FR1-A1-8 | -94.6-ΔminSENS | -94.2-ΔminSENS | -76.4 - ΔminSENS | DFT-s-OFDM NR signal, 30 kHz SCS, 5 PRBs |
| 10, 15, 20, 25, 30, 35 | 30 | G-FR1-A1-2 | -92.1-ΔminSENS | -91.7-ΔminSENS | -73.4 - ΔminSENS | DFT-s-OFDM NR signal, 30 kHz SCS, 10 PRBs |
| 40, 45, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 | -85.9-ΔminSENS | -85.5-ΔminSENS | -66.4 - ΔminSENS | DFT-s-OFDM NR signal, 30 kHz SCS, 50 PRBs |
| 10, 15, 20, 25, 30,35 | 60 | G-FR1-A1-9 | -91.5-ΔminSENS | -91.1-ΔminSENS | -73.4 - ΔminSENS | DFT-s-OFDM NR signal, 60 kHz SCS, 5 PRBs |
| 40, 45, 50, 60, 70, 80, 90, 100 | 60 | G-FR1-A1-6 | -86-ΔminSENS | -85.6-ΔminSENS | -66.6 - ΔminSENS | DFT-s-OFDM NR signal, 60 kHz SCS, 24 PRBs |
| NOTE: Wanted and interfering signal are placed adjacently around Fc, where the Fc is defined for BS channel bandwidth of the wanted signal according to the table 5.4.2.2-1 in TS 38.104 [37].The aggregated wanted and interferer signal shall be centred in the BS channel bandwidth of the wanted signal. |  |  |  |  |  |  |

Table 7.9.5.2-3: Local area BS in-channel selectivity

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) |  | Interfering signal mean power (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz |  |  |
| 5 | 15 | G-FR1-A1-7 | -90.9-ΔminSENS | -90.5-ΔminSENS | -73.4 - ΔminSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 10 PRBs |
| 7 | 15 | G-FR1-A1-7 | -90.9-ΔminSENS | -90.5-ΔminSENS | -71.6 - ΔminSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 15 RBs |
| 10, 15, 20, 25, 30, 35 | 15 | G-FR1-A1-1 | -89-ΔminSENS | -88.6-ΔminSENS | -69.4 - ΔminSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 25 PRBs |
| 40, 45, 50 | 15 | G-FR1-A1-4 | -82.6-ΔminSENS | -82.2-ΔminSENS | -63.4 - ΔminSENS | DFT-s-OFDM NR signal, 15 kHz SCS, 100 PRBs |
| 5 | 30 | G-FR1-A1-8 | -91.6-ΔminSENS | -91.2-ΔminSENS | -73.4 - ΔminSENS | DFT-s-OFDM NR signal, 30 kHz SCS, 5 PRBs |
| 10, 15, 20, 25, 30, 35 | 30 | G-FR1-A1-2 | -89.1-ΔminSENS | -88.7-ΔminSENS | -70.4 - ΔminSENS | DFT-s-OFDM NR signal, 30 kHz SCS, 10 PRBs |
| 40, 45, 50, 60, 70, 80, 90, 100 | 30 | G-FR1-A1-5 | -82.9-ΔminSENS | -82.5-ΔminSENS | -63.4 - ΔminSENS | DFT-s-OFDM NR signal, 30 kHz SCS, 50 PRBs |
| 10, 15, 20, 25, 30, 35 | 60 | G-FR1-A1-9 | -88.5-ΔminSENS | -88.1-ΔminSENS | -70.4 - ΔminSENS | DFT-s-OFDM NR signal, 60 kHz SCS, 5 PRBs |
| 40, 45, 50, 60, 70, 80, 90, 100 | 60 | G-FR1-A1-6 | -83-ΔminSENS | -82.6-ΔminSENS | -63.6 - ΔminSENS | DFT-s-OFDM NR signal, 60 kHz SCS, 24 PRBs |
| NOTE: Wanted and interfering signal are placed adjacently around Fc, where the Fc is defined for BS channel bandwidth of the wanted signal according to the table 5.4.2.2-1 in TS 38.104 [37].The aggregated wanted and interferer signal shall be centred in the BS channel bandwidth of the wanted signal. |  |  |  |  |  |  |

------------------------------ End of modified section -------------------------