3GPP TSG-RAN WG4 Meeting #116	R4-2511559

Bengaluru, India, August 25th – 29th, 2025

Title: 	Discussion on ETSI TC SES work related to the Harmonised Standards for NTN capable UE

Agenda Item:	9.4

Source: 	Huawei, HiSilicon

Document for:	Discussion

# Introduction

LS from ETSI TC SES was received in [1], where ETSI TC SES informs on the development of new harmonized standards for 3GPP-based IoT NTN UEs (ETSI EN 304 121, bands 256/255) and NR NTN UEs (ETSI EN 304 122, bands n256/n255).

ETSI TC SES informs that for frequency bands n256/256, it is intended to specify Tx and Rx requirements based on 3GPP TS 38.101-5 and 36.102 and corresponding RAN5 conformance specifications. Similar approach is intended for SEM, Tx spur, as well as Rx requirements in bands n255/255. However, the following two aspects of bands n255/255 operation may require some discussion in RAN4 based on [1]:

- Additional emissions requirements are being considered to protect 1610-1626.5 MHz frequency range based on requirements in EN 301 681 and ITU-R M.1480 - Radio astronomy in the bands 1660 MHz – 1660.5 MHz and 1668 – 1670 MHz shall be protected through the requirements associated to Tx OFF state based on 3GPP requirements.

Furthermore, ETSI TC SES is asking for feedback on the usefulness of the “nominated bandwidth” concept before SESSCN#53-e meeting scheduled 25th Sept 2025.

In this contribution, we provide initial discussion on ETSI TC SES work related to the Harmonised Standards for NTN capable UE.

# Discussion

Based on LS in [1], the following topics were identified for discussion in RAN4.

# 2.1	Tx requirements for bands n255/255

The following issues were captured in the LS in [1] for NTN UE operation in bands n255/255:

- additional emissions requirements are being considered to protect 1610-1626.5 MHz frequency range based on requirements in EN 301 681 and ITU-R M.1480,

- Radio astronomy in the bands 1660 MHz – 1660.5 MHz and 1668 – 1670 MHz shall be protected through the requirements associated to Tx OFF state based on 3GPP requirements.

Bullet 1 above implies that additional Tx emissions requirements may be missing in 38.101-5 and 36.102 specifications, for protection of 1610-1626.5 MHz. Recommendation ITU-R M.1480 lists unwanted emissions limits above 1 000 MHz and outside the band 1 626.5-1 660.5 MHz.

Limits of unwanted emissions above 1 000 MHz and outside the band 1 626.5-1 660.5 MHz for MESs with e.i.r.p. less than or equal to 15 dBW Frequency range (MHz); Carrier-on; Carrier-off. e.i.r.p. limit (dBW); Measurement bandwidth (kHz); e.i.r.p. limit (dBW)(1); Measurement bandwidth (kHz). 1 000.0-1 525.0; – 61; 1 000; – 77; 100. 1 525.0-1 559.0; – 61; 1 000; – 97; 100. 1 559.0-1 600.0; – 70; 1 000(2); – 77; 100. 1 600.0-1 605.0; – 70; 1 000; – 77; 100. 1 605.0-1 612.5; – 70 to – 58.5(3); 1 000; – 77; 100. 1 612.5-1 616.5; – 55 to – 50(3); 1 000; – 77; 100. 1 616.5-1 621.5; – 50 to – 46(3); 1 000; – 77; 100. 1 621.5-1 624.5; – 60; 30; – 77; 100. 1 624.5-1 625.0; – 60 to – 57.5(3), (4); 30; – 77; 100. 1 625.0-1 625.125; – 57.5 to – 57.2(3), (4); 30; – 77; 100. 1 625.125-1 625.8; – 57.2 to – 50(3), (4); 30; – 77; 100. 1 625.8-1 626; – 50 to – 47(3), (4); 30; – 77; 100.

Limits of unwanted emissions above 1 000 MHz and outside the band 1 626.5-1 660.5 MHz for MESs with e.i.r.p. greater than 15 dBW Frequency range (MHz); Carrier-on; Carrier-off. e.i.r.p. limit (dBW); Measurement bandwidth (kHz); e.i.r.p. limit (dBW)(1); Measurement bandwidth (kHz). 1 000.0-1 525.0; – 61; 1 000; – 72; 100. 1 525.0-1 559.0; – 61; 1 000; – 103; 3. 1 559.0-1 600.0; – 70; 1 000; – 77; 100. 1 600.0-1 605.0; – 70; 1 000; – 77; 100. 1 605.0-1 610.0; (2); 1 000; (3); 1 000. 1 610.0-1 621.5; – 46(2); 1 000; – 72; 100. 1 621.5-1 624.5; – 46 to – 40(4); 1 000; – 72; 100. 1 624.5-1 625.0; – 60 to – 57.5(4), (5), (6); 30; – 72; 100. 1 625.0-1 625.125; – 57.5 to – 57.2(4), (5), (6); 30; – 72; 100. 1 625.125-1 625.8; – 57.2 to – 50(4), (5), (6); 30; – 72; 100. 1 625.8-1 626.0; – 50 to – 47(4), (5), (6); 30; – 72; 100. 1 626.0-1 626.2; – 47 to – 40(4), (5), (6); 30; – 72; 100. 1 626.2-1 626.5; – 40(5), (6); 30; – 72; 100. 1 626.5-1 660.5; (7); (7); (7); (7). 1 660.5-1 662.5; (7); (7); (7); (7). 1 662.5-1 690.0; – 36; 1 000; – 72; 100. 1 690.0-3 400.0; – 61(8); 1 000; – 72; 100. 3 400-10 700; – 55(9), (10); 1 000; – 72; 100. 10 700-12 750; – 49; 1 000; – 76; 100.

Bullet 2 above refers to the RAS protection in 1660 MHz – 1660.5 MHz and 1668 – 1670 MHz. ETSI EN 301 681 captures related requirement for MES (Mobile Earth Station) as extracted below:

The LS [1] implies that the above requirement can rely on the Tx OFF requirement. However, it is not clear whether any related mechanism for RAS vicinity detection would need to be addressed for the purpose of the requirement referred above.

The LS [1] also indicated that bands n253/253, n251 and n250 are subject to the above-mentioned requirements (i.e., out of band emissions in 1610-1626.5 MHz, and Tx OFF for the purpose of RAS protections).

The following is proposed:

Proposal 1: Based on requirements captured in EN 301 681 and/or ITU-R M.1480, double-check necessity of additional emission requirements (i.e., out of band emissions in 1610-1626.5 MHz, and Tx OFF for the purpose of RAS protections) to be captured in TS 38.101-5 and TS 36.102 (at least for bands 255/n255).

# 2.2	Nominated bandwidth concept

ETSI TC SES asked for feedback on the usefulness of the “nominated bandwidth” concept. The “nominated bandwidth” concept was used for the purpose of defining the Spectral Emission Mask requirement and defined in the relevant existing ETSI harmonised standards (e.g., EN 301 442 and EN 301 444). The “nominated bandwidth” term was used in ETSI SES for SEM requirement definition, as well as to define the “unwanted emissions”. Below we provide definition of the “nominated bandwidth” term, as well as its interpretation:

When creating a new Harmonized Standard, the process is to reuse RAN4/RAN5 requirements and terminology, instead of trying to adopt new terms for the unwanted emissions definitions. If we would introduce “nominated bandwidth” concept for definition of UE unwanted emissions, we would need to translate/re-define the existing RAN4 requirements. This would be unnecessary complication.

As ETSI TC SES is working on creation of HS based on 3GPP RAN5 conformance specifications, the transposition process shall not modify, nor re-evaluate 3GPP conformance requirements.

Additionally, we would like to share feedback based on the experience gained during the previous work on multiple Harmonized Standards in ETSI MSG TFES. The “nominated bandwidth” value seems to be chosen/selected by the applicant (as per ETSI EN 301 442). Such approach is expected to be rejected by the HAS Consultant during the review process of the final draft of related Harmonized Standard, leading to its rejection. The underlying motivation is that the conformance requirement shall transparent, and not depend on parameter selection.

It is advised that ETSI TC SES further liaises with ETSI MSG TFES, who has extensive and up to date experience in HS drafting (please note that the latest version of ETSI EN 301 442 (developed under SES) is dated 2016, while the HS development process has been modified in the meantime).

Proposal 2: RAN4 to share feedback on the “nominated bandwidth” concept, that RAN5 conformance testing requirements (and baseline RAN4 core requirements) shall not be modified during the HS drafting process.

# 2.2	Timeline

ETSI TC SES would welcome feedback from 3GPP on the usefulness of the concept before ETSI TS SESSCN#53e (25th Sept 2025) meeting where conclusion on this topic is expected. In order to meet the timeline requested by ETSI TC SES, the following is proposed:

Proposal 3: RAN4 to send LS reply to ETSI TC SES during this meeting, providing feedback at least on the identified Tx requirements aspects, and the nominated bandwidth concept.

# Conclusions

Based on the discussion above, the following proposals were formulated:

Proposal 1: Based on requirements captured in EN 301 681 and/or ITU-R M.1480, double-check necessity of additional emission requirements to be captured in TS 38.101-5 and TS 36.102 (at least for bands 255/n255).

Proposal 2: RAN4 to share feedback on the “nominated bandwidth” concept, that RAN5 conformance testing requirements (and baseline RAN4 core requirements) shall not be modified during the HS drafting process.

Proposal 3: RAN4 to send LS reply to ETSI TC SES during this meeting, providing feedback at least on the identified Tx requirements aspects, and the nominated bandwidth concept.

# References

[1]		R4-2509023, LS on Harmonised Standard for NTN capable UE, ETSI TC SES