3GPP TSG-RAN4 Meeting #116	R4-2511572

# Bangalore, India 25th – 29rd August, 2025

Source: 	Qualcomm Incorporated

Title: 	RRM requirements for low band carrier aggregation via switching

Agenda item:	7.3.4

Document for:	Discussion

# Introduction

3GPP introduced a new work item on low band carrier aggregation via switching in RAN #106 [1]. We discuss some relevant RRM requirements for this topic.

# Past Agreements

RAN4 RRM session discussed this topic during the past meeting. The agreements and open issues are shown below:

Issue 1-1-3: switching pattern applicability for RRM requirement in R19 LB CA via switching Agreement: - For the SCell which has been activated(i.e., after UE reporting valid CQI for SCell activation in section 8.3.2 TS38.133), - The switching pattern for SDL SCell is applied. - For the SCell which is in activation procedure, - The switching pattern for SDL SCell is not applied. - RAN4 defines SCell activation requirement assuming UE utilizes the RS occasions to perform SDL SCell activation - the above RS occasions are FFS - For the SCell which is deactivated, - The switching pattern for SDL SCell is not applied. - If the deactivated SDL SCell measurement requirement is needed, FFS whether UE follows legacy deactivated SCell measurement requirements to perform deactivated SDL SCell measurement or follow a new measurement requirements for deactivated SDL SCell measurement. Note: the above FFS parts will be discussed and decided in RAN4. - Switch delay requirements Agreement: - The switching delay between operating the FDD and SDL carrier is to be discussed in the RF session. RAN4 RRM not to define additional switch delay in RRM. - Synchronization requirements for FDD and synchronization accuracy at BS antenna connectors or radiated interface boundaries Agreement: - No need to define new cell phase synchronization requirement in RRM spec for R19 LB CA via switching. - SDL SCC related RRM requirement - activated SDL SCell measurement Agreement: For R19 LB CA via switching, RAN4 to not define SDL activated SCell L3 measurement requirement if SSB-less based SCell operation is used. For R19 LB CA via switching, FFS: - Whether and how RAN4 to define SDL activated SCell L3 measurement requirement if SSB based SCell operation is used. - de-activated SDL SCell measurement Agreement: For R19 LB CA via switching, RAN4 to not define SDL deactivated SCell L3 measurement requirement if SSB-less based SCell operation is used. For R19 LB CA via switching, FFS: - Whether and how RAN4 to define SDL deactivated SCell L3 measurement requirement if SSB based SCell operation is used. - other SDL SCell requirements Agreement: - FFS: Whether and how - RAN4 to define a new SDL SCell activation/deactivation delay and related interruption for R19 LB CA - the existing SCell activation/deactivation delay and related interruption requirement can be used as baseline for R19 LB CA - FFS: Whether and how - RAN4 to define a new BFD, CBD, TCI state switch delay, PL-RS switch delay, L1-RSRP/SINR requirements for R19 LB CA - the existing BFD, CBD, TCI state switch delay, PL-RS switch delay, L1-RSRP/SINR requirement can be used as baseline for R19 LB CA Agreement: - Not to change definition of intra-frequency/inter-frequency measurement with/without gap measurement - FFS on the use case for MG application. - Potential impacts to the PCell related requirement - FFS - FFS: For a UE supporting low band CA via switching, SDL SCell operation is SSB based? or SSB-less based? or both? Agreement: - For a UE supporting low band CA via switching, RAN4 to discuss requirement for SDL SCell operation for both SSB-based SCell and SSB-less SCell based, and decide whether to define requirements for both cases or start with SSB-based in the next meeting. - (2) FFS: MRTD between the two low bands is CP? Or wait for TAE conclusion from RF session? Agreement: As generic assumption for discussing the RRM requirement, the received timing difference between the two bands at UE is within a value in option 1 or option 2, unless other TAE requirement will be defined in RF session. - Option 1: 4.67us - Option 2: 3 us RAN4 115 meeting agreements: Issue 1-1-1: SSB assumptions for R19 LB CA via switching Agreement on Monday AH: - The baseline assumption is: - SDL SCell is transmitting SSB - RAN4 to use SSB based requirement as baseline for Rel-19 low band CA via switching - requirement of PCell will still use the existing principle as baseline but the impact of carrier switching in LB CA will be considered R4-2508259 Issue 1-1-2: MRTD assumptions for R19 LB CA via switching Agreement on Monday AH: - The maximum receive timing difference for collocated FDD PCell and SDL SCell CA via switching is 3us. - Whether or not above RTD can be used as side condition for SCell activation will be discussed in another issue. Issue 1-2-1: Necessity of interruption requirement in RRM between FDD CC and SDL CC Online Agreement on Tuesday: - For SDL to FDD switching and FDD to SDL switching, RAN4 to not define interruption or scheduling restriction requirement on SDL carrier and FDD carrier. - Note: the above agreement can be revisited if not aligned with RAN1 further agreement. Issue 1-3-1: Necessity of activated SDL SCell L3 measurement and corresponding interruption requirement Online Agreement on Tuesday: - RAN4 to define the requirement for activated SDL SCell L3 measurement. Issue 1-3-2: Requirement design of activated SDL SCell L3 measurement Online Agreement on Tuesday: - For activated SDL SCell measurement, UE performs the SDL SCell measurement on the overlapped occasions of SMTC window and SDL SCell reception duration indicated by switching pattern. FFS: - If no SSB occasion of the serving cell is contained within the LB-CA switching pattern, the requirements do not apply (from Nokia proposal) - FFS on details of requirement design. Online agreement on Tuesday: - RAN4 to define the requirement for deactivated SDL SCell L3 measurement. Agreement on Thursday: - The legacy deactivated SCell measurement delay requirements are applied for UE to perform deactivated SDL SCell measurement. Agreement on Thursday: - For deactivated SDL SCell measurement, interruptions on active serving cell(s) are allowed with up to X% probability of missed ACK/NACK. - FFS: X, and X can be same or different for different SDL SCC measurement cycle (i.e., configured measCycleSCell). Issue 1-3-5: Scope of SDL SCell activation/deactivation Online agreement on Tuesday: - PUCCH SCell activation, Rel-19 fast SCell activation through EMR, and activation of multiple SCells is not in the scope of the WI. Issue 1-3-6: side condition for SDL SCell activation/deactivation requirement Agreement on Thursday: - Follow the legacy case and side conditions for SSB based inter-band FR1 SCell activation in existing spec. Issue 1-3-7: SDL SCell activation/deactivation delay requirement - Agreement on Thursday: - The R18 SSB based FR1 inter-band SCell activation delay requirement can be reused to SDL SCell activation in LB CA via switching if UE supports corresponding feature. - R16 Direct SCell activation with HO is not considered for R19 LB CA. - R17 PUCCH SCell activation is not considered for R19 LB CA. - R18 SSB-less SCell activation for NES is not considered for R19 LB CA. - R17 Fast SCell activation is not considered for R19 LB CA. - Reuse the existing SCell deactivation delay requirement for SDL SCell deactivation. - Known/unknown conditions are reused from existing SCell activation requirements - FFS: - RAN4 to discuss the UE behavior if the CQI or L1-RSRP reporting is colliding with SSB or SMTC of being-activated SDL SCell during the SDL SCell activation. (Apple) - The starting point and ending point of CSI measurement for the sake of CSI reporting should be determined, for both SSB based and SSB-less SCell activation.(ZTE) - Amend the previous agreement with the following - The UE shall continuously monitor PCell disregarding the configured switching pattern. (Nokia) Issue 1-3-10: BFD/CBD requirements for SDL SCell Agreement: - If RAN4 to define requirement for BFD/CBD for SDL SCell in LB CA via switching, the BFD/CBD on SDL carrier is performed on the overlapped occasions between SSB/CSI-RS and SDL duration in switch pattern. Issue 1-3-11: TCI state switch delay requirement for SDL SCell Agreement: - The L1-RSRP measurement and T/F tracking for TCI switching on SDL carrier is performed on the overlapped occasions between SSB/CSI-RS and SDL duration in switch pattern. Issue 1-3-13: L1-RSRP/SINR measurement requirements for SDL SCell Agreement in AH: - For L1-RSRP/SINR measurement requirement for SDL SCell, UE can only perform such measurement on the overlapped occasions of L1 RS and SDL SCell reception duration indicated by switching pattern. Issue 1-4-1: Impact on timing requirement for PCell - Agreement in AH: the existing Tx timing requirement for PCell with the same applicability condition (SSB is available in last 160ms) can apply for R19 LB CA via switching

# Discussion

According to [2], low band spectrum carries significant traffic volume in urban indoor and rural areas. However, SDL remain under-utilized since they cannot be paired with UL. That’s why, [2] proposed to introduce a network configured switching pattern in low band carrier aggregation via switching

Figure 1: RRC configured switching pattern in low band carrier aggregation via switching [2]

Figure 1 shows how network could configure this switching pattern to enable this low band carrier aggregation. This switching pattern would instruct UE to alternate between FDD DL+UL band and SDL band in a periodic manner. We discuss different RRM requirements of SCells and PCells in this scenario.

## Activation of Switching Pattern

Figure 2: Transmission of CQI and application of switching pattern during Direct SCell activation in LB-LB CA

RAN4 made the following agreement during the last meeting:

Issue 1-1-3: switching pattern applicability for RRM requirement in R19 LB CA via switching Agreement: - For the SCell which has been activated(i.e., after UE reporting valid CQI for SCell activation in section 8.3.2 TS38.133), - The switching pattern for SDL SCell is applied. - For the SCell which is in activation procedure, - The switching pattern for SDL SCell is not applied. - RAN4 defines SCell activation requirement assuming UE utilizes the RS occasions to perform SDL SCell activation - the above RS occasions are FFS - For the SCell which is deactivated, - The switching pattern for SDL SCell is not applied. - If the deactivated SDL SCell measurement requirement is needed, FFS whether UE follows legacy deactivated SCell measurement requirements to perform deactivated SDL SCell measurement or follow a new measurement requirements for deactivated SDL SCell measurement. Note: the above FFS parts will be discussed and decided in RAN4.

Above agreement shows that switching pattern for SDL SCell only applies for the active SCell, i.e., after UE reports valid CQI for SCell activation. RAN4 has allowed reduction of SCell activation timeline by not applying switching during activation procedure and SSBs of SCells may not always overlap with the ON duration of SCells of the RRC configured switching pattern.

Hence, UE will have to create an implicit switching pattern during SCell activation where the ON duration of SCell of the activation pattern is expected to include the reference signals of SCells. This means that UE will have to replace the implicit switching pattern with RRC configured switching pattern after sending non-zero CSI reports. Figure 2 shows this scenario.

Figure 3: Snapshots of SCell activation timelines in different Rel-15 scenarios

Rel-19 allows network to configure a new switching pattern only via an RRC command. On the other hand, legacy requirements allow network to trigger CSI report via DCI during SCell activation. The combination of legacy requirements and recent RAN4 agreement mean that UE’s first application of switching pattern can be dynamically triggered via DCI.

Unless periodic CSI report is not configured, network can send DCI via PCell at any time to configure CSI-RS in SCell and CSI report in PCell after UE has measured enough SCell SSB samples. RRM core spec only focuses on periodic and semi-persistent CSI-RS during SCell activation but does not limit CSI reporting to periodic or semi-persistent only. Figure 3 shows that CSI-RS is limited to periodic and semi-persistent during SCell activation but nature of CSI reporting is not captured in spec. During field deployment, gNB often uses aperiodic CSI reports during SCell activation.

Even if only periodic and semi-persistent CSI reporting is allowed during SCell activation, combination of RAN4 and RAN1 requirements force UE to update the switching pattern implicitly based on transmission of non-zero CSI reports. UE will need some time to update its scripts to change from implicit switching pattern to explicit switching pattern.

Observation 1: RAN4 has agreed that the switching pattern for SDL SCell is not applied when the SCell is in activation procedure. Switching pattern for SDL SCell applies for the active SCell (i.e. after UE reports valid CQI for SCell activation).

Observation 2: UE has to update its implicit switching pattern where SCell ON duration is obtained from the time locations of reference signals of SCell to RRC configured explicit switching pattern after sending non-zero CQI reports.

Observation 3: In Rel-19, switching pattern can only be updated via RRC after SCell gets active. UE would get 10ms (equal to RRC processing time) to update the RRC configured switching pattern when an SCell is active.

Observation 4: Core RRM requirements allow network to trigger CSI report for SCell activation via DCI.

Observation 5: Combination of legacy requirements and latest RAN4 agreements mean that the time separation between CQI reporting and the activation of explicit switching pattern can be as small as zero.

Observation 6: UE needs some time to build the scripts for explicit switching pattern and replace implicit switching pattern with that after sending non-zero CSI reports.

Note that, if RAN4 had allowed switching pattern to be applicable during SCell activation procedure, then switching pattern would have become active via MAC-CE or RRC configured SCell activation command. UE gets 3ms or 10ms after sending HARQ A/N with MAC-CE or RRC based activation command respectively to start SCell activation

Figure 4: UE’s option to apply RRC configured explicit switching pattern after sending non-zero CQI report

procedure. That means, after sending HARQ A/N corresponding to the SCell activation command, UE would have gotten 3ms or 10ms to apply switching pattern in MAC-CE or RRC based activation. Hence, it is reasonable to allow a similar amount of minimum time separation between valid CQI report and activation of switching pattern.

Proposal 1: At the end of SCell activation (i.e., after UE reporting valid CQI for SCell activation in section 8.3.2 TS38.133), spec allows UE a minimum time separation of X ms between transmitting CQI report and applying the semi-statically configured switching pattern.

- Value of X is 3ms

UE sends non-zero CQI report via PCell. Assume that spec allows a minimum time separation between sending non-zero CQI report and applying RRC configured explicit switching pattern. After the elapse of the minimum time separation, RRC configured switching pattern may tell UE to move to SCell immediately. There are two choices to tackle this scenario:

- Option 1: UE moves to SCell immediately

- Option 2: UE waits until the next transition point after sending non-zero CQI report and switches to SCell during that transition point

Figure 4 displays these two options. Option 2 is “cleaner” since it ensures UE’s transition from PCell to SCell at RRC configured switching locations. Spec should formally capture this as UE’s transition point to ensure consistency between network and UE regarding UE’s location.

Observation 7: After sending valid non-zero CQI pattern and waiting for spec allowed minimum time separation (as mentioned in proposal 1), UE might need to be in SCell if it had to follow RRC configured switching pattern. UE has two choices there:

- Option 1: UE moves to SCell immediately

- Option 2: UE waits until the next transition point from PCell to SCell and switches to SCell during that transition point

Observation 8: Option 2 of previous observation ensures UE’s transition from PCell to SCell at RRC configured switching location

Proposal 2: After sending valid non-zero CQI report and waiting for spec allowed minimum time separation), UE waits until the next transition point from PCell to SCell and switches to SCell during that transition point.

- Note: UE can be scheduled in PCell while it waits until the next transition point from PCell to SCell to switch to SCell.

## Interruption During Deactivated SCell Measurement Requirement

RAN4 made the following agreements during the last meeting:

Online agreement on Tuesday: - RAN4 to define the requirement for deactivated SDL SCell L3 measurement. Agreement on Thursday: - The legacy deactivated SCell measurement delay requirements are applied for UE to perform deactivated SDL SCell measurement. Agreement on Thursday: - For deactivated SDL SCell measurement, interruptions on active serving cell(s) are allowed with up to X% probability of missed ACK/NACK. - FFS: X, and X can be same or different for different SDL SCC measurement cycle (i.e., configured measCycleSCell).

RAN4 legacy spec allows UE to measure SCell in CA scenario. UE is allowed to interrupt PCell when measCycleSCell is greater than or equal to 640ms. This requirement was derived assuming that UE would keep its RF circuits ON to receive both PCell and deactivated SCell when measCycleSCell is less than 640ms. If measCycleSCell is greater than 640ms, UE can turn off a portion of its RF and not receive SCell in RF. Hence, when UE activates its RF to measure SCell, it causes interruption to PCell.

In this scenario, UE cannot simultaneously measure SCell and PCell. Hence, UE will cause interruption to PCell whenever it measures SSB of SCells. This interruption will happen irrespective of measCycleSCell periodicity which can range from 160ms to 1280ms. The required amount of interruption will vary based on measCycleSCell periodicity, SMTC duration and number of SSBs that need to be measured. Any potential deactivated SCell measurement needs to define interruption requirements suitably considering those factors.

Legacy spec allows UE one slot each of interruption time before and after SMTC of deactivated SCell. This legacy interruption length requirement can be reused for deactivated SCell measurement in LB-LB CA scenario.

Interruption length only focuses on the specific time location of PCell communication that get interrupted due to UE’s movement to and from PCell. Interruption length does not consider the PDCCH based grants that could be configured during UE’s switch to and from SCell and during UE’s stay at SCell. These PDCCH might have configured PDSCH in slots outside of interruption window but UE will not be able to receive those. The impact of deactivated SCell measurements on these and UE will send HARQ NACK in the corresponding locations.

Observation 9: RAN4 has to consider measCycleSCell periodicity and SMTC duration to define interruption requirements during deactivated SCell measurement.

Observation 10: Interruption length requirements do not consider the HARQ ACK/NACKs that fall outside interruption window where UE misses the relevant grants within the interruption window due to deactivated SCell measurement.

Proposal 3: Legacy interruption length requirement for measurement of deactivated SCell in intra-band CA is reused for measurement of deactivated SCell in LB-LB CA scenario.

Proposal 4:

For deactivated SDL SCell measurement, interruptions on active serving cell(s) are allowed with up to X% probability of missed ACK/NACK. The interruption rate X can be different for different SDL SCC measurement cycle (i.e., configured measCycleSCell):

  - probability of missed ACK/NACK: X= YmeasCycleSCell ;

    - Y=2* (tuning time + measurement window + retuning time), in slot level, because both scheduled PDSCH/PDCCH and ACK/NACK transmission of schedule PDSCH is interrupted

    - Tuning time from PCell to SCell is equal to the RRC configured switching gap from PCell to SCell LBCA-SwitchingGap-Duration-PCelltoSCell-r19

    - Returning time from SCell to PCell is equal to the RRC configured switching gap from SCell to PCell, i.e., LBCA-SwitchingGap- SCelltoPCell-r19

## Interruption and/or Scheduling Restriction during SCell Activation

RAN4 previously agreed that the RRC configured switching pattern is not applied during SCell activation procedure and the activation requirements are defined assuming UE utilizes the RS occasions to perform SDL SCell activation.

Issue 1-1-3: switching pattern applicability for RRM requirement in R19 LB CA via switching Agreement: - For the SCell which has been activated(i.e., after UE reporting valid CQI for SCell activation in section 8.3.2 TS38.133), - The switching pattern for SDL SCell is applied. - For the SCell which is in activation procedure, - The switching pattern for SDL SCell is not applied. - RAN4 defines SCell activation requirement assuming UE utilizes the RS occasions to perform SDL SCell activation - the above RS occasions are FFS - For the SCell which is deactivated, - The switching pattern for SDL SCell is not applied. - If the deactivated SDL SCell measurement requirement is needed, FFS whether UE follows legacy deactivated SCell measurement requirements to perform deactivated SDL SCell measurement or follow a new measurement requirements for deactivated SDL SCell measurement. Note: the above FFS parts will be discussed and decided in RAN4.

UE needs to utilize SMTCs and periodic/semi-persistent CSI-RS of SCell during SCell activation procedure. Although spec requires UE to measure CSI-RS after it has measured spec defined sufficient number of SSB samples, some UE can start measuring CSI-RS immediately after measuring one SSB. Spec or network cannot know when and how UE measures periodic/semi-persistent CSI-RS during SCell activation procedure. Hence, the safest option would be to create scheduling restriction adjacent to all SS burst set occasions and periodic/semi-persistent CSI-RS of SCell during SCell activation procedure.

Since PCell and SCells are time synchronous, UE can just use the SS burst location of SCell to measure different SSBs of the SCell. UE does not need to utilize the SMTC configured of SCell.

Observation 11: UE needs to utilize SS burst sets and periodic/semi-persistent CSI-RS of SCell during SCell activation procedure. Spec or network cannot know when and how UE measures periodic/semi-persistent CSI-RS during SCell activation procedure.

Figure 5: Implication of symbol level scheduling restriction on the granularity of switching pattern

Proposal 5: During SCell activation in LB-LB CA, scheduling restriction is defined adjacent to all SS burst set occasions and periodic/semi-persistent CSI-RS resources of SCell.

Since UE’s switch to and from SCell happens in a pre-defined manner during SCell activation, scheduling restriction requirement is more applicable for this scenario. Interruption requirement is not needed.

UE needs time to move to and from SCell. While moving to SCell, this tune time needs to consider the RF switching time and MRTD. While coming back from SCell, this tune time needs to consider RF switching time, MRTD and UL TA.  RRC configured parameters LBCA-SwitchingGap-Duration-PCelltoSCell-r19 and LBCA-SwitchingGap-SCelltoPCell-r19 would accommodate these parameters. Hence, the configured values of these two IEs can be used as the tuning time to SCell and re-tuning time from SCell.

Observation 12: UE’s tune time from PCell to SCell needs to include the RF switching time and MRTD. UE’s tune time from SCell to PCell needs to include the RF switching time, MRTD and UL TA.

- RRC configured parameters LBCA-SwitchingGap-Duration-PCelltoSCell-r19 and LBCA-SwitchingGap-SCelltoPCell-r19 would accommodate these parameters.

The next question is whether scheduling restriction in PCell would be in symbol level or slot level. Typically, RAN4 RRM spec defines scheduling restriction in symbol level. However, scheduling restriction in this scenario is stemming from an implicit switching pattern between PCell and SCell. RAN1 has agreed the RRC configured switching pattern to be in slot level granularity. This is shown in the following agreement [6]

RAN1 Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, it is not expected that the same slot (1ms) contains both symbol(s) configured as FDD carrier and symbol(s) configured as SDL carrier Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, the time unit (resolution) of the semi-static switching pattern configuration is slot (1ms for 15kHz SCS).

Allowing scheduling restriction to be in symbol level granularity will mean that UE will have to maintain a symbol level bitmap for switching pattern between PCell and SCell. This would be more complex for UE to handle than the slot level based RRC configured switching pattern. Besides, a symbol level scheduling restriction and a corresponding symbol level switching bitmap may force UE to go through multiple switches in the same slot (e.g., returning from SCell at the beginning of the slot and tuning to SCell at the end of the slot). This is also more complex than RAN1 defined explicit switching pattern that restricts the maximum number of switches to 1 due to its slot level granularity. Figure 5 shows how scheduling restriction with symbol level granularity can force UE to maintain symbol level switching pattern granularity and to support more than one switching per slot.

Figure 6: UE’s need to support bi-directional switch with a slot level granularity based scheduling restriction

Observation 13: Allowing scheduling restriction to be in symbol level granularity will force UE to maintain a symbol level bitmap for switching pattern between PCell and SCell.

Observation 14: A symbol level scheduling restriction and a corresponding symbol level switching bitmap may force UE to go through multiple switches in the same slot.

Observation 15: RAN1 has agreed the RRC configured switching pattern to be in slot level granularity.

The issues arising from symbol level granularity based scheduling restriction can be addressed in two ways. First, scheduling restriction can be adjacent to the slots containing SS burst set and periodic CSI-RS. Second, scheduling restriction can be adjacent to the specific locations of SS burst set and period CSI-RS, but they should be in slot level granularity. The second option takes us closer to restrict the number of switches per slot to one. Hence, it is preferable from that perspective.

Proposal 6: The scheduling restriction length during SDL SCell activation can be defined in the # of slots, as follows:

{(tune time FDD-to-SDL)+  SS Burst Set/CSI-RS duration + (tune time SDL-to-FDD)}, adjusted in slot level boundary.

- Note 1: “adjusted in slot level boundary” means that the scheduling restriction always starts at the 1st symbol of a slot and ends at the last symbol of the same or a different slot to accommodate the required parameters that are mentioned above.

- Note 2: Tune time from FDD to SDL is equal to the RRC configured LBCA-SwitchingGap-Duration-PCelltoSCell-r19 and tune time from SDL to FDD is equal to the RRC configured LBCA-SwitchingGap-SCelltoPCell-r19

## Interruption to Other Carriers/Bands

RAN4 RF session has focused on only two bands during this Rel-19 work item. RF session may focus on other bands, including switching between more than two bands, in the future. Since there is no 3rd band in the current release, RRM session does not need to specify interruption to 3rd band in Rel-19.

Also, this Rel-19 work item is primarily focusing on only one carrier in each band. UE cannot simultaneously operate in the relevant FDD and SDL bands. Hence, if network activates multiple carriers in FDD PCells, they will have to be turned off when UE switches to SDL band. System throughput may get impacted if network switches UE, operating with FDD bands containing multiple carriers, to SDLSCell band. Hence, RAN4 does not need to define interruption requirements to other carriers in the same band since: I) it does not seem to be within Rel-19 scope and 2) its use-case is unlikely.

Observation 16:  RAN4 RF session has focused on only two bands during this Rel-19 work item.

Observation 17: Multiple carriers in FDD band and one carrier in SDL band don’t seem to be within the scope of work item. Network is unlikely to deploy this because the switch to SDL band may impact system throughput in this scenario.

Proposal 7: RAN4 does not define interruption requirements to other bands/carriers in Rel-19.

## Measurement of PCells

PCell measurement opportunities would be impacted due to the switching pattern. Hence, PCell measurement periods need to be extended to accommodate the switching pattern at the UE.

Proposal 8: RAN4 extends all relevant measurement periods of PCell to accommodate switching pattern in LB-LB CA via switching.

# Conclusions

Observation 1: RAN4 has agreed that the switching pattern for SDL SCell is not applied when the SCell is in activation procedure. Switching pattern for SDL SCell applies for the active SCell (i.e. after UE reports valid CQI for SCell activation).

Observation 2: UE has to update its implicit switching pattern where SCell ON duration is obtained from the time locations of reference signals of SCell to RRC configured explicit switching pattern after sending non-zero CQI reports.

Observation 3: In Rel-19, switching pattern can only be updated via RRC after SCell gets active. UE would get 10ms (equal to RRC processing time) to update the RRC configured switching pattern when an SCell is active.

Observation 4: Core RRM requirements allow network to trigger CSI report for SCell activation via DCI.

Observation 5: Combination of legacy requirements and latest RAN4 agreements mean that the time separation between CQI reporting and the activation of explicit switching pattern can be as small as zero.

Observation 6: UE needs some time to build the scripts for explicit switching pattern and replace implicit switching pattern with that after sending non-zero CSI reports.

Observation 7: After sending valid non-zero CQI pattern and waiting for spec allowed minimum time separation (as mentioned in proposal 1), UE might need to be in SCell if it had to follow RRC configured switching pattern. UE has two choices there:

- Option 1: UE moves to SCell immediately

- Option 2: UE waits until the next transition point from PCell to SCell and switches to SCell during that transition point

Observation 8: Option 2 of previous observation ensures UE’s transition from PCell to SCell at RRC configured switching location

Observation 9: RAN4 has to consider measCycleSCell periodicity and SMTC duration to define interruption requirements during deactivated SCell measurement.

Observation 10: Interruption length requirements do not consider the HARQ ACK/NACKs that fall outside interruption window where UE misses the relevant grants within the interruption window due to deactivated SCell measurement.

Observation 11: UE needs to utilize SS burst sets and periodic/semi-persistent CSI-RS of SCell during SCell activation procedure. Spec or network cannot know when and how UE measures periodic/semi-persistent CSI-RS during SCell activation procedure.

Observation 12: UE’s tune time from PCell to SCell needs to include the RF switching time and MRTD. UE’s tune time from SCell to PCell needs to include the RF switching time, MRTD and UL TA.

- RRC configured parameters LBCA-SwitchingGap-Duration-PCelltoSCell-r19 and LBCA-SwitchingGap-SCelltoPCell-r19 would accommodate these parameters.

Observation 13: Allowing scheduling restriction to be in symbol level granularity will force UE to maintain a symbol level bitmap for switching pattern between PCell and SCell.

Observation 14: A symbol level scheduling restriction and a corresponding symbol level switching bitmap may force UE to go through multiple switches in the same slot.

Observation 15: RAN1 has agreed the RRC configured switching pattern to be in slot level granularity.

Observation 16:  RAN4 RF session has focused on only two bands during this Rel-19 work item.

Observation 17: Multiple carriers in FDD band and one carrier in SDL band don’t seem to be within the scope of work item. Network is unlikely to deploy this because the switch to SDL band may impact system throughput in this scenario.

Proposal 1: At the end of SCell activation (i.e., after UE reporting valid CQI for SCell activation in section 8.3.2 TS38.133), spec allows UE a minimum time separation of X ms between transmitting CQI report and applying the semi-statically configured switching pattern.

- Value of X is 3ms

Proposal 2: After sending valid non-zero CQI report and waiting for spec allowed minimum time separation), UE waits until the next transition point from PCell to SCell and switches to SCell during that transition point.

- Note: UE can be scheduled in PCell while it waits until the next transition point from PCell to SCell to switch to SCell.

Proposal 3: Legacy interruption length requirement for measurement of deactivated SCell in intra-band CA is reused for measurement of deactivated SCell in LB-LB CA scenario.

Proposal 4:

For deactivated SDL SCell measurement, interruptions on active serving cell(s) are allowed with up to X% probability of missed ACK/NACK. The interruption rate X can be different for different SDL SCC measurement cycle (i.e., configured measCycleSCell):

  - probability of missed ACK/NACK: X= YmeasCycleSCell ;

    - Y=2* (tuning time + measurement window + retuning time), in slot level, because both scheduled PDSCH/PDCCH and ACK/NACK transmission of schedule PDSCH is interrupted

    - Tuning time from PCell to SCell is equal to the RRC configured switching gap from PCell to SCell LBCA-SwitchingGap-Duration-PCelltoSCell-r19

    - Returning time from SCell to PCell is equal to the RRC configured switching gap from SCell to PCell, i.e., LBCA-SwitchingGap- SCelltoPCell-r19

Proposal 5: During SCell activation in LB-LB CA, scheduling restriction is defined adjacent to all SS burst set occasions and periodic/semi-persistent CSI-RS resources of SCell.

Proposal 6: The scheduling restriction length during SDL SCell activation can be defined in the # of slots, as follows:

{(tune time FDD-to-SDL)+  SS Burst Set/CSI-RS duration + (tune time SDL-to-FDD)}, adjusted in slot level boundary.

- Note 1: “adjusted in slot level boundary” means that the scheduling restriction always starts at the 1st symbol of a slot and ends at the last symbol of the same or a different slot to accommodate the required parameters that are mentioned above.

- Note 2: Tune time from FDD to SDL is equal to the RRC configured LBCA-SwitchingGap-Duration-PCelltoSCell-r19 and tune time from SDL to FDD is equal to the RRC configured LBCA-SwitchingGap-SCelltoPCell-r19

Proposal 7: RAN4 does not define interruption requirements to other bands/carriers in Rel-19.

Proposal 8: RAN4 extends all relevant measurement periods of PCell to accommodate switching pattern in LB-LB CA via switching.

# References

[1] RP-243317, New WID on low band carrier aggregation via switching, 3GPP TSG RAN #106.

[2] RP-242532, Motivation for a new work item on low band carrier aggregation via switching in Rel-19, 3GPP TSG RAN #106.

[3] R4-2502588, WF on RRM requirements for NR_LBCA_Sw, 3GPP TSG-RAN WG4 Meeting#114.

[4] 3GPP TSG RAN WG1 Chairman’s Notes, RAN1 #120bis.

[5] 38.133

[6] R1-2504869, LS on Low NR Band Carrier Aggregation via Switching, 3GPP TSG WG RAN1 #121