3GPP TSG-RAN WG4 Meeting #116	R4-2511575

Bengaluru, India, August 25th – August 29th, 2025

Source:	Nokia

Title:	On introducing requirements for CB-MSG3 in TS 36.133

Agenda item:	7.28.4

Document for:	Discussion

Introduction

In RAN2 #126, RAN2 has reached agreements in the Rel-19 WI related to NB-IoT over NTN (IoT_NTN_Ph3-Core) to introduce the contention-based message 3 (CB-MSG3), where the UE skips the Msg1 and Msg2 in the Random Access procedure, for faster data dump. Recently RAN2 has sent one LS to RAN4 indicating further development in the feature design (R2-2504776).

In this document we provide our views on RAN2 LS.

Discussion

RAN2 has agreed upon the introduction of CB-MSG3 and has recently sent one LS to RAN4, which states that:

- Agreements regarding TAC in MSG4 (i.e., CB-msg3 response): - The TAC is optionally used in the CB-Msg3 response. - RAN2 assumes that NTA=0 for initial CB-msg3 transmission. - RAN2 assumes the length of the TAC field is 6 bits (we can revisit this if there is major R1 impact on TA calculation)

And then, RAN2 has posed the following question:

- whether there is any concern from RAN1 and RAN4 on the agreements regarding TAC in MSG4?

From our point of view, the requirements are such that the UE is only capable of performing CB-msg3 transmission if the UE has performed pre-compensation. This reduces the necessity of TAC after RACH procedure is completed. But the agreement also provides option for TAC to be conveyed if needed. In this sense, we see no problem in having an optional TAC in the answer to CB-Msg3.

Confirm to RAN2 that there are no concerns to the the optionality of TAC in the CB-MSG3 procedure.

Conclusion

In this paper we analyzed an responded the LS received from RAN2:

Proposal 1: Confirm to RAN2 that there are no concerns to the the optionality of TAC in the CB-MSG3 procedure.

- References

- R2-2405769, “LS on UL synchronization for contention based Msg3 transmission without Msg1/Msg2”, ZTE, RAN2#126, Fukuoka, Japan, May 20 – May 24, 2024.

- R2-2504776, “LS on CB-msg3-EDT”, Huawei