3GPP TSG-RAN WG4 Meeting #116									                        R4-2511584

Bengaluru, India, August 25 – 29, 2025

Agenda item:			7.20.3

Source:	Ericsson

Title:	On 3Tx PC2 MPR requirements specification

Document for:	Approval

# Introduction

In RAN#105, a revised WID has been approved for the RAN1-led WI on “NR MIMO Phase 5” WI [1], where the two main topics of interest for RAN4 are “3 Tx” and “Asymmetric DL sTRP/UL mTRP”. In RAN4#114-bis, the discussion on “3 Tx” topic has been finalized. In RAN4#115, a draft CR to introduce FR1 3Tx RF requirements has been endorsed [2]. In this contribution, we explain why we think the agreed 3Tx PC2 MPR requirements haven’t been implemented correctly in the mentioned draft CR.

# Discussion

In RAN4#114-bis, the following was agreed [3]:

<Sub-topic 1-2> 3Tx MPR

<Agreement>: PC2 MPR for 3Tx

- Reuse 2Tx PC2 MPR

Looking at the current 2Tx PC2 MPR requirements in TS38.101-1 [4], Table 6.2D.2-1 is specified for 2Tx PC2 UEs when they don’t indicate ULFPTx ‘mode 0’ or ‘mode 2’, while Table 6.2.2-2 is referenced (1Tx PC2 MPR) for 2Tx PC2 UEs when they indicate ULFPTx ‘mode0’ or ‘mode 2’:

For UE with two or four transmit antenna connectors in closed-loop spatial multiplexing scheme, the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 6.2D.1-1 is specified in Table 6.2.2-1for PC3, Table 6.2D.2-1 for 2Tx PC2 when the UE does not indicate ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16 for the band and Table 6.2.2-2 for 2Tx PC2 when the UE indicates ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16 for the band, Table 6.2D.2-2 and Table 6.2D.2-3 for PC1.5 with 2Tx, Table 6.2D.2-4, 6.2D.2-5 for PC1.5 with 4 Tx respectively.

In the previous agreement there was no any precision regarding these details, so one can assume that the same principle should apply for 3Tx PC2 MPR (the “whole” 2Tx PC2 MPR requirement should be reused): Table 6.2D.2-1 for 3Tx PC2 UEs when they don’t indicate ULFPTx ‘mode 0’ (there’s no ‘mode 2’ for 3Tx) and Table 6.2.2-2 for 3Tx PC2 UEs when they indicate ULFPTx ‘mode 0’.

Proposal: Entire 2Tx PC2 MPR requirements shall be reused: Table 6.2D.2-1 for 3Tx PC2 when the UE does not indicate ul-FullPwrMode-r16 (‘mode 0’) for the band and Table 6.2.2-2 for 3Tx PC2 when the UE indicates ul-FullPwrMode-r16 (‘mode 0’) for the band.

The corrected part of clause 6.2D.2 could look like follows:

For UE with multiple transmit antenna connectors, up to a maximum of four, in closed-loop spatial multiplexing scheme, the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 6.2D.1-1 is specified in Table 6.2.2-1 for PC3, Table 6.2D.2-1 for 2Tx PC2 and 3Tx PC2 when the UE does not indicate ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16 for the band and Table 6.2.2-2 for 2Tx PC2 and 3Tx PC2 when the UE indicates ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16 for the band, Table 6.2D.2-1 for 3Tx PC2 when the UE indicates ul-FullPwrMode-r16, Table 6.2D.2-2 and Table 6.2D.2-3 for PC1.5 with 2Tx, Table 6.2D.2-4, 6.2D.2-5 for PC1.5 with 4 Tx respectively. For UE power class 1.5 with 2Tx, the allowed maximum power reduction (MPR) defined in Table 6.2D.2-3 is in accordance with the indicated modifiedMPR-Behaviour specified in Table L.1-1 for channel bandwidths ≤ 100 MHz. The requirements shall be met with UL MIMO configurations defined in Table 6.2D.1-2. For UE supporting UL MIMO, the maximum output power is defined as the sum of the maximum output power from both UE antenna connectors.

# Conclusion

In the previous section, we made the following proposal:

Proposal: Entire 2Tx PC2 MPR requirements shall be reused: Table 6.2D.2-1 for 3Tx PC2 when the UE does not indicate ul-FullPwrMode-r16 (‘mode 0’) for the band and Table 6.2.2-2 for 3Tx PC2 when the UE indicates ul-FullPwrMode-r16 (‘mode 0’) for the band.

# References

- RP-242394, “WID revision: NR MIMO Phase 5”, RAN#105.

- R4-2508115, “draft CR for TS 38.101-1 to introduce FR1 3Tx RF requirements”, Huawei, HiSilicon, RAN4#115.

- R4-2505111, “WF on UE RF requirements for MIMO enhancement phase 5”, Samsung, RAN4#114-bis.

- 3GPP TS38.101-1 V19.2.0, “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”.