3GPP TSG-RAN WG4 Meeting #116									                        R4-2511587

Bengaluru, India, August 25 – 29, 2025

Agenda item:			7.6.3.2.1

Source:	Ericsson

Title:	On the UE RF Tx requirements for NR-NTN HPUE

Document for:	Approval

# Introduction

In RAN#106, a revised WID has been approved for “Enhanced requirements and conductive test methodology for NR NTN and IoT NTN” WI [1]. One of the three working areas of the WI is to specify High power UE (HPUE) requirements for NR-NTN with the following objectives:

- Specify high power UE (HPUE) for NR-NTN (Non-Terrestrial Networks) and IoT-NTN (NB-IoT and eMTC based NTN) in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario

  - Support UE with power class 2 (PC2, 26 dBm) that applies to both handheld and non-handheld based on the co-existence study.

  - Support other power classes based on the co-existence and feasibility study

    - Non-hand held UE is prioritized

  - Specify RF requirements for all considered power classes for NR-NTN and IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands

    - Necessary Tx requirements, at least including

      - Investigate and if necessary, specify the solution to address potential SAR (Specific Absorption Rate restriction) requirements for handheld UE

      - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary

    - Necessary Rx requirements, at least including

      - Specify the RSD requirements on the example bands, if necessary

- Specify the necessary signaling to support the above objectives

In this contribution, we discuss the open issue on the configured Tx power equation for NR-NTN HPUE from the approved WF in RAN4#115 [2].

# Discussion

## Configured Tx power equation

One of the open issues from the approved WF is on the configured Tx power equation:

Issue 1-1: Configured Tx power equation

Agreement:

- Agree to clarify and correct the configured transmitted power equation for NTN

  - [∆TC,c], ∆TIB,c , [∆TRxSRS] are not applicable to Rel-19 NR-NTN UE

  - FFS ΔPPowerClass

  - For ΔPPowerBoost, Rel-18 power boosting should be applicable to NR-NTN.

- Regarding how to address these in CR drafting, following approach is agreed.

  - By referring to 38.101-1 and then exclude non-applicable parameters, e.g. ‘except for...’

In TS38.101-1, ∆TC,c is defined as follows:

∆TC,c = 1.5dB when NOTE 3 in Table 6.2.1-1 in 38.101-1 applies for a serving cell c, otherwise ∆TC,c = 0 dB ;

NOTE 3 in Table 6.2.1-1 in 38.101-1 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, where the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. Since such note is not present in Table 5.2.2-1 in TS38.101-5, ∆TC,c could be removed from the configured maximum output power equation.

Observation 1: ∆TC,c could be removed from the configured maximum output power equation.

In TS38.101-1, ∆TIB,c is the additional tolerance for serving cell c as specified in clauses 6.2A.4.2 for NR CA, clause 6.2C.2 for SUL, or TS 38.101-3 clause  6.2B.4.2 for EN-DC; ∆TIB,c = 0 dB otherwise. Since for NR-NTN, neither CA, SUL nor EN-DC is defined so far, ∆TIB,c could be removed from the configured maximum output power equation.

Observation 2: ∆TIB,c could be removed from the configured maximum output power equation.

∆TRxSRS is applied during SRS transmission occasions with usage in SRS-ResourceSet set as ‘antennaSwitching’. SRS-AS is in practice mostly used with TDD bands for the reciprocity-based downlink channel estimation. For that reason, even though SRS-AS is not officially precluded for FDD bands, in our view it could be removed from the configured maximum output power equation.

Observation 3: ∆TRxSRS could be removed from the configured maximum output power equation.

ΔPPowerClass is a key parameter in NR TN configured maximum output power equation to ensure that a UE falls back to a lower power class in case the percentage of uplink symbols transmitted in a certain evaluation period is larger than a certain value, no matter if the UE supports or not optional “maximum uplink duty cycle” capabilities. It is relevant for higher power classes, which are being introduced in this WI for NTN. Another use of ΔPPowerClass is for SRS-AS usage to prevent UEs which indicate PC1.5 or PC2 together with txDiversity-r16 to virtualize the SRS ports by using 2 Tx chains, but if ∆TRxSRS is removed from Pcmax equation as suggested above, that part could be removed since it would be irrelevant.

Depending on the outcome of “SAR related issues” discussion in IoT-NTN Tx requirements part, we will have a clear picture on whether ΔPPowerClass definition could be reused in its entirety, only partially or even removed (not highly likely in our view) from TS38.101-1.

Observation 4: Whether ΔPPowerClass will stay in the configured maximum output power equation will depend on the outcome of “SAR related issues” discussion. The definition from TS38.101-1 could be reused in its entirety, only partially or even removed.

Regarding ΔPPowerBoost, we don’t see an issue for it to be applicable to NR-NTN bands.

As a conclusion we propose the following:

Proposal 1: In the configured maximum output power equation in TS38.101-5, parameters ∆TC,c, ∆TIB,c and ∆TRxSRS could be removed while the status of ΔPPowerClass will depend on the outcome of “SAR related issues” discussion.

## 4Tx support for PC1

For TN, 4Tx support has been specified for non-handheld UEs in Rel-18 for up to PC1.5. The main reason for such a limitation in power class was the default PA configuration assumption of 4 x 23 dBm. The support for PC1 would require the support of 4 x 26 dBm PA configuration, where at least for TN it is assumed that it will not be widely available in the commercial UEs in Rel-19. For that reason, the proposals from the companies for the support of PC1 with 4Tx have been made in the discussion on Rel-20 scope. In our view, it would make the most sense to discuss both TN and NTN 4Tx support for PC1 in Rel-20 as a package.

Proposal 2: Postpone the introduction of 4Tx support for PC1 to Rel-20, which would be aligned with TN as they would be treated as a package.

# Conclusion

Based on the discussion in the previous sections we have made the following observations and a proposal:

Observation 1: ∆TC,c could be removed from the configured maximum output power equation.

Observation 2: ∆TIB,c could be removed from the configured maximum output power equation.

Observation 3: ∆TRxSRS could be removed from the configured maximum output power equation.

Observation 4: Whether ΔPPowerClass will stay in the configured maximum output power equation will depend on the outcome of “SAR related issues” discussion. The definition from TS38.101-1 could be reused in its entirety, only partially or even removed.

Proposal 1: In the configured maximum output power equation in TS38.101-5, parameters ∆TC,c, ∆TIB,c and ∆TRxSRS could be removed while the status of ΔPPowerClass will depend on the outcome of “SAR related issues” discussion.

Proposal 2: Postpone the introduction of 4Tx support for PC1 to Rel-20, which would be aligned with TN as they would be treated as a package.

# References

- RP-242901, “Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN”, RAN#106.

- R4-2508008, “WF on [115][128] NR_IoT_NTN_HPUE_part1”, Samsung, RAN4#115.