3GPP TSG-RAN WG4 Meeting #116									                        R4-2511589

Bengaluru, India, August 25 – 29, 2025

Agenda item:			7.6.3.3.1

Source:	Ericsson, Nokia

Title:	On the UE RF Tx requirements for IoT-NTN HPUE

Document for:	Approval

# Introduction

In RAN#106, a revised WID has been approved for “Enhanced requirements and conductive test methodology for NR NTN and IoT NTN” WI [1]. One of the three working areas of the WI is to specify High power UE (HPUE) requirements for IoT-NTN with the following objectives:

- Specify high power UE (HPUE) for NR-NTN (Non-Terrestrial Networks) and IoT-NTN (NB-IoT and eMTC based NTN) in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario

  - Support UE with power class 2 (PC2, 26 dBm) that applies to both handheld and non-handheld based on the co-existence study.

  - Support other power classes based on the co-existence and feasibility study

    - Non-hand held UE is prioritized

  - Specify RF requirements for all considered power classes for NR-NTN and IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands

    - Necessary Tx requirements, at least including

      - Investigate and if necessary, specify the solution to address potential SAR (Specific Absorption Rate restriction) requirements for handheld UE

      - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary

    - Necessary Rx requirements, at least including

      - Specify the RSD requirements on the example bands, if necessary

- Specify the necessary signaling to support the above objectives

In this contribution, we discuss the issue on relaxing the upper limitation of nominal MOP or MOP tolerance for NTN HD-FDD operation for IoT-NTN HPUE, captured in the approved WF in RAN4#115 [2].

# Discussion

In the approved WF [2], the following issue has remained open:

Issue 1-1-8: On relaxing the upper limitation of nominal MOP or MOP tolerance for NTN HD-FDD operation.

- Background (Last meeting’s WF):

  - Restrict the boosting condition to PC2 NB-Iot based Iot-NTN

  - To down-select in next meeting between:

    - Option 1: Increase [1] dB MOP upper tolerance

    - Option 2: Increase [1] dB MOP nominal power (optional with capability signalling)

- WF(NEW):

  - Restrict the boosting condition to PC2 NB-Iot based Iot-NTN

  - Increase [0.8] dB MOP or add a power boosting item in Pcmax equation (optional with capability signalling)

In the previous meetings it was recognized by one company that HD-FDD FE insertion loss (IL) is lower compared to FD-FDD IL and that the maximum output power (MOP) could be increased by 0.8 dB for such UEs.

After some discussions it was agreed to restrict this potential boosting condition only to PC2 NB-IoT based IoT-NTN. According to the last WF [2], there are two options on the table. The first one is to increase the MOP by [0.8] dB, either by changing the MOP nominal power (optional with capability signalling) or by changing the MOP upper tolerance. The second option is to introduce a power boosting item into the configured maximum output power equation (optional with capability signalling).

To our knowledge, allowing the modification of the MOP nominal power definition by a note would be unprecedented and thus shall be avoided. Nominal power definitions of different power classes are well defined and established and shall not be changed.

Observation 1: Allowing the modification of the MOP nominal power definition by a note would be unprecedented and thus shall be avoided.

Modification of MOP tolerance by a note is a common practice in NR and LTE specifications. However, the purpose of MOP tolerance is to allow for inaccuracies in the power setting due to practical limitations of the UE RF FE implementation. For example, in both LTE and NR specifications, the most typical case of a MOP tolerance modification is allowing the reduction of the lower tolerance limit by 1.5 dB for certain operating bands for the cases where the transmission bandwidths are confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. The motivation for that is that it is challenging to accurately set the output power in such a confined bandwidth under the risk of out-of-band emission towards the adjacent operating band.

Hence, allowing the modification of the MOP upper tolerance should not be used as a tool to increase the maximum output power. For the UEs which cannot even achieve 0.8 dB insertion loss improvement such requirement could be even understood as a relaxed MOP upper tolerance limit and allow for implementation of a less accurate RF FE.

Observation 2: MOP tolerances are used to allow for the inaccuracies in the output power setting and should not be used as a tool to allow for the increased maximum output power.

Finally, introducing a power boosting parameter into the configured maximum output power inequalities is also a common practice in NR at least. Such a parameter could have a precise definition and purpose and doesn’t have to be mixed (integrated into) with other parameters which have a different purpose. It would also allow for better specification readability as it is more visible, allows greater freedom in its description as it is not confined to a note, and makes future modifications and extensions easier.

Proposal: To allow the PC2 NB-IoT based IoT-NTN UE to benefit from 0.8 dB improved insertion loss, add a power boosting item in the configured maximum output power (Pcmax) equation (optional with capability signalling).

# Conclusion

Based on the discussion in the previous sections we have made the following observations:

Observation 1: Allowing the modification of the MOP nominal power definition by a note would be unprecedented and thus shall be avoided.

Observation 2: MOP tolerances are used to allow for the inaccuracies in the output power setting and should not be used as a tool to allow for the increased maximum output power.

Based on the discussion in the previous sections we have made the following proposal:

Proposal: To allow the PC2 NB-IoT based IoT-NTN UE to benefit from 0.8 dB improved insertion loss, add a power boosting item in the configured maximum output power (Pcmax) equation (optional with capability signalling).

# References

- RP-242901, “Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN”, RAN#106.

- R4-2508122, “WF on IoT-NTN HPUE”, vivo, RAN4#115.