3GPP TSG-RAN WG4 Meeting #116									                        R4-2511590

Bengaluru, India, August 25 – 29, 2025

Agenda item:			7.6.3.3.2

Source:	Ericsson

Title:	On the UE RF Rx requirements for IoT-NTN HPUE

Document for:	Approval

# Introduction

In RAN#106, a revised WID has been approved for “Enhanced requirements and conductive test methodology for NR NTN and IoT NTN” WI [1]. One of the three working areas of the WI is to specify High power UE (HPUE) requirements for IoT-NTN with the following objectives:

- Specify high power UE (HPUE) for NR-NTN (Non-Terrestrial Networks) and IoT-NTN (NB-IoT and eMTC based NTN) in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario

  - Support UE with power class 2 (PC2, 26 dBm) that applies to both handheld and non-handheld based on the co-existence study.

  - Support other power classes based on the co-existence and feasibility study

    - Non-hand held UE is prioritized

  - Specify RF requirements for all considered power classes for NR-NTN and IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands

    - Necessary Tx requirements, at least including

      - Investigate and if necessary, specify the solution to address potential SAR (Specific Absorption Rate restriction) requirements for handheld UE

      - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary

    - Necessary Rx requirements, at least including

      - Specify the RSD requirements on the example bands, if necessary

- Specify the necessary signaling to support the above objectives

In this contribution, we discuss the only open issue on the UE RF Rx requirements for IoT-NTN HPUE from the approved WF in RAN4#115 [2].

# Discussion

In the approved WF [2], the only open issue is the following:

Issue 3-1-2: PC1 RSD (Non-handheld only, full duplex eMTC based)

- Ad-hoc agreement:

  - Consider [0] dB RSD for PC1 non-handheld devices for the current NTN bands with the following.

    - To document 66~70 dB filter rejection would be needed for the non-handheld 0dB RSD in the TR.

    - Revisit this value with more inputs in next meeting.

Since full duplex eMTC based IoT-NTN is considered only for non-handheld UEs with 1Tx, in our view 0dB RSD agreement should be kept for PC1 (for all bands). That would be in line with TN PC1, where also 1Tx architecture was assumed and non-handheld UEs only - where no RSD was specified.

Proposal: Confirm 0 dB RSD for PC1 for full duplex eMTC based non-handheld UEs for IoT-NTN.

# Conclusion

Based on the discussion in the previous sections we have the following proposal:

Proposal: Confirm 0 dB RSD for PC1 for full duplex eMTC based non-handheld UEs for IoT-NTN.

# References

- RP-242901, “Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN”, RAN#106.

- R4-2508122, “WF on IoT-NTN HPUE”, vivo, RAN4#115.