# 3GPP TSG RAN WG4 Meeting#116	R4-2511592

# Bengaluru, India, Aug 25 – Aug 29, 2025

Source: 	Qualcomm Incorporated

Title: 	Discussions on channel modeling for NTN testing for NGSO

Agenda item:	7.6.5.1

Document for:	Discussion

# 1. Introduction

During the last meeting, RAN4 discussed issues related to the channel modeling for NTN testing for NGSO [1]. Most of the open issue were resolved. In this paper, we share our perspectives on the remaining open issues.

2. TE Emulated Channel Model

Propagation delay model with considering earth’s rotation during signal flight time

- agreement

  - Downlink propagation delay model at UE reception time tR with considering the Earth's rotation during signal flight time

DtRUE=ρtRECEF/c1+1cvtRECI⋅ρtRECEFρtRECEF

  - FFS whether and how to consider the propagation delay model for UL delay error for RRM uplink timing error cases

    - Option 1:

ΔtUL=RULtTc1-1cVSVtT⋅RULtTRULtT#Where

SV (Satellite)		: Receiver

UE 				: Transmitter

∆tUL= tR- tT  : Uplink total propagation delay

tT					: Time when signals are transmitted from UE

tR					: Time when signals are received by satellite

rR 					: SV position vector on ECI Frame at tT.

rT 					: UE position vector on ECI Frame at tT.

VSV				: Satellite velocity vector on ECI Frame

RUL=rR-rT	: Distance between satellite and UE at either tT or tR depending on the reference point.

c = 2.99792458 x 108 [m/s]

    - Other options are not precluded

2.1 Consideration of Earth rotation during signal flight time

During the previous RAN4 meeting, it was agreed to incorporate the effect of Earth’s rotation during signal flight time when calculating downlink propagation delay. Additionally, a proposal was introduced to apply the same consideration to the uplink delay error for use in RRM UL timing error test cases. To this end, our view is that it is unnecessary to factor in Earth’s rotation for the uplink channel. The propagation delay error introduced by Earth’s rotation during downlink signal flight time will inherently affect the uplink transmission. As such, this error will already be reflected in the RRM UL timing test cases.

Observation 1: The propagation delay error caused by Earth’s rotation during downlink signal flight time inherently impacts uplink transmission. As a result, this error will already be reflected in the RRM uplink timing test cases.

Proposal 1: Do not separately account for Earth’s rotation during signal flight time to the uplink delay error for use in RRM UL timing error test cases.

3. Conclusions

In this paper, we present our perspectives on the remaining open issues related to the channel modeling for NTN testing for NGSO. The following proposals have been made.

Observation 1: The propagation delay error caused by Earth’s rotation during downlink signal flight time inherently impacts uplink transmission. As a result, this error will already be reflected in the RRM uplink timing test cases.

Proposal 1: Do not separately account for Earth’s rotation during signal flight time to the uplink delay error for use in RRM UL timing error test cases.

4. References

[1] R4-2508727, “Way Forward for [115][324] NTN_testing_NGSO_channel_model_and_demod,” 3GPP TSG RAN WG4 Meeting #115, St. Julian’s, Malta, May 19 – 23, 2025.