# 3GPP TSG RAN WG4 Meeting#116	R4-2511593

# Bengaluru, India, Aug 25 – Aug 29, 2025

Source: 	Qualcomm Incorporated

Title: 	Discussions on demodulation performance requirements for NTN testing for NGSO

Agenda item:	7.6.5.3

Document for:	Discussion

# 1. Introduction

RAN4 is approaching the completion of channel modeling for NTN testing in NGSO scenarios and initiated the discussions on demodulation performance requirements. Several companies have proposed approaches to study the demodulation performance requirements, taking into account time-varying Doppler shifts and propagation delays [1]. This paper outlines our perspectives on how RAN4 should approach the demodulation requirements for this work item.

2. Demodulation Requirements

2.1 Impact on RAN4 demodulation requirements

Impact on RAN4 demodulation requirements with TE-emulated time vary doppler shift and propagation delay models

- agreement

  - Launch simulation campaigns to assess the impact on demodulation requirements from time-varying propagation delay and Doppler shift. Considering the residual frequency error and timing error to check whether existing NR NTN and IoT-NTN UE demodulation requirements can apply.

  - Based on simulation result, further discussion whether additional margin to consider the estimation/pre-compensation errors with the TE-emulated varying Doppler shift and propagation delay models

    - residual frequency offset and time delay

      - option 1: Fix value

        - option 1a: frequency error and timing error as 0.1 ppm = 200Hz@2GHz and 29*64*Tc = 9.44e-7s

        - option 1b: frequency error and timing error as 2Hz/s and 0.05us/s

      - option 2: time-varying propagation delay and Doppler shift.

        - FFS

  - Companies are encouraged to provide the simulation results during RAN4#116 meeting

Several proposals were submitted during RAN4#115 to assess the impact on the demodulation perforamcne requirements. The way forward is outlined below.

In our simulation study, we evaluated Option 1a to account for frequency and timing errors. The results indicate that Option 1a leads to approximately 0.5 dB degradation in demodulation performance for Test 1-1 and Test 1-2, compared to the requirements defined in Rel-17. Therefore, if Option 1a is adopted, it is important to consider incorporating an additional margin into the existing demodulation performance requirements to compensate for the impact of these errors.

Observation 1: Considering the frequency and timing errors in Option 1a, a degradation of approximately 0.5 dB is observed in comparison to the existing Rel-17 demodulation performance requirements for Test 1-1 and Test 1-2.

Proposal 1: Introduce an additional margin of at least 0.5 dB to the Rel-17 demodulation performance requirements when considering Option 1a (if agreed).

2.2 Sampling frequency offset

Doppler frequency and its rate of change (i.e. Doppler drift) are directly influenced by satellite motion, which in turn affects the sampling frequency offset observed at the receiver. Considering this relationship in the channel model ensures realistic test conditions for evaluating receiver performance.

Proposal 2: In the test, the channel model should reflect the impact of sampling frequency offset in relation to both the Doppler frequency and its rate of change (i.e., Doppler drift) caused by satellite motion.

2.3 Test applicability

We note that the satellite motion-based time-varying channel model to pre-Rel-19 test cases offers a practical and consistent approach to evaluating device performance under realistic conditions. Since the model reflects actual satellite propagation, its retroactive use enhances the relevance and accuracy of legacy test scenarios without requiring changes to the test framework. We also think that this can be based on DUT declaration, therefore, flexibility is maintained for vendors while ensuring that the test conditions align with actual deployment scenarios.

Proposal 3: The satellite motion-based time varying channel model can be retroactively applied to pre-Rel-19 test cases subject to DUT declaration.

3. Conclusions

In this paper, we present our perspectives on the performance requirements for NTN testing for UEs supporting NGSO scenarios. The following proposals have been made.

Observation 1: Considering the frequency and timing errors in Option 1a, a degradation of approximately 0.5 dB is observed in comparison to the existing Rel-17 demodulation performance requirements for Test 1-1 and Test 1-2.

Proposal 1: Introduce an additional margin of at least 0.5 dB to the Rel-17 demodulation performance requirements when considering Option 1a (if agreed).

Proposal 2: In the test, the channel model should reflect the impact of sampling frequency offset in relation to both the Doppler frequency and its rate of change (i.e., Doppler drift) caused by satellite motion.

Proposal 3: The satellite motion-based time varying channel model can be retroactively applied to pre-Rel-19 test cases subject to DUT declaration.

4. References

[1] R4-2508727, “Way Forward for [115][324] NTN_testing_NGSO_channel_model_and_demod,” 3GPP TSG RAN WG4 Meeting #115, St. Julian’s, Malta, May 19 – 23, 2025.