3GPP TSG RAN WG4 Meeting #116		R4-2511598

Bangalore, India, 25th – 29th August 2025

Source: 		Rohde & Schwarz

Title: 			Discussion on testability aspects for AI/ML based mobility

Agenda Item: 	7.19.4

Document for: 	Discussion

# Introduction

From the WF in [1], the following analysis is required with regards to testability aspects for AI/ML based mobility in NR:

### Issue 1-1-1: Prediction consistency in time domain

<Way forward >:

- Consider the propagation condition for the validation of the RRM measurement prediction requirements before modeling different time-varying characteristics per cell/site in FR1.

- Further discuss the following options on how to model different time-varying characteristics per cell/site in FR1.

  - Option 1: Change Tx power.

    - FFS: whether it is possible to change Tx power every symbol at TE using conductive test

  - Option 2: Change channel condition with fixed Tx power.

  - Option 3: Computer simulations alike test setup can be adopted. TE can transmit the signals of cell 1, 2, 3 with time-variance following a certain channel model.

  - Option 4: Select one reference UE trajectory model (e.g., from TR 38.843) to study prediction consistency impact

### Issue 1-2-1: Testing Setup in FR2 (same applies for FR1)

<Way forward >:

- For the testing setup for AI/ML mobility in FR1 and FR2, discuss the following configurations in the next meeting:

  - Number of SSBs per cell

  - Channel model

    - FFS the same or different for temporal/frequency domain prediction

  - Impact of UE trajectory

    - FFS the same or different for temporal/frequency domain prediction

  - Dynamic range of received power (consistency of channel)

  - Measurement prediction specific parameters

  - Emulation of noise and EPRE

  - L3 filter coefficient

In addition, following is captured in the TP [2] for the TR [3] with regards to testability aspects for AI/ML based mobility in NR:

6.2.3.2 Prediction consistency in time domain

To ensure prediction consistency in the time domain, it is considered how to model the different time-varying characteristics per cell/site due to moving UE trajectories. It is also considered how to incorporate controlled randomness and the extent of time-domain variation and correlation.

FR1 is prioritized during the discussion.

In this contribution, we start presenting our TE view on the topics above, especially on the yellow highlighted ones.

# Discussion

## Fast-fading (“legacy”-fading)

In our understanding, the main idea of the AIML based temporal prediction of RRM measurements for the purpose of mobility, is that the UE predicts the measurements based on repeated UE location, which is on the other side deduced by repeated cell + channel (“RF footprint”) conditions. These conditions change according to the trajectory as the UE moves across the cells. Let us focus in the following on the channel part.

Typical RRM measurements are:

- Reference Signal Received Power (RSRP)

- Reference Signal Received Quality (RSRQ)

- Signal-to-Interference and -Noise Ratio (S(I)NR)

As mentioned above, prediction of such measurements only makes sense if one assumes repeated conditions i.e. in our focus here, repeated channel conditions. The latter cannot hold true for small-scale fading, since it refers to the rapid fluctuations in signal strength that occur over short distances or short time intervals. Therefore, such fading is also called fast-fading, and it is strongly affected from the immediate environment, e.g. moving obstacles like pedestrians or vehicles. This type of channel fading is not expected to repeat if the UE repeats its trajectory and is thus not sufficient for AIML based mobility purposes.

Observation 1: Fast-fading is mostly caused due to immediate environment e.g. moving obstacles, thus does normally not repeat when UE is repeating its trajectory, and therefore is not sufficient for AIML based mobility testing.

Observation 2: The overall schematic description and implementation of fast-fading for a single fading channel with N=3 paths is shown in Figure 1, where x(t) = cell signal and h(t) = fast-fading function.

Figure 1 – Schematic description of fast-fading (symbolic obstacles)

Fast-fading can be modelled with both TDL or CDL.

- In case of TDL fading, hn(t) is a zero mean, complex Gaussian random process, with variance  and autocorrelation function autocorrelation function  , where denotes the zeroth-order Bessel function of the first kind. This autocorrelation function leads to the well-known Jakes power spectral density in the frequency domain. This fading process is also called Rayleigh fading (compare [5]).

- In case of CDL fading, hn(t) is the sum of 20 sinusoids. Their amplitudes, phases and frequencies as well as the autocorrelation function  are a complex function of numerous geometry parameters (compare [5]).

Fast-fading is represented by the legacy fading profiles and used so far for conformance requirements.

## Slow-fading (lognormal-fading)

On the other hand, there exists large-scale fading, sometimes also called slow-fading or shadow-fading, which refers to the variations in signal strength over large distances and is mainly affected from the static environment like buildings, trees or hills. Those signal variations, which are mainly due to the static environment, can be expected to repeat if the UE repeats its trajectory. Therefore, this type of channel fading becomes of interest for AIML based mobility.

Observation 3: Slow-fading is caused due to static environment e.g. static obstacles, thus does normally repeat when UE is repeating its trajectory, and therefore of interest and suitable for AIML based mobility testing.

A well accepted model for slow-fading is the so-called lognormal-fading. It is a statistical model which assumes that the signal strength follows a lognormal distribution. The simulation of a UE trajectory by repeated lognormal fading has several advantages:

- a well-accepted statistical model exists

- repetition can easily be achieved by restarting the lognormal fading process with the same seed

- easy control over fading process characteristics like average power, bandwidth and seed

- can easily be combined with fast fading

Observation 4: Lognormal-fading is a well-accepted statistical slow-fading model, assuming that the signal strength follows a lognormal distribution. It allows easy control over fading process characteristic like average power, bandwidth and seed, it can easily be repeated by restarting the lognormal fading process with the same seed, as well as it can easily be combined with fast-fading.

The effect of slow-fading is generally modelled through a lognormal process a(t), which can be expressed as:

- ν(t) is a real-valued zero-mean Gaussian random process with unit variance.

- σL is called shadow standard deviation. Typical values of σL have been found in measurement campaigns.

- mL is called area mean and is obtained by averaging the received signal strength over an area which is large enough to average over the shadowing effects. Value of mL is determined by the path loss of the link between the base station and the UE.

The distribution of a(t) follows the lognormal distribution:

The autocorrelation properties of ν(t) have been investigated by Gudmundson in [6]. Supported by his empirical studies, Gudmundson has proposed to model the autocorrelation properties of ν(t) by the negative exponential autocorrelation function rνν(τ):

- Dc>0 is called the decorrelation distance. It is an area-specific real-valued constant.

Typical values of Dc, σL and mL which have been found in measurement campaigns (compare Pätzold, M in [7]) are summarized in Table 1:

Table 1 – Typical values for Dc, σL and mL

| Area type | Dc [m] | σL [dB] | mL [dB] |
| --- | --- | --- | --- |
| Suburban | 503,9 | 7,5 | 0 |
| Urban | 8,3058 | 4,3 | 0 |

Additional considerations:

- Since the path loss is not considered in link level simulations, it is commonly assumed that mL is zero.

- The ratio between v and Dc is commonly referred to as the lognormal frequency fl i.e.:

- Using the fl, one can rewrite the autocorrelation function rνν(τ) as:

- Thus, the lognormal frequency fl, defines how fast the lognormal process changes over time.

- It can thus be seen as the slow-fading equivalent to the Doppler frequency fD, which defines how fast the fast-fading process changes over time, so in general we have fD >> fl .

Observation 5: The effect of lognormal-fading on the signal strength is modeled by the equation  , where ν(t) is a real-valued zero-mean Gaussian random process with autocorrelation properties described by the function  .  σL is the shadow standard deviation allowing the control of the power dynamic range.  fl is the lognormal frequency, allowing to control how fast the slow-fading process changes over time (similarly to Doppler-frequency fD for fast-fading, where in general fD >> fl)

## Combined of fast- and slow-fading (Suzuki-fading)

Fast- and slow-fading processes can be combined for a given fading-channel, counting for both respective effects. In this case the channel consists of two parts, the fast-fading part, as well as the slow-fading part, which describes the condition variation as the UE moves along a trajectory. The resulting fading process is also called Suzuki-fading. For each path/cluster n an independent Lognormal-fading process an(t) can be simulated.

Observation 6: Combination of fast-fading (e.g. Rayleigh) and slow-fading (Lognormal) is called Suzuki-fading.

Observation 7: The overall schematic description and implementation of Suzuki-fading for a single fading channel with N=3 paths is shown in Figure 2, where x(t) = cell signal, h(t) = fast-fading function and a(t) = slow-fading function.

Figure 2 – Schematic description of combined fast- and slow-fading (symbolic obstacles)

For a Ntx x Nrx MIMO system, C fading channels have to be simulated, where C = Ntx* Nrx. Here it is important that for given path n, the same Lognormal-fading process an(t) for all C fading channels is applied, i.e. an,c(t) = an(t) for all 1 ≤ c ≤ C. Otherwise, the fast-fading correlation between the channels will change over time.

Observation 8: For a MIMO system with C fading channels and N paths, a Suzuki-fading consist in the combination of C separate N-paths fast-fading processes (with a given correlation), with N Lognormal-fading process (same for all C fading channels).

The parameters which define the lognormal-fading process are σL, Dc and v, i.e. shadow standard deviation, decorrelation distance and speed of the UE respectively.

Since our main interest is to control how fast the lognormal-fading process changes in time as compared to the fast- fading process, we propose to define the lognormal frequency fl as configuration parameter, instead of Dc and v.

This would be consistent with the treatment of the fast-fading in 3GPP conformance test specifications, where for LTE and NR the Doppler frequency fD was chosen as a configuration parameter, rather than UE speed and carrier frequency.

Observation 9: Meaningful and feasible parameter ranges for Suzuki-fading are given in Table 2.

Table 2 – Meaningful and feasible parameter ranges for Suzuki-fading

| Parameter | Unit | Min | Max |
| --- | --- | --- | --- |
| σL | dB | 0 | 12 |
| fl | Hz | 0 | fD/32 |

A repeated UE trajectory, i.e. repeated channel conditions can be simulated by repeating the lognormal-fading process. The fast-fading can however be simulated independently for each repetition.

Observation 10: Signal variation and repetition due to repeated UE movement along a trajectory can be fully emulated by Suzuki-fading, where the Lognormal-fading part is restarted with the same seed for each repetition, while the fast-fading part runs continuously across repetitions.

Since the Suzuki-fading fully addresses the signal variation caused by the UE trajectory, there is at this stage not much expected need, to further investigate additional cell power manipulation.

Observation 11: When applying the Suzuki-fading, there is no need for further cell power manipulation for the sake of UE moving repeatedly along a trajectory.

This emphasizes a clear advantage of the usage of Suzuki-fading, as below:

Observation 12: Modeling UE movement by Suzuki-fading has the advantage that cell power and legacy fast-fading profiles remain unchanged and can be re-used. Solely additional slow-fading profile is applied on the top.

In addition, it has to be noted that:

Observation 13: Doppler effects caused due to the relative movement between UE and BTS are covered by the legacy fast-fading profiles. Also similarly as in legacy tests, resulting delay variation is neglected. This is different from the NTN scenarios, where due to the high movement speed of NGSO satellites the doppler and delay variation might need to be modeled separately.

In order do better illustrate the effects of the Suzuki-fading on the signal, we have conducted some initial simulation.

Observation 14: Figure 3 shows the simulation of a single Suzuki-fading path with one repetition of the Lognormal-fading sequence, but a contiguous Rayleigh-fading sequence with parameters: 
fD = 70 Hz, fl = 70/32 Hz, σL = 8.69 dB

Figure 3 – Simulation of a single Suzuki-fading path with one repetition

In Figure 3 can be seen that the average power variation mainly follows the lognormal slow-fading profile (red line), while the Rayleigh fast-fading fluctuates the signal with higher frequency around the slow-fading average value (blue line). After the restart (green line) the slow-fading signal repeats identically, but the overall combined-signal (blue line) repeats only at a large-scale since the fast-fading profile continues unchanged and does not repeat. This corresponds to real scenarios when a user repeats the same trajectory, where the static obstacles (buildings, trees, etc.) are the same, but moving obstacles (cars, pedestrians, etc.) have probably changed.

Observation 15: A Suzuki-faded signal, follows and repeats at large scale (w.r.t. average power) when restarting the lognormal slow-fading for a trajectory repetition, but does not remain identical due to the fast-fading running continuously across the repetitions. This corresponds to real life scenarios, where big static obstacles characterizing a trajectory do not change, but moving random obstacles do.

Another question raised in the last meeting was the power dynamic range which can be supported by the TE. As also seen in Figure 3 the dominant component of the power dynamic range is coming from the fast-fading process, already supported in the legacy tests. Adding on the top the slow-power fading, increases additionally the power dynamic range, however at a much smaller scale. As already presented Table 1, an additional dynamic range corresponding to σL = 12 dB is feasible for the TE.

Observation 16: The dominant part of the power dynamic range of a Suzuki-fading channel results from the fast-fading profiles, which are already supported in the legacy testing. Additional power dynamic range added by the Lognormal slow-fading corresponding to σL = 12 dB should also be feasible from TE perspective.

Another question to TE was about the power change rate. The value discussed was the Tx power change per symbol, which would be very challenging in case the change is done at cell power level. However, with the proposal to handle the scenario as part of channel, the power change rate becomes very flexible, up to “per sample” rate.

Observation 17: Addressing UE movement as part of the channel allows to maximize the power change rate, up to “per sample”, which is far beyond the “per symbol” rate.

Finally, depending on the implementation of the power variation, there was the uncertainty about the duration of such a power variation model. Again, handling the scenario as part of channel, expands the possibilities considerably, since the lognormal/slow-fading model has practically no duration limitation (i.e. until it naturally repeats), when comparing with the conformance test duration.

Observation 18: The lognormal/slow-fading model has practically no duration limitation (i.e. until it naturally repeats), when comparing with conformance test duration. Repetition is triggered by the test case only on purpose, in order to repeat the UE movement trajectory.

Considering all the technical capabilities, suitability and advantages of the Lognormal slow-fading and the resulting Suzuki-fading, we make the following proposals:

Proposal 1: Model the time variation of the DL signal as the UE moves repeatedly along a trajectory applying Suzuki-fading. This means Lognormal slow-fading is applied on the top of the legacy configuration with fast-fading per cell.

Proposal 2: Define Lognormal slow-fading profiles (key parameters σL   fl) to count for the intended scenarios and performance for AIML based mobility.

Proposal 3: Emulate repetition of the UE trajectory for AIML training / ground truth / inference purposes, by solely restarting the Lognormal slow-fading process with the same seed, while the fast-fading process continues independently unchanged.

# Conclusions

In this contribution, the following observations and proposals are made:

Observation 1: Fast-fading is mostly caused due to immediate environment e.g. moving obstacles, thus does normally not repeat when UE is repeating its trajectory, and therefore is not sufficient for AIML based mobility testing.

Observation 2: The overall schematic description and implementation of fast-fading for a single fading channel with N=3 paths is shown in Figure 1, where x(t) = cell signal and h(t) = fast-fading function.

Figure 1 – Schematic description of fast-fading (symbolic obstacles)

Observation 3: Slow-fading is caused due to static environment e.g. static obstacles, thus does normally repeat when UE is repeating its trajectory, and therefore of interest and suitable for AIML based mobility testing.

Observation 4: Lognormal-fading is a well-accepted statistical slow-fading model, assuming that the signal strength follows a lognormal distribution. It allows easy control over fading process characteristic like average power, bandwidth and seed, it can easily be repeated by restarting the lognormal fading process with the same seed, as well as it can easily be combined with fast-fading.

Observation 5: The effect of lognormal-fading on the signal strength is modeled by the equation  , where ν(t) is a real-valued zero-mean Gaussian random process with autocorrelation properties described by the function  .  σL is the shadow standard deviation allowing the control of the power dynamic range.  fl is the lognormal frequency, allowing to control how fast the slow-fading process changes over time (similarly to Doppler-frequency fD for fast-fading, where in general fD >> fl)

Observation 6: Combination of fast-fading (e.g. Rayleigh) and slow-fading (Lognormal) is called Suzuki-fading.

Observation 7: The overall schematic description and implementation of Suzuki-fading for a single fading channel with N=3 paths is shown in Figure 2, where x(t) = cell signal, h(t) = fast-fading function and a(t) = slow-fading function.

Figure 2 – Schematic description of combined fast- and slow-fading (symbolic obstacles)

Observation 8: For a MIMO system with C fading channels and N paths, a Suzuki-fading consist in the combination of C separate N-paths fast-fading processes (with a given correlation), with N Lognormal-fading process (same for all C fading channels).

Observation 9: Meaningful and feasible parameter ranges for Suzuki-fading are given in Table 2.

Table 2 – Meaningful and feasible parameter ranges for Suzuki-fading

| Parameter | Unit | Min | Max |
| --- | --- | --- | --- |
| σL | dB | 0 | 12 |
| fl | Hz | 0 | fD/32 |

Observation 10: Signal variation and repetition due to repeated UE movement along a trajectory can be fully emulated by Suzuki-fading, where the Lognormal-fading part is restarted with the same seed for each repetition, while the fast-fading part runs continuously across repetitions.

Observation 11: When applying the Suzuki-fading, there is no need for further cell power manipulation for the sake of UE moving repeatedly along a trajectory.

Observation 12: Modeling UE movement by Suzuki-fading has the advantage that cell power and legacy fast-fading profiles remain unchanged and can be re-used. Solely additional slow-fading profile is applied on the top.

Observation 13: Doppler effects caused due to the relative movement between UE and BTS are covered by the legacy fast-fading profiles. Also similarly as in legacy tests, resulting delay variation is neglected. This is different from the NTN scenarios, where due to the high movement speed of NGSO satellites the doppler and delay variation might need to be modeled separately.

Observation 14: Figure 3 shows the simulation of a single Suzuki-fading path with one repetition of the Lognormal-fading sequence, but a contiguous Rayleigh-fading sequence with parameters:  fD = 70 Hz, fl = 70/32 Hz, σL = 8.69 dB

Figure 3 – Simulation of a single Suzuki-fading path with one repetition

Observation 15: A Suzuki-faded signal, follows and repeats at large scale (w.r.t. average power) when restarting the lognormal slow-fading for a trajectory repetition, but does not remain identical due to the fast-fading running continuously across the repetitions. This corresponds to real life scenarios, where big static obstacles characterizing a trajectory do not change, but moving random obstacles do.

Observation 16: The dominant part of the power dynamic range of a Suzuki-fading channel results from the fast-fading profiles, which are already supported in the legacy testing. Additional power dynamic range added by the Lognormal slow-fading corresponding to σL = 12 dB should also be feasible from TE perspective.

Observation 17: Addressing UE movement as part of the channel allows to maximize the power change rate, up to “per sample”, which is far beyond the “per symbol” rate.

Observation 18: The lognormal/slow-fading model has practically no duration limitation (i.e. until it naturally repeats), when comparing with conformance test duration. Repetition is triggered by the test case only on purpose, in order to repeat the UE movement trajectory.

Proposal 1: Model the time variation of the DL signal as the UE moves repeatedly along a trajectory applying Suzuki-fading. This means Lognormal slow-fading is applied on the top of the legacy configuration with fast-fading per cell.

Proposal 2: Define Lognormal slow-fading profiles (key parameters σL   fl) to count for the intended scenarios and performance for AIML based mobility.

Proposal 3: Emulate repetition of the UE trajectory for AIML training / ground truth / inference purposes, by solely restarting the Lognormal slow-fading process with the same seed, while the fast-fading process continues independently unchanged.

# References

- R4-2508269, “WF on FS_NR_AIML_Mob_Part2”, OPPO, 3GPP TSG RAN WG4 Meeting #115, May 2025

- R4-2508419, “TP on RAN4 aspects for TR 38.744”, OPPO, 3GPP TSG RAN WG4 Meeting #115, May 2025

- 3GPP TR 38.744 V1.0.0, “Technical Specification Group Radio Access Network; Study on Artificial Intelligence (AI)/Machine Learning (ML) for Mobility in NR (Release 19)”

- 3GPP TR 38.843 V18.0.0, “Technical Specification Group Radio Access Network; Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18)”

- 3GPP TR 38.901 V18.0.0, “Study on channel model for frequencies from 0.5 to 100 GHz”

- Gudmundson, M.: “Correlation Model for Shadow Fading in Mobile Radio Systems. Electronics Letters”, vol. 27 (1991), p. 2145-2146.

- Pätzold, M.: “Mobile Radio Channels”, 2nd Edition, Wiley, 2012