3GPP TSG-RAN WG4 Meeting #116                                          R4-2511599

# Bengaluru, India, 25th – 29th of August 2025

Title: 	Suggestions for improvement of VSAT UE RF requirements in Ku-band

Agenda Item:	7.7.5 – UE RF requirements

Release:	Rel-19

Source: 	THALES

Document for:	Discussion

# Information

The intention of this document is to provide some information and some proposals for the computation of essential requirements for VSAT UE RF in Ku-band (under NR_NTN_Ku_bands WID).

# Discussion

Observation 1: VSAT EIRPmin should be based on the following equation (or similar):

EIRPmin=-174dBm/Hz+10log10BW+NF+SNR+IM-SANAntennaGain+PathlossFreeSpace+AtmosphereLoss

Where:

- NF is the GEO/LEO Noise Figure in Ku-band (assumption with same noise figure 3dB for all orbits)

- BandWidth

  - can be assumed as 100 MHz, since 100 MHz is defined for both FR1-NTN and FR2-NTN

  - could be differentiated between FR1-NTN (e.g. 50 MHz) and FR2-NTN (e.g. 200 MHz)

- SNR is assumed as -1dB which is applicable for QPSK with 1/3 coding rate;

- IM is the implementation margin for low modulation order, 2dB could be considered;

- SANAntennaGain is for instance:

  - 44-[3/0]dB; where 44dBi is the GEO Rx antenna gain in Ku-band

  - 38.5-[3/0]dB; where 38.5dBi is the LEO Rx antenna gain in Ku-band

    - 3dB margin is considered for VSAT is located at the beamprint edge which is corresponding to 3dB beamwidth of radiation beam.

    - If VSAT is located at the center of beamprint, 0dB margin could be considered;

- An AtmosphereLoss of 2dB could be considered

- PathlossFreeSpace=32.44+20*log10(D2)+20*log10(fMHz);

- fMHz is assumed as 14.5 GHz for UL direction (worst case for Ku-band);

- D2 is slant distance between VSAT and SAN and could be derived as following:

D2=(B*sinθ)2+r2-B2-B*sinθ

Where:

- h is assumed as

  - 35786km for GEO,

  - 1200km for LEO@1200km

  - 600km for LEO@600km

- B is assumed as 6378km for Earth radius

- r is assumed as h+B;

- θ is the elevation angle

Observation 2: Companies to further discuss the elevation angle θ to be used to derive the requirement. θ could be e.g. 35° elevation angle, 75° or higher, but 90° elevation angle should be avoided.

Proposal 1: RAN4 to mention in the specification the CBW that was used as reference value for computing the EIRPmin requirement, even if other configurations or values are not precluded.

Proposal 2: For EIS VSAT computation, one can use EIS=-174+10*log10(BW)+NF+SINR+IM-G as done for the Ka-band:

- Where G is including antenna gain and [polarization gain]

- Target SINR=-1dB

- IM=2dB

NOTE: For Rx gain without polarization gain for Ka-band it was used:

  - (for all types except type 3) the assumption in the coexistence study 39.7dBi for NF=2.5dB;

  - (for type 3 only) 17dBi for VSAT connecting with LEO only with NF 6dB;

Proposal 3: For e.g. EISREFSENS_50MHz (dBm) in Ku-band, consider one unique value based on worst case GSO (and not 2 values based on NGSO and GSO) for terminals Type 4 and 5 which can connect with both GSO and NGSO.

NOTE: Same process was used for Ka-band.

Proposal 4: When possible, consider a scaling factor for EIRPmax in Ku-band based on a value X (determined for a certain BW e.g. 100 MHz, antenna gain and transmission power) + 10log10(NRB x SCS x 12 / factor) in order to cover situations with higher BW for the terminal (e.g. BW=400 MHz), where the factor has been determined on the reference scenario X (with BW e.g. 100 MHz).

Proposal 5: In this specific case, the “factor” represents the normalized factor to scale EIRPmax for different (Channel bandwidth, SCS) configurations. The value of factor could be based on 120 KHz SCS e.g. 66 RBs x 120 kHz SCS x 12, i.e. 95040 kHz or even based on 30 kHz SCS e.g. 273 RBs x 30 kHz x 12, i.e. 98280 with an equivalent number of RBs for an equivalent BW of 100 MHz.

Proposal 6: As done in the past for Ka-band, Ku-band should also consider relaxing the ACS VSAT requirement, otherwise the implementation cost may be too costly. As a matter of fact, different strategies (or combination of them) could be imagined in order to achieve better protection and decrease ACS VSAT value around 25 dB or less:

1/ For a constant BS TN EIRP, increase the number of antenna elements and decrease the transmission power (for a narrower beam with same EIRP);

2/ Increase transmission power of the satellite;

3/ Increase the antenna gain of the satellite;

4/ Add a guard band and model the guard band with realistic adjacent channel OFDM spectrum shape;

5/ Increase the VSAT antenna gain,

6/ Increasing the ACLR of the TN BS (deployments are foreseen for IMT-2030 technology and not necessary in the vicinity of Ku-band UE),

7/ Considering some isolation distance,

8/ Any of the combination above, etc.

For reference, see for instance R4-2405972 “On the ACS requirement issue for VSAT UE in above 10 GHz” (THALES, Magister Solutions Ltd, Eutelsat Group, ESA, Inmarsat, Viasat, Novamint, EchoStar, Amazon).

For instance, at RAN4#110, the following agreement has been made:

With the following notes:

It has also been agreed that the respective notes should be included in the TR 38.863, where (at the time of the Ka-band specification):

NOTE 1:              At the time of this 3GPP co-existence study, there is no TN band defined or planned near 17 GHz. The parameters are derived based on 3GPP coexistence scenarios in which a TN system is simulated to be operating in the band directly adjacent to the proposed NTN system as well as technical assumptions that may or may not be applicable in practice. The results of the study are not intended to address coexistence issues from a regulatory standpoint.

NOTE 2:             There are existing non-3GPP VSAT UE operating in Ka band at present and will likely continue operating in the future, with ACS performance lower than the values proposed.

NOTE 3:              Additional solutions could be further considered to address coexistence issues if and when TN is deployed in 17 GHz.