3GPP TSG-RAN WG4 Meeting #116	R4-2511607

Bengaluru, IN, August 25 – 29, 2025

Agenda Item:	7.24.5.2

Source:	MediaTek Inc.

Title:	Discussion on the RRM core requirements for LP-WUS

Document for:	Discussion

# Introduction

In this paper, we further discuss the remaining issues in the RRM core part and provide our view. The achieved agreements in the last meeting are captured in the WF [1].

# Discussion

## EMR with LPWUS

It is known that for a UE which supports idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16 the UE can perform idle mode CA/DC measurements on the serving cell. However, with LPWUS UE, EMR may happen in both cases: Case#1 (RRM offloading) and Case#3 (RRM relaxation). To define UE behavior when EMR is configured will depend on whether T331 is running or not for both cases. If we follow the legacy principle, UE need to measure NR inter-frequency carriers configured for idle mode CA/DC measurements without relaxation.

Proposal 1: On EMR for LPWUS and when T331 is running, MR is expected to perform measurements on NR inter-frequency carriers configured for idle mode CA/DC measurements without relaxation for both Case#1 (RRM offloading) and Case#3 (RRM relaxation).

On the other hand, when T331 is not running:

- For Case#1 (RRM offloading): RAN4 assumed in this case Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ is always met. Therefore, the UE shall search for inter-frequency layers configured for idle mode CA/DC measurements by following the relaxed higher priority frequency measurement requirements for Case#1 (i.e., every 1 hr)

- Case#3 (RRM relaxation).

  - If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ

    - the UE shall search for inter-frequency layers configured for idle mode CA/DC measurements by following the relaxed higher priority frequency measurement requirements for Case#3 (i.e., every 1 hr)

  - if Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ

    - the UE shall search for inter-frequency layers configured for idle mode CA/DC measurements by following the relaxed higher priority frequency measurement requirements for Case#3 (i.e., 16 times)

Proposal 2: On EMR for LPWUS and when T331 is not running in Case#1 (RRM offloading):

- RAN4 assumed in this case Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ is always met. Therefore, the UE shall search for inter-frequency layers configured for idle mode CA/DC measurements by following the relaxed higher priority frequency measurement requirements for Case#1 (i.e., every 1 hr).

Proposal 3: On EMR for LPWUS and when T331 is not running in Case#3 (RRM relaxation):

- If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ

  - the UE shall search for inter-frequency layers configured for idle mode CA/DC measurements by following the relaxed higher priority frequency measurement requirements for Case#3 (i.e., every 1 hr)

- if Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ

  - the UE shall search for inter-frequency layers configured for idle mode CA/DC measurements by following the relaxed higher priority frequency measurement requirements for Case#3 (i.e., 16 times)

## RedCap with LPWUS

In R17, RedCap was introduced to support reduced capability devices. R17 RedCap UEs can achieve up to 6 times RRM relaxation when either of stationary or not at the cell edge criteria are met. Or 4 hours RRM relaxation when both the aforementioned criteria are met. We understand that introducing LPWUS with RedCap can bring further RRM relaxation. However, given that this is a last meeting for core part we are not confident that whether the agreed requirements for LPWUS can be reused directly to RedCap. Especially with issues related to the number of SSB samples required for synchronization for different wake-up delay capabilities.

Proposal 4: The agreed requirements for LP-WUS cannot be reused directly for RedCap. Due to the short time RAN4 to deprioritize LP-WUS requirements for RedCap.

## Entry/Exit conditions

Following the progress in RAN4, LP-WUS capable UE may operate under three scenarios (see Figure 1):

- RRM offloading (Case#1): when MR is deactivated and LR is activated.

- RRM relaxation (Case#3): when both MR and LR are activated.

- Legacy UE: when MR is activated and LR is deactivated.

Figure 1: Different cases of LP-WUR UE

For both cases (Case#1 and Case#3) RAN4 can define same evaluation requirements for exit conditions based on LR measurements only. On the other hand, the entry conditions can be evaluated by MR against MR threshold for both Case#1 and Case#3, however, if LR threshold is also configured then the LR evaluation can also be considered. Note that the evaluation of MR is based on DRX cycle (similar to legacy serving cell measurements Nserv), while LR evaluation should be based on LO cycle regardless of whether LP-SS is configured or not (Note: DRX cycle and LO periodicity are the same based on RAN1 agreement).

| Cases | Entry condition | Exit condition |
| --- | --- | --- |
| Case#1 (only LR) | - Evaluation based on MR using DRX cycle - Optional evaluation based on LR using LO cycle | - Evaluation based on LR using LO cycle |
| Case#3 (LR+MR) | - Evaluation based on MR using DRX cycle - Optional evaluation based on LR using LO cycle | - Evaluation based on LR using LO cycle |

Another issue needs to be discussed is that how to avoid the ping pong behaviour when transitioning between different cases. Activating and deactivating MR can consume more power and takes longer time compared to LR. Therefore, it is important to ensure UE can confidently switch to any case without having ping pong impact.

Proposal 5: Discuss how to avoid the ping pong behaviour when transitioning between different cases based on RRM measurements.

Since in Case#1 RRM measurements are fully offloaded to LR, UE need to be confident enough to enter Case#1 before deactivating MR. Therefore, we propose that RAN4 to study allowing the UE entering Case#3 first before entering Case#1 after a period of time (i.e., even when signal quality is higher than the threshold). This will help UE switching to Case#1 for RRM fully offloading more confidently after ensuring that signal quality condition does not change during Case#3 for a given period of time.

Proposal 6: RAN4 to study switching between Case#3 and Case#1 based on a timer. Since UE need to be confident enough to enter Case#1 before deactivating MR, it is better for the UE to enter Case#3 first for a period of time (confidence period) before UE can enter Case#1.

## Other issues

The following issues were also discussed:

Issue 1-1-9: LP-WUR status at legacy case (not at LP-WUS monitoring case/fully offloading(case 1) case/RRM relaxation (case 3) case)

In our view, when UE operate in the legacy status, the LR requirements should not apply, and UE is not mandated to activate LR or meeting the corresponding requirements for LPWUS.

Proposal 7: No LR measurement and evaluation requirements apply at the legacy state.

Regarding the evaluation requirement, RAN4 previous agreement:

RAN4#114-bis Issue 1-2-5-2: On MR evaluation requirements Agreement: - Reuse legacy Nserv - Note: the legacy values are Nserv = 4 when DRX = 0.32s and 0.64s, Nserv = 2 when DRX = 1.28 and 2.56s RAN4#114 Issue 1-2-4-2-3: On how to define LR evaluation requirements Agreement: LR evaluation duration is [x1 samples]*LP-SS (for OOK LR) or [y1 samples] *LO (for SSB LR), assuming x or y samples are used to satisfy accuracy requirement and x1 > x and y1>y. Number x and y are defined as measurement requirement as [x samples]*LP-SS (for OOK LR) or [y samples] *LO (for SSB LR)

RAN4 need to discuss the number of samples required to define the evaluation duration requirements for LR for both types OOK LR and OFDM LR. RAN4 to consider using x1=2*x and y1=2*y for the evaluation requirement, where x and y are decided based on simulation results.

Proposal 8: RAN4 to consider using x1=2*x and y1=2*y for the evaluation requirement, where x and y are decided based on simulation results.

Also, regarding the applicability of MR and LR evaluation, in our view MR evaluation apply for the entry to case#1 and case#3, while LR evaluation apply to exit cases from case#1/3 to legacy case; and (optional) switching between case3# and case#1.

Proposal 9: On the applicability of MR and LR evaluation:

- MR evaluation applies to the entry to case#1 and case#3

- LR evaluation applies to exiting from case#1 and case#3 to legacy case; and (optional) switching between case3# to case#1

Issue 1-2-1-1: Detail on LR accuracy and side conditions requirements - Proposals - P1: Use 2.5 dB as the RF impairment margin for LP-RSRP accuracy requirements. (vivo Huawei Apple) - P2: Consider 3.5 dB RF impairment margin for LR (xiaomi)

In our view, the RF impairment of LR can be worse than MR, which is dependent on how UE implement those RF components. Therefore, the accuracy of LR should reflect this degradation in the RF impairment margin, i.e., we support to use 3.5 dB margin to accommodate RF impairment.

Proposal 10: Consider 3.5 dB RF impairment margin for LR.

Issue 1-2-2-2: Upper bound on SSB-based LP-WUR measurement periodicity - Proposals - P1: No need to define upper bound for SSB-based LP-WUR measurement periodicity. (oppo xiaomi Nokia CT Huawei Apple) - P2: Prefer the periodicity for measurement and evaluation delay requirements for LP-WUR based on SSB in IDLE/Inactive mode is 1.28s if the LO periodicity is 2.56s. (vivo ZTE)

For Issue 1-2-2-2, we also support no need to define upper bound for SSB-based LP-WUR measurement periodicity. Even if LO periodicity is 2.56s, delay requirement can still apply based on this periodicity.

Proposal 11: No need to define upper bound for SSB-based LP-WUR measurement periodicity.

Also whether to define requirements for LPWUS in FR2, we prefer to prioritize FR1 in this release and deprioritize FR2.

Proposal 12: Prioritize FR1 requirements for R19 LPWUS and deprioritize FR2.

# Summary

In this contribution, the following conclusions are provided:

Proposal 1: On EMR for LPWUS and when T331 is running, MR is expected to perform measurements on NR inter-frequency carriers configured for idle mode CA/DC measurements without relaxation for both Case#1 (RRM offloading) and Case#3 (RRM relaxation).

Proposal 2: On EMR for LPWUS and when T331 is not running in Case#1 (RRM offloading):

- RAN4 assumed in this case Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ is always met. Therefore, the UE shall search for inter-frequency layers configured for idle mode CA/DC measurements by following the relaxed higher priority frequency measurement requirements for Case#1 (i.e., every 1 hr).

Proposal 3: On EMR for LPWUS and when T331 is not running in Case#3 (RRM relaxation):

- If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ

  - the UE shall search for inter-frequency layers configured for idle mode CA/DC measurements by following the relaxed higher priority frequency measurement requirements for Case#3 (i.e., every 1 hr)

- if Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ

  - the UE shall search for inter-frequency layers configured for idle mode CA/DC measurements by following the relaxed higher priority frequency measurement requirements for Case#3 (i.e., 16 times)

Proposal 4: The agreed requirements for LP-WUS cannot be reused directly for RedCap. Due to the short time RAN4 to deprioritize LP-WUS requirements for RedCap.

Proposal 5: Discuss how to avoid the ping pong behaviour when transitioning between different cases based on RRM measurements.

Proposal 6: RAN4 to study switching between Case#3 and Case#1 based on a timer. Since UE need to be confident enough to enter Case#1 before deactivating MR, it is better for the UE to enter Case#3 first for a period of time (confidence period) before UE can enter Case#1.

Proposal 7: No LR measurement and evaluation requirements apply at the legacy state.

Proposal 8: RAN4 to consider using x1=2*x and y1=2*y for the evaluation requirement, where x and y are decided based on simulation results.

Proposal 9: On the applicability of MR and LR evaluation:

- MR evaluation applies to the entry to case#1 and case#3

- LR evaluation applies to exiting from case#1 and case#3 to legacy case; and (optional) switching between case3# to case#1

Proposal 10: Consider 3.5 dB RF impairment margin for LR.

Proposal 11: No need to define upper bound for SSB-based LP-WUR measurement periodicity.

Proposal 12: Prioritize FR1 requirements for R19 LPWUS and deprioritize FR2.

# References

- R4-2504908, WF on RRM requirements for NR_LPWUS, vivo.

- RP-241824, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), vivo, NTT DOCOMO.

- R2-2409212, Report from session on Rel-18 MIMO and MUSIM, Rel-19 MIMO, LPWUS, and SBFD, CATT