3GPP TSG-WG4 Meeting #116	R4-2511623

# Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

# Title:	Remaining items on HPUE MSD Simplification

# Agenda Item:	7.1.2.1.1

# Source:	Skyworks Solutions, Inc.

# Document for:	Approval

- Introduction

This contribution captures our views on the remaining open items for HPUE MSD simplification in response to the agreed upon way forward (WF) [1] and as a companion to the draft CR [3]. This document also discusses if the NR-CA MSD simplification tables should be ported to EN-DC 38.101-3.

- Discussion

Figure 1 reproduces the WF agreement #1 and #2 [1].

Figure 1: WF agreement #1 (top) and #2 on HPUE MSD requirements [1].

For agreement #1 and #2 RAN4 did not discuss the following remaining questions:

Q1: How to simplify the 2UL PC1.5 MSD requirements in case the PC2 MSD requirements are specified and the PC3 MSD requirements are not specified?

Q2: What does the WF [1] agreement #2 “small PC2/PC1.5 MSD” mean where PC3 MSD is not specified and how to use the agreed LUTs?

Also, the agreed 2UL IMD LUTs “automatically” specify the HPUE MSD requirements for all IMD orders specified in the legacy PC3 MSD tables. For some CA combinations, the legacy PC3 MSD requirements are specified for multiple IMD orders, e.g. IMD2, IMD4, IMD5. Yet the RAN4 PRD rules restrict to a maximum of 2 the number of MSD test points for each affected DL band:

- 1 MSD test point for the lowest even IMD order,

- 1 MSD test point for the lowest odd IMD order.

Q3: How to ensure that the MSD requirements calculated with LUT will comply to the PRD rules?

This paper presents our views on these three questions.

  - On specifying the PC1.5 MSD when there PC3 MSD is not specified and the PC2 MSD is specified.

For these exception cases, we propose to re-use the concept agreed in [2] for 1UL band 1UL CC MSD simplification by making the following observation.

Observation 1: The PC1.5 MSD due to 2UL IMD interference can be specified by using the PC2 MSD requirement specified in the legacy MSD tables as input variable to the 2UL LUTs entitled “MSDmax 6, 9, 12, 15” since:

- for IMD2, the offset between PC1.5 and PC2 is 6dB,

- for IMD3, the offset between PC1.5 and PC2 is 9dB,

- for IMD4, the offset between PC1.5 and PC2 is 12dB,

- for IMD5, the offset between PC1.5 and PC2 is 15dB.

Proposal 1: Consider adopting the following 2UL PC1.5 MSD requirements simplification.

“As an exception, for cases where:

- the PC2 MSD is specified in Table 7.3A.5-1a or in Table 7.3A.5-2a, and

- the PC3 MSD is not specified in Table 7.3A.5-1 or in Table 7.3A.5-2, and

- the PC1.5 MSD is not specified in Table 7.3A.5-1b or in Table 7.3A.5-2b, then

the PC1.5 MSD is specified as PC1.5 MSD = PC2 MSD + MSD, where,

- MSD is specified in Table 7.3A.2.3-3 output columns denoted “MSDmax 6, 9, 12, 15” and where the input column uses the specified PC2 MSD specified in Table 7.3A.5-1a or Table 7.3A.5-2a instead of PC3 MSD, and,

- the correspondence between the MSDmax specified in Table 7.3A.2.3-3 and the dual UL IMD order is specified in Table 7.3A.2.3-4 using the input row specified for PC2, and

- these PC1.5 MSD requirements apply for the same uplink/downlink configurations as those specified in the PC2 MSD requirements of Table 7.3A.5-1a or Table 7.3A.5-2a.

Table 7.3A.2.3-3: MSDmax look-up-table for MSD due to 2UL CA intermodulation interference

| PC3 MSD (dB) | MSDmax (dB) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 6 | 9 | 12 | 15 | 18 | 24 | 30 |
| 0.1≤ PC3 MSD ≤0.4 | 1.2 | 2.3 | 3.8 | 5.7 | 7.8 | 13 | 18.7 |
| 0.5≤ PC3 MSD ≤1.4 | 1.9 | 3.5 | 5.4 | 7.6 | 10 | 15.6 | 21.4 |
| 1.5≤ PC3 MSD ≤2.4 | 2.9 | 4.8 | 7.1 | 9.6 | 12.4 | 18.1 | 24.1 |
| 2.5≤ PC3 MSD ≤3.4 | 3.5 | 5.7 | 8.2 | 10.9 | 13.7 | 19.6 | 25.5 |
| 3.5≤ PC3 MSD ≤4.4 | 3.9 | 6.3 | 8.9 | 11.7 | 14.6 | 20.5 | 26.5 |
| 4.5≤ PC3 MSD ≤5.4 | 4.3 | 6.8 | 9.5 | 12.4 | 15.3 | 21.3 | 27.3 |
| 5.5≤ PC3 MSD ≤6.4 | 4.6 | 7.2 | 10 | 12.9 | 15.9 | 21.8 | 27.8 |
| 6.5≤ PC3 MSD ≤7.4 | 4.8 | 7.6 | 10.4 | 13.4 | 16.3 | 22.3 | 28.3 |
| 7.5≤ PC3 MSD ≤8.4 | 5.1 | 7.9 | 10.8 | 13.7 | 16.7 | 22.6 | 28.6 |
| 8.5≤ PC3 MSD ≤9.4 | 5.2 | 8.1 | 11 | 14 | 16.9 | 22.9 | 28.9 |
| PC3 MSD ≥9.5 | 6 | 9 | 12 | 15 | 18 | 24 | 30 |

Table 7.3A.2.3-4: 2UL band 2UL CC MSDmax correspondence look-up-table

| Inter-band CA 2UL band 2CC power-class | MSDmax |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 6 | 9 | 12 | 15 | 18 | 24 | 30 |
| PC2 | IMD2 | IMD3 | IMD4 | IMD5 |  |  |  |
| PC1.5 |  |  | IMD2 |  | IMD3 | IMD4 | IMD5 |

  - On “small PC2/PC1.5 MSD” when no PC3 MSD is specified.

Based on P1 “Table 7.3A.2.3-3”, we assume that in case no PC3 MSD is simplified (i.e. PC3 MSD =0dB), then the PC2 MSD is at worst-case:

- 1.2dB for IMD2

- 2.3dB for IMD3

- 3.8dB for IMD4

- 5.7dB for IMD5.

In our views, a small 2UL IMD PC2 MSD is an MSD less than 3dB. In which case we propose the following.

Proposal 2: When no 2UL IMD PC3 MSD is specified, capture the following guidelines in the RAN4 PRD.

For PC2:

- Do not specify the 2UL PC2 MSD for IMD2 and IMD3,

- The PC2 2UL IMD4 MSD cannot exceed 3.8dB, i.e. 2UL IMD4 MSD <= 3.8dB

- The PC2 2UL IMD5 MSD cannot exceed 5.7dB, i.e. 2UL IMD5 MSD <= 5.7dB,

For PC1.5,

- Specify the 2UL IMD4 and 2UL IMD5 MSD using proposal 1.

  - On ensuring a maximum of two 2UL IMD MSD test points per affected DL band.

Figure 2 reproduces the band n1 PC3 2UL IMD MSD test points from Table 7.3A.5-1.

Figure 2: Example of multiple PC3 2UL IMD MSD test points affecting DL band n1 in CA_n1-n77

The downlink Band n1 is affected by multiple 2UL IMD products:

- 29.8dB MSD due to IMD2 2UL transmissions of band n1 and n77,

- 8.0dB MSD due to IMD4 2UL transmissions of band n1 and n77,

- 17.0dB MSD due to IMD5 from intra-band UL CA_n77 transmission.

The PC3 2UL IMD5 MSD requirement is not eligible to MSD simplification via LUT, while both the IMD2 and the IMD4 are eligible. Yet according to the RAN4 PRD, when the DL band is affected by multiple even order 2UL IMD products, only 1 MSD requirement should be specified: that corresponding to the lowest IMD order. It seems necessary to remove any redundant 2UL PC3 MSD test points that remain in the legacy PC3, PC2 and PC1.5 MSD tables.

Proposal 3: Bring clean-up CR to remove the redundant 2UL IMD MSD test points from the PC3, PC2 and PC1.5 MSD tables. Exceptions to these clean-ups are test points which cannot be verified due to regional frequency range restrictions – see the Annex for detailed list of changes.

  - On porting the NR-CA MSD LUTs to EN-DC

In theory, the NR-CA MSD LUTs could be ported to simplify the EN-DC HPUE MSD requirements. There are however some subtle differences that prevent a simple copy and paste of the NR-CA draftCR [3] to 38.101-3, namely.

Observation:

- For 1UL band 1UL CC, the NR-CA eligibility criteria of sub-clause 7.3A.2.3.1 [3] can not be simply ported to ENDC because:

  - The 38.101-3 clause 5 inter-band EN-DC configuration tables mostly specify “single UL switched” operation is supported, and only few band combinations have footnotes indicating if HPUE is supported or not,

  - For EN-DC, only the clause 6 maximum output power table can be used to detect what are the valid 1UL band or 2UL band HPUE configurations.

- Also, the highest inter-band ENDC power class supported as of today is PC2. So, the NR-CA MSD simplification would have to be simplified to restrict the scope to only PC2.

- Precondition to any initiative would be to implement the proposal 3 clean-up CR to EN-DC MSD legacy tables.

Proposal 4: Based on the above observation, RAN4 to postpone the introduction of the MSD simplification LUTs to TS 38.101-3. We invite interested companies to share their views and motivations for such effort.

- Conclusion

This contribution proposes to complete the introduction of the HPUE MSD simplification in Rel-19 by adopting the following proposals.

Proposal 1: Consider adopting the following 2UL PC1.5 MSD requirements simplification.

“As an exception, for cases where:

- the PC2 MSD is specified in Table 7.3A.5-1a or in Table 7.3A.5-2a, and

- the PC3 MSD is not specified in Table 7.3A.5-1 or in Table 7.3A.5-2, and

- the PC1.5 MSD is not specified in Table 7.3A.5-1b or in Table 7.3A.5-2b, then

the PC1.5 MSD is specified as PC1.5 MSD = PC2 MSD + MSD, where,

- MSD is specified in Table 7.3A.2.3-3 output columns denoted “MSDmax 6, 9, 12, 15” and where the input column uses the specified PC2 MSD specified in Table 7.3A.5-1a or Table 7.3A.5-2a instead of PC3 MSD, and,

- the correspondence between the MSDmax specified in Table 7.3A.2.3-3 and the dual UL IMD order is specified in Table 7.3A.2.3-4 using the input row specified for PC2, and

- these PC1.5 MSD requirements apply for the same uplink/downlink configurations as those specified in the PC2 MSD requirements of Table 7.3A.5-1a or Table 7.3A.5-2a.

Table 7.3A.2.3-3: MSDmax look-up-table for MSD due to 2UL CA intermodulation interference

| PC3 MSD (dB) | MSDmax (dB) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 6 | 9 | 12 | 15 | 18 | 24 | 30 |
| 0.1≤ PC3 MSD ≤0.4 | 1.2 | 2.3 | 3.8 | 5.7 | 7.8 | 13 | 18.7 |
| 0.5≤ PC3 MSD ≤1.4 | 1.9 | 3.5 | 5.4 | 7.6 | 10 | 15.6 | 21.4 |
| 1.5≤ PC3 MSD ≤2.4 | 2.9 | 4.8 | 7.1 | 9.6 | 12.4 | 18.1 | 24.1 |
| 2.5≤ PC3 MSD ≤3.4 | 3.5 | 5.7 | 8.2 | 10.9 | 13.7 | 19.6 | 25.5 |
| 3.5≤ PC3 MSD ≤4.4 | 3.9 | 6.3 | 8.9 | 11.7 | 14.6 | 20.5 | 26.5 |
| 4.5≤ PC3 MSD ≤5.4 | 4.3 | 6.8 | 9.5 | 12.4 | 15.3 | 21.3 | 27.3 |
| 5.5≤ PC3 MSD ≤6.4 | 4.6 | 7.2 | 10 | 12.9 | 15.9 | 21.8 | 27.8 |
| 6.5≤ PC3 MSD ≤7.4 | 4.8 | 7.6 | 10.4 | 13.4 | 16.3 | 22.3 | 28.3 |
| 7.5≤ PC3 MSD ≤8.4 | 5.1 | 7.9 | 10.8 | 13.7 | 16.7 | 22.6 | 28.6 |
| 8.5≤ PC3 MSD ≤9.4 | 5.2 | 8.1 | 11 | 14 | 16.9 | 22.9 | 28.9 |
| PC3 MSD ≥9.5 | 6 | 9 | 12 | 15 | 18 | 24 | 30 |

Table 7.3A.2.3-4: 2UL band 2UL CC MSDmax correspondence look-up-table

| Inter-band CA 2UL band 2CC power-class | MSDmax |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 6 | 9 | 12 | 15 | 18 | 24 | 30 |
| PC2 | IMD2 | IMD3 | IMD4 | IMD5 |  |  |  |
| PC1.5 |  |  | IMD2 |  | IMD3 | IMD4 | IMD5 |

Proposal 2: When no 2UL IMD PC3 MSD is specified, capture the following guidelines in the RAN4 PRD.

For PC2:

- Do not specify the 2UL PC2 MSD for IMD2 and IMD3,

- The PC2 2UL IMD4 MSD cannot exceed 3.8dB, i.e. 2UL IMD4 MSD <= 3.8dB

- The PC2 2UL IMD5 MSD cannot exceed 5.7dB, i.e. 2UL IMD5 MSD <= 5.7dB,

For PC1.5,

- Specify the 2UL IMD4 and 2UL IMD5 MSD using proposal 1.

Proposal 3: Bring clean-up CR to remove the redundant 2UL IMD MSD test points from the PC3, PC2 and PC1.5 MSD tables. Exceptions to these clean-ups are test points which cannot be verified due to regional frequency range restrictions – see the Annex for detailed list of changes.

Observation:

- For 1UL band 1UL CC, the NR-CA eligibility criteria of sub-clause 7.3A.2.3.1 [3] cannot be simply ported to ENDC because:

  - The 38.101-3 clause 5 inter-band EN-DC configuration tables mostly specify “single UL switched” operation is supported, and only few band combinations have footnotes indicating if HPUE is supported or not,

  - For EN-DC, only the clause 6 maximum output power table can be used to detect what are the valid 1UL band or 2UL band HPUE configurations.

- Also, the highest inter-band ENDC power class supported as of today is PC2. So, the NR-CA MSD simplification would have to be simplified to restrict the scope to only PC2.

- Precondition to any initiative would be to implement the proposal 3 clean-up CR to EN-DC MSD legacy tables.

Proposal 4: Based on the above observation, RAN4 to postpone the introduction of the MSD simplification LUTs to TS 38.101-3. We invite interested companies to share their views and motivations for such effort

# References

- R4-2507935 WF on new MSD rules, 3GPP TSG-RAN WG4 Meeting # 115, St Julian's, Malta, Skyworks, Murata Manufacturing Co Ltd., Nokia, Samsung, Qualcomm, Apple, Vivo, ZTE, Mediatek, Huawei, Oppo.

- R4-2507916 DraftCR to 38101-1-j10 on introducing the 1UL band 1CC HPUE MSD requirements simplification, 3GPP TSG-RAN WG4 Meeting # 115, St Julian's, Malta, Skyworks Solutions, Inc., Nokia, Samsung, Murata Manufacturing Co Ltd., Qualcomm France, ZTE, Huawei, Apple, Vivo, Mediatek.

- R4-2511602, Draft CR to 38101-1-j20 on introducing the HPUE MSD requirements simplification, 3GPP TSG-RAN WG4 Meeting # 116, Bangaluru, India, Skyworks Solutions, Inc., Nokia, T-Mobile USA, ZTE, Oppo, Samsung, Murata Manufacturing Co Ltd., MediaTek, vivo, Huawei, Apple, Qualcomm France.

# Annex – MSD clean-up overview

# Table 7.3A.5-1: Removal of following MSD for DL band / IMD order.

- CA_n1-n77: n1 IMD4

- CA_n2-n66: n66 IMD5

- CA_n2-n77: n2 IMD4

- CA_n3-n77: n3 IMD4

- CA_n3-n78: n3 IMD4

- CA_n20-n41: n20 IMD5

- CA_n25-n66: n66 IMD5

- CA_n25-n77: n25 IMD4

- CA_n28-n50: n28 IMD4

- CA_n28-n74: n28 IMD4

Table 7.3A.5-1a: Removal of following MSD for DL band / IMD order.

- CA_n1-n77: n1 IMD4

- CA_n2-n77: n2 IMD4

- CA_n3-n77: n3 IMD4

- CA_n3-n78: n3 IMD4

- CA_n25-n66: n66 IMD5

- CA_n25-n77: n25 IMD4

Table 7.3A.5-1b: Removal of following MSD for DL band / IMD order.

- CA_n2-n77: n2 IMD4

- CA_n3-n78: n3 IMD4

- CA_n25-n77: n25 IMD4

Table 7.3A.5-2: Removal of following MSD for DL band / IMD order.

- CA_n1-n3-n78: 		n78 IMD4

- CA_n2-n66-n77: 		n66 IMD4

- CA_n2-n66-n77: 		n77 IMD4

- CA_n3-n5-n78: 		n78 IMD5

- CA_n3-n8-n78: 		n78 IMD5

- CA_n3-n26-n78: 		n78 IMD5

- CA_n5-n7-n78: 		n78 IMD4

- CA_n5-n66-n77: 		n78 IMD5

- CA_n7-n28-n78: 		n78 IMD4

- CA_n20-n41-n78:	n78 IMD4

- CA_n25-n41-n77: 	n77 IMD5

- CA_n25-n41-n78: 	n78 IMD5

- CA_n25-n66-n77: 	n66 IMD4

- CA_n25-n66-n77: 	n77 IMD4

- CA_n26-n66-n77: 	n77 IMD5

- CA_n41-n71-n77: 	n77 IMD4

- CA_n41-n71-n78: 	n77 IMD4

- CA_n66-n70-n78: 	n70 IMD4

- CA_n70-n71-n77: 	n77 IMD5

The following redundant test points cannot be removed due to regional frequency range restrictions.

- CA_n2-n66-n77: 	n2 IMD4

- CA_n25-n66-n77: 	n25 IMD4

- CA_n26-n70-n77: 	n77 IMD5

Table 7.3A.5-2a: Removal of following MSD for DL band / IMD order.

- CA_n25-n41-n77: 	n77 IMD5

- CA_n25-n66-n77: 	n66 IMD4

- CA_n41-n71-n77: 	n77 IMD4

The following redundant test point cannot be removed due to regional frequency range restrictions.

- CA_n25-n66-n77: 	n25 IMD4

Table 7.3A.5-2b: Removal of following MSD for DL band / IMD order.

- CA_n25-n41-n77: 	n77 IMD5

- CA_n41-n71-n77: 	n77 IMD4