3GPP TSG-WG4 Meeting #116	R4-2511628

# Bengaluru, India, 25th Aug 2025 - 29th Aug 2025

# Title:	SUL REFSENS and missing PC2 MSD

# Agenda Item:	6.5.3

# Source:	Skyworks Solutions, Inc.

# Document for:	Approval

- Introduction

This contribution discusses how to best introduce the missing power class 3 (PC3) and PC2 one uplink (1UL) maximum sensitivity degradation (MSD) requirements for SUL with downlink CA. We also propose to clarify if PC3 or PC2 REFSENS requirements are missing for some “SUL operation” configurations.

- Discussion

This contribution focuses on two areas:

- How to introduce the missing PC3 and missing PC2 MSD requirements due to 1UL SUL band for SUL operation with downlink CA? This is a follow up of our previous contribution [1],

- How to ensure preconditions are met prior to introducing HPUE / PC2 MSD requirements by:

  - Making sure the REFSENS requirements are complete, both for PC3 and PC2

  - Making sure there are no missing PC3 MSD requirements

## Missing PC2 1UL SUL band MSD requirements

The parsing of Rel 19.2.0 shows that the conclusions presented in [1] remain valid at this meeting (i.e., the following PC2 1UL SUL band MSD requirements are missing in TS 38.101-1):

- MSD due to UL harmonic interference where two MSD test points are needed per DL band to mirror their NR-CA counterparts for:

  - Band n77 UL2/DL1 MSD for SUL_n77-n80

  - Band n77 UL2/DL1 MSD for SUL_n77-n84

  - Band n78 UL2/DL1 MSD for SUL_n78-n80

  - Band n79 UL2/DL1 MSD for SUL_n79-n97

- MSD due to RX harmonic mixing, where two MSD points are needed for Band n41 due to UL4/DL3 for SUL_n41-n98 as agreed for its NR CA counterpart CA_n39-n41,

- MSD due to cross-band isolation for:

  - Band n1 MSD for SUL_n1-n80

  - Band n3 MSD for SUL_n3-n84

  - Band n41 MSD for SUL_n41-n80

  - Band n41 MSD for SUL_n41-n95

  - Band n41 MSD for SUL_n41-n98

MSD requirements were proposed in [1] and it was proposed to introduce these missing requirements with new legacy format MSD tables. Since then, RAN4 has reached agreements on calculating any missing PC2 or PC1.5 1UL band 1UL CC MSD requirements from the specified PC3 MSD requirements. It may be simpler at this stage for RAN4 to consider using the NR-CA HPUE MSD LUT for the missing SUL MSD requirements.

Observation 1: Several SUL PC2 MSD requirements are missing for MSD due to harmonic, harmonic mixing, and cross-band isolation. RAN4 may consider two ways to address this issue:

- Option 1:  Introduce the missing PC2 MSD requirements with new “legacy format” MSD tables, as proposed in [1]

- Option 2: Do not introduce the missing PC2 MSD requirements with additional legacy format MSD tables. Instead, specify the missing PC2 1UL “SUL band” MSD requirements by porting the NR-CA HPUE MSD LUT to SUL clause 7.3C.

Proposal 1: Due to few meetings remaining before the end of Release-19, it is proposed to specify the missing PC2 1UL SUL band MSD requirements by porting the NR-CA HPUE MSD Look-Table into clause 7.3C.

## Preconditions to port the NR-CA HPUE MSD LUTs to SUL

### No missing PC3 MSD requirements

Since the highest power class supported by SUL is currently PC2, any missing PC2 MSD requirement can be calculated if a PC3 “baseline” or PC3 LUT input value is specified. We make the following observation based on the previous analysis in [1]:

Observation 2:

- The Band n79 PC3 UL2/DL1 MSD requirement needs to be corrected for SUL_n79-n97 to:

  - Change the band n79 10 MHz CBW to 35.5 dB

  - Change the SUL RB allocation to 12 RB for the band n79 100 MHz CBW MSD test point

- The Band n41 PC3 UL4/DL3 MSD requirement needs to be introduced for SUL_n41-n98 based on the specified MSD requirements for its NR-CA counterpart CA_n39-n41

Proposal 2 based on observation 2:

- Change the titles of Table 7.3C.2-2 and Table 7.3C.2-4 to clarify that these are the PC3 MSD requirements

- Correct the PC3 SUL_n79-n97 MSD test points

- Introduce a new PC3 legacy format RX harmonic mixing MSD table to capture the missing Band n41 MSD for SUL_n41-n98 as proposed in the table below.

Table 7.3C.2-5: PC3 reference sensitivity and uplink/downlink configurations for SUL operation (exceptions due to harmonic mixing)

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n98 | n41 | 5 | 15 | 6 | 10 | 9.3 | NOTE 2 | UL4/DL3 |
| n98 | n41 | 5 | 15 | 6 | 100 | 2.2 | NOTE 2 | UL4/DL3 |
| NOTE 1: The requirements should be verified using RBstart = floor((NRB-LCRB)/2), where floor(x) is the greatest integer less than or equal to x, and where the UL parameters NRB and LCRB are respectively, the transmission bandwidth configuration and the number of RB’s for the specified UL band channel bandwidth and the UL band subcarrier spacing. NOTE 2: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. |  |  |  |  |  |  |  |  |

### SUL operation PC3, PC2 REFSENS

For SUL operation, clause 7.3C.2 specifies that REFSENS requirements for downlink bands specified in Table 7.3.2-1a, Table 7.3.2-1b, Table 7.3.2-2, and Table 7.3.2-2a are to be met. This means that only PC3 REFSENS requirements are specified for SUL although some SUL “bands” do support PC2, (i.e., it seems that some PC2 REFSENS requirements are missing, namely for the configurations annotated with Note 1 in Table 7.3C.2-1):

- SUL_n1-n84, whose NR counterpart would be Band n1, where n84 supports PC2

- SUL_n3-n80, whose NR counterpart would be Band n3, where n80 supports PC2

Regarding these latter configurations, it is not clear why:

- SUL_n24-n99 is specified in the BCS Table 5.5C-1 while SUL_n1-n84, SUL_n3-n80, and SUL_n8-n81 are not

- SUL_n5-n89 (~NR n5), SUL_n20_n82 (~NR n20), SUL_n28-n83 (~NR n28), and SUL_n66-n86 (~ NR n66) are neither specified in the BCS Table 5.5C-1 nor in the REFSENS uplink configuration Table 7.3C.2-1.

Observation 3: The PC2 REFSENS requirements seem missing for SUL_n1-n84 and SUL_n3-n80

Observation 4: It is not clear why

- SUL_n24-n99 is specified in the BCS Table 5.5C-1, but SUL_n1-n84, SUL_n3-n80, and SUL_n8-n81 are not. Do these configurations need to be added to Table 5.5C-1?

- SUL_n5-n89 (~NR n5), SUL_n20_n82 (~NR n20), SUL_n28-n83 (~NR n28), and SUL_n66-n86 (~ NR n66) are neither specified in the BCS Table 5.5C-1 nor in the REFSENS uplink configuration Table 7.3C.2-1.

Does that mean that PC3 REFSENS requirements do not apply to these configurations?

Proposal 3: Based on observation 3 and 4, consider clarifying the PC3 and PC2 REFSENS requirements for the identified SUL configurations.

- Conclusion

This contribution proposes to introduce missing PC2 MSD requirements for SUL operation with downlink CA by porting the NR-CA HPUE MSD LUT to clause 7.3C. A precondition to this simplification is to make sure PC3 MSD requirements are not missing. RAN4 may also consider clarifying how to resolve the observations relative to PC3 or PC2 REFSENS requirements for SUL.

Observation 1: Several SUL PC2 MSD requirements are missing for MSD due to harmonic, harmonic mixing, and cross-band isolation. RAN4 may consider two ways to address this issue:

- Option 1:  Introduce the missing PC2 MSD requirements with new legacy format MSD tables, like proposed in [1]

- Option 2: Do not introduce the missing PC2 MSD requirements with additional legacy format MSD tables. Instead, specify the missing PC2 1UL SUL band MSD requirements by porting the NR-CA HPUE MSD LUT to SUL clause 7.3C.

Proposal 1: Due to few meetings remaining before the end of Release-19, it is proposed to specify the missing PC2 “1UL SUL band” MSD requirements by porting the NR-CA HPUE MSD Look-Table into clause 7.3C.

Proposal 2 based on observation 2:

- Change the titles of Table 7.3C.2-2 and Table 7.3C.2-4 to clarify that these are the PC3 MSD requirements

- Correct the PC3 SUL_n79-n97 MSD test points

- Introduce a new PC3 legacy format RX harmonic mixing MSD table to capture the missing Band n41 MSD for SUL_n41-n98 as proposed in the table below

Table 7.3C.2-5: PC3 reference sensitivity and uplink/downlink configurations for SUL operation (exceptions due to harmonic mixing)

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n98 | n41 | 5 | 15 | 6 | 10 | 9.3 | NOTE 2 | UL4/DL3 |
| n98 | n41 | 5 | 15 | 6 | 100 | 2.2 | NOTE 2 | UL4/DL3 |
| NOTE 1: The requirements should be verified using RBstart = floor((NRB-LCRB)/2), where floor(x) is the greatest integer less than or equal to x, and where the UL parameters NRB and LCRB are respectively, the transmission bandwidth configuration and the number of RB’s for the specified UL band channel bandwidth and the UL band subcarrier spacing. NOTE 2: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. |  |  |  |  |  |  |  |  |

Proposal 3: Based on observation 3 and 4, consider clarifying the PC3 and PC2 REFSENS requirements for the identified SUL configurations.

# References

- R4-2502172 HPUE MSD for SUL, 3GPP TSG RAN Meeting #114, Athens, Greece, Skyworks Solutions, Inc.