3GPP TSG-RAN WG4 Meeting #116	R4-2511635

Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.26.3

Source:	Qualcomm Incorporated

Title: 		RRM core requirements for XR enhancements

Document for:	Discussion

# Introduction

RAN4 achieved multiple agreements over the last few meetings on DCI based MG cancellation mechanism introduced by RAN1. In this contribution paper, we further discuss the remaining few issues identified in the WF [1] from the last meeting.

# Type of measurement gaps

RAN4 discussed the applicability of the cancellation DCI to concurrent MGs and agreed to the following during the last meeting:

<Agreement>: - MG cancelling DCI applies to concurrent MGs and cancels the MG surviving the collision handling rules. - The timeline X ms for the MG cancelling the colliding MGs is based on the one that survives collision handling rules. - FFS the applicability of the gap cancellation if two non-colliding concurrent gaps are separated by less than X ms

With concurrent MGs, the applicability of MG cancelling DCI for the colliding MGs is clear that the DCI applies to the M G that survives the collision handling rules. What is not clear is the applicability of the gap cancellation if the two MGs are non-colliding and are separated by less than X ms (X= 5ms or 3ms). Note that the gaps are considered colliding if they are separated by less than 4 ms.

X ms

X ms

MG 2

MG 1

< X ms

Point B

Point A

As shown in the above figure, MG1 and MG2 are two such MGs which are separated by less X ms. Point A and point B correspond to the time instances X ms before the corresponding MGs. It’s clear from RAN1 specification [TS 38.213, Clause 10.6], that any DCI received before point A corresponds to MG1 and any DCI received after point A and before point B should correspond to MG2. So we think, there’s no more clarification needed to handle this case and the MG cancelling DCI is applicable for this case

[TS 38.213, Clause 10.6] A measurement gap cancellation field in a DCI format [5, TS 38.212] provided by a PDCCH reception is associated to an earliest RRM measurement gap occasion that starts at least 3 msec or 5 msec, as determined by a reported UE capability, after the end of the PDCCH reception

Observation 1: RAN1 specification already clarifies that a measurement gap cancellation field in a DCI is associated to an earliest RRM measurement gap occasion that starts at least 3 msec or 5 msec.

Proposal 1: MG cancelling DCI also applies to non-colliding concurrent MGs separated by <X ms and no spec change is needed in RAN4.

# Impact on RRM requirements

Since RAN4 agreed that the MG cancellation applies only to L3 measurements and to follow NR-U-like approach for designing the formulas for measurement delay extension, we now need to discuss the exact formula to extend the L3 measurement requirements. We tend to think that it’s straightforward to use NR-U approach and add the number of skipped MGs to the number of SSB samples for each requirement.

Some companies argued that this may cause unnecessary extension of the measurement periods because of CSSF scaling and hence proposed to not apply the CSSF to the delay extension. We understand this motivation but since the network doesn’t know which MO a UE is measuring in a particular MG, it’s difficult take out CSSF from the delay extension.

Proposal 2: For DCI based dynamic indication, RAN4 to extend the L3 delays by adding the number of MGs deactivated during the L3 period to the number of measurement samples.

As an example, Table 9.2.6.2-1 can be modified as below:

Table 9.2.6.2-1: Time period for PSS/SSS detection (FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max(600 ms, (5+Nskip) x Kgap x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320 ms | max(600 ms, ceil(M2Note 1x (5+Nskip) x Kgap) x max(MGRP, SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320 ms | Ceil((5+Nskip) x Kgap ) x max(MGRP, DRX cycle) x CSSFintra |

Some companies also brought various conditions to enable the MG skipping. These included – maximum number of MGs skipped during the measurement period, maximum acceptable time separation between two non-cancelled MGs etc. We are okay to specify the limit on maximum number of skipped MGs within a measurement period and maximum acceptable time separation between two non-cancelled MGs as a part of the requirements but since we agreed to indicate the preferred percentage of MG cancellations within a time period as a UAI, perhaps it’s also okay to not specify such conditions.

Proposal 3: Given that a UE can send preferred percentage of MG cancellations within a time period as a UAI, no need to define additional conditions on maximum number of MGs skipped during the measurement period and maximum acceptable time separation between two non-cancelled MGs etc.

# Conclusion

Observation 1: RAN1 specification already clarifies that a measurement gap cancellation field in a DCI is associated to an earliest RRM measurement gap occasion that starts at least 3 msec or 5 msec.

Proposal 1: MG cancelling DCI also applies to non-colliding concurrent MGs separated by <X ms and no spec change is needed in RAN4.

Proposal 2: For DCI based dynamic indication, RAN4 to extend the L3 delays by adding the number of MGs deactivated during the L3 period to the number of measurement samples.

Proposal 3: Given that a UE can send preferred percentage of MG cancellations within a time period as a UAI, no need to define additional conditions on maximum number of MGs skipped during the measurement period and maximum acceptable time separation between two non-cancelled MGs etc.

# References

[1] R4-2508285, “WF on RRM requirements for XR_Ph3”, Nokia, RAN4#115