3GPP TSG RAN WG4 #116			   R4-2511640

Bengaluru, India, Aug 25th – 29th, 2025

Agenda item:	7.21.5

Source: 	Qualcomm Incorporated

Title: 	Simulation results for L1-SRS-RSRP measurements

Document for:	Discussion

# Introduction

In this contribution, we will provide the simulation results for L1-SRS-RSRP measurement accuracy requirements based on the agreed link level simulation assumptions [1].

# Evaluation results

In this section, we will provide evaluation results for the following configurations in [1].

- Timing error: Case 1 and Case 2

- Number of PRBs: 24 and 48

- SRS SNR: 1dB, 2dB and 3dB

- FR1 and SCS 15KHz, 30KHz and 60KHz.

- Channel: AWGN

## Case 1

The measurement performance for Case 2 under SNR = 1, 2 and 3 shown in Table 1 to Table 3 respectively.

Table 1: L1-SRS-RSRP performance for Case 1, AWGN channel, and SNR equals 1 dB

| SCS (KHz) | PRB number | Sample number | L1-SRS-RSRP error (dB) |  |
| --- | --- | --- | --- | --- |
|  |  |  | 5% | 95% |
| 15 | 24 | 1 | -0.5214 | -0.0123 |
|  | 48 | 1 | -0.5055 | -0.0122 |
| 30 | 24 | 1 | -1.0090 | -0.0445 |
|  | 48 | 1 | -0.9782 | -0.0356 |
| 60 | 24 | 1 | -2.1007 | -0.0442 |
|  | 48 | 1 | -2.0165 | -0.0830 |

Table 2: L1-SRS-RSRP performance for Case 1, AWGN channel, and SNR equals 2 dB

| SCS (Khz) | PRB number | Sample number | L1-SRS-RSRP error (dB) |  |
| --- | --- | --- | --- | --- |
|  |  |  | 5% | 95% |
| 15 | 24 | 1 | -0.5086 | -0.0218 |
|  | 48 | 1 | -0.5088 | -0.0256 |
| 30 | 24 | 1 | -0.9989 | -0.0376 |
|  | 48 | 1 | -0.9727 | -0.0459 |
| 60 | 24 | 1 | -2.1625 | -0.0755 |
|  | 48 | 1 | -2.0311 | -0.0741 |

Table 3: L1-SRS-RSRP performance for Case 1, AWGN channel, and SNR equals 3 dB

| SCS (Khz) | PRB number | Sample number | L1-SRS-RSRP error (dB) |  |
| --- | --- | --- | --- | --- |
|  |  |  | 5% | 95% |
| 15 | 24 | 1 | -0.5056 | -0.0234 |
|  | 48 | 1 | -0.5010 | -0.0242 |
| 30 | 24 | 1 | -0.9990 | -0.0514 |
|  | 48 | 1 | -0.9931 | -0.0347 |
| 60 | 24 | 1 | -2.1616 | -0.0762 |
|  | 48 | 1 | -2.0638 | -0.0755 |

## Case 2

The measurement performance for Case 2 under SNR = 1, 2 and 3 shown in Table 4 to Table 6 respectively.

Table 4: L1-SRS-RSRP performance for Case 2, AWGN channel, and SNR equals 1 dB

| SCS (Khz) | PRB number | Sample number | L1-SRS-RSRP error (dB) |  |
| --- | --- | --- | --- | --- |
|  |  |  | 5% | 95% |
| 15 | 24 | 1 | -0.2375 | 0.0340 |
|  | 48 | 1 | -0.2442 | 0.0331 |
| 30 | 24 | 1 | -0.4051 | 0.0292 |
|  | 48 | 1 | -0.4065 | 0.0379 |
| 60 | 24 | 1 | -0.7593 | 0.0334 |
|  | 48 | 1 | -0.6910 | 0.0345 |

Table 5: L1-SRS-RSRP performance for Case 2, AWGN channel, and SNR equals 2 dB

| SCS (Khz) | PRB number | Sample number | L1-SRS-RSRP error (dB) |  |
| --- | --- | --- | --- | --- |
|  |  |  | 5% | 95% |
| 15 | 24 | 1 | -0.2500 | 0.0243 |
|  | 48 | 1 | -0.2278 | 0.0208 |
| 30 | 24 | 1 | -0.4166 | 0.0241 |
|  | 48 | 1 | -0.3875 | 0.0350 |
| 60 | 24 | 1 | -0.7433 | 0.0134 |
|  | 48 | 1 | -0.6899 | 0.0191 |

Table 6: L1-SRS-RSRP performance for Case 2, AWGN channel, and SNR equals 3 dB

| SCS (Khz) | PRB number | Sample number | L1-SRS-RSRP error (dB) |  |
| --- | --- | --- | --- | --- |
|  |  |  | 5% | 95% |
| 15 | 24 | 1 | -0.2191 | 0.0201 |
|  | 48 | 1 | -0.2143 | 0.0190 |
| 30 | 24 | 1 | -0.3832 | 0.0200 |
|  | 48 | 1 | -0.3848 | 0.0140 |
| 60 | 24 | 1 | -0.7348 | 0.0149 |
|  | 48 | 1 | -0.6955 | 0.0082 |

# Conclusion

In this contribution, we provided our evaluation analysis on the accuracy of L1-SRS-RSRP under various agreed scenarios.

# Reference

- R4-2416910, Simulation assumption for evaluating L1- SRS-RSRP measurement requirements, Ericsson