3GPP TSG-RAN WG4 Meeting #116	R4-2511701

# Bangalore, India,  25th August – 29th August 2025				(revision of R4-2509539)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.741 | CR | 0011 | rev | - | Current version: | 18.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of NTN HPUE to TR 38.741 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple, Globalstar |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_IoT_NTN_req_test_enh-Core |  |  |  |  |  | Date: |  |  | 2025-08-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | NTN HPUE feature enables support of the PC2 power class for the satellite bands. Thus, in this CR we add the corresponding PC2 power class definitions for the NTN L-/S-band. |  |  |  |  |  |  |  |  |
| Summary of change: |  | PC2 power class, 26dBm, is added to the list of supported power classes for this NTN band. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PC2 power class definition will be missing in the TR. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1.1, 6.2.1.2a(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 6.2.1	UE transmitter characteristics

#### 6.2.1.1	Maximum output power

Table 6.2.1.1-1: NTN L-/S-band n254 Power Class

| NR satellite band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- |
| n254 | 26 | ±2 | 23 | ±2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 3 is default power class unless otherwise stated |  |  |  |  |

******************* NEXT CHANGED SECTION *******************

#### 6.2.1.2a	MPR/A-MPR for PC2

##### 6.2.1.2a.1	Maximum power reduction

Maximum power reduction for the NTN L-/S-band is the same as for the TN bands, as specified in TS 38.101-1 sub-clause 6.2.2, MPR values from which are presented below in Table 6.2.1.2a.1-1.

Table 6.2.1.2a.1-1: Maximum power reduction (MPR) for power class 2.

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.5 | ≤ 0.5 | 01 |
|  | QPSK | ≤ 3.5 | ≤ 1 | 02 |
|  | 16 QAM | ≤ 3.5 | ≤ 2 | ≤ 1 |
|  | 64 QAM | ≤ 3.5 | ≤ 2.5 |  |
| CP-OFDM | QPSK | ≤ 3.5 | ≤ 3 | ≤ 1.5 |
|  | 16 QAM | ≤ 3.5 | ≤ 3 | ≤ 2 |
|  | 64 QAM | ≤ 3.5 |  |  |
| NOTE 1: Applicable for a UE indicating support for UE capability powerBoosting-pi2BPSK-QPSK-r18 or powerBoosting-pi2BPSK-QPSK-Modified-r18 and if the IE powerBoostPi2BPSK-r18 is set to 1. The reference power is increased by [ΔPPowerBoost - ΔPPowerClass] NOTE 2: Applicable for a UE indicating support for UE capability powerBoosting-pi2BPSK-QPSK-r18 or powerBoosting-pi2BPSK-QPSK-Modified-r18 and if the IE powerBoostQPSK-r18 is set to 1. The reference power is increased by [ΔPPowerBoost - ΔPPowerClass] |  |  |  |  |

##### 6.2.1.2a.2	Additional maximum power reduction (company #1)

6.2.1.2a.2.1		Configuration cases

Power back-off simulation scenarios and assumptions are like the ones presented in sub-clause 6.2.1.2.2.1 with the main difference is that power class 2 is considered.

###### 6.2.1.2a.2.2	Power back-off results for ETSI sub-range 1

Figures below present 5MHz simulation results for NS_04N, which corresponds to the first frequency sub-range 1610-1618MHz under the ETSI requirements. The simulation results are close to the ones obtained for the PC3 power class with the only major difference that power back-off values are larger for large RB allocations. Nevertheless, for small RB allocations they are typically within MPR limits. If the 5MHz channel is shifted further from the left edge by 1.4MHz, then small RB allocations will need only small power back-off.

Figure 6.2.1.2a.2.2-1: Power back-off results for 5MHz at Fc=1612.5MHz.

Figure 6.2.1.2a.2.2-2: Power back-off results for 5MHz at Fc=1613.9MHz.

###### 6.2.1.2a.2.3	Power back-off results for ETSI sub-range 2

Figures below present simulations results for NS_05N, which is the second frequency sub-range 1618-1626.5MHz operating under the ETSI requirements. Like for PC3, additional power back-off is needed for large allocations and for some small allocations residing at the right edge of the channel. The reason for the latter case is strict ETSI out-of-band emission requirements for the upper edge of this band. If the 5MHz transmission channel is shifted from the upper edge of the band, then required power back-off reduces significantly, and for instance small RB allocations are mostly within the existing MPR limits.

Figure 6.2.1.2a.2.3-1: Power back-off results for 5MHz at Fc=1624MHz.

Figure 6.2.1.2a.2.3-2: Power back-off results for 5MHz at Fc=1622.4MHz.

Next, there are simulation results for the 10MHz and 15MHz channels. According to the ETSI regulations, if the channel is not fully contained within the first ETSI sub-range, then the requirements from the second range should be followed. That is the reason why we present here 10 and 15MHz channel results, which reside at the lower and the upper edge of the band, under NS_05N. As can be seen from the figures, the overall trend is similar to what we observed earlier with PC3. While small allocations have more or less small back-off, large allocations especially for the band lower edge may require extremely high power back-off values reaching extremely high values up to 15dB.

Figure 6.2.1.2a.2.3-3: Power back-off results for 10MHz at Fc=1615MHz.

Figure 6.2.1.2a.2.3-4: Power back-off results for 10MHz at Fc=1621.5MHz.

Figure 6.2.1.2a.2.3-5: Power back-off results for 15MHz at Fc=1617.5MHz.

Figure 6.2.1.2a.2.3-6: Power back-off results for 15MHz at Fc=1619MHz.

##### 6.2.1.2a.3	Additional maximum power reduction (company #2)

##### 6.2.1.2a.4	Additional maximum power reduction (company #3)

##### 6.2.1.2a.5	Harmonised A-MPR values