| 3GPP TR 38.792 V0.3.0 (2025-08) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; High power UE (power class 1.5) for NR Inter-band Carrier Aggregation (CA)/Dual connectivity (DC) with high power on both FDD and TDD bands (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	7

3.3	Abbreviations	7

4	Background	7

4.1	Introduction	7

4.2	TR Maintenance	7

5	PC1.5 FDD+TDD CA band combinations with high power on both FDD and TDD bands	7

5.x	CA_nX-nY (Example)	7

5.1	CA_n25-n41-n66	8

5.2	CA_n25-n66-n77	10

5.3	CA_n25-n71-n77	11

5.4	CA_n66-n71-n77	12

6	PC1.5 FDD+TDD NR DC band combinations with high power on both FDD and TDD bands	14

6.x	DC_nx-nY (Example)	14

Annex A:	 Valid UL configurations	15

Annex B (informative): Change history	15

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document is a technical report for UE requirements for PC1.5 inter-band UL CA and DC with HPUE in both FDD and TDD bands under Rel-19 time-frame. The purpose is to gather the relevant background information and studies of these 3Tx band combinations.

This TR contains the RF requirements of band combination specific part. The actual requirements are added to the corresponding technical specifications.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[3]	3GPP TS 38.101-3: "NR; User Equipment (UE) radio transmission and reception; Part 3:  Range 1 and Range 2 Interworking operation with other radios".

[4]	WI description, RP-231719: "WID revision for Core part 4Rx handheld UE for low NR bands (1GHz) and/or 3Tx for NR inter-band UL Carrier Aggregation (CA) and EN-DC"

[5]	WI description, RP-232674: "New WID: Rel-18 new basket WI for 3Tx inter-band UL CA and EN-DC"

[6]	WI description, RP-241679: "New WID: Rel-19 HPUE (PC1.5 or 2) for NR intra-band CA or NR inter-band CA/DC band combinations with/without NR SUL"

[7]	3GPP R4-2507935 WF on new MSD rules, Skyworks, Murata Manufacturing Co Ltd., Nokia, Samsung, Qualcomm, Apple, Vivo, ZTE, Mediatek, Huawei, Oppo, RAN4#115 2025-05

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

3Tx: UE transmit with one Tx chain on one band and two Tx chains on the other band in CA or EN-DC band combinations.

Carrier aggregation: Aggregation of two or more component carriers in order to support wider transmission bandwidths.

Inter-band carrier aggregation: Carrier aggregation of component carriers in different operating bands.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

ΔRIB,c	Allowed reference sensitivity relaxation due to support for inter-band CA operation, for serving cell c

ΔTIB,c	Allowed maximum configured output power relaxation due to support for inter-band CA operation, inter-band NR-DC operation and due to support for SUL operations, for serving cell c

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

CA	Carrier Aggregation

DC	Dual Connectivity

IMD	Inter-modulation

MSD	Maximum Sensitivity Deduction

SCS	Subcarrier spacing

# 4	Background

## 4.1	Introduction

General requirements for 3Tx feature was introduced in Rel-18 RAN4 specification 38.101-1 and 38.101-3 via WI [4]. After that, some band combinations are proposed and introduced via Rel-18 basket WI [5]. In Rel-19, basket WI [6] will introduce more band combination for PC1.5 with high power on both FDD and TDD bands.

The present document is a technical report for the Rel-19 basket WI and covers band combinations for PC1.5 with high power on both FDD and TDD bands.

It covers the RF requirements for each band combination specific issues (i.e. one sub-clause defined per band combination) mainly MSD analysis caused by harmonic mixing, cross band leakage and IMD issue.

For the band combinations whose 2Tx MSD is not specified in the spec yet, 3Tx MSD needs to be evaluated. And it is encouraged to align the requirements between 2Tx and 3Tx band combinations.

## 4.2	TR Maintenance

A single company is responsible for introducing all approved TPs in the current TR, i.e. TR editor. However, it is the responsibility of the contact person of each band combination to ensure that the TPs related to the band combination have been implemented.

# 5	PC1.5 FDD+TDD CA band combinations with high power on both FDD and TDD bands

## 5.x	CA_nX-nY (Example)

### 5.x.1	Operating bands for CA

Table 5.x.1-1:  CA band combination of band nX+nY

| NR CA Band combination | NR Band |
| --- | --- |
| CA_nX-nY | nX, nY |

### 5.x.2	Maximum output power for inter-band CA

Table 5.x.2-1: UE CA Power Class

| NR UL CA Configuration | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| CA_nXA-nYA | 29a | +2/-3 | 26 | +2/-3 | 23 | +2/-3 |
| NOTE a: The maximum output power in NR band X and band Y are PC2 or PC1.5. |  |  |  |  |  |  |

### 5.x.3	MSD scenario studies

Table 5.x.3-1 summarizes frequency ranges where harmonics mixing, cross band leakage, IMD interferences occur for CA_ nX-nY.

Table 5.x.3-1: MSD scenarios for CA_nX-nY with total power class 1.5

|  | Aggressor Tx | Victim Rx | Whether 2Tx requirements exists |
| --- | --- | --- | --- |
| Harmonic mixing | M order of nY UL | N order of nX DL | Yes or No? |
|  | … | … | … |
| Cross band leakage | nY UL | nX DL | Yes or No? |
|  | … | … | … |
| IMD | IMD order (M order of nX UL + N order of nY UL) | nX DL | Yes or No? |
|  | … | … | … |

Editor's note: only list the MSD scenario that exists for this band combination.

### 5.x.4	REFSENS requirements

Editor's note1: Make it clear whether the existing 2Tx requirements are reused or newly analysed due to for example missing of the 2Tx requirements.

Editor's note2: If reused, just say to reuse the existing 2Tx MSD requirements for MSD type harmonic mixing, or cross band leakage or IMD.

Editor’s note3: If MSD is newly specified, either the LUT (look up table) approach [7] or calculation can be used to get the MSD requirements, and if calculation is used it would be useful to give some key parameters as reference.

#### 5.x.4.1	REFSENS requirements for total power class 1.5

## 5.1	CA_n25-n41-n66

### 5.1.1	Configurations

Table 5.1.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n25A-n41A-n66A | n257 n417,9 n667 CA_n25A-n41A7 CA_n25A-n66A7 CA_n41A-n66A7,13,14 | n25 | 5, 10, 15, 20 | 0 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |
|  |  | n66 | 5, 10, 15, 20, 40 |  |
|  |  | n25 | 5, 10, 15, 20, 25, 30, 40 | 1 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n41 | n41 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 7: Minimum requirements for Power Class 2 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Minimum requirements for Power Class 1.5 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 13: Minimum requirements for Power Class 2 are applicable for this uplink configuration with 1Tx antenna connector in one band and 2Tx antenna connectors in the other band. NOTE 14 Minimum requirements for Power Class 1.5 are applicable for this uplink configuration with 1Tx antenna connector in one band and 2Tx antenna connectors in the other band. |  |  |  |  |

### 5.1.2	Maximum output power

<Editor’s note: In table 5.1.2-1, the power class 2 cases supported by the uplink CA should be kept as the same numbering and others that not supported should be removed. >

Table 5.1.2-1 UE Power Class 2 for uplink inter-band CA (Three bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n41-n66 | Case d | 26dBm | 26dBm | 26dBm |

Table 5.1.2-2 UE Power Class 1.5 for uplink inter-band CA (Three bands)

| Uplink CA configuration | Power class 1.5 cases for CA_n41-n66 | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n41-n66 | Case a | 29dBm | 26dBm | 26dBm |

### 5.1.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 5.1.3.1	Power class 2 case d

There is an IMD4 from UL CA_n41A-n66A into the n25 DL. The PC3 MSD is 11.0 dB. The PC2 MSD is 22.7dB in Table 7.3A.5-2a. The MSD can also be calculated with the LUT approach.

#### 5.1.3.2	Power class 1.5 case a

There is an IMD4 from UL CA_n41A-n66A into the n25 DL. The PC3 MSD is 11.0 dB. The PC1.5 MSD can be calculated with the LUT approach.

### 5.1.4	∆TIB and ∆RIB values

There is no change compared to the values for PC3 CA.

## 5.2	CA_n25-n66-n77

### 5.2.1	Configurations

Table 5.2.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n25A-n66A-n77A | n257 n667 n777,9 CA_n25A-n66A7 CA_n25A-n77A7,13,14 CA_n66A-n77A7,13,14 | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n66 | 5, 10, 15, 20, 25, 30, 40 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 7: Minimum requirements for Power Class 2 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Minimum requirements for Power Class 1.5 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 13: Minimum requirements for Power Class 2 are applicable for this uplink configuration with 1Tx antenna connector in one band and 2Tx antenna connectors in the other band. NOTE 14 Minimum requirements for Power Class 1.5 are applicable for this uplink configuration with 1Tx antenna connector in one band and 2Tx antenna connectors in the other band. |  |  |  |  |

### 5.2.2	Maximum output power

<Editor’s note: In table 5.2.2-1, the power class 2 cases supported by the uplink CA should be kept as the same numbering and others that not supported should be removed. >

Table 5.2.2-1 UE Power Class 2 for uplink inter-band CA (Three bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n25-n77 | Case d | 26dBm | 26dBm | 26dBm |
| CA_n66-n77 | Case d | 26dBm | 26dBm | 26dBm |

Table 5.2.2-2 UE Power Class 1.5 for uplink inter-band CA (Three bands)

| Uplink CA configuration | Power class 1.5 cases for CA_n41-n66 | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n25-n77 | Case a | 29dBm | 26dBm | 26dBm |
| CA_n66-n77 | Case a | 29dBm | 26dBm | 26dBm |

### 5.2.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 5.2.3.1	Power Class 2 and Power Class 1.5 MSD

There ae several IMDs for the three different uplink configurations. The Look Up Table approach is used to calculate the PC2 and PC1.5 MSD.

Table 5.2.3-1 UE Power Class 2 for uplink inter-band CA (Three bands)

| Uplink CA configuration | Victim Band | IMD | PC3 MSD (dB) |
| --- | --- | --- | --- |
| CA_n25-n77 | n66 | IMD2 | 29.2 |
|  |  | IMD4 | 10.4 |
|  |  | IMD5 | 4.0 |
| CA_n66-n77 | n25 | IMD2 | 32.1 |
|  |  | IMD4 | 9.1 |
|  |  | IMD5 | 2.1 |

### 5.2.4	∆TIB and ∆RIB values

There is no change compared to the values for PC3 CA.

## 5.3	CA_n25-n71-n77

### 5.3.1	Configurations

Table 5.3.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n25A-n71A-n77A | n257 n717 n777,9 CA_n25A-n71A7 CA_n25A-n77A7,13,14 CA_n71A-n77A7,13,14 | n25 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n25 | n25 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 7: Minimum requirements for Power Class 2 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Minimum requirements for Power Class 1.5 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 13: Minimum requirements for Power Class 2 are applicable for this uplink configuration with 1Tx antenna connector in one band and 2Tx antenna connectors in the other band. NOTE 14 Minimum requirements for Power Class 1.5 are applicable for this uplink configuration with 1Tx antenna connector in one band and 2Tx antenna connectors in the other band. |  |  |  |  |

### 5.3.2	Maximum output power

Only UL combinations with IMD MSD are included in this section.

Table 5.3.2-1 UE Power Class 2 for uplink inter-band CA (Three bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n71-n77 | Case d | 26dBm | 26dBm | 26dBm |

Table 5.3.2-2 UE Power Class 1.5 for uplink inter-band CA (Three bands)

| Uplink CA configuration | Power class 1.5 cases for CA_n41-n66 | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n71-n77 | Case a | 29dBm | 26dBm | 26dBm |

### 5.3.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 5.3.3.1	Power Class 2 and Power Class 1.5 MSD

The Look Up Table approach is used to calculate the PC2 and PC1.5 MSD. There is no MSD for UL CA_n25A-n77A.

Table 5.3.3-1 UE PC2 and PC1.5 LUT MSD

| Uplink CA configuration | Victim Band | IMD | PC3 MSD (dB) |
| --- | --- | --- | --- |
| CA_n71-n77 | n25 | IMD3 | 16.5 |

### 5.3.4	∆TIB and ∆RIB values

There is no change compared to the values for PC3 CA.

## 5.4	CA_n66-n71-n77

### 5.4.1	Configurations

Table 5.4.1-1: NR CA configurations and bandwidth combinations sets defined for inter-band CA (three bands)

| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n66A-n71A-n77A | n667 n717 n777,9 CA_n66A-n71A7 CA_n66A-n77A7,13,14 CA_n71A-n77A7,13,14 | n66 | 5, 10, 15, 20, 25, 30, 40 | 0 |
|  |  | n71 | 5, 10, 15, 20 |  |
|  |  | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  |
|  |  | n66 | n66 channel bandwidths in Table 5.3.5-1 | 4 and 5 |
|  |  | n71 | n71 channel bandwidths in Table 5.3.5-1 |  |
|  |  | n77 | n77 channel bandwidths in Table 5.3.5-1 |  |
| NOTE 7: Minimum requirements for Power Class 2 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 9: Minimum requirements for Power Class 1.5 are applicable for this uplink combination or single uplink carrier in this downlink/uplink combination NOTE 13: Minimum requirements for Power Class 2 are applicable for this uplink configuration with 1Tx antenna connector in one band and 2Tx antenna connectors in the other band. NOTE 14 Minimum requirements for Power Class 1.5 are applicable for this uplink configuration with 1Tx antenna connector in one band and 2Tx antenna connectors in the other band. |  |  |  |  |

### 5.4.2	Maximum output power

Table 5.4.2-1 UE Power Class 2 for uplink inter-band CA (Three bands)

| Uplink CA configuration | Power class 2 cases for CA_nX-nY | CA power class | Carrier X power class | Carrier Y power class |
| --- | --- | --- | --- | --- |
| CA_n66-n77 | Case d | 26dBm | 26dBm | 26dBm |
| CA_n71-n77 | Case d | 26dBm | 26dBm | 26dBm |

Table 5.4.2-2 UE Power Class 1.5 for uplink inter-band CA (Three bands)

| Uplink CA configuration | Power class 1.5 cases for CA_n41-n66 | CA power class | Carrier X power class | Carrier Y power class | Status |
| --- | --- | --- | --- | --- | --- |
| CA_n66-n77 | Case a | 29dBm | 26dBm | 26dBm | New |
| CA_n71-n77 | Case a | 29dBm | 26dBm | 26dBm | New |

### 5.4.3	REFSENS requirements

Analysis of REFSENS exceptions or MSD requirements is needed due to higher power uplink.

#### 5.4.3.1	Power Class 2 and Power Class 1.5 MSD

There are IMDs for two of the uplink configurations. The Look Up Table approach is used to calculate the PC2 and PC1.5 MSD.

Table 5.4.3-1 UE PC2 and PC1.5 LUT MSD

| Uplink CA configuration | Victim Band | Power class 2 cases for CA_nX-nY | PC3 MSD (dB) |
| --- | --- | --- | --- |
| CA_n66-n77 | n71 | IMD3 | 15.3 |
| CA_n71-n77 | n66 | IMD3 | 15.5 |

### 5.4.4	∆TIB and ∆RIB values

There is no change compared to the values for PC3 CA.

# 6	PC1.5 FDD+TDD NR DC band combinations with high power on both FDD and TDD bands

## 6.x	DC_nx-nY (Example)

### 6.x.1	Operating bands for NR DC

Table 6.x.1-1:  DC band combination of band nX+nY

| NR CA Band combination | Bands |
| --- | --- |
| DC_nX-nY | nX, nY |

### 6.x.2	Maximum output power for inter-band NR DC

Table 6.x.2-1: UE Power Class for inter-band NR DC

| DC configuration | Power class 1.5 (dBm) | Tolerance (dB) | Power class 2 (dBm) | Tolerance (dB) | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| DC_nXA-nYA | 29a | +2/-3 | 26 | +2/-3 | 23 | +2/-3 |
| NOTE a: The maximum output power in NR band X and band Y are PC2 or PC1.5. |  |  |  |  |  |  |

### 6.x.3	MSD scenario studies

Table 6.x.3-1 summarizes frequency ranges where harmonics mixing, cross band leakage, IMD interferences occur for DC_bX-nY.

Table 6.x.3-1: MSD scenarios for DC_nX-nY with total power class 1.5

|  | Aggressor Tx band | Victim Rx band | Whether 2Tx requirements exists |
| --- | --- | --- | --- |
| Harmonic mixing | M order of nY UL | N order of bX DL | Yes or No? |
|  | … | … | … |
| Cross band leakage | nY UL | bX DL | Yes or No? |
|  | … | … | … |
| IMD | IMD order (M order of bX UL + N order of nY UL) | bX DL | Yes or No? |
|  | … | … | … |

Editor's note: only list the MSD scenario that exists for this band combination.

### 6.x.4	REFSENS requirements

Editor's note1: Make it clear whether the existing 2Tx requirements are reused or newly analysed due to for example missing of the 2Tx requirements.

Editor's note2: If reused, just say to reuse the existing 2Tx MSD requirements for MSD type harmonic mixing, or cross band leakage or IMD.

Editor’s note3: If MSD is newly specified, it would be useful to give some key parameters as reference.

#### 6.x.4.1	REFSENS requirements for total power class 1.5

# Annex A:		Valid UL configurations

For 3Tx PC1.5 inter-band UL CA with 2 bands UL, the following UL configurations are applicable:

-  two band UL with one CC per band: CA_nXA-nYA

-  two band UL with two CC in one band: CA_nXB-nYA, CA_nXC-nYA, CA_nXA-nYB, CA_nXA-nYC

The following three UL cluster cases are not supported: CA_nX(2A)-nYA, CA_nXA-nY(2A)

Combinations with four UL CCs are not supported.

# Annex B (informative): Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2024-08 | RAN4 #112 | R4-24xxxxx |  |  |  | TR skeleton | 0.0.1 |
| 2024-11 | RAN4#113 | R4-2417918 |  |  |  | Capture the TP to TR from R4-2418709 | 0.1.0 |
| 2025-05 | RAN4#115 | R4-2508043 |  |  |  | TP to TR38.792 on MSD definition approach for PC1.5 with HPUE on both FDD and TDD | 0.2.0 |
| 2025-08 | RAN4#116 | R4-2512879 |  |  |  | TP for TR 38.792: DL CA_n25-n41-n66 with UL CA_n41A-n66A | 0.3.0 |
| 2025-08 | RAN4#116 | R4-2512882 |  |  |  | TP for TR 38.792: DL CA_n25-n66-n77 | 0.3.0 |
| 2025-08 | RAN4#116 | R4-2512885 |  |  |  | TP for TR 38.792: DL CA_n25-n71-n77 | 0.3.0 |
| 2025-08 | RAN4#116 | R4-2512888 |  |  |  | TP for TR 38.792: DL CA_n66-n71-n77 | 0.3.0 |